Storage Management Technical Specification, Part 2 Common Architecture

Version 1.8.0, Revision 5

Abstract: This SNIA Techncial Position defines an interface between WBEM-capable clients and servers for the secure, extensible, and interoperable management of networked storage.

This document has been released and approved by the SNIA. The SNIA believes that the ideas, methodologies and technologies described in this document accurately represent the SNIA goals and are appropriate for widespread distribution. Suggestions for revision should be directed to http:// www.snia.org/feedback/.

SNIA Technical Position

23 March 2020

- USAGE

- Copyright © 2018 SNIA. All rights reserved. All other trademarks or registered trademarks are the property of their 3	respective owners. 4

The SNIA hereby grants permission for individuals to use this document for personal use only, and for corporations

5 and other business entities to use this document for internal use only (including internal copying, distribution, and display) provided that: 6

- 1)	Any text, diagram, chart, table or definition reproduced shall be reproduced in its entirety with no alter-

- ation, and,

9

- 2) Any document, printed or electronic, in which material from this document (or any portion hereof) is reproduced shall acknowledge the SNIA copyright on that material, and shall credit the SNIA for granting

- permission for its reuse.

- Other than as explicitly provided above, you may not make any commercial use of this document or any portion 13 thereof, or distribute this document to third parties. All rights not explicitly granted are expressly reserved to SNIA.

14 Permission to use this document for purposes other than those enumerated above may be requested by e-mailing 15 tcmd@snia.org. Please include the identity of the requesting individual and/or company and a brief description of 16 the purpose, nature, and scope of the requested use.

17

All code fragments, scripts, data tables, and sample code in this SNIA document are made available under the 18	following license:

19	BSD 3-Clause Software License

20

21	Copyright (c) 2018, The Storage Networking Industry Association.

22

Redistribution and use in source and binary forms, with or without modification, are permitted provided that the 23 following conditions are met: 24

25 • Redistributions of source code must retain the above copyright notice, this list of conditions and the following disclaimer. 26

27 • Redistributions in binary form must reproduce the above copyright notice, this list of conditions and the 28 following disclaimer in the documentation and/or other materials provided with the distribution.

29 • Neither the name of The Storage Networking Industry Association (SNIA) nor the names of its contributors may 30 be used to endorse or promote products derived from this software without specific prior written permission.

31

32	THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS" AND ANY

EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF 33 MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL 34 THE COPYRIGHT OWNER OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, 35 SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO,

- PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR

BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN

- CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

38

39

40

41

42

43

44

45

46	DISCLAIMER

47

48 The information contained in this publication is subject to change without notice. The SNIA makes no 49 warranty of any kind with regard to this specification, including, but not limited to, the implied warranties 50 of merchantability and fitness for a particular purpose. The SNIA shall not be liable for errors contained 51 herein or for incidental or consequential damages in connection with the furnishing, performance, or use 52 of this specification.

53	Suggestions for revisions should be directed to http://www.snia.org/feedback/.

54

Copyright © 2003-2020 SNIA. All rights reserved. All other trademarks or registered trademarks are the

55 property of their respective owners. 56

- Portions of the CIM Schema are used in this document with the permission of the Distributed

- Management Task Force (DMTF). The CIM classes that are documented have been developed and reviewed by both the SNIA and DMTF Technical Working Groups. However, the schema is still in

- development and review in the DMTF Working Groups and Technical Committee, and subject to change.

60

61

62

63

64

65

66

67

68

69

70

71

72

73

74

75

# REVISION HISTORY

Revision 1

Date

April 14, 2017

SCRs Incorporated and other changes

Annex A (informative) SMI-S Information Model

- This standard is now based on DMTF’s CIM schema Version 2.50.

Comments

Editorial notes and DRAFT material are displayed.

Revision 2

Date

February 16, 2018

SCRs Incorporated and other changes None.

Comments

Substantial editorial clean up to align CIM references with existing XML.

Revision 3

Date

August 14, 2018

SCRs Incorporated and other changes None.

Comments None.

Revision 4

Date

October 24, 2019

SCRs Incorporated and other changes None.

Comments

Minor editorial clean up to create corrected technical position.

Revision 5

Date

March 23, 2020

SCRs Incorporated and other changes

None.

Comments

Include references to most recent versions of SNIA TLS Specification for Storage Systems.

Suggestions for revisions should be directed to http://www.snia.org/feedback/.

- INTENDED AUDIENCE

- This document is intended for use by individuals and companies engaged in developing, deploying, and 3 promoting interoperable multi-vendor SANs through the Storage Networking Industry Association (SNIA) 4 organization.

- CHANGES TO THE SPECIFICATION

- Each publication of this specification is uniquely identified by a three-level identifier, comprised of a 7 version number, a release number and an update number. The current identifier for this specification is 8 version 1.8.0. Future publications of this specification are subject to specific constraints on the scope of 9 change that is permissible from one publication to the next and the degree of interoperability and

10 backward compatibility that should be assumed between products designed to different publications of this standard. The SNIA has defined three levels of change to a specification:

11

• Major Revision: A major revision of the specification represents a substantial change to the underlying scope 12 or architecture of the SMI-S API. A major revision results in an increase in the version number of the version

13 identifier (e.g., from version 1.x.x to version 2.x.x). There is no assurance of interoperability or backward compatibility between releases with different version numbers.

14

15 • Minor Revision: A minor revision of the specification represents a technical change to existing content or an 16 adjustment to the scope of the SMI-S API. A minor revision results in an increase in the release number of

17 the specification’s identifier (e.g., from x.1.x to x.2.x). Minor revisions with the same version number preserve interoperability and backward compatibility.

18

• Update: An update to the specification is limited to minor corrections or clarifications of existing specification 19 content. An update will result in an increase in the third component of the release identifier (e.g., from x.x.1 to

20	x.x.2). Updates with the same version and minor release levels preserve interoperability and backward 21	compatibility.

22

23	TYPOGRAPHICAL CONVENTIONS

24

Maturity Level 25

26 In addition to informative and normative content, this specification includes guidance about the maturity 27 of emerging material that has completed a rigorous design review but has limited implementation in

28 commercial products. This material is clearly delineated as described in the following sections. The typographical convention is intended to provide a sense of the maturity of the affected material, without 29 altering its normative content. By recognizing the relative maturity of different sections of the standard, an 30 implementer should be able to make more informed decisions about the adoption and deployment of 31 different portions of the standard in a commercial product.

32

This specification has been structured to convey both the formal requirements and assumptions of the 33 SMI-S API and its emerging implementation and deployment lifecycle. Over time, the intent is that all 34 content in the specification will represent a mature and stable design, be verified by extensive 35 implementation experience, assure consistent support for backward compatibility, and rely solely on

36 content material that has reached a similar level of maturity. Unless explicitly labeled with one of the subordinate maturity levels defined for this specification, content is assumed to satisfy these 37 requirements and is referred to as “Finalized”. Since much of the evolving specification

38 content in any given release will not have matured to that level, this specification defines three 39 subordinate levels of implementation maturity that identify important aspects of the content’s increasing 40 maturity and stability. Each subordinate maturity level is defined by its level of implementation 41 experience, its stability and its reliance on other emerging standards. Each subordinate maturity level is 42 identified by a unique typographical tagging convention that clearly distinguishes content at one maturity 43 model from content at another level.

44

45

- Experimental Maturity Level

- No material is included in this document unless its initial architecture has been completed and reviewed. 48 Some content included in this document has complete and reviewed design, but lacks implementation 49 experience and the maturity gained through implementation experience. This content is included in order 50 to gain wider review and to gain implementation experience. This material is referred to as 51 “Experimental”. It is presented here as an aid to implementers who are interested in likely future

52 developments within the SMI specification. The contents of an Experimental profile may change as implementation experience is gained. There is a high likelihood that the changed content will be included 53 in an upcoming revision of the specification. Experimental material can advance to a higher maturity level

54 as soon as implementations are available. Figure 1 is a sample of the typographical convention for 55

Experimental content.

56

EXPERIMENTAL

Experimental content appears here.

EXPERIMENTAL

57

58

59

60

61

62

63

64	Figure 1 - Experimental Maturity Level Tag

65

66

- Implemented Maturity Level

- Profiles for which initial implementations have been completed are classified as “Implemented”. This

- indicates that at least two different vendors have implemented the profile, including at least one provider implementation. At this maturity level, the underlying architecture and modeling are stable, and changes in future revisions will be limited to the correction of deficiencies identified through additional implementation experience. Should the material become obsolete in the future, it must be deprecated in a minor revision of the specification prior to its removal from subsequent releases. Figure 2 is a sample of the typographical convention for Implemented content.

IMPLEMENTED

Implemented content appears here.

IMPLEMENTED

70

71

72	Figure 2 - Implemented Maturity Level Tag

73

74

75	Stable Maturity Level

Once content at the Implemented maturity level has garnered additional implementation experience, it can be tagged at the Stable maturity level. Material at this maturity level has been implemented by three different vendors, including both a provider and a client. Should material that has reached this maturity level become obsolete, it may only be deprecated as part of a minor revision to the specification. Material at this maturity level that has been deprecated may only be removed from the specification as part of a major revision. A profile that has reached this maturity level is guaranteed to preserve backward compatibility from one minor specification revision to the next. As a result, Profiles at or above the Stable 76 maturity level shall not rely on any content that is Experimental. Figure 3 is a sample of the typographical 77 convention for Implemented content.

STABLE

Stable content appears here.

STABLE

Figure 3 - Stable Maturity Level Tag

- Finalized Maturity Level

- Content that has reached the highest maturity level is referred to as “Finalized.” In addition to satisfying 80 the requirements for the Stable maturity level, content at the Finalized maturity level must solely depend 81 upon or refine material that has also reached the Finalized level. If specification content depends upon 82 material that is not under the control of the SNIA, and therefore not subject to its maturity level 83 definitions, then the external content is evaluated by the SNIA to assure that it has achieved a 84 comparable level of completion, stability, and implementation experience. Should material that has 85 reached this maturity level become obsolete, it may only be deprecated as part of a major revision to the 86 specification. A profile that has reached this maturity level is guaranteed to preserve backward

- compatibility from one minor specification revision to the next. Over time, it is hoped that all specification content will attain this maturity level. Accordingly, there is no special typographical convention, as there is with the other, subordinate maturity levels. Unless content in the specification is marked with one of the typographical conventions defined for the subordinate maturity levels, it should be assumed to have reached the Finalized maturity level.

Deprecated Material

Non-Experimental material can be deprecated in a subsequent revision of the specification. Sections identified as “Deprecated” contain material that is obsolete and not recommended for use in new development efforts. Existing and new implementations may still use this material, but shall move to the newer approach as soon as possible. The maturity level of the material being deprecated determines how long it will continue to appear in the specification. Implemented content shall be retained at least until the next revision of the specialization, while Stable and Finalized material shall be retained until the next

- major revision of the specification. Providers shall implement the deprecated elements as long as it 89 appears in the specification in order to achieve backward compatibility. Clients may rely on deprecated 90 elements, but are encouraged to use non-deprecated alternatives when possible.

91

92 Deprecated sections are documented with a reference to the last published version to include the 93 deprecated section as normative material and to the section in the current specification with the 94 replacement. Figure 4 contains a sample of the typographical convention for deprecated content.

95

DEPRECATED

Content that has been deprecated appears here.

DEPRECATED

96

97

98

99

100

101

102

103	Figure 4 - Deprecated Tag

104

105

106

107

108

109

110

111

112

113

114

115

116

117

118

119

120

121

122

123

124

125

126

127

128

129

130

131

Contents

Revision History	5

List of Figures	15

List of Tables	17

Foreword	23

1 Scope	25

2 Normative references	27

2.1 General	27

2.2 Approved references	27

2.3 DMTF references (Final)	27

2.4 IETF references	28

2.5 References under development	29

2.6 Other references	29

3 Terms, definitions, symbols, abbreviations, and conventions	31

3.1 Terms and definitions	31

3.2 Acronyms and abbreviations	37

3.3 Keywords	37

3.4 Conventions	38

4 Transport and Reference Model	41

4.1 Introduction	41

4.1.1 Overview	41

4.1.2 Language Requirements	41

4.1.3 Communications Requirements	41

4.1.4 XML Message Syntax and Semantics	41

4.2 Transport Stack	42

4.3 Reference Model	42

4.3.1 Overview	42

4.3.2 Roles for Interface Constituents	43

4.3.3 Cascaded Agents	43

5 Health and Fault Management	45

5.1 Objectives	45

5.2 Overview	45

5.3 Terms	45

5.4 Description of Health and Fault Management	46

5.4.1 Operational Status and Health State (Polling)	46

5.4.2 Standard Errors and Events	47

5.4.3 Indications	47

5.4.4 Event Correlation and Fault Containment	47

5.4.5 Fault Regions	50

5.4.6 Examples	52

6 Object Model General Information	55

6.1 Model Overview (Key Resources)	55

6.1.1 Overview	55

6.1.2 Introduction to CIM UML Notation	55

6.2 Techniques	56

6.2.1 CIM Fundamentals	56

6.2.2 Modeling Profiles	58

6.2.3 CIM Naming	58

7 Correlatable and Durable Names	59

7.1 Overview	59

7.2 Guidelines for SCSI Logical Unit Names	60

7.3 Guidelines for FC-SB-2 Device Names	60

7.4 Guidelines for Port Names	61

7.5 Guidelines for Storage System Names	61

7.6 Standard Formats for Correlatable Names	62

7.6.1 General	62

7.6.2 Standard Formats for Logical Unit Names	63

7.6.3 Standard Formats for Port Names	64

7.6.4 Standard Formats for Fabric Names	65

7.6.5 Standard Formats for Storage System Names	65

7.6.6 Operating System Device Names	67

7.6.7 Case Sensitivity	68

7.7 Testing Equality of correlatable Names	68

7.8 iSCSI Names	69

8 Standard Messages	71

8.1 Overview	71

8.2 Registries for Standard Messages	71

8.3 SNIA Standard Messages	71

8.3.1 Common Profile-related Messages	71

8.3.2 Block Storage Messages	85

8.3.3 Fabric Messages	118

8.3.4 Filesystem Messages	123

8.3.5 Host Messages	135

8.3.6 Media Library Messages	138

9 Service Discovery	171

9.1 Objectives	171

9.2 Overview	171

9.3 SLP Messages	173

9.4 Scopes	174

9.5 Services Definition	175

9.5.1 SLP Terms	175

9.5.2 Service Type	175

9.5.3 Service Attributes	175

9.6 User Agents (UA)	175

9.7 Service Agents (SAs)	176

9.8 Directory Agents (DAs)	177

9.9 Service Agent Server (SA Server)	177

9.9.1 General Information	177

9.9.2 SA Server (SAS) Implementation	177

9.9.3 SA Server (SAS) Clients	178

9.9.4 SA Server Configuration	178

9.9.5 SA Server Discovery	180

9.9.6 SAS Client Registration	180

9.10 Configurations	180

9.10.1 Overview	180

9.10.2 Multicast Configurations	180

9.10.3 No Multicast configuration	181

9.10.4 Multicast Islands	182

10 Indications	185

10.1 Indications profile	185

10.1.1 Profile Support	185

10.1.2 Creating a client defined indication and subscription	185

10.1.3 ListenerDestination	185

10.2 Indication Filter Strings	185

10.2.1 Definition Syntax	185

10.2.2 Instance Creation	186

10.2.3 Instance Deletion	186

10.2.4 Modification of any value in an array property	186

10.2.5 Modification to either of Two Specific values in an Array Property	186

10.2.6 Alert	187

11 SMI-S Roles	189

11.1 Introduction	189

11.2 SMI-S Client	190

11.2.1 Overview	190

11.2.2 SLP Functions	190

11.2.3 Generic Operations	190

11.2.4 Security Considerations	190

11.2.5 Lock Management Functions	190

11.3 Dedicated SMI-S Server	190

11.3.1 Overview	190

11.3.2 SLP Functions	191

11.3.3 Generic Operations	191

11.3.4 Security Considerations	192

11.3.5 Lock Management Functions	192

11.4 General Purpose SMI-S Server	192

11.4.1 Overview	192

11.4.2 SLP Functions	192

11.4.3 Generic Operations	193

11.4.4 Lock Management Functions	193

11.4.5 Provider Sub-role	193

11.5 Directory Server	193

11.5.1 SLP Functions	193

11.5.2 Generic Operations	193

11.5.3 Security Considerations	193

11.5.4 Lock Management Functions	194

11.6 Combined Roles on a Single System	194

11.6.1 Overview	194

11.6.2 General Purpose SMI-S Server as a Profile Aggregator	194

12 Installation and Upgrade	195

12.1 Introduction	195

12.2 Role of the Administrator	195

12.3 Goals	195

12.3.1 Non-Disruptive Installation and De-installation	195

12.3.2 Plug-and-Play	195

12.4 Server Deployment	196

12.4.1 General	196

12.4.2 Controlled Environment	196

12.4.3 Multiple WBEM Server systems	196

12.4.4 Shared WBEM Server	197

12.4.5 Uninstallation	198

12.4.6 Update	198

12.4.7 Reconfiguration	198

12.5 WBEM Service Support & Related Functions	198

12.5.1 Installation	198

12.5.2 Multiple WBEM Servers on a Single Server System	199

12.5.3 Uninstallation/Upgrade	199

12.5.4 Reconfiguration	199

12.5.5 Failure	199

12.6 Client	199

12.6.1 Uninstallation	199

12.6.2 Reconfiguration	199

12.7 Directory Service	199

12.7.1 Installation	199

12.7.2 Uninstallation/Failure	200

12.8 Issues with Discovery Mechanisms	200

13 Security	201

13.1 Requirements	201

13.1.1 Overview	201

13.1.2 General Requirements for HTTP Implementations	202

13.2 Description of SMI-S Security	202

13.2.1 Security Scope	202

13.2.2 Transport Security	203

13.2.3 Authentication	203

13.2.4 Service Discovery	204

Annex A (informative) SMI-S Information Model	207

Annex B (normative) Compliance with the SNIA SMI Specification	209

# LIST OF FIGURES

Figure 1  -  Experimental Maturity Level Tag.........................................................................................8 Figure 2  -  Implemented Maturity Level Tag .........................................................................................8 Figure 3  -  Stable Maturity Level Tag....................................................................................................9 Figure 4  -  Deprecated Tag...................................................................................................................9 Figure 5  -  Reference Model. ..............................................................................................................42 Figure 6  -  Basic Fault Detection.........................................................................................................46 Figure 7  -  Health Lifecycle .................................................................................................................49 Figure 8  -  Continuum .........................................................................................................................50 Figure 9  -  Application Fault Region....................................................................................................51 Figure 10  -  Switch Example...............................................................................................................53 Figure 11  -  Lines that Connect Classes.............................................................................................55 Figure 12  -  iSCSI Qualified Names (iqn) Examples...........................................................................69 Figure 13  -  iSCSI EUI Name Example...............................................................................................69 Figure 14  -  iSCSI 64-bit NAA Name Example....................................................................................70 Figure 15  -  iSCSI 128-bit NAA Name Example..................................................................................70 Figure 16  -  SA Server Configuration................................................................................................180

Figure 17  -  Multicast Configuration..................................................................................................181 Figure 18  -  No Multicast configuration .............................................................................................182 Figure 19  -  Multicast Islands............................................................................................................183 Figure 20  -  SMI-S Roles ..................................................................................................................189

Figure B.1 Provider Migration 211

# LIST OF TABLES

| Table 1 - | OperationalStatus for Disk Drive .........................................................................................................46 |
| --- | --- |
| Table 2 - | Standard Formats for StorageVolume Names ....................................................................................63 |
| Table 3 - | Standard Formats for Port Names.......................................................................................................64 |
| Table 4 - | Standard Formats for Storage System Names....................................................................................66 |
| Table 5 - | Standard Operating System Names for Tape Devices........................................................................67 |
| Table 6 - | LogicalDisk.Name for disk partitions ...................................................................................................68 |
| Table 7 - | GenericDiskParittion.Name for disk partitions.....................................................................................68 |
| Table 8 - | Standard Operating System Names for Unpartitioned DIsks ..............................................................68 |
| Table 9 - | Redundancy Message Arguments ......................................................................................................71 |

Table 10  - Redundancy Alert Information.............................................................................................................72 Table 11  - Environmental Message Arguments....................................................................................................72 Table 12  - Environmental Alert Information ..........................................................................................................73 Table 13  - FRU Operation Message Arguments ..................................................................................................73 Table 14  - FRU Operation Alert Information.........................................................................................................74 Table 15  - Password change Message Arguments..............................................................................................74 Table 16  - Password change Alert Information.....................................................................................................74 Table 17  - User or Account Operation Message Arguments................................................................................75 Table 18  - User or Account Operation Alert Information.......................................................................................75 Table 19  - User Login Message Arguments .........................................................................................................76 Table 20  - User Login Alert Information................................................................................................................76 Table 21  - Proxy Agent Device Communication Message Arguments.................................................................76 Table 22  - Proxy Agent Device Communication Alert Information........................................................................77 Table 23  - Port Status Changed Message Arguments.........................................................................................77 Table 24  - Port Status Changed Alert Information................................................................................................78 Table 25  - Datacheck Error Message Arguments.................................................................................................78 Table 26  - Datacheck Error Alert Information .......................................................................................................78 Table 27  - User Login Failure Message Arguments.............................................................................................79 Table 28  - User Login Failure Alert Information....................................................................................................79 Table 29  - Drive not responding Message Arguments .........................................................................................80 Table 30  - Drive not responding Alert Information................................................................................................80 Table 31  - Fan Failure Alert Information...............................................................................................................80 Table 32  - Power Supply Failure Alert Information...............................................................................................81 Table 33  - Drive Power Consumption Alert Information .......................................................................................81 Table 34  - Drive Voltage Alert Information............................................................................................................81 Table 35  - Predictive Failure Alert Information .....................................................................................................82 Table 36  - Diagnostics Required Alert Information...............................................................................................82 Table 37  - Drive is responding Message Arguments............................................................................................82 Table 38  - Drive is responding Alert Information ..................................................................................................83 Table 39  - Cooling Fan Issues Cleared Alert Information.....................................................................................83 Table 40  - Power Supply Issues Cleared Message Arguments ...........................................................................83 Table 41  - Power Supply Issues Cleared Alert Information..................................................................................84 Table 42  - Controller Failure Message Arguments...............................................................................................84 Table 43  - Controller Failure Alert Information......................................................................................................84 Table 44  - Controller Issues Cleared Message Arguments..................................................................................84 Table 45  - Controller Issues Cleared Alert Information.........................................................................................85 Table 46  - Device Not ready Message Arguments...............................................................................................85 Table 47  - Error Properties for Device Not ready .................................................................................................86 Table 48  - Error Properties for Internal Bus Error.................................................................................................86 Table 49  - Error Properties for DMA Overflow......................................................................................................87 Table 50  - Error Properties for Firmware Logic Error ...........................................................................................87 Table 51  - Front End Port Error Message Arguments..........................................................................................88 Table 52  - Front End Port Error Alert Information.................................................................................................88 Table 53  - Back End Port Error Message Arguments...........................................................................................88 Table 54  - Back End Port Error Alert Information .................................................................................................88 Table 55  - Remote Mirror Error Message Arguments...........................................................................................89 Table 56  - Error Properties for Remote Mirror Error.............................................................................................89 Table 57  - Remote Mirror Error Alert Information .................................................................................................89 Table 58  - Error Properties for Cache Memory Error............................................................................................90 Table 59  - Error Properties for Unable to Access Remote Device .......................................................................90 Table 60  - Error Reading Data Alert Information ..................................................................................................91 Table 61  - Error Writing Data Alert Information ....................................................................................................91 Table 62  - Error Validating Write (CRC) Alert Information....................................................................................92 Table 63  - Error Properties for Copy Operation Failed.........................................................................................92

Table 64  - Error Properties for RAID Operation Failed.........................................................................................93 Table 65  - Error Properties for Invalid RAID Type................................................................................................93 Table 66  - Error Properties for Invalid Storage Element Type..............................................................................94

Table 67  - Error Properties for Configuration Change Failed ...............................................................................94 Table 68  - Error Properties for Buffer Overrun .....................................................................................................95 Table 69  - Stolen Capacity Message Arguments .................................................................................................95 Table 70  - Error Properties for Stolen Capacity....................................................................................................95 Table 71  - Invalid Extent passed Message Arguments ........................................................................................96 Table 72  - Error Properties for Invalid Extent passed...........................................................................................96 Table 73  - Error Properties for Invalid Deletion Attempted...................................................................................97 Table 74  - Error Properties for Job Failed to Start................................................................................................97 Table 75  - Job was Halted Message Arguments..................................................................................................98 Table 76  - Invalid State Transition Message Arguments......................................................................................98 Table 77  - Error Properties for Invalid State Transition ........................................................................................98 Table 78  - Invalid SAP for Method Message Arguments......................................................................................99 Table 79  - Error Properties for Invalid SAP for Method ........................................................................................99 Table 80  - Resource Not Available Message Arguments.....................................................................................99 Table 81  - Error Properties for Resource Not Available .....................................................................................100 Table 82  - Resource Limit Exceeded Message Arguments................................................................................100 Table 83  - Error Properties for Resource Limit Exceeded..................................................................................100 Table 84  - Thin Provision Capacity Warning Message Arguments ....................................................................101 Table 85  - Thin Provision Capacity Warning Alert Information...........................................................................101

Table 86  - Provision Capacity Critical Message Arguments...............................................................................101 Table 87  - Provision Capacity Critical Alert Information .....................................................................................102 Table 88  - Thin Provision Capacity Okay Message Arguments .........................................................................102 Table 89  - Thin Provision Capacity Okay Alert Information................................................................................102 Table 90  - Masking Group Membership Changed Message Arguments............................................................103 Table 91  - Masking Group Membership Changed Alert Information ..................................................................103 Table 92  - StorageVolume Relocation Starts Message Arguments ...................................................................103 Table 93  - StorageVolume Relocation Starts Alert Information..........................................................................103 Table 94  - StorageVolume Relocation Ends Message Arguments.....................................................................104 Table 95  - StorageVolume Relocation Ends Alert Information ...........................................................................104 Table 96  - StoragePool Relocation Starts Message Arguments ........................................................................104 Table 97  - StoragePool Relocation Starts Alert Information...............................................................................104 Table 98  - StoragePool Relocation Ends Message Arguments..........................................................................105 Table 99  - StoragePool Relocation Ends Alert Information................................................................................105 Table 100  - LogicalDisk Relocation Starts Message Arguments.........................................................................105 Table 101  - LogicalDisk Relocation Starts Alert Information................................................................................105 Table 102  - LogicalDisk Relocation Ends Message Arguments..........................................................................106 Table 103  - LogicalDisk Relocation Ends Alert Information.................................................................................106 Table 104  - Volume or pool degraded Message Arguments ...............................................................................106 Table 105  - Volume or pool degraded Alert Information......................................................................................106 Table 106  - Volume or pool failed Message Arguments......................................................................................107 Table 107  - Volume or pool failed Alert Information.............................................................................................107 Table 108  - Volume or pool issues cleared Message Arguments........................................................................107 Table 109  - Volume or pool issues cleared Alert Information..............................................................................108 Table 110  - The StoragePool is healthy Message Arguments.............................................................................108 Table 111  - The StoragePool is healthy Alert Information...................................................................................108 Table 112  - StoragePool is dependent on an element with problems Message Arguments ...............................109 Table 113  - StoragePool is dependent on an element with problems Alert Information......................................109 Table 114  - The StoragePool is being serviced Message Arguments.................................................................110 Table 115  - The StoragePool is being serviced Alert Information........................................................................110 Table 116  - The OperationalStatus of the Pool is impacting an element allocated from it Message Arguments111

Table 117  - The OperationalStatus of the Pool is impacting an element allocated from it Alert Information.......111 Table 118  - The StoragePool OperationalStatus may be corrected by applying a spare Message Arguments112

Table 119  - The StoragePool OperationalStatus may be corrected by applying a spare Alert Information.........112 Table 120  - The StoragePool OperationalStatus may be corrected by relocating the pool Message Arguments113

Table 121  - The StoragePool OperationalStatus may be corrected by relocating the pool Alert Information......113

Table 122  - Pool experiencing interference from system workload Message Arguments...................................114 Table 123  - Pool experiencing interference from system workload Alert Information..........................................114 Table 124  - Pool performance degraded by component element Message Arguments......................................115

Table 125  - Pool performance degraded by component element Alert Information ............................................115 Table 126  - Pool degraded due to loss of RAID protection Message Arguments................................................116 Table 127  - Pool degraded due to loss of RAID protection Alert Information......................................................116 Table 128  - Pool degraded due to loss of port redundancy Message Arguments...............................................117 Table 129  - Pool degraded due to loss of port redundancy Alert Information......................................................117 Table 130  - Pool predicting failure due lack of available capacity Message Arguments .....................................117 Table 131  - Pool predicting failure due lack of available capacity Alert Information............................................118 Table 132  - Zone Database Changed Message Arguments................................................................................118 Table 133  - Zone Database Changed Alert Information......................................................................................118

Table 134  - ZoneSet Activated Message Arguments ..........................................................................................119 Table 135  - ZoneSet Activated Alert Information.................................................................................................119

Table 136  - Error Properties for Session Locked.................................................................................................120

Table 137  - Error Properties for Session Aborted................................................................................................120 Table 138  - Switch Status Changed Message Arguments ..................................................................................121 Table 139  - Switch Status Changed Alert Information.........................................................................................121 Table 140  - Fabric Merge/Segmentation Message Arguments ...........................................................................121 Table 141  - Fabric Merge/Segmentation Alert Information..................................................................................121 Table 142  - Switch Added/Removed Message Arguments .................................................................................122 Table 143  - Switch Added/Removed Alert Information........................................................................................122 Table 144  - Fabric Added/Removed Message Arguments..................................................................................122 Table 145  - Fabric Added/Removed Alert Information.........................................................................................122 Table 146  - Security Policy change Message Arguments ...................................................................................123 Table 147  - Security Policy change Alert Information..........................................................................................123 Table 148  - System OperationalStatus Bellwether Message Arguments............................................................124 Table 149  - System OperationalStatus Bellwether Alert Information...................................................................124 Table 150  - NetworkPort OperationalStatus Bellwether Message Arguments ....................................................124 Table 151  - NetworkPort OperationalStatus Bellwether Alert Information...........................................................125 Table 152  - LogicalDisk OperationalStatus Bellwether Message Arguments......................................................125 Table 153  - LogicalDisk OperationalStatus Bellwether Alert Information ............................................................125 Table 154  - CopyState is set to Broken Message Arguments.............................................................................126 Table 155  - CopyState is set to Broken Alert Information....................................................................................126 Table 156  - Not Enough Space Message Arguments..........................................................................................127 Table 157  - Not Enough Space Alert Information................................................................................................127 Table 158  - The changes in RemoteReplicationCollection Message Arguments................................................127 Table 159  - The changes in RemoteReplicationCollection Alert Information.......................................................128 Table 160  - The changes in ProtocolEndpoint Message Arguments...................................................................128 Table 161  - The changes in ProtocolEndpoint Alert Information .........................................................................128 Table 162  - CopyState is set to Broken Message Arguments.............................................................................129 Table 163  - CopyState is set to Broken Alert Information....................................................................................129 Table 164  - CopyState is set to Invalid Message Arguments..............................................................................129 Table 165  - CopyState is set to Invalid Alert Information.....................................................................................130 Table 166  - CopyState is set to Inactive Message Arguments............................................................................130 Table 167  - CopyState is set to Inactive Alert Information...................................................................................130 Table 168  - CopyState is set to Split Message Arguments..................................................................................131 Table 169  - CopyState is set to Split Alert Information........................................................................................131 Table 170  - CopyState alert has been cleared Message Arguments ..................................................................131 Table 171  - CopyState alert has been cleared Alert Information.........................................................................132 Table 172  - Available Space Changed Message Arguments ..............................................................................132 Table 173  - Available Space Changed Alert Information.....................................................................................132 Table 174  - Filesystem Inaccessible Message Arguments..................................................................................133 Table 175  - Filesystem Inaccessible Alert Information ........................................................................................133 Table 176  - Filesystem is Online Message Arguments........................................................................................133 Table 177  - Filesystem is Online Alert Information..............................................................................................133 Table 178  - Fileshare is degraded Message Arguments.....................................................................................134 Table 179  - Fileshare is degraded Alert Information............................................................................................134 Table 180  - Fileshare in normal state Message Arguments ................................................................................134 Table 181  - Fileshare in normal state Alert Information.......................................................................................135 Table 182  - Required Firmware Version Message Arguments............................................................................135 Table 183  - Required Firmware Version Alert Information...................................................................................135 Table 184  - Recommended Firmware Version Message Arguments..................................................................136 Table 185  - Recommended Firmware Version Alert Information.........................................................................136 Table 186  - Controller OK Message Arguments..................................................................................................136 Table 187  - Controller OK Alert Information.........................................................................................................136 Table 188  - Controller not OK Message Arguments............................................................................................137 Table 189  - Controller not OK Alert Information...................................................................................................137 Table 190  - Bus rescan complete Alert Information.............................................................................................137 Table 191  - Disk initialize Failed Message Arguments........................................................................................137 Table 192  - Disk initialize Failed Alert Information...............................................................................................138 Table 193  - Read Warning Alert Information .......................................................................................................138 Table 194  - Write Warning Alert Information........................................................................................................138 Table 195  - Hard Error Alert Information..............................................................................................................139 Table 196  - Media Alert Information.....................................................................................................................139 Table 197  - Read Failure Alert Information..........................................................................................................139 Table 198  - Write Failure Alert Information..........................................................................................................140 Table 199  - Media Life Alert Information..............................................................................................................140 Table 200  - Not Data Grade Alert Information.....................................................................................................140 Table 201  - Write Protect Alert Information..........................................................................................................141 Table 202  - No Removal Alert Information...........................................................................................................141 Table 203  - Cleaning Media Alert Information .....................................................................................................141 Table 204  - Unsupported Format Alert Information .............................................................................................142 Table 205  - Recoverable Snapped Tape Alert Information..................................................................................142 Table 206  - Unrecoverable Snapped Tape Alert Information ..............................................................................142 Table 207  - Memory Chip In Cartridge Failure Alert Information.........................................................................143 Table 208  - Forced Eject Alert Information..........................................................................................................143 Table 209  - Read Only Format Alert Information.................................................................................................143 Table 210  - Directory Corrupted On Load Alert Information................................................................................144 Table 211  - Nearing Media Life Alert Information................................................................................................144 Table 212  - Clean Now Alert Information.............................................................................................................144 Table 213  - Clean Periodic Alert Information.......................................................................................................145 Table 214  - Expired Cleaning Media Alert Information........................................................................................145 Table 215  - Invalid Cleaning Media Alert Information..........................................................................................145

Table 216  - Retention Requested Alert Information.............................................................................................146

Table 217  - Dual-Port Interface Error Alert Information.......................................................................................146 Table 218  - Drive Maintenance Alert Information ................................................................................................146 Table 219  - Hardware A Alert Information ...........................................................................................................147

Table 220  - Hardware B Alert Information ...........................................................................................................147 Table 221  - Interface Alert Information ................................................................................................................147 Table 222  - Eject Media Alert Information............................................................................................................148 Table 223  - Download Failure Alert Information ..................................................................................................148 Table 224  - Loader Hardware A Alert Information...............................................................................................148

Table 225  - Loader Stray Media Alert Information...............................................................................................149 Table 226  - Loader Hardware B Alert Information...............................................................................................149 Table 227  - Loader Door Alert Information ..........................................................................................................149 Table 228  - Loader Hardware C Alert Information...............................................................................................150 Table 229  - Loader Magazine Alert Information...................................................................................................150 Table 230  - Loader Predictive Failure Alert Information ......................................................................................150 Table 231  - Load Statistics Alert Information.......................................................................................................151 Table 232  - Media Directory Invalid at Unload Alert Information .........................................................................151 Table 233  - Media System area Write Failure Alert Information..........................................................................151 Table 234  - Media System Area Read Failure Alert Information .........................................................................152 Table 235  - No Start of Data Alert Information.....................................................................................................152

Table 236  - Loading Failure Alert Information......................................................................................................152 Table 237  - Library Hardware A Alert Information ...............................................................................................153 Table 238  - Library Hardware B Alert Information ...............................................................................................153 Table 239  - Library Hardware C Alert Information...............................................................................................153

Table 240  - Library Hardware D Alert Information...............................................................................................154 Table 241  - Library Diagnostic Required Alert Information..................................................................................154 Table 242  - Library Interface Alert Information ....................................................................................................154 Table 243  - Failure Prediction Alert Information ..................................................................................................155 Table 244  - Library Maintenance Alert Information..............................................................................................155 Table 245  - Library Humidity Limits Alert Information..........................................................................................155 Table 246  - Library Voltage Limits Alert Information............................................................................................156 Table 247  - Library Stray Media Alert Information...............................................................................................156 Table 248  - Library Pick Retry Alert Information..................................................................................................156 Table 249  - Library Place Retry Alert Information................................................................................................157 Table 250  - Library Load Retry Alert Information.................................................................................................157 Table 251  - Library Door Alert Information...........................................................................................................157 Table 252  - Library Mailslot Alert Information......................................................................................................158 Table 253  - Library Magazine Alert Information...................................................................................................158 Table 254  - Library Security Alert Information .....................................................................................................158 Table 255  - Library Security Mode Alert Information ...........................................................................................159 Table 256  - Library Offline Alert Information........................................................................................................159 Table 257  - Library Drive Offline Alert Information...............................................................................................159 Table 258  - Library Scan Retry Alert Information.................................................................................................160 Table 259  - Library Inventory Alert Information....................................................................................................160 Table 260  - Library Illegal Operation Alert Information........................................................................................160 Table 261  - Pass Through Mechanism Failure Alert Information.........................................................................161 Table 262  - Cartridge in Pass-through Mechanism Alert Information..................................................................161 Table 263  - Unreadable barcode Labels Alert Information..................................................................................161 Table 264  - Throughput Threshold Warning Alert Message Arguments..............................................................162 Table 265  - Throughput Threshold Warning Alert Alert Information....................................................................162 Table 266  - Throughput Threshold Critical Alert Message Arguments................................................................162 Table 267  - Throughput Threshold Critical Alert Alert Information.......................................................................163 Table 268  - Physical Capacity Threshold Warning Alert Message Arguments....................................................163 Table 269  - Physical Capacity Threshold Warning Alert Alert Information..........................................................163 Table 270  - Physical Capacity Threshold Critical Alert Message Arguments......................................................164 Table 271  - Physical Capacity Threshold Critical Alert Alert Information.............................................................164 Table 272  - Logical Capacity Threshold Warning Alert Message Arguments......................................................165 Table 273  - Logical Capacity Threshold Warning Alert Alert Information............................................................165 Table 274  - Logical Capacity Threshold Critical Alert Message Arguments........................................................165 Table 275  - Logical Capacity Threshold Critical Alert Alert Information...............................................................166 Table 276  - System Ratio Threshold Warning Alert Message Arguments...........................................................166 Table 277  - System Ratio Threshold Warning Alert Alert Information.................................................................166 Table 278  - System Ratio Threshold Critical Alert Message Arguments.............................................................167 Table 279  - System Ratio Threshold Critical Alert Alert Information....................................................................167 Table 280  - Deduplication Ratio Threshold Warning Alert Message Arguments.................................................168 Table 281  - Deduplication Ratio Threshold Warning Alert Alert Information .......................................................168 Table 282  - Deduplication Ratio Threshold Critical Alert Message Arguments...................................................168 Table 283  - Deduplication Ratio Threshold Critical Alert Alert Information..........................................................169 Table 284  - Replication Traffic Threshold Warning Alert Message Arguments...................................................169 Table 285  - Replication Traffic Threshold Warning Alert Alert Information..........................................................169 Table 286  - Replication Traffic Threshold Critical Alert Message Arguments......................................................170 Table 287  - Replication Traffic Threshold Critical Alert Alert Information............................................................170 Table 288  - Message Types ................................................................................................................................174 Table 289  - Required Configuration Properties for SA as DA..............................................................................178 Table 290  - Required Configuration Properties for SA ........................................................................................179 Table 291  - Create an IndicationFilter and subscribe to it ...................................................................................185

# FOREWORD

Storage Management Technical Specification, Part 2 Common Architecture, 1.8.0 Rev 4 defines the core 3 architecture of SMI-S. This includes the protocols (WBEM, SLP,...); the model is defined in the other 4 specification parts.

5

Parts of this Standard

6

This standard is subdivided in the following parts:

7

- Storage Management Technical Specification, Part 1 Overview, 1.8.0 Rev 4

8

- Storage Management Technical Specification, Part 2 Common Architecture, 1.8.0 Rev 4

9

- Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4

10

- Storage Management Technical Specification, Part 4 Block Devices, 1.8.0 Rev 4

11

- Storage Management Technical Specification, Part 5 Filesystems, 1.8.0 Rev 4

12

- Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4

13

- Storage Management Technical Specification, Part 7 Host Elements, 1.8.0 Rev 4

14

- Storage Management Technical Specification, Part 8 Media Libraries, 1.8.0 Rev 4

15

SNIA Web Site 16

Current SNIA practice is to make updates and other information available through their web site at 17 http://www.snia.org

18

SNIA Address 19

Requests for interpretation, suggestions for improvement and addenda, or defect reports are welcome. They 20 should be sent via the SNIA Feedback Portal at http://www.snia.org/feedback/ or by mail to the Storage Networking

21

Industry Association, 4360 ArrowsWest Drive, Colorado Springs, Colorado 80907, U.S.A.

# 1  Scope

Storage Management Technical Specification, Part 2 Common Architecture, 1.8.0 Rev 4 defines the core 3 architecture and protocols in SMI-S. The components of SMI-S architecture include:

- • Transport - communicating management information between constituents of the management system

- • Health and fault management - detecting failures through monitoring the state of storage components

- • General information about the object model

- • Names - how SMI-S uses names to allow applications to correlate across SMI-S and to other standards

- • Standard messages - how exceptions are presented to client applications

- • Service discovery - techniques clients use to discover SMI-S services

- • Installation and upgrade - recommendations for implementations

- • Compliance - requirement for compliance to the standard

# 2  Normative references

## 2.1	General

3 The following referenced documents are indispensable for the application of this document. For dated 4 references, only the edition cited applies. For undated references, the latest edition of the referenced 5 document (including any amendments) applies.

## 6	2.2	Approved references

- ISO/IEC 14776-413, SCSI Architecture Model - 3 (SAM-3) [ANSI INCITS 402-200x]

- ISO/IEC 14776-452, SCSI Primary Commands - 3 (SPC-3) [ANSI INCITS.351-2005]

- ANSI/INCITS 374:2003, Information technology - Fibre Channel Single - Byte Command Set-3 (FC-SB-3)

- ISO/IEC 20648, Information technology — TLS specification for storage systems

- SNIA TLS Specification for Storage Systems http://www.snia.org/https://www.snia.org/tech_activities/standards/curr_standards/tls

12

## 13	2.3	DMTF references (Final)

14	DMTF Final documents are accepted as standards. For DMTF Draft or Preliminary documents, see 2.5.

15

DMTF DSP0004, CIM Infrastructure Specification 3.0.1

16	http://www.dmtf.org/sites/default/files/standards/documents/DSP0004_3.0.1.pdf

17

DMTF DSP0200, CIM Operations over HTTP 1.4

18	http://www.dmtf.org/sites/default/files/standards/documents/DSP0200_1.4.pdf

19

DMTF DSP0201 Representation of CIM in XML 2.4

20	http://www.dmtf.org/sites/default/files/standards/documents/DSP0201_2.4.pdf

21

DMTF DSP0202 CIM Query Language Specification 1.0

22	http://www.dmtf.org/standards/published_documents/DSP0202_1.0.0.pdf

23

DMTF DSP0205 WBEM Discovery Using the Service Location Protocol 1.0.1 24	http://dmtf.org/sites/default/files/standards/documents/DSP0205_1.0.1.pdf

25

DMTF DSP0206 WBEM SLP Template 1.0

26	http://www.dmtf.org/sites/default/files/standards/documents/wbem.1.0.en

27

DMTF DSP0207, 1.0.1 WBEM URI Mapping Specification

- http://www.dmtf.org/sites/default/files/standards/documents/DSP0207_1.0.1.pdf

- DMTF DSP0210 CIM-RS Protocol 1.0 http://www.dmtf.org/sites/default/files/standards/documents/DSP0210_1.0.pdf

30

- DMTF DSP0211 CIM-RS Payload Representation in JSON 1.0

- http://www.dmtf.org/sites/default/files/standards/documents/DSP0211_1.0.pdf

- DMTF DSP0221 Managed Object Format (MOF) 3.0.1

- http://www.dmtf.org/sites/default/files/standards/documents/DSP0221_3.0.1.pdf

- DMTF DSP0223 Generic Operations 2.0.0 http://www.dmtf.org/sites/default/files/standards/documents/DSP0223_2.0.0.pdf

36

37	DMTF DSP0226, WS-Management Protocol Specification 1.1.1 http://www.dmtf.org/sites/default/files/standards/documents/DSP0226_1.1.1.pdf

38

39

- DMTF DSP0227, WS-Management CIM Binding Specification 1.2

- http://www.dmtf.org/sites/default/files/standards/documents/DSP0227_1.2.pdf

- DMTF DSP0228, Message Registry Schema, 1.1.0

- http://schemas.dmtf.org/wbem/messageregistry/1/dsp0228_1.1.0.xsd

- DMTF DSP2011, Standard Messages Whitepaper 1.0

- http://www.dmtf.org/sites/default/files/standards/documents/DSP2011.pdf

- DMTF DSP0230, WS-CIM Mapping Specification 1.1.0

- http://www.dmtf.org/sites/default/files/standards/documents/DSP0230_1.1.0.pdf

- DMTF DSP8000 1.2.0 Message Registry Print XSLT Stylesheet

- http://schemas.dmtf.org/wbem/messageregistry/1/dsp8000_1.2.0.xsl

- DMTF DSP8016 2.0.0 WBEM Operations Message Registry

- http://schemas.dmtf.org/wbem/messageregistry/1/dsp8016_2.0.0.xml

52

## 2.4	IETF references

53

For IETF Informational documents and proposed standards, see 2.5.

54

IETF RFC 2045, Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message 55 Bodies

56 http://www.ietf.org/rfc/rfc2045.txt

57

IETF RFC 2246, The TLS Protocol Version 1.0

58 http://www.ietf.org/rfc/rfc2246.txt

59

IETF RFC 4291, IP Version 6 Addressing Architecture 60 IETF RFC 2396, Uniform Resource Identifiers (URI)

61 http://www.ietf.org/rfc/rfc2396.txt

62

63	IETF RFC 2608, Service Location Protocol, Version 2 http://www.ietf.org/rfc/rfc2608.txt

64

65	IETF RFC 2609, Service Templates and Service: Schemes http://www.ietf.org/rfc/rfc2609.txt 66

67	IETF RFC 2610, DHCP Options for Service Location Protocol http://www.ietf.org/rfc/rfc2610.txt

68

69	IETF RFC 2616, Hypertext Transfer Protocol -- HTTP/1.1 http://www.ietf.org/rfc/rfc2616.txt

70

71	IETF RFC 2617, HTTP Authentication: Basic ad Digest Access Authentication http://www.ietf.org/rfc/rfc2617.txt

72

IETF RFC 2445, Internet Calendaring and Scheduling Core Object Specification (iCalendar)

- http://www.ietf.org/rfc/rfc2445.txt

- IETF RFC 3280, Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile

- http://www.ietf.org/rfc/rfc3280.txt 76 IETF RFC 3723, Securing Block Storage Protocols over IP

77 http://www.ietf.org/rfc/rfc3723.txt 78

79	IETF RFC 3986, Definitions of Managed Objects for the DS3/E3 Interface Type 80	http://www.ietf.org/rfc/rfc3986.txt

- IETF RFC 4291, IP Version 6 Addressing Architecture

- http://www.ietf.org/rfc/rfc4291.txt

- IIETF RFC 4514, Lightweight Directory Access Protocol (LDAP): String Representation of Distinguished

- Names

http://www.ietf.org/rfc/rfc4514.txt

85

## 86	2.5	References under development

The following documents (and their web addresses) are subject to change.

DMTF DSP8055 1.0.0d Diagnostic Message Registry

http://www.dmtf.org/sites/default/files/standards/documents/DSP8055_1.0.0d.xml

## 2.6	Other references

IETF RFC 1945 Hypertext Transfer Protocol -- HTTP/1.0 http://www.ietf.org/rfc/rfc1945.txt

IETF RFC 2614 An API for Service Location http://www.ietf.org/rfc/rfc2614.txt

UML (Universal Modeling Language) Specifications

http://www.omg.org/technology/documents/modeling_spec_catalog.htm#UML

ITU-T Recommendation X.509 (1997 E): Information Technology - Open Systems Interconnection - The Directory: Authentication Framework

PKCS #12, Personal Information Exchange Syntax http://www.rsasecurity.com/rsalabs/node.asp?id=2138

# 1	3  Terms, definitions, symbols, abbreviations, and conventions

## 2	3.1	Terms and definitions

3	3.1.1

access control 4 means to ensure authorized access and to prevent unauthorized access to resources relevant to information security based on the business and security requirements 5

- 3.1.2

- account

an established relationship between a user and a computer, network or information service 8

3.1.3

- accountability

Information Security property that establishes responsibility for the effects of action taken by individuals,

- organizations or communities with an explanation as to how and why the action took place. 11	[SOURCE: ISO/IEC 27000]

12

- 3.1.4

administrator

- a person charged with the installation, configuration, and management of a computer system, network, 15 storage subsystem, database, or application

16

3.1.5

agent 17 an Object Manager that includes the provider service for a limited set of resources

18

3.1.6

- aggregation

- a strong form of an association

3.1.7

- audit Log

- logs collecting the evidence of selected user activities, exceptions, and information security events

- 3.1.8

- authentication the act of verifying the identity claimed by a party to a communication

25

3.1.9

26	authentication mechanism process for determining and validating a user (or device) identity 27

- 3.1.10

authorization

- the process of granting a right or permission to access a system resource

- 3.1.11

bidirectional authentication

- See  "mutual authentication"

32

3.1.12

- CIM Server See "WBEM Server".

- client

a process that issues requests for service

- 3.1.13

- Common Information Model

- an object-oriented description of the entities and relationships in a business' management environment maintained by the Distributed Management Task Force

38

3.1.14

39 dedicated SMI-S Server a WBEM Server that is dedicated to supporting a single device or subsystem

40

- 3.1.15 digested password

- the hashed form of a cleartext password

- 3.1.16

- discovery

a process which provides information about what physical and logical storage entities have been found 45 within the management domain

46

- 3.1.17

Distributed Management Task Force (DMTF)

- an industry organization that develops management standards for computer system and enterprise

environments

49

- 3.1.18 dynamic host control protocol (DHCP)

- an Internet protocol that allows nodes to dynamically acquire (“lease”) network addresses for periods of 52 time rather than having to pre-configure them

53	3.1.19 embedded SMI-S Server 54 a WBEM Server that is embedded in the device or subsystem for which it provides management

- 3.1.20

- enclosure

a box or cabinet

57

3.1.21

58 entity authentication corroboration that an entity is the one claimed.

59

[SOURCE: ISO/IEC 9798]

60

3.1.22

- enumerate

- an operation used to enumerate subclasses, subclass names, instances and instance names 3.1.23

- event

- an occurrence of a phenomenon of interest.

- 3.1.24

- eXtensible Markup Language a universal format for structured documents and data on the World Wide Web 67

3.1.25

- extent

a set of consecutively addressed disk blocks.

- 3.1.26

- external authentication

- authentication which relies on an authentication service separate from (or external to) an entity

3.1.27

- extrinsic method

- A method defined as part of CIM Schema

74

3.1.28

75	fabric

Any interconnect between two or more Fibre Channel N_Ports, including point-to-point, loop, and 76

Switched Fabric. 77

3.1.29

- FICON™

- Fibre Channel storage protocol used in IBM mainframe computers and peripheral devices such as ECKD 80 storage arrays and tape drives

81

3.1.30

- general purpose SMI-S Server

- an SMI-S Server that is not dedicated to supporting a single device or subsystem, and may support multiple devices or subsystems.

84

3.1.31

85	grammar

a formal definition of the syntactic structure of a language (see  "syntax"), normally given in terms of 86 production rules that specify the order of constituents and their sub-constituents in a sentence (a well87 formed string in the language)

- 3.1.32 host bus adapter (HBA)

- card that contains ports for host systems

- 3.1.33

- Hypertext Transfer Protocol (HTTP) request-reply protocol used for internet communications

92

3.1.34

- identity

- representation of an actual user (or application or service or device)

- 3.1.35

- interconnect element non-terminal network elements (Switches, hubs, routers, directors).

97

3.1.36

98 interface definition language (IDL)

high-level declarative language that provides the syntax for interface declarations 99

- 3.1.37

- intrinsic method operations made against a WBEM server and a CIM namespace independent of the implementation of the schema defined in the server

- 3.1.38

- logical unit number

- a SCSI logical unit or logical unit number.

3.1.39

- mutual authentication

- authentication that provides both parties (users or entities) with assurance of each other's identity. 107

- 3.1.40

- Network Address Authority (NAA) a four bit identifier to denote a network address authority (i.e., an organization such as CCITT or IEEE 110 that administers network addresses)

111

3.1.41

non-repudiation 112 the ability to prove an action or event has taken place, so that this event or action cannot be repudiated

- later

- [SOURCE: ISO/IEC 27000]

- 3.1.42

- out-of-band transmission of management information for storage components outside of the data path, typically over 117	Ethernet.

118 Also: use of mechanisms other than the ones required on a communications channel to transmit 119 information.

[SOURCE: ISO/IEC 24767]

120

- 3.1.43

- partition collection of contiguous block on a disk or virtual disk

123

3.1.44

124	password

a secret sequence of characters or a word that a user submits to a system for purposes of authentication, 125 validation, or verification.

126

3.1.45

127	path

combination of initiator and target ports and logical unit

128

- 3.1.46 privacy

- the right of an entity (normally an individual or an organization), acting on its own behalf, to determine the 131 degree to which the confidentiality of their private information is maintained.

132	3.1.47

privileged user 133 a user who, by virtue of function, and/or seniority, has been allocated powers within a system, which are significantly greater than those available to the majority of users 134

- 3.1.48

- protocol

- a set of rules that define and constrain data, operations, or both

- 3.1.49 proxy SMI-S Server

- an SMI-S Server that does not run on the device or subsystem which it supports

- 3.1.50

- public key infrastructure (PKI)

- a framework established to issue, maintain, and revoke public key certificates accommodating a variety of security technologies.

143

3.1.51

144	SAN

a group of fabrics that have common leaf elements.

145

- 3.1.52

- SCSI Parallel Interface (SPI)

The family of SCSI standards that define the characteristics of the parallel version of the SCSI interface. 148

3.1.53

149	Secure Sockets Layer (SSL)

A suite of cryptographic algorithms, protocols and procedures used to provide security for 150	communications used to access the world wide web.

151

Note 1 to entry: More recent versions of SSL are known as TLS (Transport Level Security) and are standardized by the Internet Engineering Task Force (IETF).

152

3.1.54

153

Service Access Point

- the network address of a process offering a service.

- 3.1.55 156	shared secret

157 a pre-shared key that has been allocated to communicating parties prior to the communication process starting.

158

3.1.56

159

Simple Network Management Protocol (SNMP)

- an IETF protocol for monitoring and managing systems and devices in a network

- 3.1.57

- SMI-S server a WBEM Server that supports SMI-S (Storage Management Initiative Specification) profiles for management 163	of a device or subsystem

164

3.1.58

SNMP trap

165 a type of SNMP message used to signal that an event has occurred

166

3.1.59 167	soft zone

168 a zone consisting of zone Members that are made visible to each other through Client Service requests 169

3.1.60

Storage Management Initiative Specification (SMI-S)

170 an interface between WBEM-capable clients and servers for the secure, extensible, and interoperable 171 management of networked storage (this standard)

- 3.1.61

- Storage Networking Industry Association (SNIA) an association of producers and consumers of storage networking products whose goal is to further 174 storage networking technology and applications

175

176

- 3.1.62

- storage resource management (SRM)

- management of physical and logical storage resources, including storage elements, storage devices, appliances, virtual devices, disk volume and file resources

180

3.1.63

181	switch

fibre channel interconnect element that supports a mesh topology

182

- 3.1.64 switched fabric

- a fabric comprised of one or more Switches

- 3.1.65 syntax

- the structure of strings in some language. A language's syntax is described by a grammar 187

- 3.1.66

- threat

- a potential source of an incident that may result in adverse changes to an asset, a group of assets or an

- organization

- [SOURCE: ISO/IEC 27000]

193

3.1.67

- unidirectional authentication

- authentication that provides one party (user or entity) with assurance of the other's identity 196

- 3.1.68

- User Datagram Protocol

- an Internet protocol that provides connectionless datagram delivery service to applications 200

- 3.1.69

- vulnerability

- weakness in an information system, system security procedures, internal controls, or implementation that 204	could be exploited or triggered by a threat

205	[SOURCE: ISO/IEC 27000]

206

- 3.1.70

- Web Based Enterprise Management (WBEM)

- a set of management and Internet standard technologies from DMTF developed to unify the management 210	of distributed computing environments

211

3.1.71

212

WBEM Server

213 a server that provides support for CIM requests and provides CIM responses using web protocols 214

- 3.1.72

- web service

- a software system designed to support interoperable machine-to-machine interaction over a network 218

- 3.1.73

- zone

- a group of ports and switches that allow access. Defined by a zone definition 222

- 3.1.74

- zone set

- one or more zones that may be activated or deactivated as a group

226

## 227	3.2	Acronyms and abbreviations

- AAA	Authentication, Authorization, and Accounting

- ACL	Access Control List

API	application programming interface

230

CA	Certificate Authority CIM	Common Information Model

231

CRL	Certificate Revocation List

- DHCP	dynamic host control protocol

- FC	Fibre Channel

HBA	host bus adapter

- HMAC	keyed-Hash Message Authentication Code

- HTTPS	Hypertext Transfer Protocol Secure 236	IDL	interface definition language

IETF	Internet Engineering Task Force

- IMA	iSCSI Management API

- IP	Internet Protocol

#### 239	IPsec	Internet Protocol Security iSCSI	Internet SCSI

240	LDAP	Lightweight Directory Access Protocol

MAC	Message Authentication Code

241

OS	operating system

PKI	Public Key Infrastructure

242

RADIUS	Remote Authentication Dial In User Service

- RBAC	Role-base Access Control RFC	Request for Comments

- SAM-3	SCSI Architecture Model

- SAN	storage area network

- SB	Single Byte (command set)

SCSI	Small Computer System Interface

- SES	SCSI Enclosure Services

- SLP	Service Location Protocol

SMI-S	Storage Management Initiative - Specification

- SNIA	Storage Networking Industry Association

SPC-3	SCSI Primary Commands-3

250

SSL	Secure Socket Layer

251

SSO	Single Sign-on

252

SSP	Storage Service Provider TC	Technical Committee

253

TCP	Transmission Control Protocol

- TLS	Transport Layer Security

- UDP	User Datagram Protocol

- URL	Uniform Resource Locator

- WBEM 	Web-Based Enterprise Management

- XML	Extensible Markup Language

## 259	3.3	Keywords

- expected

- a keyword used to describe the behavior of the hardware or software in the design models presumed by this standard 262

263	Other hardware and software design models may also be implemented.

264

265	3.3.1

invalid 266 a keyword used to describe an illegal or unsupported bit, byte, word, field or code value

267

- Note 1 to entry: Receipt of an invalid bit, byte, word, field or code value shall be reported as an error.

- 3.3.2 mandatory

- a keyword indicating an item that is required to be implemented as defined in this standard to claim compliance with this standard.

271

3.3.3

272	may

a keyword that indicates flexibility of choice with no implied preference 273

- 3.3.4 may not

- keywords that indicates flexibility of choice with no implied preference

276

3.3.5

277	obsolete a keyword indicating that an item was defined in prior standards but has been removed from this standard

3.3.6

opaque

a keyword indicating that value has no semantics or internal structure

3.3.7 optional a keyword that describes features that are not required to be implemented by this standard

Note 1 to entry: However, if any optional feature defined by this standard is implemented, it shall be implemented as defined in this standard.

3.3.8 reserved

a keyword referring to bits, bytes, words, fields and code values that are set aside for future standardization

Note 1 to entry: Their use and interpretation may be specified by future extensions to this or other standards.

Note 2 to entry: A reserved bit, byte, word or field shall be set to zero, or in accordance with a future extension to this standard. Recipients are not required to check reserved bits, bytes, words or fields for zero values.

Note 3 to entry: Receipt of reserved code values in defined fields shall be reported as an error.

3.3.9 shall

a keyword indicating a mandatory requirement

Designers are required to implement all such requirements to ensure interoperability with other products that conform to this standard.

3.3.10 should

a keyword indicating flexibility of choice with a preferred alternative; equivalent to the phrase “it is recommended”

## 3.4	Conventions

Certain words and terms used in this American National Standard have a specific meaning beyond the normal English meaning. These words and terms are defined either in 3 Terms, definitions, symbols, abbreviations, and conventions or in the text where they first appear.

Numbers that are not immediately followed by lower-case b or h are decimal values.

Numbers immediately followed by lower-case b (xxb) are binary values.

Numbers immediately followed by lower-case h (xxh) are hexadecimal values.

Hexadecimal digits that are alphabetic characters are upper case (i.e., ABCDEF, not abcdef).

Hexadecimal numbers may be separated into groups of four digits by spaces. If the number is not a multiple of four digits, the first group may have fewer than four digits (e.g., AB CDEF 1234 5678h) Decimal fractions are initiated with a comma (e.g., two and one half is represented as 2,5).

Decimal numbers having a value exceeding 999 are separated with a space(s) (e.g., 24 255).

See also “ Typographical Conventions “(in front matter) for typographical conventions.

# 1	4  Transport and Reference Model

## 2	4.1	Introduction

### 3	4.1.1	Overview

4 The interoperable management of storage devices and network elements in a distributed storage network 5 requires a common transport for communicating management information between constituents of the 6 management system. This section of the specification details the design of this transport, as well as the 7 roles and responsibilities of constituents that use the common transport (i.e., a reference model).

### 8	4.1.2	Language Requirements

- To express management information across the interface, a language is needed that:

- • Can contain platform independent data structures,

- • Is self describing and easy to debug,

- • Can be extended easily for future needs.

### 13	4.1.3	Communications Requirements

- Communications protocols to carry the XML based management information are needed that:

- • Can take advantage of the existing ubiquitous IP protocol infrastructures,

- • Can be made to traverse inter- and intra-organizational firewalls, 17 • Can easily be embedded in low cost devices.

18 The Hyper Text Transport Protocol (HTTP) was chosen for the messaging protocol and TCP was chosen 19 for the base transfer protocol to carry the XML management information for this interface as they meet 20 the requirements in 4.1.3.

### 21	4.1.4	XML Message Syntax and Semantics

22 In order to be successful, the expression of XML management information (messages) across this 23 interface needs to follow consistent rules for semantics and syntax. These rules are detailed in this 24 specification. They are of sufficient quality, extensibility, and completeness to allow their wide adoption by 25 storage vendors and management software vendors in the industry. In addition, to facilitate rapid 26 adoption, existing software that can parse, marshal, un-marshal, and interpret these XML messages 27 should be widely available in the market such that vendor implementations of the interface are 28 accelerated. The message syntax and semantics selected should:

- • Be available on multiple platforms,

- • Have software implementations that are Open source (i.e., collaborative code base),

- • Have software implementations available in Java and C++,

- • Leverage industry standards where applicable, 33 • Conform with W3C standards for XML use.

- • Be object model independent (i.e., be able to express any object model).

- Virtually the only existing industry standard in this area is the WBEM standards as developed and 36 maintained by the DMTF.

## 37	4.2	Transport Stack

38 It is the primary objective of this interface to drive seamless interoperability across vendors as 39 communications technology and the object model underlying this interface evolves. Accordingly, the 40 transport stack has been layered such that (if required) other protocols can be added as technology 41 evolves. For example, should SOAP or IIOP become prominent, the content in the stack could be

- expanded with minimal changes to existing product implementations in the market.This specification relies on the DMTF WBEM Protocol Specifications.

- To be compliant with this specification, CIM-XML shall be supported.

Optionally, other WBEM protocols, such as WS-Management or CIM-RS, may additionally be supported.

44

- It should be noted that this specification places no restriction on the physical network selected to carry this transport stack. For example, a vendor can choose to use in-band communication over Fibre-channel

- as the backbone for this interface. Another vendor could exclusively (and wisely) choose out-of-band communication over Ethernet to implement this management interface. Additionally, select vendors could 47 choose a mix of in-band and out-of-band physical network to carry this transport stack.

## 48	4.3	Reference Model

49

### 50	4.3.1	Overview

51

- As shown in Figure 7, the Reference Model shows all possible constituents of the management environment in the presence of the transport stack for this interface.

Client

- Figure 7 illustrates that the transport for this interface uses a WBEM Protocol to execute intrinsic and extrinsic methods against the schema for this interface.

54

NOTE  It is envisioned that a more complete version of this reference model would include a Lock Manager. However, in this 55 version of SMI-S, a Lock Manager is not specified. As a result, it is shown as a dotted box to illustrate where the role would fit.

56

### 4.3.2	Roles for Interface Constituents

4.3.2.1	Client

A Client is the consumer of the management information in the environment. It provides an API (language binding in Java or C++ for example) for overlying management applications (like backup engines, graphical presentation frameworks, and volume managers) to use.

4.3.2.2	SMI-S Server

An SMI-S Server is a WBEM Server. Often, an SMI-S Server controls only one device or subsystem, and is incapable of providing support for complex intrinsic methods like schema traversal. An SMI-S Server can be embedded in a device (like a Fibre Channel Switch) or provide a proxy on a host that communicates to a device over a legacy or proprietary interconnect (like a SCSI based array controller).

Embedding an SMI-S Server directly in a device or subsystem reduces the management overhead seen by a customer and eliminates the requirement for a stand-alone host (running the proxy agent) to support the device.

Embedded SMI-S Servers are the desired implementation for “plug and play” support in an SMI-S managed environment. However, proxy SMI-S Servers are a practical concession to the legacy devices that are already deployed in storage networked environments. In either case, the minimum CIM support for SMI-S Servers applies to either SMI-S Server deployments.

4.3.2.3	General Purpose SMI-S Server

A General Purpose SMI-S Server serves management information from one or more devices or underlying subsystems through providers. As such a General Purpose SMI-S Server is an aggregator that enables proxy access to devices/subsystems and can perform more complex operations like schema traversals. A General Purpose SMI-S Server typically includes a standard provider interface to which device vendors adapt legacy or proprietary product implementations.

4.3.2.4	Provider

A provider expresses management information for a given resource such as a storage device or 57 subsystem exclusively to a WBEM Server. The resource may be local to the host that runs the Object 58 Manager or may be remotely accessed through a distributed systems interconnect.

59

60	4.3.2.5	Lock Manager

This version of the specification does not support a lock manager.

61

4.3.2.6	Directory Server (SLP Directory Agent)

62	A directory server provides a common service for use by clients for locating services in the management 63	environment.

64

### 65	4.3.3	Cascaded Agents

66	This specification discusses constituents in the SMI-S environment in the context of Clients and Servers.

This version of the specification also allows constituents in a SMI-S management environment to function 67 as both client and server.

68

69

70

Directory Server (SLP Directory Agent)

# 1	5  Health and Fault Management

## 2	5.1	Objectives

3 Health and Fault Management is the activity of anticipating or detecting failures through monitoring the 4 state of the storage network and its components and intervening before services can be interrupted. A 5 service in this case is the realization of storage through several interconnected devices connected, 6 configured for a dedicated purpose. The purpose is the delivery of software application functionality in 7 support of some business function.

## 8	5.2	Overview

- • Express states and statuses with standard meanings.

- • Define the use of comprehensive error reporting in determining the type, category, and source of failures.

- • Define the quality associated with errors rather than qualities.

- • Define explicit failure scopes rather than requiring HFM enabled application to construct them.

## 13	5.3	Terms

5.3.1	error

14 An unexpected condition, result, signal or datum. An error is usually caused by an underlying problem in 15 the system such as a hardware fault or software defect. Errors can be classified as correctable 16 (recoverable) or uncorrectable, detectable or undetectable.

17

5.3.2	fault

18 A problem that occurs when something is broken and therefore not functioning in the manner it was 19 intended to function. A fault may cause an error to occur.

20	5.3.3	fault region

Many devices or applications can attempt to fix themselves upon encountering some adverse condition. 21 The set of components which the device or application can attempt to fix is called the Fault Region. The 22 set may include part or all of other devices or applications. Having the Fault Regions declared helps a 23 HFM application, acting as a doctor, to do no harm by attempting to interfere and thereby adversely affect 24 the corrective action being attempted.

25

26	5.3.4	Health and Fault Management (HFM)

Health and Fault Management is the activity of anticipating or detecting debilitating failures through 27 monitoring the state of the storage network and its components and intervening in before services can be 28 interrupted. A service in this case is the realization of storage utilization through several interconnected 29 devices connected, configured for a dedicated purpose. The purpose is the delivery of software 30 application functionality in support of some business function.

- 5.3.5	operational status

- These values indicate the current status(es) of the element. Various operational statuses are defined (e.g., OK, starting, stopping, stopped, In Service, No Contact).

33

- 5.3.6	health state

- These values indicate the current health of the element. This attribute expresses the health of this element but not necessarily that of its subcomponents.

36

37

38

## 39	5.4	Description of Health and Fault Management

40 The goal of effective administration requires devices and applications that comprise storage services to 41 report their status and the nature of their errors in standard terms. These terms need to be 42 understandable by a client without device-specific knowledge.

Fault Detection Indication; CIM_Error; Poll for text Health State; SubComponentInError Association..

Figure 8 - Basic Fault Detection

43 There are four basic ways for a SMI-S client to detect an error or fault condition. Figure 8 lists the four 44 basic methods for fault detection. These are:

- • Health state and Operational status - Polling.

- • Error - Standard errors returned from CIM operations.

- • Indications - Subscribe for and receive asynchronous Indications.

- • Fault Regions (experimental) - Walk the CIM model looking for RelatedElementCausingError associations.

### 49	5.4.1	Operational Status and Health State (Polling)

50 Operational Status and Health State are the two properties that will be used to monitor health. These two 51 properties could convey very different statuses and may at times be related or independent of each other. 52 For example, you may have a disk drive with the Operational Status of “Stopped” and the HealthState of 53 30 (Non-recoverable error) or 5 (OK). Now the reason the disk drive is stopped could vary from the fact 54 that it had a head crash (HealthState = 30) to the situation where it was stopped for the routine 55 maintenance (HealthState = 5).

56 Table 1 is an example of how HealthState can disambiguate health for a disk drive, various values for 57 OperationalStatus and HealthState:

58	Table 1 shows, for a disk drive, various possible values for OperationalStatus and HealthState. Note that there are many cases not shown.:

Table 1 - OperationalStatus for Disk Drive

| Operational Status | Description | HealthState | Description | Comment |
| --- | --- | --- | --- | --- |
| 2 | OK | 5 | OK | Everything is fine |
| 2 | OK | 10 | Degraded/Warning | Some soft errors |
| 3 or 5 | Degraded or Predicted Failure | 15 | Minor Failure | Many soft errors |
| 3 or 5 | Degraded or Predicted Failure | 20 | Major Failure | Some hard errors |
| 3 | Degraded | 10 | Good | A subcomponent has failed (no data loss) |

Table 1 - OperationalStatus for Disk Drive (Continued)

| Operational Status | Description | HealthState | Description | Comment |
| --- | --- | --- | --- | --- |
| 10 | Stopped | 5 | OK | Drive spun down normally |
| 10 | Stopped | 30 | Non-recoverable Error | Head crash |
| 8 | Starting | 10 | Degraded/Warning | Will update HealthState once fully started |
| 4 | Stressed | 5 | OK | Too many I/O in progress, but the drive is fine. |
| 15 | Dormant | 5 | OK | The drive is not needed currently |

59

60

61 The property OperationalStatus is multi-valued and more dynamic. It tends to emphasize the current 62 status and potentially the immediate status leading to the current status; whereas, the property

- HealthState is less dynamic and tends to imply the health over a longer period of time. Again, in the disk drive example, the disk drive's operational status may change many times in a given time period.

- However, in the same time period, the health of the same drive may not change at all.

65

### 66	5.4.2	Standard Errors and Events

67 Standardization of error and events are required so that the meaning is unambiguous and is given to comparisons.

68

- Error and Alert indications

- HFM clients shall not be required to be embodied with specific knowledge of the devices and applications 71 in order to derive the quality of the error from the datum. The device and application shall express the 72 quality of the error rather than the quantity interpreted with a priori knowledge to determine that error 73 condition is present. For example, a device needs to express that it is too hot rather than requiring the 74 HFM enabled application to determine this from the temperature datum and device specific knowledge of 75 acceptable operating conditions.

76	Standard errors are defined for each Profile. The definitions will be contained in the profiles. Standard errors are not the only error codes that can be returned, but are the only codes that a generic client will 77	understand.

78

### 79	5.4.3	Indications

80 Indications are asynchronous messages from WBEM Servers to clients. A client must register for them. Each SMI-S profile contains lists of indication filters that clients use to indicate the information it is interested in. The message itself is defined in the SMI-S Indications Profile (in Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4).

81

82	EXPERIMENTAL

83

### 84	5.4.4	Event Correlation and Fault Containment

85 Automation will require that an error arising through control and configuration activities, as a side effect of 86 them, or by failures caused by defects can be directly correlatable. Error categories like network cabling 87 failures or network transmission errors will help organize the types of error that can be produced. 88 Standard errors, like impending disk media failure, will be required as well.

- Once the errors have been collected and correlated, the HFM enabled application can produce an impact list sorted by likelihood. Some of the error correlation can be determined by the common affect through

- the manifestation of the RelatedElementCausingError association to be described later. The alerts 91 themselves can report its correlation with other alerts.

92

Potential faults can then be derived from errors for each component. Deriving such a list may require a 93 dialog between the HFM enabled application and the device or application in question such that the HFM 94 enabled application is assisted in the production of the list.

95

96 If permitted, then control and configuration operations may be executed to contain the fault. The pallet of these operations will be those operations already available through SMI-S. However, special operations 97 may arise from the HFM design work as well. Fault containment will include the reconfiguration of the 98 storage service with alternative components, leaving failing components or interconnections isolated.

99

100	Much like a physician, the HFM enabled application is notified or consulted when symptoms appear. The

HFM enabled application then develops a prognosis based on the manifestation of the ailment. At times, 101 the HFM enabled application will perform diagnostic procedures. The end result of the process is to 102 produce a list of possible causes, ranked by probability, and associated recommended procedures.

103

- Also like a doctor, the HFM enabled application will settle for enabling the patients to heal themselves.

That is the HFM enabled applications cannot be expected to heal the device in all cases. A significant

- portion of all possible corrective actions will require the intervention of people or device unique knowledge. 106

- The simplified state diagram shown in Figure 9 follows the fault mitigation life cycle for HFM.

- The device or application manifests an event, either by a state change, error returned from a WBEM 109 operation, or an alert indication. 110

111	The event is recognized by the HFM enabled application and accessed by the HFM enabled application.

It may be that the event indication does the represent the existence of an error. An error condition may be 112 heralded by a single or multiple events occurring in some order. The process of examining and 113 characterizing event as errors is called error handling.

114

- Once it is determined that an error condition is present, then possible causes are sought and ranked by likelihood. The causes themselves describe a potential problem or fault with the component in question. Alternatively, the device or application may report the fault directly, through an alert indication, optionally with recommended actions.

Event

Received

indicate

event

has

occurred

Error

Derived

Fault

Derived

Fault

Resolution

assess

event

conditions

find

and

rank

possible

causes

recommend

actions

Figure 9 - Health Lifecycle

- Fault resolution may not require the intervention of an operator or field technician. It is these faults that 117 can be handled entirely by the HFM enabled application. Otherwise, the HFM enabled application cannot 118 actively participate in whole fault resolution life cycle. In this case, the HFM enabled application would 119 wait for the end state of fault resolution to come to being before ending its fault mitigation exercise.

120 Faults are contained and components repaired or replaced. The instructions to the HFM enabled 121 application for what can be done to repair the fault are the recommended actions. Fault Containment 122 includes fencing off the faulty component and maintaining the service. To be minimally effective, the HFM 123 enabled application contains the fault. The repair may or may not be done with human intervention.

124 The devices and application that comprise a storage system have themselves some level of self 125 diagnostics and report functionality.

Figure 10 - Continuum

126 There is a range of ability of devices and applications to recover from failures and to report on the error 127 recovery actions taken. See Figure 10. The variance of capabilities for device and applications can be 128 plotted on a continuum. At one end of continuum, the device or application recognizes a fault condition 129 and takes action, reporting on the action taken and any further action required to service it. At the other 130 end of the continuum, the device can only report on that states and requires intervention both in the 131 detection of fault conditions and taking corrective action.

Continuum

of

fault

recovery

‐

Reports

on

states

‐

Requires

intervention

‐

Reports

on

action

was

taken

‐

Self

‐

Healing

132 There are limits to what an HFM enabled application can do. Obviously, if the device or application cannot 133 report states, errors and alerts in a standard way or cannot report this data at all, then there is little an 134 external implementation can do.

135	However, few, if any, of these devices and applications can monitor and correct the service as a whole. It 136	is for this reason, the HFM implementation is needed to augment the effectiveness of the administrator.

### 137	5.4.5	Fault Regions

138 A scope can be applied to the effect of errors and the associated fault. A fault may affect a component, a 139 device or application, storage service, or all the above. This scope defines the area of influence for fault 140 containment. For example, the device itself may monitor its components and perform fault mitigation on 141 its own. The plot of components whose errors are handled by a given fault mitigation entity is the fault 142 region. The scope of effect of this fault region shall be defined.

143 Figure 11 illustrates the scope of fault regions in a simplified SAN example and how they may be 144 recursive in nature. AN HFM application has the widest scope of concern in this example.

HFM

Application

Fault

Region

Host

Fault

Region

Array

Fault

Region

Switch

Fault

Region

Host

Array

Switch

Webserver

Routing

and

Zoning

Block

Server

Filesystem

Inband

Protocol

Inband

Protocol

Host

HFM

Application

Out

of

band

protocol

Out

of

band

protocol

Figure 11 - Application Fault Region

145 Error handling is initiated by the interception of error events. For example, a switch may recognize the 146 failure of one it ports and reroute traffic to a working port. In this case, the fault region is defined as the 147 switch itself. If the failure event is publicly consumable, other fault mitigation entities can also handle the 148 error as well. The failure of a drive may be mitigated one way in the array fault region and mitigated 149 differently in the HFM enabled application fault region. For example, the array fault mitigation entity can 150 bring a volume off line if the failure of the disk brings the set of disks below the minimum required for 151 quorum. At the same time, the HFM enabled application can reconfigure the storage service to create a 152 replacement volume and then restore the failed volume's data from backup.

153 The HFM enabled application is one of the several possible storage network scope fault mitigation 154 entities. As discussed previously, this broad scope is necessary to mitigate faults where the faults cannot 155 be entirely mitigated by the storage device or application alone. It is necessary that fault mitigation 156 entities like the HFM enabled application can observe the activities of the fault mitigation entities 157 contained within their fault regions such that they do no harm. Device or application should express what 158 error conditions are to be handled inside their own fault domain and how an HFM enabled application can 159 detect that such fault containment is occurring. State changes on components may BE sufficient 160 representation of these activities.

#### Array Example

161 In general, the HFM enabled application fault region mitigation may not necessarily include the same 162 actions that the host, switch, or array may take to fix them.

EXPERIMENTAL

### 163	5.4.6	Examples

- 5.4.6.1	Array Example

- The scenario presented is related to a storage array that contains one or many ports. A port is off-line. 166 This port effects access to a volume from a host

An indication is produced by the array notifying the HFM enabled application of the failure. The indication reports the name of the port instance that has failed.

A client that gets the top-level ComputerSystem instance should see an operational status of Degraded. At the same time, reading the instance of the LogicalPort subclass (e.g., FCPort) representing the failed port would see an operational status of Stopped.

167

168

169

170

171

172

173

174

175

176

177

178

179

180

181

182

183

Switch Example

5.4.6.2	Switch Example

The scenario presented is related to a FC Switch that contains many ports. See Figure 12. One of the ports is off-line.

184

Figure 12 - Switch Example

Element

StatisticalData

Product

ComputerSystem

Dedicated="switch"

FCPort

FCPortStatistics

FCPort

Element

StatisticalData

FCPortStatistics

Software

InstalledOn

System

Firmware

(

)

SoftwareIdentity

PhysicalPackage

ProductPhysical

Component

System

Device

ComputerSystem

Package

.

.

.

FCPortSettings

FCPortCapabilities

Element

Capabilities

Element

SettingData

FCPortSettings

FCPortCapabilities

FCSwitchSettings

EnabledLogical

ElementCapabilities

Element

SettingData

Element

Capabilities

Element

SettingData

Element

Capabilities

FCPortRate

Statistics

FCPortRate

Statistics

StatisticsCollection

MemberOf

Collection

185

186

187

188

189

190

191

192

5.4.6.2.1	Indication

An AlertIndication is produced by the switch notifying the HFM enabled client of the failure. The indication reports the Object Name of the FC port (FCPort) that has failed through its AlertingManagedElement property.

5.4.6.2.2	Standard Errors

A call to Port settings, port capabilities, or statistics cause an Error to be reported. The error reports the Object Name of the FCPort that has failed through the ErrorSource property.

#### Switch Example

193 It is mandatory to report error conditions through both AlertIndication and Error in those cases where 194 Error is returned when the method call failed for reasons other than the method call itself. For example, if

195 the device is over heat, then a method call can fail because of this condition. It is expected that the device will report an over heat AlertIndication to listening clients as well.

EXPERIMENTAL

5.4.6.2.3	Fault Region

The RelatedElementCausingError association defines the relationship between a CIM Instance that is reporting an error status and the component that is the cause of the reported status. The failed port would report error status and the RelatedElementCausingError association reports that the PortStatistics and PortSettings are affected. The switch itself would be thereby degraded.

The RelatedElementCausingError association is independent of all other associations. It is only use to report error associations and comes into existence only when necessary. Once the error has been handled, the association is removed from the model.

EXPERIMENTAL

# 1	6  Object Model General Information

## 2	6.1	Model Overview (Key Resources)

### 3	6.1.1	Overview

4	The SMI-S object model is based on the Common Information Model (CIM), developed by the DMTF. For 5	a more complete discussion of the full functionality of CIM and its modeling approach, see 6	 http://www.dmtf.org/standards/standard_cim.php.

7	Readers seeking a more complete understanding of the assumptions, standards and tools that assisted in 8	the creation of the SMI-S object model are encouraged to review the following: 9 • CIM Tutorial (http://www.wbemsolutions.com/tutorials/CIM/index.html)

- • CIM UML Diagrams and MOFs (http://www.dmtf.org/standards/standard_cim.php)

- Managed Object File (MOF) is a way to describe CIM object definitions in a textual form. A MOF can be 12 encoded in either Unicode of UTF-8. A MOF can be used as input into an MOF editor, parser or compiler 13 for use in an application.

14 The SMI-S model is divided into several profiles, each of which describes a particular class of SAN entity 15 (such as disk arrays or FibreChannel Switches). These profiles allow for differences in implementations 16 but provide a consistent approach for clients to discover and manage SAN resources. IN DMTF parlance, 17 a provider is the instrumentation logic for a profile. In many implementations, providers operate in the 18 context of a WBEM Server that is the infrastructure for a collection of providers. A WBEM client interacts 19 with one or more WBEM Servers.

### 20	6.1.2	Introduction to CIM UML Notation

- CIM diagrams use a subset of Unified Modeling Language (UML) notation.

PhysicalPackage

- Most classes are depicted in rectangles. The class name is in the upper part and properties 23 (also known as attributes or fields) are listed in the lower part. A third subdivision added for methods, if 24 they are included. A special type of class, called an association, is used to describe the relationship 25 between two or more CIM classes

26	Three types of lines connect classes, as shown in Figure 13.

Inheritence

Association

Aggregation

Figure 13 - Lines that Connect Classes

The CIM documents generally follow the convention of using blue arrows for inheritance, red lines for associations and green lines for aggregation. The color-coding makes large diagrams much easier to 27 read but is not a part of the UML standard.

28

The ends of some associations have numbers (cardinality) indicating the valid count of object instances.

29

Cardinality is expressed either as a single value (such as 1), or a range of values (0..1 or 1..4); “*” is shorthand for 0..n.

##### Scope

30 Some associations and aggregations are marked with a “W” at one end indicating that the identity of this 31 class depends on the class at the other end of the association. For example, fans may not have 32 worldwide unique identifiers; they are typically identified relative to a chassis.

33	This document uses two other UML conventions.

34

| DIsk1 |  |
| --- | --- |

35

36 The UML Package symbol  is used as a shortcut representing a group of classes that work together as an entity. For example, several classes model different aspects of a disk drive. After the initial explanation of these objects, a single disk package symbol is used to represent the entire group of objects. 37

38 Schema diagrams include all of a profile’s classes and associations; the class hierarchy is included and 39 each class is depicted one time in the schema diagram. Instance diagrams also contain classes and

40 associations but represent a particular configuration; multiple instances of an object may be depicted in an instance diagram. An instance may be named with an instance name followed by a colon and a class 41 name (underlined). For example,

42

| Array: ComputerSystem |  | Switch: ComputerSystem |
| --- | --- | --- |

43

44

45 represents an array and a switch – two instances of <COMPUTER SYSTEM> objects.

## 6.2	Techniques

### 46	6.2.1	CIM Fundamentals

- 6.2.1.1	Scope

This section provides a rudimentary introduction to some of the modeling techniques used in CIM, and is

- intended to speed understanding of the SMI-S object model.

49

50	6.2.1.2	Associations as Classes

CIM presents relationships between objects with specialized classes called associations and 51 aggregations. In addition to references to the related objects, the association or aggregations may also 52 contain domain-related properties. For example, ControlledBy associates a controller and a device. There 53 is a many-to-many cardinality between controllers and devices (i.e., a controller may control multiple 54 devices and multi-path devices connect to multiple controllers); each controller/device connection has a 55 separate activity state. This state corresponds to the AcccessState property of ControlledBy association 56 linking the device and the controller.

57

58	6.2.1.3	Logical and Physical Views

CIM separates physical and logical views of a system component, and represents them as different 59 objects – the “realizes” association ties these logical and physical objects together.

60

61	6.2.1.4	Identity

Different agents may each have information about the same organic object and may need to instantiate 62 different model objects representing the same thing. Access control is one example: a switch zone 63 defines which host device ports may access a device port. The switch agent creates partially populated 64 port objects that are also created by the HBA and storage system agents. The ConcreteIdentity 65 association is used to indicate the associated object instances are the same thing. ConcreteIdentity is 66 also used as a language-independent alternative to multiple inheritance. For example, a FibreChannel 67 port inherits from a generic port and also has properties of a SCSI controller. CIM models this as FCPort 68 and ProtocolController objects associated by ConcreteIdentity.

Extensibility

- 6.2.1.5	Extensibility

- CIM makes allowances for additional values in enumerations that were not specified in the class

Derivation by adding a property to hold arbitrary additional values for an enumeration. This property is 71 usually named OtherXXXX (where XXXX is the name of the enumeration property) and specifying “other” 72 as the value in the enumeration property indicates its use. For an example see the ConnectorType and 73 OtherTypeDescription properties of Slot object in the CIM_Physical MOF.

74

- 6.2.1.6	Value/ValueMap Arrays

- CIM uses a pair of arrays to represent enumerated types. ValueMap is an array of integers; Values is an array of strings that map to the equivalent entry in ValueMap. For example, PrinterStatus (in the 77 CIM_Device MOF) is defined as follows:

78

- ValueMap {“1”, “2”, “3”, “4”, “5”, “6”, “7”},

- Values {“Other”, “Unknown”, “Idle”, “Printing”, “Warm-up,

“Stopped Printing”, “Offline”},

81

A status value of 6 means “Stopped Printing”. A client application can automatically convert the integer 82 status value to a human-readable message using this information from the MOF. 83

- 6.2.1.7	Return Codes

- When a class definition includes a method, the MOF includes Value/ValueMap arrays representing the possible return codes. These values are partitioned into ranges of values; values from 0 to 0x1000 are 86 used for return codes that may be common to various methods. Interoperable values that are specific to a 87 method start at 0x1001; and vendor-specific values may be defined starting at 0x8000. Here’s an 88 example of return codes for starting a storage volume.

89

- ValueMap {“0”, “1”, “2”, “4”, “5”, “.”, “0x1000”,

- “0x1001”, “…”, “0x8000..”},

Values {“Success”, “Not Supported”, “Unknown”, “Time-out,

92

“Failed”, “Invalid Parameter”, “DMTF_Reserved”,

93

“Method parameters checked - job started”,

94

“Size not supported”,

95

“Method_Reserved”, "Vendor_Specific”}]

96

- 6.2.1.8	Model Conventions

- This is a summary of objects and associations that are common to multiple profiles.

- PhysicalPackage represents the physical storage product. PhysicalPackage may be sub-classed to 100	ChangerDevice, but PhysicalPackage accommodates products deployed in multiple chassis.

- Product models asset information including vendor and product names. Product is associated with

- PhysicalPackage.

- SoftwareIdentity models firmware and optional software packages. InstalledSoftwareIdentity associates

- SoftwareIdentity and ComputerSystem, ElementSoftwareIdentity associates SoftwareIdentity and LogicalDevices (a superclass of devices and ports).

105

- Service models a configuration interface (for example, a switch zoning service or an array access control

- service). Services typically have methods and properties describing the capabilities of the service. A storage system may have multiple services; for example, an array may have separate services for LUN 108	Masking and LUN creation. A client can test for the existence of a named service to see if the agent is 109	providing this capability.

110

- LogicalDevice (for example, FCPort) is a superclass with device subclasses (like and DiskDrive and

- TapeDrive) and also intermediate nodes like Controller and FCPort. Each LogicalDevice subclass shall Model Conventions

- be associated to a ComputerSystem with a SystemDevice aggregation. Due to the large number of 114	LogicalDevice subclasses, SystemDevice aggregations are often omitted in instance diagrams in this

115	specification.

116

117 This specification covers many common storage models and management interfaces, but some implementations include other objects and associations not detailed in the specification. In some cases, 118 these are modeled by CIM schema elements not covered by this document. When vendor-specific 119 capabilities are needed, they should be modeled in subclasses of CIM objects. These subclasses may 120 contain vendor-specific properties and methods and vendor-specific associations to other classes. 121

### 122	6.2.2	Modeling Profiles

In addition to modeling SAN components, SMI-S servers shall model the profiles they provide. This 123 information is used two ways:

124

- • Clients can quickly determine which profiles are available.

- • An SLP component can query the SMI-S Server and automatically determine the appropriate SLP Service Template information (see Clause 9 Service Discovery).

127

128 A client can traverse the Server Profile in each SMI-S server to see which profiles (and objects) claim SMI-S compliance. RegisteredProfile describes the profiles that a WBEM server claims are supported. 129 The RegisteredSubprofile is used to define the optional features supported by the system being modeled.

130 A client can traverse the associations in the Server Profile to see which profiles claim SMI-S compliance. 131

### 132	6.2.3	CIM Naming

There may be multiple SMI-S servers in any given storage network environment. It is not sufficient to

133 think of the name of an object as just the combination of its key values. The name also serves to identify 134 the Server that is responsible for the object. The name of an object (instance) consists of the namespace 135 and the model. The namespace provides access to a specific SMI-S server implementation and is used to 136 locate a particular namespace within a server. The model provides full navigation within the CIM Schema 137 and is the concatenation of the class name and key-qualified properties and values.

138

139 The namespace has special rules. It should uniquely identify a SMI-S server. However, a SMI-S server may support multiple namespaces. How an implementation defines Namespaces within a SMI-S server is 140 not restricted. However, to ease interoperability, SMI-S implementations should manage all objects within 141 a profile in one namespace.

142

143

# 1	7  Correlatable and Durable Names

## 2	7.1	Overview

3 Management applications often read and write information about managed objects in multiple CIM 4 namespaces or between CIM and some other storage management namespace. When an object in one 5 namespace is associated with an object in another namespace, each namespace may represent some 6 amount of information about the same managed resource using different objects. A management 7 application understands when objects in different namespaces represent the same managed resource by 8 the use of a unique common identifier, referred to as a “correlatable name”. A correlatable name is 9 designated as a mandatory property for any objects representing managed resources that may be seen 10 from multiple points of view. These durable names are used by management applications for object 11 coordination.

12 A related concept is referred to as “durability”. Some names may be correlatable at a particular point in 13 time, but may change over time (e.g., a durable name is a hardware-assigned port or volume name and a 14 correlatable, non-durable ID is a DHCP IP address). No name is permanently durable (e.g., even a name 15 derived from hardware may change due to FRU replacement). A client application should assume that a 16 stored durable name remains valid over time where a non-durable may not remain valid over time.

17 Correlatable names are unique within a defined namespace. In some cases, that namespace is world18 wide; requiring compliance to standards defined by a naming authority. In other cases, the namespace is 19 the hosting system or some set of connected systems (e.g., operating system device names are unique to 20 the containing host).

21 A name may be expressed in different formats (e.g., numeric value are sometimes displayed as decimal 22 or hexadecimal, the hexadecimal value sometimes has a leading “0x” or a trailing “h”). To assure 23 interoperability, mandatory formats are specified by this standard.

24 A necessary technique associated with correlatable names involves the use of CIM properties that 25 describe the format or namespace from which the name is derived. CIM key-value combinations are 26 unique across instances of a class, but CIM does not fully address cases where different types of 27 identifiers are possible on different instances of an object. It is therefore necessary to ensure that multiple 28 sources of information about managed resources use the same approach for forming correlatable names 29 whenever different types of identifiers are possible.

30 When different types of identifiers are possible, the profile specifies the possible name formats and 31 namespaces for durable and correlatable IDS, the preferred order that each implementation should use if

32 multiple namespaces are available, and the related properties that a client uses to determine the 33 namespace.

- Correlatable, durable names are mandatory for these objects:

- • SCSI logical units or (such as storage volumes or tape drives) that are exported from storage systems; also 36 SB (Single Byte Command Code Sets)

- • SB control unit issues

- • External Ports on hosts and storage devices

- • Fibre Channel ports on interconnect elements

- • Fibre Channel fabric (modeled as AdminDomain)

- • ComputerSystem objects that server as top-level systems for all SMI-S profiles

- • Operating System Device Names

- CIM keys and correlatable names are not tightly coupled. For some classes, they may be the same, but 44 this is not mandatory as long as all correlatable names are unique and management applications are able 45 to determine when objects in different namespaces are providing information about the same managed 46 resource.

47 The common types of information used for names include the SCSI Device Identifiers from the 48 Identification Vital Product Data page (i.e., VPD page 83h), SB Node Element Descriptors from Read49 Configuration Data, the response from ATA IDENTIFY commands, Fibre Channel Name_Identifiers (i.e., 50 World Wide Names), Fully Qualified Domain Names, and IP Address information. See 7.2, 7.3, 7.4, and 51 7.5 for general information on the advantages and disadvantages of certain types of names. The details 52 for each class requiring durable correlatable names are provided in the profiles subclauses of this 53 document.

54 If the name used in the instrumentation in binary, the CIM representation is an upper case hexadecimal55 encoded representation of the value returned. For example, decimal 27 is hexadecimal 1b and will be 56 represented by the string “1B”. Note that each binary byte requires two ASCII characters using this 57 representation. If the name used in the instrumentation is ASCII text, the case of the characters is 58 preserved in the CIM property.

## 59	7.2	Guidelines for SCSI Logical Unit Names

- The preferred logical unit identifier is returned from a SCSI INQUIRY command in VPD page 83h.

- NOTE  Legacy systems may lack correlatable names as SCSI standards prior to SAM-3 and SPC-3 did not clearly define 62 logical unit names, however this has been clarified to be logical unit names and recent systems have converged in compliance.

63 The Unit Serial Number VPD page (i.e., SCSI Inquiry VPD Page 80h) returns a serial number, but the 64 SPC-3 standard allows this either be a serial number for a single logical unit or a serial number of the 65 target device. There's no mechanism to discover which approach the device is using. If a client is not 66 coded to understand which products provide per-logical unit or per-target serial numbers, then it should 67 not use the Unit Serial Number VPD page as a logical unit name.

68 The Identification Vital Product Data page (i.e., VPD page 83h) returns a list of identifiers with metadata 69 describing each identifier. The metadata includes:

- • Code Set (binary versus ASCII)

- • Association (indicates the SCSI object to which the identifier applies, e.g., for a logical unit, port, or target

- device)

- • Type (the naming authority for identifiers of the structure of information about target ports)

- • Protocol Identifier (indicates the SCSI transport protocol to which the identifier applies)

- To identify a logical unit name the Association shall be set to zero. The preferred Types for logical units 76 are 3 (NAA), 2 (EUI), and 8 (SCSI Name). However type 1 (T10) is allowed. If the code set in the inquiry 77 response indicates the identifier is binary, the CIM representation is hexadecimal-encoded.

## 78	7.3	Guidelines for FC-SB-2 Device Names

79 FC-SB-2 devices and control unit images use the node-element descriptor (NED) name format. NEDs are 80 retrieved within a configuration record retrieved by the READ-CONFIGURATION DATA command. A 81 configuration record contains information describes the internal configuration of the device, where the 82 information retrieved describes the corresponding node elements that are accessed when an I/O 83 operation is performed.

84

85

- NEDs are 32 bytes and contain these fields:

- • 4 bytes (flags, type, class, reserved) - binary

- • 6 byte "type number" - string

- • 3 byte "model number" - string

- • 3 byte "manufacturer" - string

- • 2 byte "plant of manufacture"- string

- • 12 byte sequence number" - string

93

• 2 byte tag - binary

94

95 The I/O-Device NED is used for identifying devices. The Token NED is used for identifying control-unit images.

The Name property for LogicalDevices representing SB devices is world-wide unique value formed by 96

composing these fields.

## 7.4	Guidelines for Port Names

97	The following is a list of optimal names for ports based on the transport type:

- Fibre Channel ports use Port World Wide Names (i.e., FC Name_Identifier)

98

- iSCSI has three types of ports

99

• the combination of IP address and TCP port number serve as the primary correlatable name for iSCSI target

- ports. Note that this information is stored in two separate properties and hence there is no single correlatable name.

- • the logical element (iSCSIProrotolEndpoint) that represents the SCSI port The SCSI logical port shall be

- named with an iSCSI name. 103

- • the underlying physical ports (typically Ethernet ports). Ethernet ports names shall use the MAC address.

- 3)	Parallel SCSI (SPI) and ATA ports typically do not have names, they are identified by a bus-relative

- address typically set with jumpers. In configurations where these drives are not shared by multiple hosts, the host-relative name acts as the name.

- 4) CIM port classes do not include NameFormat; the appropriate format is determined by the transport 108 implied by the port subclass. 109

- SCSIProtocolEndpoint represents SCSI protocol running through a port. In many cases, there is one-toone mapping between SCSIProtocolEndpoint and some subclass of LogicalPort and the name

- requirements are identical. For iSCSI, there may be multiple Ethernet ports per SCSIProtocolEndpoint

112

instance. The IP address and TCP port number are modeled in IPProtocolEndpoint and 113 TCPProtocolEndpoint. iSCSIProtocolEndpoint Name holds the iSCSI initiator or target name.

114

SBProtocolEndpoint represents SB protocol running through a port. In many cases, there is a one to-one

115 mapping between SBProtocolEndpoint and some subclass of LogicalPort and the name requirements are 116 identical.

117

## 118	7.5	Guidelines for Storage System Names

119 Each profile has a ComputerSystem or AdminDomain instance that represents the entire system. There 120 are a variety of standard and proprietary names used to name storage systems. Unlike SCSI logical units 121	and ports, there is no particular name format in common use. There are advantages and disadvantages to

certain types of names. 122

- IP addresses have an advantage in human recognition; (e.g., administrators are accustomed to referring

- to systems by their IP addresses). The downsides are that IP addresses are not necessarily durable (e.g.,

- DHCP) are not necessarily system-wide (e.g., some storage systems have multiple network interfaces), and are not necessarily unique (e.g., NAT allows the same IP address to be used in multiple network 126 zones). 127

128 Full Qualified Domain Names are friendlier than IP addresses and may fix the durability issue of IP 129 addresses (e.g., a host name may be constant even when the IP address changes). But storage systems

130 do not necessarily have access to their network names. Network names are typically handled through a central service such as DNS. When a client application opens a connection to a remote system, it asks 131 the local system to resolve the name to an IP address, the local system redirects the request to the DNS 132 server, the IP address is returned and the client application opens the connection. If the remote system is 133 the storage system, this sequence requires the DNS server to know about the storage system, but not 134 vice-versa. A storage system is only required to know about DNS if software on the storage system acts

- as a network client using host names. And, like IP addresses, a storage system may have several

- network interfaces with different FQDNs. 137

138 Transport-specific names are specific to a particular storage transport (e.g., Fibre Channel or iSCSI). 139 There are some good standard names (e.g., FC platform names or iSCSI Network Entity names). The

- disadvantage of transport-specific names is that they are not able to be consistently used on storage systems supporting multiple transports or in configurations with transport bridges (e.g., a client may have

- no mechanism to issue FC commands to an FC device behind an FC/iSCSI bridge).

142

143 SCSI target names solve the transport-specific issue. Before the SAM-3 and SPC-3 standards there was 144 not a standard SCSI system name, however with SPC-3, the Identification Vital Product Data page

- association value 2 was defined for a target name. At this time, the SPC-3 standard is too new to be in common use. Most storage systems include some vendor-specific way to get a target name, but client is

- not able to use these names without specific knowledge of the vendor-specific interface.

147

148 At this time, no single storage system name format is in common use. The best approach is for 149 implementations to expose several names, along with information that tells the client how to interpret the

- name. The OtherIdentifyingInfo and IdentifyingDescriptions array properties of ComputerSystem provide the list of names and interpretations. However, IdentifyingDescriptions is not an enumerated type; and as

- a result, any string is valid from a CIM perspective.

152

## 153	7.6	Standard Formats for Correlatable Names

154

### 155	7.6.1	General

- Correlatable names shall be used and formatted consistently. Storage volume names are more complex that other element names (i.e., the same format may be used in different namespaces). For example

- several common INQUIRY Vital Product Data page names use the IEEE NAA format and as a result a

client is not able to correlate names from different namespaces.

158

159

160

161

### 162	7.6.2	Standard Formats for Logical Unit Names

163 For disks and arrays, multiple name formats are in common use. Table 2 specifies standard formats for 164 storage volume names.

Table 2 - Standard Formats for StorageVolume Names

| Description | Format property and value(valuemap) | Format of Name |
| --- | --- | --- |
| SCSI VPD page 83 type 3, Association 0, NAA 0101b | NameFormat = NAA(9), NameNamespace = VPD83Type3(1) | NAA name with first nibble of 5. Recommended format (8 bytes long) when the ID is directly associated with a hardware component. Formatted as 16 un-separated upper case hex digits (e.g., '21000020372D3C73') |
| SCSI VPD page 83, type 3h, Association=0, NAA 0110b | NameFormat = NAA(9), NameNamespace= VPD83Type3(1) | NAA name with first nibble of 6. Recommended format (16 bytes long) when IDs are generated dynamically. Formatted as 32 unseparated upper case hex digits. |
| SCSI VPD page 83, type 3h, Association=0, NAA 0010b | NameFormat = NAA(9), NameNamespace = VPD83Type3(1) | NAA name with first nibble of 2. Formatted as 16 un-separated upper case hex digits |
| SCSI VPD page 83, type 3h, Association=0, NAA 0001b | NameFormat = NAA(9), NameNamespace = VPD83Type3(2) | NAA name with first nibble of 1. Formatted as 16 un-separated upper case hex digits |
| SCSI VPD page 83, type 2h, Association=0 | NameFormat = EUI64(10), NameNamespace = VPD83Type2(3) | Formatted as 16, 24, or 32 un-separated upper case hex digits |
| SCSI VPD page 83, type 1h, Association=0 | NameFormat = T10VID(11), NameNamespace = VPD83Type1(4) | Formatted as 1 to 252 bytes of ASCII. |
| SCSI VPD page 80, serial number | NameFormat = Other(1), NameNamespace = VPD80(5) | Only if serial number refers to logical units rather than the enclosure. 1-252 ASCII characters |
| SB I/O Device NED | NameFormat=SBDevice(13), NameNamespace=SB | 64 un-separated upper case hex digits. The tag subfield contains CU_image+device_address |
| SB Token NED | NameFormat=SBToken(14), NameNamespace=SB | 64 un-separated upper case hex digits. The tag sub-field contains the CU_image |
| SCSI Concatenation of Vendor,Product, SerialNumber | NameFormat = SNVM(7), NameNamespace = SNVM(7) | A concatenation of three strings representing the vendor name, product name within the vendor namespace, and serial number within the model namespace. These strings come from SCSI standard INQUIRY response data. Strings are delimited with a ‘+’ and spaces are included. Vendor and Product are fixed length: Vendor ID is 8 bytes, Product is 16 bytes. SerialNumber is variable length and may be up to 252 bytes in length. If one of these fields contains a plus sign, it shall be escaped with a backslash ('\+'). The concatenation is done to provide world-wide uniqueness; clients should not parse this name. |

Table 2 - Standard Formats for StorageVolume Names

| Description | Format property and value(valuemap) | Format of Name |
| --- | --- | --- |
| ATA Concatenation of, Model, SerialNumber | NameFormat=ATA, NameNamespace=ATA | A concatenation of three strings representing the vendor and model names and serial number within the model namespace. The manufacturer name is not based on a specific standard. The model name and serial number strings come from ATA IDENTIFY DEVICE response data. Strings are delimited with a ‘+’ and spaces are included. The vendor is 20 characters, model is 40 characters, and serial number is 20 characters. If one of these fields contains a plus sign, it shall be escaped with a backslash ('\+'). The concatenation is done to provide uniqueness; clients should not parse this name. Note that ATA standards do not require any interface to return a manufacturer ID; many implementations put a manufacturer name in the model string. |
| FC Node WWN | NameFormat = NodeWWN(8) NameNamespace = NodeWWN(6) | 16 un-separated upper case hex digits (e.g., '21000020372D3C73') |

165

Storage volumes may have multiple standard names. A page 83 logical unit identifier shall be placed in the Name property with NameFormat and Namespace set as specified in Table 2. Each additional name 166 should be placed in an element of OtherIdentifyingInfo and the corresponding element in 167 IdentifyingDescriptions shall contain a string from the Values lists from NameFormat and

- NameNamespace, separated by a semi-colon. For example, an identifier from SCSI VPD page 83 with type 3, association 0, and NAA 0101b - the corresponding entry in IdentifyingDescriptions[] shall be “NAA;VPD83Type3”.

- For other types of devices, the logical unit name shall be in the Name property; NameFormat and NameNamespace are not valid properties of these other device classes.

### 170	7.6.3	Standard Formats for Port Names

171

172	Table 3 specifies standard formats for port names. 173

Table 3 - Standard Formats for Port Names

174

| An IP interface's MAC | Network Port Permanent Address property; no corresponding format property | Six upper case hex bytes, bytes are delimited by colons ':' |
| --- | --- | --- |
| World Wide Name (i.e., FC Name_Identifier) | FCPort Permanent Address property; no corresponding format property | 16 un-separated upper case hex digits (e.g., '21000020372D3C73') |
|  | ProtocolEndpoint Name property; ConnectionType = 2 (Fibre Channel) | 16 un-separated upper case hex digits (e.g., '21000020372D3C73') |
| Parallel SCSI Name | SPI Port Name property; no corresponding format property | String - platform-specific name representing the name. Note that this name is only correlatable relative to the system containing the port. |
|  | SCSIProtocolEndpoint Name property; ConnectionType = 3 (Parallel SCSI) | String - platform-specific name representing the name. |

175

176

177

178

179	Table 3 - Standard Formats for Port Names

| An IP interface's MAC | Network Port Permanent Address property; no corresponding format property | Six upper case hex bytes, bytes are delimited by colons ':' |
| --- | --- | --- |
| iSCSI Port Name | iSCSIProtocolEndpoint Name | < iSCSI node name > + ’ i, ’ + ISID for initiators, < iSCSI node name > + ’ t, ’ + TPGT for target ports, where < iSCSI node name> may be any of the standard iSCSI name namespaces (e.g., iqn, eui); and includes the namespace prefix. |
| SAS Port Names | SASPort Name property; no corresponding format property | SAS Address, 16 un-separated upper case hex digits |
|  | SCSIProtocolEndpoint Name property; ConnectionType = 8 (SAS) | SAS Address, 16 un-separated upper case hex digits |
| ATA Port Name | ATAPort or SASSATAPort Name property; no corresponding format property | String - platform-specific name representing the name. Note that this name is only correlatable relative to the system containing the port. |
|  | ATAProtocolEndpoint Nameproperty | String - platform-specific name representing the name. |

180

Note that iSCSI Network Portals do not have a single correlatable name. The combination of IPProtocolEndpoint IPv4Address or IPv6Address and TCPProtocolEndpoint PortNumber uniquely identifies the network portal, but since these are two properties, they do not form a correlatable name.

### 7.6.4	Standard Formats for Fabric Names

A fabric is modeled as AdminDomain. AdminDomain.Name shall hold the fabric name (i.e., WWN) and AdminDomain.NameFormat shall be set to “WWN”. AdminDomain.Name shall be formatted as 16 unseparated upper case hex digits.

### 7.6.5	Standard Formats for Storage System Names

Due to the limited list of possible formats, the Name property is not considered an essential identifier for SMI-S. SMI-S clients should use OtherIdenfyiingInfo property as described in Table 4.

Providers shall supply at least one Durable or Correlatable Name as an element in the

IdentifyingDescriptions[] array. The corresponding array elements of OtherIdentifyingInfo[] shall include a 181 value from Table 4 for all elements of IdentifyingDescriptions[]. The elements in the 182 IdentifyingDescriptions array are strings and may contain white space between words. Whenever white-

- space appears, it shall consist of a single blank; other white-space characters and multiple consecutive blanks shall not be used.

- At least one of the values in IdentifyingDescriptions[] shall be something other than “SCSI Vendor Specific Name” or “Other Vendor Specific Name”.

- OtherIdentifyingInfo[0] should be assigned the most preferable name by the instrumentation.

- In all cases, if the name is returned to the instrumentation in binary, the corresponding entry in 187 OtherIdentifyingInfo holds an upper-case hexadecimal-encoded representation of the value returned. 188 Standard names defined in binary are called out in Table 4.

- Other ComputerSystem properties should be set as follows:

- Name is a CIM key and shall be unique for ComputerSystem instances within the CIM namespace. SMI-S 191 clients should not assume Name is either durable or correlatable.

192 NameFormat is an enumerated type describing the Name property. Only a few of the defined values are 193 appropriate for storage systems. Use “IP” if Name is derived from an IP address of Fully Qualified Domain 194 Name. Use “HID” if Name is derived from a hardware ID. Use “OID” if Name is a unique ID determined by 195 some unique ID generating logic. 196

197 ElementName is a friendly name; SMI-S clients should not assume that ElementName is unique, correlatable, or durable since a customer may provide the same info for multiple systems.

198

199	Table 4 - Standard Formats for Storage System Names

| IdentifyingDescriptions [x] value | Description |  | Format of OtherIdentifyinginfo[x] |
| --- | --- | --- | --- |
| T10 Target Name Type 1 | An identifier from a Identification Vital Product Data page response with Association equal to 2 | Type 1 (T10) | 1 to 252 bytes of ASCII |
| T10 Target Name Type 2 |  | Type 2 (EUI) | 16, 24, or 32 un-separated upper case hex digits (e.g., '21000020372D3C73') |
| T10 Target Name Type 3 |  | Type 3 (NAA) | 16 or 32 un-separated upper case hex digits (e.g., '21000020372D3C73') |
| T10 Target Name Type 8 |  | Type 8 (SCSI Names) | iSCSI Names (see 7.8) |
| T11 FC-GS-4 Platform Name | A platform name as defined in T11 FC-GS-4 standard |  | Up to 508 hex digits (254 bytes) as specified by T11 FC-GS-4 subclause on Platform Name. Format as unseparated as hex digits. Platform Name Format Byte shall be included. |
| T11 RNID Name | The sixteen byte Vendor Unique name from the General Topology Discovery format RNID response as defined in T11 FC LS standard. This name format should only be used if the storage system supports RNID General Topology Discovery and provides a meaning system identifier in the Vendor Unique field. |  | 32 unseparated hex digits. |
| iSCSI Network Entity Name | An iSCSI Network Entity name. |  | iSCSI Names (see 7.8) |
| Ipv4 Address | An IP V4 name |  | Four decimal bytes delimited with dots ('.') |
| Ipv6 Address | An IP V6 name |  | ‘x:x:x:x:x:x:x:x’, where the 'x's are the uppercase hexadecimal values of the eight 16-bit pieces of the address. Examples: ‘FEDC:BA98:7654:3210:FEDC: BA98:7654:3210’, ‘1080:0:0:0:8:800:200C:417A’ Leading zeros in individual fields should not be included and there shall be at least one numeral in every field. (This format is compliant with RFC 4291.) In addition, omitting groups of zeros or using dotted decimal format for an embedded IPv4 address is prohibited. |
| Fully Qualified Domain Name | A fully qualified domain name. |  | A legal DNS name (fully qualified) consisting of strings delimited by periods. |

200

201

202

203

204

205

206

207

208

209

210

211

212

Table 4 - Standard Formats for Storage System Names (Continued)

| IdentifyingDescriptions [x] value | Description |  | Format of OtherIdentifyinginfo[x] |
| --- | --- | --- | --- |
| Node WWN | The Fibre Channel Node WWN. The provider shall assure that the same Node WWN shall be available through all FC ports within a target device. |  | 16 un-separated upper case hex digits (e.g., '21000020372D3C73') |
| T10 Unit Serial Number VPD page | SCSI Inquiry VPD page 80 response is a serial number This name may be unique for a specific logical unit or for the target (e.g., storage system). These names are only valid if the instrumentation is certain that all logical units in a system return the same value. Since there is no mechanism to test whether the value is unique per target or per logical unit, this value is not interoperably correlatable and should not be used |  | 1-252 ASCII characters |
| SCSI Vendor Specific Name | This is a name accessible through a vendor-specific SCSI command | A client with a priori knowledge may be able to correlate this based on vendor and Product IDs. | unknown |
| Other Vendor Specific Name | This is a name accessible through some non-SCSI vendor-specific interface. |  | unknown |

### 7.6.6	Operating System Device Names

Each operating system has different conventions for naming devices. Many operating systems provide multiple names for the same device instance. In this version of the specification, operating system device name formats are recommended.

The case of names specified by operating system interfaces shall be preserved.

Operating system device names are unique within the namespace of the scoping system and are not unique between systems.

Table 5 specifies the format for names of tape devices.

Table 5 - Standard Operating System Names for Tape Devices

| Operating System | Format | Notes |
| --- | --- | --- |
| AIX | /dev/rmtX | X represents a hexadecimal number and may be more than one character |
| HP-UX | /dev/rmn/Xm | X represents a hexadecimal number and may be more than one character |
| Linux | /dev/stX | X represents one or two lower case alphabetic characters |
| Solaris | /dev/rmt/Xn | X represents a hexadecimal number and may be more than one character |
| WIndows | \\.\\TAPEX | X represents a decimal number |

213

214

215

216

217

218

219

Some operating systems treat disk partitions as virtual devices; applications operate on partitions as if they were disks. The model requires two classes for each partition, LogicalDisk and GenericDiskPartition. Other operating systems allow applications to operate on the entire disk without partitions. Linux allows both.

- Table 6 specifies the format for LogicalDisk.Name of disk partitions

Table 6 - LogicalDisk.Name for disk partitions

| Operating System | Format | Notes |
| --- | --- | --- |
| Linux | dev/sdXY or /dev/hdXY | where X represents one or two lower case alphabetic characters and Y represents an integer between 1 and 15 |
| Solaris | /dev/dsk/cXtXdXsX | X represents one or two lower case alphabetic characters |
| WIndows | C: or the file name of mount point | C represents an uppercase letter |
| zSeries | CC:SS:DDDD or CC:DDDD | CC represents a Channel Subsystem Identifier, SS is a subchannel set (within the channel subsystem), and DDDD is the device number. SS is optional for subchannel set zero. |

Table 7 specifies the format for GernericDiskParition.Name and DeviceId properties for disk partitions

- Table 7 - GenericDiskParittion.Name for disk partitions

222

| Operating System | Format | Notes |
| --- | --- | --- |
| Linux | sdXY or hdXY | X represents one or two lower case alphabetic characters |
| Solaris | /dev/dsk/cXtXdXsX | where X represents one or two lower case alphabetic characters and Y represents an integer between 1 and 15 |
| WIndows | Disk #X, Partition #X | X represents a decimal digit |

223

224

225

Table 8 specifies the format for LogicalDisk.Name for unpartitioned disks.

Table 8 - Standard Operating System Names for Unpartitioned DIsks

| Operating System | Format | Notes |
| --- | --- | --- |
| AIX | /dev/hdiskX | X represents a hexadecimal number and may be more than one character |
| HP-UX | /dev/dsk/cXtYdZ | X, Y, and Z represents hexadecimal number and may be more than one character in length |
| Linux | /dev/sdX or /dev/hdX | X represents one or two lower case alphabetic characters |
| Windows | \.\PHYSICALDRIVEx | x represents a a decimal number and may be more than one character |

226

### 7.6.7	Case Sensitivity

Names and NameFormats are case sensitive and the cases provided in Table 8 shall be used If not otherwise specified, uppercase should be used.

## 7.7	Testing Equality of correlatable Names

The implementation shall only compare objects of the same class or parent class. For objects that do not require the use of additional properties, a simple direct comparison is sufficient, providing the format for the mandatory correlatable name as identified in this section or the specific profile is adhered to.

For objects that do require the use of additional properties (e.g., NameFormat), the correlatable names of objects representing the same entity should compare positively, negatively, or indicate clearly when a comparison is ambiguous:

227 • If the two objects have the same NameFormat and Name, then they refer to the same resource.

- If the two objects have the same NameFormat and different Names, then they refer to different resources.

- If the two objects have different NameFormats, whether the Names are the same or different, then it is unknown whether they refer to the same resource.

This reduces the possibility that a match is missed by a string equals comparison simply because of an incompatibility of formats rather than non-equality of the data.

## 7.8	iSCSI Names

The iSCSI standards define three text formats for names that apply to various iSCSI elements. The three formats are: iSCSI qualified name (iqn), IEEE Extended Unique Identifier (eui), and ANSI T10 NAA. The format is included in the name as a three-letter prefix. The three formats are explained in more detail.

The iSCSI qualified name (iqn) format is defined in [iSCSI] and contains (in order):

228	1)	1 - The string “iqn.” 229

2)	2 - A date code specifying the year and month in which the organization registered the domain or

- sub-domain name used as the naming authority string.

- 3)	3 - The organizational naming authority string, which consists of a valid, reversed domain or subdo-

- main name.

233

234 Optionally, a ':', followed by a string of the assigning organization's choosing, which shall make each assigned iSCSI name unique. 235

236 Figure 14 contains examples of iSCSI-qualified names that may be generated by “EXAMPLE Storage, 237 Inc.”

Organizational Subgroup Naming Authority Naming and/or string Defined by Type Date Auth Org. or Local Naming Authority +--++-----+ +---------+ +--------------------------------+ ¦ ¦¦ ¦ ¦ ¦ ¦ ¦ iqn.2001-04.com.example:diskarrays-sn-a8675309 iqn.2001-04.com.example iqn.2001-04.com.example:storage.tape1.sys1.xyz iqn.2001-04.com.example:storage.disk2.sys1.xyz

238

239

240

241

242

243

244

245

246	Figure 14 - iSCSI Qualified Names (iqn) Examples

247

248 The IEEE Registration Authority provides a service for assigning globally unique identifiers [EUI]. The EUI-64 format is used to build a global identifier in other network protocols.

249

The format is "eui." followed by an EUI-64 identifier. Figure 15 contains an example.

250

Type EUI-64 identifier (ASCII-encoded hexadecimal) +--++--------------+ ¦ ¦¦ ¦ eui.02004567A425678D

251

252

253

254

- Figure 15 - iSCSI EUI Name Example

- Type "naa." - Network Address Authority

257

The ANSI T10 FC-FS standard defines a format for constructing globally unique identifiers [FC-FS] referred to as an Network Address Authority (NAA) format. The iSCSI name format is "naa." followed by an NAA identifier (ASCII-encoded hexadecimal digits).

Figure 16 contains an example of an iSCSI name with a 64-bit NAA value: type NAA identifier (ASCIIencoded hexadecimal).

+--++--------------+          |  ||              |          naa.52004567BA64678D

Figure 16 - iSCSI 64-bit NAA Name Example

Figure 17 contains an example of an iSCSI name with a 128-bit NAA value: type NAA identifier (ASCIIencoded hexadecimal).

258

+--++------------------------------+ ¦ ¦¦ ¦ naa.62004567BA64678D0123456789ABCDEF

259

260

Figure 17 - iSCSI 128-bit NAA Name Example

iSCSI names are composed only of displayable characters. iSCSI names allow the use of international character sets but are not case sensitive. No whitespace characters are used in iSCSI names.

261

262

263

264

265

266

267

268

Message: Redundancy

# 1	8  Standard Messages

## 2	8.1	Overview

3 Standard Messages provide an interoperable design for event reporting and are documented in DMTF 4 DSP2011, Standard Messages Whitepaper. SMI-S profiles reference standard messages for alert 5 indications and error reporting.

6

## 7	8.2	Registries for Standard Messages

- Message registries are machine-readable lists of standard messages specified in DMTF DSP0228 1.1.0.

- Instances of CIM_AlertIndication or CIM_Error classes in implementations of this standard should 10	conform to the provisions specified by the referenced DMTF message registries, or 8.3 "SNIA Standard 11	Messages". Implementations of this standard may utilize the machine-readable capability of message 12	registries, but may use other techniques to assure message payload conforms to provisions of message 13	registries.

14 CIM_AlertIndication instances referenced in SMI-S profile include OwningEntity and MessageID 15 properties. If OwningEntity is DMTF and the MessageID starts with WIPG, see DSP8016 WBEM 16 Operations Message Registry 1.1. If OwningEntity is DMTF and the MessageID starts with DIAG, see 17 DSP8055 Diagnostics Message Registry. SNIA standard messages are documented in 8.3 "SNIA 18 Standard Messages".

## 19	8.3	SNIA Standard Messages

20

### 8.3.1	Common Profile-related Messages

21

22	8.3.1.1	Message: Redundancy

23

Owning Entity: SNIA

- Message ID: Core1

- Message Format String: <Device Type> <Device Unique Identifier> had redundancy failure for

- <Component Type> at <Component Location Or Identifier>

27

28

A message indicating a redundancy failure in a set of redundant components in a device. Table 9 29	describes the message arguments.

30

Table 9 - Redundancy Message Arguments

31

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | HBA |
|  |  |  | Array |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| Component Type | string | The type of component a redundancy failure has occurred for. Typically the string would contain one of the following: "Power Component", "Fan Component", "Board Component", Cross Bar", "System Clock' | Power Component |
|  |  |  | Fan Component |
|  |  |  | Board Component |
|  |  |  | Cross Bar |

32

33

34

35

36

37

38

39

40

41

42

43

44

##### Message: Environmental

45	Table 9 - Redundancy Message Arguments

46

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
|  |  |  | System Clock |
|  |  |  | Communications Port |
| Component Location Or Identifier | string | Location or identifier of the component |  |

47

48

49

50

51

52

53	Table 10 describes the alerts that are associated with this message.

54

55	Table 10 - Redundancy Alert Information

56

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system containing the device. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 4 | Minor |

57

58

59

60

61

62

- 8.3.1.2	Message: Environmental

- Owning Entity: SNIA

- Message ID: Core2

Message Format String: <Device Type> <Device Unique Identifier> had an environmental problem of type

66

<SensorType> of <Environmental Issue> <Sensor Location Or Identifier>

67 68	A message indicating a environmental issue with a device. Table 11 describes the message arguments.

69

70	Table 11 - Environmental Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | HBA |
|  |  |  | Array |
|  |  |  | Tape Library |
|  |  |  | Drive |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| SensorType | string | The sensor type. | temperature |
|  |  |  | humidity |
| Environmental Issue | string | The environmental issue. |  |
| Sensor Location Or Identifier | string | Location or identifier of the Sensor |  |

71

Message: FRU Operation

72	Table 12 describes the alerts that are associated with this message.

73

74	Table 12 - Environmental Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  |  |
| ALERT_TYPE | Y |  | Environmental Alert |
| PERCEIVED_SEVERITY | Y | 4 | Minor |

EXPERIMENTAL

8.3.1.3	Message: FRU Operation

Owning Entity: SNIA

Message ID: Core3

Message Format String: <Device Type> <Device Unique Identifier> had a Field Replaceable Unit (FRU)

75

<The FRU Operation> on <FRU Type> at <FRU Location Or Identifier>

76

77

A message indicating an manual operation occured with a Field Replaceable Unit (FRU) that resulted in a change. Table 13 describes the message arguments.

78

79	Table 13 - FRU Operation Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | HBA |
|  |  |  | Array |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| The FRU Operation | string | The operation on the FRU that is the basis of the message | removed |
|  |  |  | added |
|  |  |  | replaced |
|  |  |  | incompatible |
| FRU Type | string | The Type of FRU |  |
| FRU Location Or Identifier | string | Location or the Identifier of the FRU |  |

80

81

82

83

84

85

86

87

88

89

90

91

92

#### Message: Password change

Table 14 describes the alerts that are associated with this message.

Table 14 - FRU Operation Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  |  |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.1.4	Message: Password change

Owning Entity: SNIA

Message ID: Core4

Message Format String: <Device Type> <Device Unique Identifier> password has change for user <User Identification>

A message indicating a user or account password has change. Table 15 describes the message arguments.

Table 15 - Password change Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | HBA |
|  |  |  | Array |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| User Identification | string | User or Account Identification |  |

Table 16 describes the alerts that are associated with this message.

Table 16 - Password change Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  |  |
| ALERT_TYPE | Y |  |  |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

Message: User or Account Operation

EXPERIMENTAL

8.3.1.5	Message: User or Account Operation

Owning Entity: SNIA

Message ID: Core5

Message Format String: <Device Type> <Device Unique Identifier> user <User Identification> <User Operation>

A message indicating a user or account password has added, removed, or disabled. Table 17 describes the message arguments.

Table 17 - User or Account Operation Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | HBA |
|  |  |  | Array |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| User Identification | string | User or Account Identification |  |
| User Operation | string | Operation on User | removed |
|  |  |  | disabled |
|  |  |  | added |

Table 18 describes the alerts that are associated with this message.

Table 18 - User or Account Operation Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  |  |
| ALERT_TYPE | Y |  | Security Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.1.6	Message: User Login

Owning Entity: SNIA

Message ID: Core6

Message Format String: <Device Type> <Device Unique Identifier> user <User> <Login Operation> Message: Proxy Agent Device Communication

A message indicating user or account login activity including logging into or off of a device. Table 19 describes the message arguments.

Table 19 - User Login Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | HBA |
|  |  |  | Array |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| User | string | The user or account name |  |
| Login Operation | string | Operation on User | logged in |
|  |  |  | logged out |

Table 20 describes the alerts that are associated with this message.

Table 20 - User Login Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name must reference the top-most computer system that is shutting down. If the computer system is cluster, then the cluster computer system must be referenced. |
| ALERT_TYPE | Y |  | Security Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.1.7	Message: Proxy Agent Device Communication

Owning Entity: SNIA

Message ID: Core7

Message Format String: Agent <Agent Identifier> <Agent Connectivity> communication with <Device Type> <Device Unique Identifier>

If an agent is acting as a proxy to a device, this message is used if the connection is lost between the proxy and the device. Table 21 describes the message arguments.

Table 21 - Proxy Agent Device Communication Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Agent Identifier | string | An identifier for the SMI Agent |  |
| Agent Connectivity | string | A description for the connectivity | lost |
|  |  |  | regained |

Message: Port Status Changed

Table 21 - Proxy Agent Device Communication Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | Array |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |

Table 22 describes the alerts that are associated with this message.

Table 22 - Proxy Agent Device Communication Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  |  |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.1.8	Message: Port Status Changed

Owning Entity: SNIA

Message ID: Core8

Message Format String: FC Port <Port Identifier> in <Device Type> <Device Unique Identifier> status changed to <Port Status>

The fabric has detected a change in status of a Fibre Channel port in the fabric. Table 23 describes the message arguments.

Table 23 - Port Status Changed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Port Identifier | string | The Fibre Channel Port Name. |  |
| Device Type | string |  | Switch |
|  |  |  | HBA |
|  |  |  | Array |
|  |  |  | Fabric |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| Port Status | string | Fibre Channel Port Status. This should be the same value as the OperationalStatus for the FCPort. |  |

#### Message: Datacheck Error

Table 24 describes the alerts that are associated with this message.

Table 24 - Port Status Changed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  |  |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.1.9	Message: Datacheck Error

Owning Entity: SNIA

Message ID: Core9

Message Format String: <Device Type> <Device Unique Identifier> data check ( <Data Check Type>

A data check error occurred on a device. The error could be a checksum error, CRC error, or some other kind of error where there was some determination that the data transmitted or stored was not correct. Table 25 describes the message arguments.

Table 25 - Datacheck Error Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | HBA |
|  |  |  | Array |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| Data Check Type | string | The type of data check that occured. | Switch |
|  |  |  | CRC |

Table 26 describes the alerts that are associated with this message.

Table 26 - Datacheck Error Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  |  |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

Message: User Login Failure

EXPERIMENTAL

8.3.1.10	Message: User Login Failure

Owning Entity: SNIA

Message ID: Core10

Message Format String: <Device Type> <Device Unique Identifier> user <User> had login failure.

A message indicating user or account login failure into a device. Table 27 describes the message arguments.

Table 27 - User Login Failure Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Type | string | A description of the type of element | Switch |
|  |  |  | HBA |
|  |  |  | Array |
| Device Unique Identifier | string | An identifier for the device (host, array, switch, etc.). |  |
| User | string | The user or account name |  |

Table 28 describes the alerts that are associated with this message.

Table 28 - User Login Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name must reference the top-most computer system that is shutting down. If the computer system is cluster, then the cluster computer system must be referenced. |
| ALERT_TYPE | Y |  | Security Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.1.11	Message: Drive not responding

Owning Entity: SNIA

Message ID: Core12

Message Format String: <Type of Drive> drive is not responding. Drive Identifier: <Device Unique

Identifier>

#### Message: Fan Failure

A message indicating a drive is not responding to I/O commands. Table 29 describes the message arguments.

Table 29 - Drive not responding Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Type of Drive | string | Type of drive not responding. | Disk |
|  |  |  | Tape |
|  |  |  | CD |
|  |  |  | DVD |
| Device Unique Identifier | string | An identifier for the drive. |  |

Table 30 describes the alerts that are associated with this message.

Table 30 - Drive not responding Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | Reference to the drive |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

EXPERIMENTAL

8.3.1.12	Message: Fan Failure

Owning Entity: SNIA

Message ID: Core13

Message Format String: Fan failure.

Table 31 describes the alerts that are associated with this message.

Table 31 - Fan Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_Fan (if modelled) or CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.1.13	Message: Power Supply Failure

Owning Entity: SNIA

Message ID: Core14

Message Format String: Power supply unit failure.

Message: Drive Power Consumption

Table 32 describes the alerts that are associated with this message.

Table 32 - Power Supply Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_PowerSupply (if modelled) or CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.1.14	Message: Drive Power Consumption

Owning Entity: SNIA

Message ID: Core15

Message Format String: Power consumption of the drive is outside specified range.

Table 33 describes the alerts that are associated with this message.

Table 33 - Drive Power Consumption Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.1.15	Message: Drive Voltage

Owning Entity: SNIA

Message ID: Core17

Message Format String: Drive voltage limits exceeded.

Table 34 describes the alerts that are associated with this message.

Table 34 - Drive Voltage Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.1.16	Message: Predictive Failure

Owning Entity: SNIA

Message ID: Core18

Message Format String: Predictive failure of drive hardware.

#### Message: Diagnostics Required

Table 35 describes the alerts that are associated with this message.

Table 35 - Predictive Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.1.17	Message: Diagnostics Required

Owning Entity: SNIA

Message ID: Core19

Message Format String: The drive may have a hardware fault that may be identified by extended diagnostics (i.e., SEND DIAGNOSTIC command).

Table 36 describes the alerts that are associated with this message.

Table 36 - Diagnostics Required Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

EXPERIMENTAL

8.3.1.18	Message: Drive is responding

Owning Entity: SNIA

Message ID: Core20

Message Format String: <Type of Drive> drive issues have cleared. Drive Identifier: <Device Unique Identifier>

A message indicating a drive has resumed responding to I/O commands. Table 37 describes the message arguments.

Table 37 - Drive is responding Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Type of Drive | string | Type of drive responding. | Disk |
|  |  |  | Tape |
|  |  |  | CD |
|  |  |  | DVD |
| Device Unique Identifier | string | An identifier for the drive. |  |

Message: Cooling Fan Issues Cleared

Table 38 describes the alerts that are associated with this message.

Table 38 - Drive is responding Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | Reference to the drive |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.1.19	Message: Cooling Fan Issues Cleared

Owning Entity: SNIA

Message ID: Core21

Message Format String: Fan issues cleared:

Table 39 describes the alerts that are associated with this message.

Table 39 - Cooling Fan Issues Cleared Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_Fan (if modeled) or CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.1.20	Message: Power Supply Issues Cleared

Owning Entity: SNIA

Message ID: Core22

Message Format String: Power supply unit issues cleared: <Device Unique Identifier>

Table 40 describes the message arguments.

Table 40 - Power Supply Issues Cleared Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Unique Identifier | string | An identifier for the power supply. |  |

Message: Controller Failure

Table 41 describes the alerts that are associated with this message.

Table 41 - Power Supply Issues Cleared Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_PowerSupply (if modelled) or CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

8.3.1.21	Message: Controller Failure

Owning Entity: SNIA

Message ID: Core23

Message Format String: Controller failure: <Device Unique Identifier>

Table 42 describes the message arguments.

Table 42 - Controller Failure Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Unique Identifier | string | An identifier for the controller. |  |

Table 43 describes the alerts that are associated with this message.

Table 43 - Controller Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

8.3.1.22	Message: Controller Issues Cleared

Owning Entity: SNIA

Message ID: Core24

Message Format String: Controller issues resolved: <Device Unique Identifier>

Table 44 describes the message arguments.

Table 44 - Controller Issues Cleared Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device Unique Identifier | string | An identifier for the controller. |  |

Message: Device Not ready

Table 45 describes the alerts that are associated with this message.

Table 45 - Controller Issues Cleared Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

### 8.3.2	Block Storage Messages

EXPERIMENTAL

8.3.2.1	Message: Device Not ready

Owning Entity: SNIA

Message ID: DRM1

Message Format String: Device <Device ID> not ready because of <StatusOrStatus> state or status.

A message indicating a device is not ready. Table 46 describes the message arguments.

Table 46 - Device Not ready Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device ID | string | LogicalDevice.DeviceID, PhysicalElement.Tag, or ComputerSystem.Name |  |
| StatusOrStatus | string | Relavent State or Status the most explains the reason for the production of this message. |  |

Message: Internal Bus Error

Table 47 describes the error properties.

Table 47 - Error Properties for Device Not ready

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 5(Hardware Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the device, which is typically the computer system instance | Existence is required |
| PERCEIVED_SEVERITY | 4(Minor) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.2	Message: Internal Bus Error

Owning Entity: SNIA

Message ID: DRM2

Message Format String: Internal Bus Error

Table 48 describes the error properties.

Table 48 - Error Properties for Internal Bus Error

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 5(Hardware Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the device, which is typically the computer system instance | Existence is required |
| PERCEIVED_SEVERITY | 4(Minor) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.3	Message: DMA Overflow

Owning Entity: SNIA

Message ID: DRM3

Message Format String: DMA Overflow

Message: Firmware Logic Error

Table 49 describes the error properties.

Table 49 - Error Properties for DMA Overflow

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the device, which is typically the computer system instance | Existence is required |
| PERCEIVED_SEVERITY | 4(Minor) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.4	Message: Firmware Logic Error

Owning Entity: SNIA

Message ID: DRM4

Message Format String: Firmware Logic Error

Table 50 describes the error properties.

Table 50 - Error Properties for Firmware Logic Error

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the device, which is typically the computer system instance | Existence is required |
| PERCEIVED_SEVERITY | 4(Minor) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.5	Message: Front End Port Error

Owning Entity: SNIA

Message ID: DRM5

Message Format String: Front End Port Error on Device identified by <Device ID> Message: Back End Port Error

A message indicating front end port is in error. Table 51 describes the message arguments.

Table 51 - Front End Port Error Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device ID | string | LogicalDevice.DeviceID |  |

Table 52 describes the alerts that are associated with this message.

Table 52 - Front End Port Error Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | Object Name for the top-level object for the device, which is typically the computer system instance |
| ALERT_TYPE | Y | 2 | Communications Alert |
| PERCEIVED_SEVERITY | Y | 4 | Minor |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.6	Message: Back End Port Error

Owning Entity: SNIA

Message ID: DRM6

Message Format String: Back End Port Error on Device identified by <Device ID>

A message indicating a back end port is in error. Table 53 describes the message arguments.

Table 53 - Back End Port Error Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Device ID | string | LogicalDevice.DeviceID |  |

Table 54 describes the alerts that are associated with this message.

Table 54 - Back End Port Error Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | Object Name for the top-level object for the device, which is typically the computer system instance |
| ALERT_TYPE | Y | 2 | Communications Alert |
| PERCEIVED_SEVERITY | Y | 4 | Minor |

EXPERIMENTAL

Message: Remote Mirror Error

EXPERIMENTAL

8.3.2.7	Message: Remote Mirror Error

Owning Entity: SNIA

Message ID: DRM7

Message Format String: Error detected associated with remote volume, <Remote Volume Name>

A message indicating an error associated with remote volume. Table 55 describes the message arguments.

Table 55 - Remote Mirror Error Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Remote Volume Name | string | StorageVolume.Name |  |

Table 56 describes the error properties.

Table 56 - Error Properties for Remote Mirror Error

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 5(Hardware Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the remote block server, which is typically the computer system instance. The implementation will have to implement the Cascading Subprofile. | Existence is optional |
| PERCEIVED_SEVERITY | 3(Degraded/Warning) | Existence is required |

Table 57 describes the alerts that are associated with this message.

Table 57 - Remote Mirror Error Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | N |  | Object Name for the top-level object for the remote block server, which is typically the computer system instance. The implementation will have to implement the Cascading Subprofile. |
| ALERT_TYPE | Y |  |  |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.8	Message: Cache Memory Error

Owning Entity: SNIA

Message: Unable to Access Remote Device

Message ID: DRM8

Message Format String: Cache Memory Error

Table 58 describes the error properties.

Table 58 - Error Properties for Cache Memory Error

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 5(Hardware Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the device, which is typically the computer system instance | Existence is required |
| PERCEIVED_SEVERITY | 3(Degraded/Warning) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.9	Message: Unable to Access Remote Device

Owning Entity: SNIA

Message ID: DRM9

Message Format String: Unable to Access Remote Device

Table 59 describes the error properties.

Table 59 - Error Properties for Unable to Access Remote Device

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 5(Hardware Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the remote block server, which is typically the computer system instance. The implementation will have to implement the Cascading Subprofile. | Existence is optional |
| PERCEIVED_SEVERITY | 3(Degraded/Warning) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.10	Message: Error Reading Data

Owning Entity: SNIA

Message ID: DRM10

Message: Error Writing Data

Message Format String: Error Reading Data

Table 60 describes the alerts that are associated with this message.

Table 60 - Error Reading Data Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | Object Name for the top-level object for the device, which is typically the computer system instance |
| ALERT_TYPE | Y | 2 | Communications Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.11	Message: Error Writing Data

Owning Entity: SNIA

Message ID: DRM11

Message Format String: Error Writing Data

Table 61 describes the alerts that are associated with this message.

Table 61 - Error Writing Data Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | Object Name for the top-level object for the device, which is typically the computer system instance |
| ALERT_TYPE | Y | 2 | Communications Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.12	Message: Error Validating Write (CRC)

Owning Entity: SNIA

Message ID: DRM12

Message Format String: Error Validating Write

Message: Copy Operation Failed

Table 62 describes the alerts that are associated with this message.

Table 62 - Error Validating Write (CRC) Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | Object Name for the top-level object for the device, which is typically the computer system instance |
| ALERT_TYPE | Y | 2 | Communications Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.13	Message: Copy Operation Failed

Owning Entity: SNIA

Message ID: DRM13

Message Format String: Copy Operation Failed

Table 63 describes the error properties.

Table 63 - Error Properties for Copy Operation Failed

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 5(Hardware Error) | Existence is required |
| ERROR_SOURCE |  | Existence is discouraged |
| PERCEIVED_SEVERITY | 3(Degraded/Warning) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.14	Message: RAID Operation Failed

Owning Entity: SNIA

Message ID: DRM14

Message Format String: RAID Operation Failed

Message: Invalid RAID Type

Table 64 describes the error properties.

Table 64 - Error Properties for RAID Operation Failed

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 5(Hardware Error) | Existence is required |
| ERROR_SOURCE |  | Existence is discouraged |
| PERCEIVED_SEVERITY | 3(Degraded/Warning) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.15	Message: Invalid RAID Type

Owning Entity: SNIA

Message ID: DRM15

Message Format String: Invalid RAID Type

Table 65 describes the error properties.

Table 65 - Error Properties for Invalid RAID Type

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 10(Unsupported Operation Error) | Existence is required |
| ERROR_SOURCE |  | Existence is discouraged |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.16	Message: Invalid Storage Element Type

Owning Entity: SNIA

Message ID: DRM16

Message Format String: Invalid Device Type

Message: Configuration Change Failed

Table 66 describes the error properties.

Table 66 - Error Properties for Invalid Storage Element Type

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 10(Unsupported Operation Error) | Existence is required |
| ERROR_SOURCE |  | Existence is discouraged |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.17	Message: Configuration Change Failed

Owning Entity: SNIA

Message ID: DRM17

Message Format String: Configuration Change Failed

Table 67 describes the error properties.

Table 67 - Error Properties for Configuration Change Failed

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the device, which is typically the computer system instance | Existence is required |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.18	Message: Buffer Overrun

Owning Entity: SNIA

Message ID: DRM18

Message Format String: Buffer Overrun

Message: Stolen Capacity

Table 68 describes the error properties.

Table 68 - Error Properties for Buffer Overrun

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | Object Name for the top-level object for the device, which is typically the computer system instance | Existence is required |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.19	Message: Stolen Capacity

Owning Entity: SNIA

Message ID: DRM19

Message Format String: The capacity requested, <Requested Capacity> , that was requested is no longer available.

A message indicating an invalid capacity was requested. Table 69 describes the message arguments.

Table 69 - Stolen Capacity Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Requested Capacity | sint64 | Capacity requested in bytes expressed in powers of 10. |  |

Table 70 describes the error properties.

Table 70 - Error Properties for Stolen Capacity

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | The pool, volume, or logical disk being modified, or, in the case of element creation the parent pool from which capacity is being drawn. | Existence is required |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

#### Message: Invalid Extent passed

EXPERIMENTAL

8.3.2.20	Message: Invalid Extent passed

Owning Entity: SNIA

Message ID: DRM20

Message Format String: One or more of the extents passed can not be used to create or modify storage elements. <Invalid Extents Array>

A message indicating an invalid extent was passed as an argument. Table 71 describes the message arguments.

Table 71 - Invalid Extent passed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Invalid Extents Array | reference | Array of references to the all Extents that can not be used in the specified manner (ex. CreateOrModifyStroragePool or CreateOrModifyElementsFromElements). |  |

Table 72 describes the error properties.

Table 72 - Error Properties for Invalid Extent passed

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | A reference to the storage configuration service instance on which the method was called that caused this error. | Existence is required |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.21	Message: Invalid Deletion Attempted

Owning Entity: SNIA

Message ID: DRM21

Message Format String: Existing pool or storage element (StorageVolume or LogicalDisk) may not be deleted because there are existing Storage Extents which relay on it.

Message: Job Failed to Start

Table 73 describes the error properties.

Table 73 - Error Properties for Invalid Deletion Attempted

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | A reference to one of the dependent StorageExtents. | Existence is required |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.22	Message: Job Failed to Start

Owning Entity: SNIA

Message ID: DRM22

Message Format String: Job failed to start because resources required for method execution are no longer available.

Table 74 describes the error properties.

Table 74 - Error Properties for Job Failed to Start

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 8(Oversubscription Error) | Existence is required |
| ERROR_SOURCE | Reference to Job instance which failed to start for this reason if a Job instance was created because of the time required to make this resource assessment. If a Job instance was not created, because the assessment was fast enough, then this property must be NULL. | Existence is required |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.23	Message: Job was Halted

Owning Entity: SNIA

Message ID: DRM23

Message Format String: Job was <Reason for Job halt>

#### Message: Invalid State Transition

A message indicating that a job was halted. Table 75 describes the message arguments.

Table 75 - Job was Halted Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Reason for Job halt | string | A Job may be stopped by a client using the RequestedStateChange method. If the job stopped executing for other reasons, then use a different message. | killed |
|  |  |  | terminated |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.24	Message: Invalid State Transition

Owning Entity: SNIA

Message ID: DRM24

Message Format String: An invalid state transition, <Invalid Sync State> , was requested given current state, <Current Sync State>

A message indicating an invalid state transition occurred. Table 76 describes the message arguments.

Table 76 - Invalid State Transition Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Invalid Sync State | string | The textual equivalent (Value) for StorageSynchronized.SyncState value requested. |  |
| Current Sync State | string | The textual equivalent (Value) for the current StorageSynchronized.SyncState value |  |

Table 77 describes the error properties.

Table 77 - Error Properties for Invalid State Transition

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | Reference to the StorageSynchronized instance in question. | Existence is required |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

Message: Invalid SAP for Method

EXPERIMENTAL

8.3.2.25	Message: Invalid SAP for Method

Owning Entity: SNIA

Message ID: DRM25

Message Format String: Invalid type of copy services host. The host must be a <Host Type>

A message indicating an invalid copy services host. Table 78 describes the message arguments.

Table 78 - Invalid SAP for Method Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Host Type | string | The type of copy services on which the method was invoked. | source |
|  |  |  | target |

Table 79 describes the error properties.

Table 79 - Error Properties for Invalid SAP for Method

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | Reference to the Computer System host which is of the wrong type. | Existence is required |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.26	Message: Resource Not Available

Owning Entity: SNIA

Message ID: DRM26

Message Format String: <Resource Needed>

A message indicating that a resource was not available for replication. Table 80 describes the message arguments.

Table 80 - Resource Not Available Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Resource Needed | string |  | No replication log available. |
|  |  |  | Special replica pool required. |

#### Message: Resource Limit Exceeded

Table 81 describes the error properties.

Table 81 - Error Properties for Resource Not Available

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | Nothing to reference. | Existence is discouraged |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.27	Message: Resource Limit Exceeded

Owning Entity: SNIA

Message ID: DRM27

Message Format String: <Reason>

A message indicating a resource limit has been reached. Table 82 describes the message arguments.

Table 82 - Resource Limit Exceeded Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Reason | string | The reasons for the lack of resources for copy services operation. | Insufficient pool space. |
|  |  |  | Maximum replication depth exceeded. |
|  |  |  | Maxium replicas exceeded for source element. |

Table 83 describes the error properties.

Table 83 - Error Properties for Resource Limit Exceeded

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | 1(CIM_ERR_FAILED) | Existence is required |
| ERROR_TYPE | 4(Software Error) | Existence is required |
| ERROR_SOURCE | Nothing to reference. | Existence is discouraged |
| PERCEIVED_SEVERITY | 2(Information) | Existence is required |

EXPERIMENTAL

8.3.2.28	Message: Thin Provision Capacity Warning

Owning Entity: SNIA

Message ID: DRM28

Message: Provision Capacity Critical

Message Format String: <Element type> with identifier <Device or Pool ID or Name> is nearing its available capacity limit.

The actual capacity of a volume or pool is nearing a limit (e.g., actual usage of containing pool is nearing SpaceLimit). Table 84 describes the message arguments.

Table 84 - Thin Provision Capacity Warning Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Element type | string | A 'volume', 'pool' or 'filesystem' | volume |
|  |  |  | pool |
|  |  |  | filesystem |
| Device or Pool ID or Name | string | Disk Name. |  |

Table 85 describes the alerts that are associated with this message.

Table 85 - Thin Provision Capacity Warning Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The pool, volume or filesystem ID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

8.3.2.29	Message: Provision Capacity Critical

Owning Entity: SNIA

Message ID: DRM29

Message Format String: <Element type> with identifier <Device or Pool ID or Name> has exceeded the available capacity limit.

the actual capacity of a volume or pool has reached a limit (e.g., actual usage of containing pool is equal to SpaceLimit). Write commands from hosts to the volume or pool are failing. . Table 86 describes the message arguments.

Table 86 - Provision Capacity Critical Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Element type | string | A 'volume', 'pool' or 'filesystem' | volume |
|  |  |  | pool |
|  |  |  | filesystem |
| Device or Pool ID or Name | string | Disk Name. |  |

#### Message: Thin Provision Capacity Okay

Table 87 describes the alerts that are associated with this message.

Table 87 - Provision Capacity Critical Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The pool, volume or filesystem ID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.2.30	Message: Thin Provision Capacity Okay

Owning Entity: SNIA

Message ID: DRM30

Message Format String: <Thin element type> with identifier <Device or Pool ID> capacity condition cleared.

The actual capacity of a volume or pool is no longer in a capacity warning or critical state. Table 88 describes the message arguments.

Table 88 - Thin Provision Capacity Okay Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Thin element type | string | A 'volume', 'pool' or 'filesystem' | volume |
|  |  |  | pool |
|  |  |  | filesystem |
| Device or Pool ID | string | Disk Name. |  |

Table 89 describes the alerts that are associated with this message.

Table 89 - Thin Provision Capacity Okay Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The pool, volume or filesystem ID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.2.31	Message: Masking Group Membership Changed

Owning Entity: SNIA

Message ID: DRM31

Message Format String: There is a change in membership of masking group with identifier <InstanceID> , and with ElementName <ElementName>

Message: StorageVolume Relocation Starts

The membership of a masking group has changed. Table 90 describes the message arguments.

Table 90 - Masking Group Membership Changed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| InstanceID | string | The instance ID of masking group | InstanceID |
| ElementName | string | The ElementName of masking group | ElementName |

Table 91 describes the alerts that are associated with this message.

Table 91 - Masking Group Membership Changed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The masking group object name. |
| ALERT_TYPE | Y | 2 | Model Change |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.2.32	Message: StorageVolume Relocation Starts

Owning Entity: SNIA

Message ID: DRM32

Message Format String: Relocation is starting for the Storage Volume with identifier <StorageVolume DeviceID>

The relocation of a storage volume is starting. Table 92 describes the message arguments.

Table 92 - StorageVolume Relocation Starts Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| StorageVolume DeviceID | string | DeviceID. |  |

Table 93 describes the alerts that are associated with this message.

Table 93 - StorageVolume Relocation Starts Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The StorageVolume DeviceID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.2.33	Message: StorageVolume Relocation Ends

Owning Entity: SNIA

Message ID: DRM33

Message Format String: Relocation has ended for the Storage Volume with identifier <StorageVolume DeviceID>

#### Message: StoragePool Relocation Starts

The relocation of a storage volume has ended. Table 94 describes the message arguments.

Table 94 - StorageVolume Relocation Ends Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| StorageVolume DeviceID | string | DeviceID. |  |

Table 95 describes the alerts that are associated with this message.

Table 95 - StorageVolume Relocation Ends Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The StorageVolume DeviceID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.2.34	Message: StoragePool Relocation Starts

Owning Entity: SNIA

Message ID: DRM34

Message Format String: Relocation is starting for the Storage Pool with identifier <StoragePool PoolID>

The relocation of a storage pool is starting. Table 96 describes the message arguments.

Table 96 - StoragePool Relocation Starts Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| StoragePool PoolID | string | PoolID. |  |

Table 97 describes the alerts that are associated with this message.

Table 97 - StoragePool Relocation Starts Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The StoragePool PoolID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.2.35	Message: StoragePool Relocation Ends

Owning Entity: SNIA

Message ID: DRM35

Message Format String: Relocation has ended for the Storage Pool with identifier <StoragePool PoolID>

Message: LogicalDisk Relocation Starts

The relocation of a storage pool has ended. Table 98 describes the message arguments.

Table 98 - StoragePool Relocation Ends Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| StoragePool PoolID | string | PoolID. |  |

Table 99 describes the alerts that are associated with this message.

Table 99 - StoragePool Relocation Ends Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The StoragePool PoolID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.2.36	Message: LogicalDisk Relocation Starts

Owning Entity: SNIA

Message ID: DRM36

Message Format String: Relocation is starting for the Logical Disk with identifier <LogicalDisk DeviceID>

The relocation of a logical disk is starting. Table 100 describes the message arguments.

Table 100 - LogicalDisk Relocation Starts Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| LogicalDisk DeviceID | string | DeviceID. |  |

Table 101 describes the alerts that are associated with this message.

Table 101 - LogicalDisk Relocation Starts Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The LogicalDisk DeviceID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.2.37	Message: LogicalDisk Relocation Ends

Owning Entity: SNIA

Message ID: DRM37

Message Format String: Relocation has ended for the Logical Disk with identifier <LogicalDisk DeviceID> Message: Volume or pool degraded

The relocation of a logical disk has ended. Table 102 describes the message arguments.

Table 102 - LogicalDisk Relocation Ends Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| LogicalDisk DeviceID | string | DeviceID. |  |

Table 103 describes the alerts that are associated with this message.

Table 103 - LogicalDisk Relocation Ends Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The LogicalDisk DeviceID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

8.3.2.38	Message: Volume or pool degraded

Owning Entity: SNIA

Message ID: DRM38

Message Format String: Volume or pool <Element type> with identifier <Device or Pool ID> is in a degraded state.

The status of a volume or pool is degraded. . Table 104 describes the message arguments.

Table 104 - Volume or pool degraded Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Element type | string | A 'volume' or 'pool' | volume |
|  |  |  | pool |
| Device or Pool ID | string | Disk Name. |  |

Table 105 describes the alerts that are associated with this message.

Table 105 - Volume or pool degraded Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The pool or volume ID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 4 | Degraded/Warning |

EXPERIMENTAL

Message: Volume or pool failed

EXPERIMENTAL

8.3.2.39	Message: Volume or pool failed

Owning Entity: SNIA

Message ID: DRM39

Message Format String: Volume or pool <Element type> with identifier <Device or Pool ID> has failed and is not accessible.

The status of a volume or pool is failed. . Table 106 describes the message arguments.

Table 106 - Volume or pool failed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Element type | string | A 'volume' or 'pool' | volume |
|  |  |  | pool |
| Device or Pool ID | string | Disk Name. |  |

Table 107 describes the alerts that are associated with this message.

Table 107 - Volume or pool failed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The pool or volume ID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.40	Message: Volume or pool issues cleared

Owning Entity: SNIA

Message ID: DRM40

Message Format String: Volume or pool <Element type> with identifier <Device or Pool ID> is no longer degraded.

The status of a volume or pool is normal. . Table 108 describes the message arguments.

Table 108 - Volume or pool issues cleared Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Element type | string | A 'volume' or 'pool' | volume |
|  |  |  | pool |
| Device or Pool ID | string | Disk Name. |  |

#### Message: The StoragePool is healthy

Table 109 describes the alerts that are associated with this message.

Table 109 - Volume or pool issues cleared Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The pool or volume ID. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.41	Message: The StoragePool is healthy

Owning Entity: SNIA

Message ID: DRM101

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> has found that the pool has an OK OperationalStatus.

The test ran to completion and found the OperationalStatus to be OK. Table 110 describes the message arguments.

Table 110 - The StoragePool is healthy Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |

Table 111 describes the alerts that are associated with this message.

Table 111 - The StoragePool is healthy Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (StoragePool is OK) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

Message: StoragePool is dependent on an element with problems

EXPERIMENTAL

8.3.2.42	Message: StoragePool is dependent on an element with problems

Owning Entity: SNIA

Message ID: DRM102

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> has an

OperationalStatus of <OperationalStatus1> , because <Named Element> <Element Moniker> has an OperationalStatus of <OperationalStatus2>

The test found that an element the pool is dependent on has a non-OK OperationalStatus. Table 112 describes the message arguments.

Table 112 - StoragePool is dependent on an element with problems Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperationalStatus1 | string | The OperationalStatus of the pool under test. |  |
| Named Element | string | The common name (e.g. Storage Pool or Disk Drive) for the non-OK element |  |
| Element Moniker | string | A unique name for the contributing element with the non-OK state. | The object path of the contributing element |
|  |  |  | The ElementName of the contributing element |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperationalStatus2 | string | The OperationalStatus of the contributing element. |  |

Table 113 describes the alerts that are associated with this message.

Table 113 - StoragePool is dependent on an element with problems Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (StoragePool element problem) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

#### Message: The StoragePool is being serviced

EXPERIMENTAL

8.3.2.43	Message: The StoragePool is being serviced

Owning Entity: SNIA

Message ID: DRM103

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> has an OperationalStatus of <OperatonalStatus> , because it is being <Service Action>

The test found that the StoragePool is being serviced, which results in the pool OperationalStatus. Table 114 describes the message arguments.

Table 114 - The StoragePool is being serviced Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperatonalStatus | string | The OperationalStatus of the storage pool under test. |  |
| Service Action | string | The temporary service that is in progress. | Rebuilt |
|  |  |  | Relocated |
|  |  |  | Tested |
|  |  |  | Serviced |

Table 115 describes the alerts that are associated with this message.

Table 115 - The StoragePool is being serviced Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (StoragePool servicing in progress) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.44 Message: The OperationalStatus of the Pool is impacting an element allocated from it Owning Entity: SNIA

Message: The OperationalStatus of the Pool is impacting an element allocated from it

Message ID: DRM104

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> has an OperationalStatus of <OperatonalStatus1> that is causing <Named Element> <Element Moniker> to have an OperationalStatus of <OperatonalStatus2>

The test found that the OperationalStatus of the StoragePool is impacting an element that is allocated from the pool. Table 116 describes the message arguments.

Table 116 - The OperationalStatus of the Pool is impacting an element allocated from it Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperatonalStatus1 | string | The OperationalStatus of the storage pool under test. |  |
| Named Element | string | The common name (e.g. Storage Pool or Storage Volume) for the non-OK element | Storage Pool |
|  |  |  | Storage Volume |
|  |  |  | Logical Disk |
|  |  |  | Filesystem |
| Element Moniker | string | A unique name for the allocated element with the non-OK state. | The object path of the allocated element |
|  |  |  | The ElementName of the allocated element |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperatonalStatus2 | string | The OperationalStatus of the element allocated from the pooll under test. |  |

Table 117 describes the alerts that are associated with this message.

Table 117 - The OperationalStatus of the Pool is impacting an element allocated from it Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (Element Impacted) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

#### Message: The StoragePool OperationalStatus may be corrected by applying a spare

EXPERIMENTAL

8.3.2.45	Message: The StoragePool OperationalStatus may be corrected by applying a spare

Owning Entity: SNIA

Message ID: DRM105

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> has an

OperationalStatus of <OperatonalStatus> can be fixed by applying one of the following spare <Named Element> <List of Spares>

The test found that applying a spare will correct the OperationalStatus of the Pool. Table 118 describes the message arguments.

Table 118 - The StoragePool OperationalStatus may be corrected by applying a spare Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperatonalStatus | string | The OperationalStatus of the storage pool under test. |  |
| Named Element | string | The common name (e.g. Storage Extent or Disk Drive) for the spare element | Storage Extent |
|  |  |  | Disk Drive |
| List of Spares | string | An array of element monikers of the available spares. | The object path of the spare element |
|  |  |  | The ElementName of the spare element |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |

Table 119 describes the alerts that are associated with this message.

Table 119 - The StoragePool OperationalStatus may be corrected by applying a spare Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (Deploy Spare) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

Message: The StoragePool OperationalStatus may be corrected by relocating the pool

EXPERIMENTAL

8.3.2.46	Message: The StoragePool OperationalStatus may be corrected by relocating the pool

Owning Entity: SNIA

Message ID: DRM106

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> has an OperationalStatus of <OperatonalStatus> can be fixed by relocating the storage pool.

The test found that relocating the pool will correct the OperationalStatus of the pool. Table 120 describes the message arguments.

Table 120 - The StoragePool OperationalStatus may be corrected by relocating the pool Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperatonalStatus | string | The OperationalStatus of the storage pool under test. |  |

Table 121 describes the alerts that are associated with this message.

Table 121 - The StoragePool OperationalStatus may be corrected by relocating the pool Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (StoragePool may be relocated) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.47	Message: Pool experiencing interference from system workload

Owning Entity: SNIA

Message ID: DRM107

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> has an OperationalStatus of <OperatonalStatus> is experiencing interferance from other system workloads.

Message: Pool performance degraded by component element

The test found that the storage pool has its operational status because of interference from system workloads. Table 122 describes the message arguments.

Table 122 - Pool experiencing interference from system workload Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperatonalStatus | string | The OperationalStatus of the storage pool under test. |  |

Table 123 describes the alerts that are associated with this message.

Table 123 - Pool experiencing interference from system workload Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (Workload Interference) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.48	Message: Pool performance degraded by component element

Owning Entity: SNIA

Message ID: DRM108

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> has an

OperationalStatus of degraded due to conditions in <Named Element> <Element Moniker> has an OperationalStatus of <OperationalStatus>

Message: Pool degraded due to loss of RAID protection

The test found that the activity in the storage pool may be experiencing performance problems because a component element (e.g., parent storage pool or disk drive) has a non-OK OperationalStatus. Table 124 describes the message arguments.

Table 124 - Pool performance degraded by component element Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| Named Element | string | The common name (e.g. Storage Extent or Disk Drive) for the non-OK element | Storage Extent |
|  |  |  | Disk Drive |
|  |  |  | Back-end port |
|  |  |  | Storage Pool |
| Element Moniker | string | A unique name for the contributing element with the non-OK state. | The object path of the contributing element |
|  |  |  | The ElementName of the contributing element |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |
| OperationalStatus | string | The OperationalStatus of the contributing element. |  |

Table 125 describes the alerts that are associated with this message.

Table 125 - Pool performance degraded by component element Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (Degraded by Element) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.49	Message: Pool degraded due to loss of RAID protection

Owning Entity: SNIA

Message ID: DRM109

#### Message: Pool degraded due to loss of port redundancy

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> is degraded due to loss of RAID protection.

The test found that the storage pool is degraded due to the loss of RAID protection (PackageRedundancy or DataRedundancy). Table 126 describes the message arguments.

Table 126 - Pool degraded due to loss of RAID protection Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |

Table 127 describes the alerts that are associated with this message.

Table 127 - Pool degraded due to loss of RAID protection Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (Loss of RAID protection) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.50	Message: Pool degraded due to loss of port redundancy

Owning Entity: SNIA

Message ID: DRM110

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> is degraded due to loss of port redundancy for disk access.

Message: Pool predicting failure due lack of available capacity

The test found that the storage pool is degraded due to disk access degradation due to a failing port. Table 128 describes the message arguments.

Table 128 - Pool degraded due to loss of port redundancy Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |

Table 129 describes the alerts that are associated with this message.

Table 129 - Pool degraded due to loss of port redundancy Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (Loss of Port redundancy) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.2.51	Message: Pool predicting failure due lack of available capacity

Owning Entity: SNIA

Message ID: DRM111

Message Format String: The <Test> test on the selected storage pool to test <Pool Moniker> is predicting failure because the available capacity is low.

The test found that the storage pool is predicting failure because it is running low on available capacity. Table 130 describes the message arguments.

Table 130 - Pool predicting failure due lack of available capacity Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Test | string | The Name property value of the StoragePoolDiagnosticTest instance invoked. |  |
| Pool Moniker | string | A unique name for the pool under test that was specified. | The object path of the pool under test |

#### Message: Zone Database Changed

Table 130 - Pool predicting failure due lack of available capacity Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
|  |  |  | The ElementName of the pool under test |
|  |  |  | A unique user friendly name not in the model (such as, asset name) |

Table 131 describes the alerts that are associated with this message.

Table 131 - Pool predicting failure due lack of available capacity Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The object name of the pool under test. |
| ALERT_TYPE | Y | 1 | Other (Low Available Capacity) |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

### 8.3.3	Fabric Messages

8.3.3.1	Message: Zone Database Changed

Owning Entity: SNIA

Message ID: FC1

Message Format String: Zone database changed for <Fabric Identity Type> <WWN>

An Indication when the fabric or switch has determined that the Zone Database has been modified. Table 132 describes the message arguments.

Table 132 - Zone Database Changed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Fabric Identity Type | string | Defines the type of fabric entity names by the following WWN. | fabric |
|  |  |  | switch |
| WWN | string | World Wide name identifier. The required form of the WWN is defined by this regular expression, "^[0123456789ABCDEF]{16}$" |  |

Table 133 describes the alerts that are associated with this message.

Table 133 - Zone Database Changed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | A reference to the switch or fabric which is named by the WWN. |

Message: ZoneSet Activated

Table 133 - Zone Database Changed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERT_TYPE | Y |  | Environmental Alert |
| PERCEIVED_SEVERITY | Y |  | Informational |

8.3.3.2	Message: ZoneSet Activated

Owning Entity: SNIA

Message ID: FC2

Message Format String: ZoneSet <ZoneSet Name> was activated for fabric <WWN>

An Indication when the fabric has determined that a ZoneSet has been activated. Table 134 describes the message arguments.

Table 134 - ZoneSet Activated Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| ZoneSet Name | string | CIM_ZoneSet.ElementName attribute |  |
| WWN | string | World Wide name identifier. The required form of the WWN is defined by this regular expression, "^[0123456789ABCDEF]{16}$" |  |

Table 135 describes the alerts that are associated with this message.

Table 135 - ZoneSet Activated Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | A reference to the fabric which is named by the WWN. |
| ALERT_TYPE | Y |  | Environmental Error |
| PERCEIVED_SEVERITY | Y |  | Minor |

EXPERIMENTAL

8.3.3.3	Message: Session Locked

Owning Entity: SNIA

Message ID: FC3

Message Format String: Operation blocked by session lock.

#### Message: Session Aborted

Table 136 describes the error properties.

Table 136 - Error Properties for Session Locked

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | ( CIM_ERR_FAILED ) | Existence is required |
| ERROR_TYPE | ( Software Error ) | Existence is required |
| ERROR_SOURCE |  | Existence is required |
| PERCEIVED_SEVERITY | (Degraded/Warning) | Existence is required |

EXPERIMENTAL

EXPERIMENTAL

8.3.3.4	Message: Session Aborted

Owning Entity: SNIA

Message ID: FC4

Message Format String: Operation by another client caused the session to be aborted.

Table 137 describes the error properties.

Table 137 - Error Properties for Session Aborted

| Property | Value | Description |
| --- | --- | --- |
| CIMSTATUSCODE | ( CIM_ERR_FAILED ) | Existence is required |
| ERROR_TYPE | ( Software Error ) | Existence is required |
| ERROR_SOURCE |  | Existence is required |
| PERCEIVED_SEVERITY | (Degraded/Warning) | Existence is required |

EXPERIMENTAL

8.3.3.5	Message: Switch Status Changed

Owning Entity: SNIA

Message ID: FC5

Message Format String: Switch <Switch Unique Identifier> in Fabric <Fabric Name> status changed to <Switch OperationalStatus>

Message: Fabric Merge/Segmentation

The fabric has detected a change in status of a switch in the fabric. Table 138 describes the message arguments.

Table 138 - Switch Status Changed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Switch Unique Identifier | string | The Switch Name (WWN). |  |
| Fabric Name | string | Fabric name. Typically the principal switch's WWN |  |
| Switch OperationalStatus | string | Switch Status |  |

Table 139 describes the alerts that are associated with this message.

Table 139 - Switch Status Changed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The Fabric |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y |  | Major |

8.3.3.6	Message: Fabric Merge/Segmentation

Owning Entity: SNIA

Message ID: FC6

Message Format String: <Fabric Name> has detected a <Fabric Change>

The fabric has detected either two fabrics have merged into a single fabric or a single fabric has segmented. . Table 140 describes the message arguments.

Table 140 - Fabric Merge/Segmentation Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Fabric Name | string | Fabric name. Typically the principal switch's WWN |  |
| Fabric Change | string | A value of merge or segmentation | merge |
|  |  |  | segmentation |

Table 141 describes the alerts that are associated with this message.

Table 141 - Fabric Merge/Segmentation Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The SAN |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y |  | Minor |

8.3.3.7	Message: Switch Added/Removed

Owning Entity: SNIA

#### Message: Fabric Added/Removed

Message ID: FC7

Message Format String: The fabric <Fabric Name> has detected switch <Switch Unique Identifier> has been <Fabric Change Type>

A Switch has been added or removed from the fabric. Table 142 describes the message arguments.

Table 142 - Switch Added/Removed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Fabric Name | string | Fabric name. Typically the principal switch's WWN |  |
| Switch Unique Identifier | string | The Switch Name (WWN). |  |
| Fabric Change Type | string | A value of added or removed | added |
|  |  |  | removed |

Table 143 describes the alerts that are associated with this message.

Table 143 - Switch Added/Removed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The Fabric |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y |  | Minor |

8.3.3.8	Message: Fabric Added/Removed

Owning Entity: SNIA

Message ID: FC8

Message Format String: Fabric <Fabric Identifier> was <Change Type>

The agent has detected the addition or removal of a fabric from the SAN. This message can also be used for Virtual Fabrics. Table 144 describes the message arguments.

Table 144 - Fabric Added/Removed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Fabric Identifier | string | The Fabric Identity |  |
| Change Type | string | A value of Added or Removed | added |
|  |  |  | removed |

Table 145 describes the alerts that are associated with this message.

Table 145 - Fabric Added/Removed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The SAN |

Message: Security Policy change

Table 145 - Fabric Added/Removed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y |  | Major |

EXPERIMENTAL

8.3.3.9	Message: Security Policy change

Owning Entity: SNIA

Message ID: FC9

Message Format String: Fabric Security Policy changed in <Fabric Name>

The fabric has detected a change in the Security Database. Table 146 describes the message arguments.

Table 146 - Security Policy change Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Fabric Name | string | Fabric name. Typically the principal switch's WWN |  |

Table 147 describes the alerts that are associated with this message.

Table 147 - Security Policy change Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The Fabric |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y |  | Minor |

EXPERIMENTAL

### 8.3.4	Filesystem Messages

EXPERIMENTAL

8.3.4.1	Message: System OperationalStatus Bellwether

Owning Entity: SNIA

Message ID: FSM1

Message Format String: The OperationalStatus of the <System Name> system has changed. Related elements will not report the change in their OperationalStatus.

#### Message: NetworkPort OperationalStatus Bellwether

A message indicating the change in OperationalStatus of a system is a bellwether alert. Table 148 describes the message arguments.

Table 148 - System OperationalStatus Bellwether Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| System Name | string | The Name property of the system whose OperationalStatus has changed. | The Name of a NAS System |
|  |  |  | The Name of a Component System |
|  |  |  | The Name of a Base Server System |
|  |  |  | The Name of a Virtual File Server System |

Table 149 describes the alerts that are associated with this message.

Table 149 - System OperationalStatus Bellwether Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.2	Message: NetworkPort OperationalStatus Bellwether

Owning Entity: SNIA

Message ID: FSM2

Message Format String: The OperationalStatus of the <NetworkPort Name> network port on the <System Name> system has changed. Related elements will not report the change in their OperationalStatus.

A message indicating the change in OperationalStatus of a NetworkPort is a bellwether alert. Table 150 describes the message arguments.

Table 150 - NetworkPort OperationalStatus Bellwether Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| NetworkPort Name | string | The ElementName property of the network port whose OperationalStatus has changed. |  |
| System Name | string | The Name property of the system on which the port exists | The Name of a NAS System |
|  |  |  | The Name of a Component System |
|  |  |  | The Name of a Base Server System |
|  |  |  | The Name of a Virtual File Server System |

Message: LogicalDisk OperationalStatus Bellwether

Table 151 describes the alerts that are associated with this message.

Table 151 - NetworkPort OperationalStatus Bellwether Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.3	Message: LogicalDisk OperationalStatus Bellwether

Owning Entity: SNIA

Message ID: FSM3

Message Format String: The OperationalStatus of the <LogicalDisk Name> logical disk on the <System Name> system has changed. Related elements will not report the change in their OperationalStatus.

A message indicating the change in OperationalStatus of a LogicalDisk is a bellwether alert. Table 152 describes the message arguments.

Table 152 - LogicalDisk OperationalStatus Bellwether Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| LogicalDisk Name | string | The Name property of the logical disk whose OperationalStatus has changed. |  |
| System Name | string | The Name property of the system on which the logical disk is known. | The Name of a NAS System |
|  |  |  | The Name of a Component System |
|  |  |  | The Name of a Base Server System |
|  |  |  | The Name of a Virtual File Server System |

Table 153 describes the alerts that are associated with this message.

Table 153 - LogicalDisk OperationalStatus Bellwether Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

#### Message: CopyState is set to Broken

EXPERIMENTAL

8.3.4.4	Message: CopyState is set to Broken

Owning Entity: SNIA

Message ID: FSM4

Message Format String: CopyState of an alerting Managed Element, which is an instance of CIM_Synchronized containing a SystemElement.ElementName= <Source Element Name> and SyncedElement.ElementName= <Target Element Name> , is set to Broken.

CopyState is set to Broken. Table 154 describes the message arguments.

Table 154 - CopyState is set to Broken Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Source Element Name | string | The textual equivalent (Value) for SystemElement.ElementName value |  |
| Target Element Name | string | The textual equivalent (Value) for SyncedElement.ElementName value |  |

Table 155 describes the alerts that are associated with this message.

Table 155 - CopyState is set to Broken Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.5	Message: Not Enough Space

Owning Entity: SNIA

Message ID: FSM5

Message Format String: Remaining pool space either below the warning threshold set for the

<StoragePool Name> or there is no remaining space in the <StoragePool Name>

Message: The changes in RemoteReplicationCollection

Not Enough Space. Table 156 describes the message arguments.

Table 156 - Not Enough Space Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| StoragePool Name | string | The textual equivalent (Value) for StoragePool.Name |  |
| StoragePool Name | string | The textual equivalent (Value) for StoragePool.Name |  |

Table 157 describes the alerts that are associated with this message.

Table 157 - Not Enough Space Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.6	Message: The changes in RemoteReplicationCollection

Owning Entity: SNIA

Message ID: FSM6

Message Format String: The collection of the paths that provide the access to a remote system for replication operations in <RemoteReplicationCollection Name> are changed

The changes in RemoteReplicationCollection. Table 158 describes the message arguments.

Table 158 - The changes in RemoteReplicationCollection Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| RemoteReplicationCollecti on Name | string | The textual equivalent (Value) for Synchronized.SystemElement value |  |

#### Message: The changes in ProtocolEndpoint

Table 159 describes the alerts that are associated with this message.

Table 159 - The changes in RemoteReplicationCollection Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.7	Message: The changes in ProtocolEndpoint

Owning Entity: SNIA

Message ID: FSM7

Message Format String: The networking protocol <ProtocolEndpoint Name> which enables a replication service to reach a remote element is changed.

The changes in ProtocolEndpoint. Table 160 describes the message arguments.

Table 160 - The changes in ProtocolEndpoint Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| ProtocolEndpoint Name | string | The textual equivalent (Value) for ProtocolEndpoint.Name |  |

Table 161 describes the alerts that are associated with this message.

Table 161 - The changes in ProtocolEndpoint Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.8	Message: CopyState is set to Broken

Owning Entity: SNIA

Message ID: FSM8

Message: CopyState is set to Invalid

Message Format String: CopyState of an alerting Managed Element, which is an instance of CIM_Synchronized containing a SystemElement.ElementName= <Source Element Name> and SyncedElement.ElementName= <Target Element Name> , is set to Fractured.

CopyState is set to Fractured. Table 162 describes the message arguments.

Table 162 - CopyState is set to Broken Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Source Element Name | string | The textual equivalent (Value) for SystemElement.ElementName value |  |
| Target Element Name | string | The textual equivalent (Value) for SyncedElement.ElementName value |  |

Table 163 describes the alerts that are associated with this message.

Table 163 - CopyState is set to Broken Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.9	Message: CopyState is set to Invalid

Owning Entity: SNIA

Message ID: FSM9

Message Format String: CopyState of an alerting Managed Element, which is an instance of CIM_Synchronized containing a SystemElement.ElementName= <Source Element Name> and SyncedElement.ElementName= <Target Element Name> , is set to Invalid.

CopyState is set to Invalid. Table 164 describes the message arguments.

Table 164 - CopyState is set to Invalid Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Source Element Name | string | The textual equivalent (Value) for SystemElement.ElementName value |  |
| Target Element Name | string | The textual equivalent (Value) for SyncedElement.ElementName value |  |

#### Message: CopyState is set to Inactive

Table 165 describes the alerts that are associated with this message.

Table 165 - CopyState is set to Invalid Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.10	Message: CopyState is set to Inactive

Owning Entity: SNIA

Message ID: FSM10

Message Format String: CopyState of an alerting Managed Element, which is an instance of CIM_Synchronized containing a SystemElement.ElementName= <Source Element Name> and SyncedElement.ElementName= <Target Element Name> , is set to Inactive.

CopyState is set to Inactive. Table 166 describes the message arguments.

Table 166 - CopyState is set to Inactive Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Source Element Name | string | The textual equivalent (Value) for SystemElement.ElementName value |  |
| Target Element Name | string | The textual equivalent (Value) for SyncedElement.ElementName value |  |

Table 167 describes the alerts that are associated with this message.

Table 167 - CopyState is set to Inactive Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 4 | Minor |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.11	Message: CopyState is set to Split

Owning Entity: SNIA

Message: CopyState alert has been cleared

Message ID: FSM11

Message Format String: CopyState of an alerting Managed Element, which is an instance of CIM_Synchronized containing a SystemElement.ElementName= <Source Element Name> and SyncedElement.ElementName= <Target Element Name> , is set to Split.

CopyState is set to Split. Table 168 describes the message arguments.

Table 168 - CopyState is set to Split Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Source Element Name | string | The textual equivalent (Value) for SystemElement.ElementName value |  |
| Target Element Name | string | The textual equivalent (Value) for SyncedElement.ElementName value |  |

Table 169 describes the alerts that are associated with this message.

Table 169 - CopyState is set to Split Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 4 | Minor |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.12	Message: CopyState alert has been cleared

Owning Entity: SNIA

Message ID: FSM12

Message Format String: CopyState of an alerting Managed Element, which is an instance of

CIM_Synchronized containing a SystemElement.ElementName= <Source Element Name> and SyncedElement.ElementName= <Target Element Name> , is set to a non-adverse condition.

CopyState adverse condition cleared. Table 170 describes the message arguments.

Table 170 - CopyState alert has been cleared Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Source Element Name | string | The textual equivalent (Value) for SystemElement.ElementName value |  |
| Target Element Name | string | The textual equivalent (Value) for SyncedElement.ElementName value |  |

#### Message: Available Space Changed

Table 171 describes the alerts that are associated with this message.

Table 171 - CopyState alert has been cleared Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.13	Message: Available Space Changed

Owning Entity: SNIA

Message ID: FSM13

Message Format String: The available space of the filesystem <Filesystem Name> has changed

The available space of a filesystem has changed. Table 172 describes the message arguments.

Table 172 - Available Space Changed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Filesystem Name | string | The textual equivalent (Value) for Filesystem.Name |  |

Table 173 describes the alerts that are associated with this message.

Table 173 - Available Space Changed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.14	Message: Filesystem Inaccessible

Owning Entity: SNIA

Message ID: FSM14

Message Format String: The filesystem <Filesystem Name> is not accessible

Message: Filesystem is Online

The filesystem is not accessible. Table 174 describes the message arguments.

Table 174 - Filesystem Inaccessible Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Filesystem Name | string | The textual equivalent (Value) for Filesystem.Name |  |

Table 175 describes the alerts that are associated with this message.

Table 175 - Filesystem Inaccessible Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.15	Message: Filesystem is Online

Owning Entity: SNIA

Message ID: FSM15

Message Format String: The filesystem <Filesystem Name> is back online

The filesystem is online. Table 176 describes the message arguments.

Table 176 - Filesystem is Online Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Filesystem Name | string | The textual equivalent (Value) for Filesystem.Name |  |

Table 177 describes the alerts that are associated with this message.

Table 177 - Filesystem is Online Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

#### Message: Fileshare is degraded

EXPERIMENTAL

8.3.4.16	Message: Fileshare is degraded

Owning Entity: SNIA

Message ID: FSM16

Message Format String: The fileshare <Fileshare Name> is in a degraded state

The fileshare state is degraded. Table 178 describes the message arguments.

Table 178 - Fileshare is degraded Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Fileshare Name | string | The textual equivalent (Value) for Fileshare.Name |  |

Table 179 describes the alerts that are associated with this message.

Table 179 - Fileshare is degraded Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.4.17	Message: Fileshare in normal state

Owning Entity: SNIA

Message ID: FSM17

Message Format String: The fileshare <Fileshare Name> has returned to normal conditions.

The fileshare has returned to normal operations. Table 180 describes the message arguments.

Table 180 - Fileshare in normal state Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Fileshare Name | string | The textual equivalent (Value) for Fileshare.Name |  |

Message: Required Firmware Version

Table 181 describes the alerts that are associated with this message.

Table 181 - Fileshare in normal state Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

### 8.3.5	Host Messages

8.3.5.1	Message: Required Firmware Version

Owning Entity: SNIA

Message ID: Host1

Message Format String: Controller firmware is older than required. Current Version: <Current Version> Minimum required version: <Minimum required version>

A message indicating the controller firmware is older than required. Table 182 describes the message arguments.

Table 182 - Required Firmware Version Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Current Version | string | The current firmware version number. |  |
| Minimum required version | string | The minimum required version number |  |

Table 183 describes the alerts that are associated with this message.

Table 183 - Required Firmware Version Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The controller. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

8.3.5.2	Message: Recommended Firmware Version

Owning Entity: SNIA

Message ID: Host2

Message Format String: Controller firmware is older than recommended. Current Version: <Current

Version> Minimum recommended version: <Minimum recommended version>

#### Message: Controller OK

A message indicating the controller firmware is older than recommended. Table 184 describes the message arguments.

Table 184 - Recommended Firmware Version Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Current Version | string | The current firmware version number. |  |
| Minimum recommended version | string | The minimum recommended version number |  |

Table 185 describes the alerts that are associated with this message.

Table 185 - Recommended Firmware Version Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The controller. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 4 | Minor |

8.3.5.3	Message: Controller OK

Owning Entity: SNIA

Message ID: Host3

Message Format String: Controller health is ok. Controller Name: <Controller Name>

Table 186 describes the message arguments.

Table 186 - Controller OK Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Controller Name | string | Controller Name. |  |

Table 187 describes the alerts that are associated with this message.

Table 187 - Controller OK Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The controller. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

8.3.5.4	Message: Controller not OK

Owning Entity: SNIA

Message ID: Host4

Message Format String: Controller health is not ok. Controller Name: <Controller Name>

Message: Bus rescan complete

Table 188 describes the message arguments.

Table 188 - Controller not OK Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Controller Name | string | Controller Name. |  |

Table 189 describes the alerts that are associated with this message.

Table 189 - Controller not OK Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The controller. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

8.3.5.5	Message: Bus rescan complete

Owning Entity: SNIA

Message ID: Host5

Message Format String: Bus rescan complete

Table 190 describes the alerts that are associated with this message.

Table 190 - Bus rescan complete Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The controller. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 4 | Minor |

8.3.5.6	Message: Disk initialize Failed

Owning Entity: SNIA

Message ID: Host6

Message Format String: Disk Initialize Failed. Disk name: <Disk Name>

Table 191 describes the message arguments.

Table 191 - Disk initialize Failed Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| Disk Name | string | Disk Name. |  |

Message: Read Warning

Table 192 describes the alerts that are associated with this message.

Table 192 - Disk initialize Failed Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | The system containing the controller. |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Major |

### 8.3.6	Media Library Messages

8.3.6.1	Message: Read Warning

Owning Entity: SNIA

Message ID: SML1

Message Format String: The drive is having severe trouble reading.

Table 193 describes the alerts that are associated with this message.

Table 193 - Read Warning Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.2	Message: Write Warning

Owning Entity: SNIA

Message ID: SML2

Message Format String: The drive is having severe trouble writing.

Table 194 describes the alerts that are associated with this message.

Table 194 - Write Warning Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 4 | Warning |

8.3.6.3	Message: Hard Error

Owning Entity: SNIA

Message ID: SML3

Message Format String: The drive had a hard read or write error.

Message: Media

Table 195 describes the alerts that are associated with this message.

Table 195 - Hard Error Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 5 | Warning |

8.3.6.4	Message: Media

Owning Entity: SNIA

Message ID: SML4

Message Format String: Media can no longer be written/read, or performance is severely degraded.

Table 196 describes the alerts that are associated with this message.

Table 196 - Media Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.5	Message: Read Failure

Owning Entity: SNIA

Message ID: SML5

Message Format String: The drive can no longer read data from the storage media.

Table 197 describes the alerts that are associated with this message.

Table 197 - Read Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.6	Message: Write Failure

Owning Entity: SNIA

Message ID: SML6

Message Format String: The drive can no longer write data to the media.

#### Message: Media Life

Table 198 describes the alerts that are associated with this message.

Table 198 - Write Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.7	Message: Media Life

Owning Entity: SNIA

Message ID: SML7

Message Format String: The media has exceeded its specified life.

Table 199 describes the alerts that are associated with this message.

Table 199 - Media Life Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.8	Message: Not Data Grade

Owning Entity: SNIA

Message ID: SML8

Message Format String: The cartridge is not data-grade. Any data you write to the media is at risk. Replace the cartridge with a data-grade media.

Table 200 describes the alerts that are associated with this message.

Table 200 - Not Data Grade Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.9	Message: Write Protect

Owning Entity: SNIA

Message ID: SML9

Message Format String: Write command is attempted to a write protected media.

Message: No Removal

Table 201 describes the alerts that are associated with this message.

Table 201 - Write Protect Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.10	Message: No Removal

Owning Entity: SNIA

Message ID: SML10

Message Format String: Manual or software unload attempted when prevent media removal is on.

Table 202 describes the alerts that are associated with this message.

Table 202 - No Removal Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.6.11	Message: Cleaning Media

Owning Entity: SNIA

Message ID: SML11

Message Format String: Cleaning media loaded into drive

Table 203 describes the alerts that are associated with this message.

Table 203 - Cleaning Media Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.6.12	Message: Unsupported Format

Owning Entity: SNIA

Message ID: SML12

Message Format String: Attempted load of unsupported media format (e.g., DDS2 in DDS1 drive).

#### Message: Recoverable Snapped Tape

Table 204 describes the alerts that are associated with this message.

Table 204 - Unsupported Format Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.6.13	Message: Recoverable Snapped Tape

Owning Entity: SNIA

Message ID: SML13

Message Format String: Tape snapped/cut in the drive where media can be de-mounted.

Table 205 describes the alerts that are associated with this message.

Table 205 - Recoverable Snapped Tape Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.14	Message: Unrecoverable Snapped Tape

Owning Entity: SNIA

Message ID: SML14

Message Format String: Tape snapped/cut in the drive where media cannot be de-mounted.

Table 206 describes the alerts that are associated with this message.

Table 206 - Unrecoverable Snapped Tape Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.15	Message: Memory Chip In Cartridge Failure

Owning Entity: SNIA

Message ID: SML15

Message Format String: Memory chip failed in cartridge.

Message: Forced Eject

Table 207 describes the alerts that are associated with this message.

Table 207 - Memory Chip In Cartridge Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.16	Message: Forced Eject

Owning Entity: SNIA

Message ID: SML16

Message Format String: Manual or forced eject while drive actively writing or reading.

Table 208 describes the alerts that are associated with this message.

Table 208 - Forced Eject Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.17	Message: Read Only Format

Owning Entity: SNIA

Message ID: SML17

Message Format String: Media loaded that is read-only format.

Table 209 describes the alerts that are associated with this message.

Table 209 - Read Only Format Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.18	Message: Directory Corrupted On Load

Owning Entity: SNIA

Message ID: SML18

Message Format String: Drive powered down while loaded, or permanent error prevented the directory being updated.

#### Message: Nearing Media Life

Table 210 describes the alerts that are associated with this message.

Table 210 - Directory Corrupted On Load Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.19	Message: Nearing Media Life

Owning Entity: SNIA

Message ID: SML19

Message Format String: Media may have exceeded its specified number of passes.

Table 211 describes the alerts that are associated with this message.

Table 211 - Nearing Media Life Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.6.20	Message: Clean Now

Owning Entity: SNIA

Message ID: SML20

Message Format String: The drive thinks it has a head clog or needs cleaning.

Table 212 describes the alerts that are associated with this message.

Table 212 - Clean Now Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.21	Message: Clean Periodic

Owning Entity: SNIA

Message ID: SML21

Message Format String: The drive is ready for a periodic cleaning.

Message: Expired Cleaning Media

Table 213 describes the alerts that are associated with this message.

Table 213 - Clean Periodic Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.22	Message: Expired Cleaning Media

Owning Entity: SNIA

Message ID: SML22

Message Format String: The cleaning media has expired.

Table 214 describes the alerts that are associated with this message.

Table 214 - Expired Cleaning Media Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.23	Message: Invalid Cleaning Media

Owning Entity: SNIA

Message ID: SML23

Message Format String: Invalid cleaning media type used.

Table 215 describes the alerts that are associated with this message.

Table 215 - Invalid Cleaning Media Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.24	Message: Retention Requested

Owning Entity: SNIA

Message ID: SML24

Message Format String: The drive is having severe trouble reading or writing, which will be resolved by a retention cycle.

#### Message: Dual-Port Interface Error

Table 216 describes the alerts that are associated with this message.

Table 216 - Retention Requested Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Information |

8.3.6.25	Message: Dual-Port Interface Error

Owning Entity: SNIA

Message ID: SML25

Message Format String: Failure of one interface port in a dual-port configuration (i.e., Fibre Channel)

Table 217 describes the alerts that are associated with this message.

Table 217 - Dual-Port Interface Error Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.26	Message: Drive Maintenance

Owning Entity: SNIA

Message ID: SML26

Message Format String: The drive requires preventive maintenance (not cleaning).

Table 218 describes the alerts that are associated with this message.

Table 218 - Drive Maintenance Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.27	Message: Hardware A

Owning Entity: SNIA

Message ID: SML27

Message Format String: The drive has a hardware fault that requires reset to recover.

Message: Hardware B

Table 219 describes the alerts that are associated with this message.

Table 219 - Hardware A Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.28	Message: Hardware B

Owning Entity: SNIA

Message ID: SML28

Message Format String: The drive has a hardware fault that is not read/write related or requires a power cycle to recover.

Table 220 describes the alerts that are associated with this message.

Table 220 - Hardware B Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.29	Message: Interface

Owning Entity: SNIA

Message ID: SML29

Message Format String: The drive has identified an interface fault.

Table 221 describes the alerts that are associated with this message.

Table 221 - Interface Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.30	Message: Eject Media

Owning Entity: SNIA

Message ID: SML30

Message Format String: Error recovery action: Media Ejected

#### Message: Download Failure

Table 222 describes the alerts that are associated with this message.

Table 222 - Eject Media Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.31	Message: Download Failure

Owning Entity: SNIA

Message ID: SML31

Message Format String: Firmware download failed.

Table 223 describes the alerts that are associated with this message.

Table 223 - Download Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degraded/Warning |

8.3.6.32	Message: Loader Hardware A

Owning Entity: SNIA

Message ID: SML32

Message Format String: Loader mechanism is having trouble communicating with the drive.

Table 224 describes the alerts that are associated with this message.

Table 224 - Loader Hardware A Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.33	Message: Loader Stray Media

Owning Entity: SNIA

Message ID: SML33

Message Format String: Stray media left in loader after previous error recovery.

Message: Loader Hardware B

Table 225 describes the alerts that are associated with this message.

Table 225 - Loader Stray Media Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.34	Message: Loader Hardware B

Owning Entity: SNIA

Message ID: SML34

Message Format String: Loader mechanism has a hardware fault.

Table 226 describes the alerts that are associated with this message.

Table 226 - Loader Hardware B Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.35	Message: Loader Door

Owning Entity: SNIA

Message ID: SML35

Message Format String: Changer door open.

Table 227 describes the alerts that are associated with this message.

Table 227 - Loader Door Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.36	Message: Loader Hardware C

Owning Entity: SNIA Message ID: SML36

Message Format String: The loader mechanism has a hardware fault that is not mechanically related.

Message: Loader Magazine

Table 228 describes the alerts that are associated with this message.

Table 228 - Loader Hardware C Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.37	Message: Loader Magazine

Owning Entity: SNIA

Message ID: SML37

Message Format String: Loader magazine not present.

Table 229 describes the alerts that are associated with this message.

Table 229 - Loader Magazine Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.38	Message: Loader Predictive Failure

Owning Entity: SNIA

Message ID: SML38

Message Format String: Predictive failure of loader mechanism hardware

Table 230 describes the alerts that are associated with this message.

Table 230 - Loader Predictive Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.39	Message: Load Statistics

Owning Entity: SNIA

Message ID: SML39

Message Format String: Drive or library powered down with media loaded.

Message: Media Directory Invalid at Unload

Table 231 describes the alerts that are associated with this message.

Table 231 - Load Statistics Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice or CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.40	Message: Media Directory Invalid at Unload

Owning Entity: SNIA

Message ID: SML40

Message Format String: Error preventing the media directory being updated on unload.

Table 232 describes the alerts that are associated with this message.

Table 232 - Media Directory Invalid at Unload Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.41	Message: Media System area Write Failure

Owning Entity: SNIA

Message ID: SML41

Message Format String: Write errors while writing the system area on unload.

Table 233 describes the alerts that are associated with this message.

Table 233 - Media System area Write Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.42	Message: Media System Area Read Failure

Owning Entity: SNIA

Message ID: SML42

Message Format String: Read errors while reading the system area on load.

#### Message: No Start of Data

Table 234 describes the alerts that are associated with this message.

Table 234 - Media System Area Read Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.43	Message: No Start of Data

Owning Entity: SNIA

Message ID: SML43

Message Format String: Media damaged, bulk erased, or incorrect format.

Table 235 describes the alerts that are associated with this message.

Table 235 - No Start of Data Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.44	Message: Loading Failure

Owning Entity: SNIA

Message ID: SML44

Message Format String: The drive is unable to load the media

Table 236 describes the alerts that are associated with this message.

Table 236 - Loading Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_MediaAccessDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.45	Message: Library Hardware A

Owning Entity: SNIA

Message ID: SML45

Message Format String: Changer mechanism is having trouble communicating with the internal drive

Message: Library Hardware B

Table 237 describes the alerts that are associated with this message.

Table 237 - Library Hardware A Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.46	Message: Library Hardware B

Owning Entity: SNIA

Message ID: SML46

Message Format String: Changer mechanism has a hardware fault

Table 238 describes the alerts that are associated with this message.

Table 238 - Library Hardware B Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.47	Message: Library Hardware C

Owning Entity: SNIA Message ID: SML47

Message Format String: The changer mechanism has a hardware fault that requires a reset to recover.

Table 239 describes the alerts that are associated with this message.

Table 239 - Library Hardware C Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.48	Message: Library Hardware D

Owning Entity: SNIA

Message ID: SML48

Message Format String: The changer mechanism has a hardware fault that is not mechanically related or requires a power cycle to recover.

#### Message: Library Diagnostic Required

Table 240 describes the alerts that are associated with this message.

Table 240 - Library Hardware D Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.49	Message: Library Diagnostic Required

Owning Entity: SNIA

Message ID: SML49

Message Format String: The changer mechanism may have a hardware fault which would be identified by extended diagnostics.

Table 241 describes the alerts that are associated with this message.

Table 241 - Library Diagnostic Required Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.50	Message: Library Interface

Owning Entity: SNIA

Message ID: SML50

Message Format String: The library has identified an interface fault

Table 242 describes the alerts that are associated with this message.

Table 242 - Library Interface Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.51	Message: Failure Prediction

Owning Entity: SNIA

Message ID: SML51

Message Format String: Predictive failure of library hardware

Message: Library Maintenance

Table 243 describes the alerts that are associated with this message.

Table 243 - Failure Prediction Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ChangerDevice |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.52	Message: Library Maintenance

Owning Entity: SNIA

Message ID: SML52

Message Format String: Library preventative maintenance required.

Table 244 describes the alerts that are associated with this message.

Table 244 - Library Maintenance Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.53	Message: Library Humidity Limits

Owning Entity: SNIA

Message ID: SML53

Message Format String: Library humidity limits exceeded

Table 245 describes the alerts that are associated with this message.

Table 245 - Library Humidity Limits Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.54	Message: Library Voltage Limits

Owning Entity: SNIA

Message ID: SML54

Message Format String: Library voltage limits exceeded

#### Message: Library Stray Media

Table 246 describes the alerts that are associated with this message.

Table 246 - Library Voltage Limits Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.55	Message: Library Stray Media

Owning Entity: SNIA

Message ID: SML55

Message Format String: Stray cartridge left in library after previous error recovery

Table 247 describes the alerts that are associated with this message.

Table 247 - Library Stray Media Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.56	Message: Library Pick Retry

Owning Entity: SNIA

Message ID: SML56

Message Format String: Operation to pick a cartridge from a slot had to perform an excessive number of retries before succeeding

Table 248 describes the alerts that are associated with this message.

Table 248 - Library Pick Retry Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.57	Message: Library Place Retry

Owning Entity: SNIA

Message ID: SML57

Message Format String: Operation to place a cartridge in a slot had to perform an excessive number of retries before succeeding

Message: Library Load Retry

Table 249 describes the alerts that are associated with this message.

Table 249 - Library Place Retry Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.58	Message: Library Load Retry

Owning Entity: SNIA

Message ID: SML58

Message Format String: Operation to load a cartridge in a drive had to perform an excessive number of retries before succeeding

Table 250 describes the alerts that are associated with this message.

Table 250 - Library Load Retry Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.59	Message: Library Door

Owning Entity: SNIA

Message ID: SML59

Message Format String: Library door open is preventing the library from functioning

Table 251 describes the alerts that are associated with this message.

Table 251 - Library Door Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.60	Message: Library Mailslot

Owning Entity: SNIA

Message ID: SML60

Message Format String: Mechanical problem with import/export mailslot

Message: Library Magazine

Table 252 describes the alerts that are associated with this message.

Table 252 - Library Mailslot Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.61	Message: Library Magazine

Owning Entity: SNIA

Message ID: SML61

Message Format String: Library magazine not present

Table 253 describes the alerts that are associated with this message.

Table 253 - Library Magazine Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.62	Message: Library Security

Owning Entity: SNIA

Message ID: SML62

Message Format String: Library door opened then closed during operation

Table 254 describes the alerts that are associated with this message.

Table 254 - Library Security Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.63	Message: Library Security Mode

Owning Entity: SNIA

Message ID: SML63

Message Format String: Library security mode changed

Message: Library Offline

Table 255 describes the alerts that are associated with this message.

Table 255 - Library Security Mode Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.6.64	Message: Library Offline

Owning Entity: SNIA

Message ID: SML64

Message Format String: Library manually turned offline

Table 256 describes the alerts that are associated with this message.

Table 256 - Library Offline Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.6.65	Message: Library Drive Offline

Owning Entity: SNIA

Message ID: SML65

Message Format String: Library turned internal drive offline.

Table 257 describes the alerts that are associated with this message.

Table 257 - Library Drive Offline Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

8.3.6.66	Message: Library Scan Retry

Owning Entity: SNIA

Message ID: SML66

Message Format String: Operation to scan the bar code on a cartridge had to perform an excessive number of retries before succeeding

#### Message: Library Inventory

Table 258 describes the alerts that are associated with this message.

Table 258 - Library Scan Retry Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.67	Message: Library Inventory

Owning Entity: SNIA

Message ID: SML67

Message Format String: Inconsistent media inventory

Table 259 describes the alerts that are associated with this message.

Table 259 - Library Inventory Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.68	Message: Library Illegal Operation

Owning Entity: SNIA

Message ID: SML68

Message Format String: Illegal operation detected

Table 260 describes the alerts that are associated with this message.

Table 260 - Library Illegal Operation Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Degrading/Warning |

8.3.6.69	Message: Pass Through Mechanism Failure

Owning Entity: SNIA

Message ID: SML69

Message Format String: Error occurred in pass-through mechanism during self test or while attempting to transfer a cartridge between library modules

Message: Cartridge in Pass-through Mechanism

Table 261 describes the alerts that are associated with this message.

Table 261 - Pass Through Mechanism Failure Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.70	Message: Cartridge in Pass-through Mechanism

Owning Entity: SNIA

Message ID: SML70

Message Format String: Cartridge left in the pass-through mechanism between two library modules

Table 262 describes the alerts that are associated with this message.

Table 262 - Cartridge in Pass-through Mechanism Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

8.3.6.71	Message: Unreadable barcode Labels

Owning Entity: SNIA

Message ID: SML71

Message Format String: Unable to read a bar code label on a cartridge during library inventory/scan

Table 263 describes the alerts that are associated with this message.

Table 263 - Unreadable barcode Labels Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 2 | Information |

EXPERIMENTAL

8.3.6.72	Message: Throughput Threshold Warning Alert

Owning Entity: SNIA

Message ID: SML72

Message Format String: The throughput threshold has exceeded the warning level

<ThroughputWarningAlertThreshold> of the <Computer System> system

Message: Throughput Threshold Critical Alert

Table 264 describes the message arguments.

Table 264 - Throughput Threshold Warning Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| ThroughputWarningAlertT hreshold | string | A string rendering of the CIM_VTLResourceUsage.ThroughputWar ningAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 265 describes the alerts that are associated with this message.

Table 265 - Throughput Threshold Warning Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.6.73	Message: Throughput Threshold Critical Alert

Owning Entity: SNIA

Message ID: SML73

Message Format String: The throughput threshold has exceeded the critical level <ThroughputCriticalAlertThreshold> of the <Computer System> system

Table 266 describes the message arguments.

Table 266 - Throughput Threshold Critical Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| ThroughputCriticalAlertThr eshold | string | A string rendering of the CIM_VTLResourceUsage.ThroughputCriti calAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Message: Physical Capacity Threshold Warning Alert

Table 267 describes the alerts that are associated with this message.

Table 267 - Throughput Threshold Critical Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

EXPERIMENTAL

EXPERIMENTAL

8.3.6.74	Message: Physical Capacity Threshold Warning Alert

Owning Entity: SNIA

Message ID: SML74

Message Format String: The physical capacity threshold has exceeded the warning level <PhysicalCapacityWarningAlertThreshold> of the <Computer System> system

Table 268 describes the message arguments.

Table 268 - Physical Capacity Threshold Warning Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| PhysicalCapacityWarningA lertThreshold | string | A string rendering of the CIM_VTLResourceUsage.PhysicalCapaci tyWarningAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 269 describes the alerts that are associated with this message.

Table 269 - Physical Capacity Threshold Warning Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Warning |

EXPERIMENTAL

Message: Physical Capacity Threshold Critical Alert

EXPERIMENTAL

8.3.6.75	Message: Physical Capacity Threshold Critical Alert

Owning Entity: SNIA

Message ID: SML75

Message Format String: The physical capacity threshold has exceeded the critical level <PhysicalCapacityCriticalAlertThreshold> of the <Computer System> system

Table 270 describes the message arguments.

Table 270 - Physical Capacity Threshold Critical Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| PhysicalCapacityCriticalAl ertThreshold | string | A string rendering of the CIM_VTLResourceUsage.PhysicalCapaci tyCriticalAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 271 describes the alerts that are associated with this message.

Table 271 - Physical Capacity Threshold Critical Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

EXPERIMENTAL

EXPERIMENTAL

8.3.6.76	Message: Logical Capacity Threshold Warning Alert

Owning Entity: SNIA

Message ID: SML76

Message Format String: The logical capacity threshold has exceeded the warning level

<LogicalCapacityWarningAlertThreshold> of the <Computer System> system

Message: Logical Capacity Threshold Critical Alert

Table 272 describes the message arguments.

Table 272 - Logical Capacity Threshold Warning Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| LogicalCapacityWarningAl ertThreshold | string | A string rendering of the CIM_VTLResourceUsage.LogicalCapacity WarningAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 273 describes the alerts that are associated with this message.

Table 273 - Logical Capacity Threshold Warning Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.6.77	Message: Logical Capacity Threshold Critical Alert

Owning Entity: SNIA

Message ID: SML77

Message Format String: The logical capacity threshold has exceeded the critical level <LogicalCapacityCriticalAlertThreshold> of the <Computer System> system

Table 274 describes the message arguments.

Table 274 - Logical Capacity Threshold Critical Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| LogicalCapacityCriticalAler tThreshold | string | A string rendering of the CIM_VTLResourceUsage.LogicalCapacity CriticalAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Message: System Ratio Threshold Warning Alert

Table 275 describes the alerts that are associated with this message.

Table 275 - Logical Capacity Threshold Critical Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

EXPERIMENTAL

EXPERIMENTAL

8.3.6.78	Message: System Ratio Threshold Warning Alert

Owning Entity: SNIA

Message ID: SML78

Message Format String: The system ratio has fallen below the warning level threshold <SystemRatioWarningAlertThreshold> of the <Computer System> system

Table 276 describes the message arguments.

Table 276 - System Ratio Threshold Warning Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| SystemRatioWarningAlert Threshold | string | A string rendering of the CIM_VTLResourceUsage.LogicalCapacity CriticalAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 277 describes the alerts that are associated with this message.

Table 277 - System Ratio Threshold Warning Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Warning |

EXPERIMENTAL

Message: System Ratio Threshold Critical Alert

EXPERIMENTAL

8.3.6.79	Message: System Ratio Threshold Critical Alert

Owning Entity: SNIA

Message ID: SML79

Message Format String: The system ratio threshold has fallen below the critical level threshold <SystemRatioCriticalAlertThreshold> of the <Computer System> system

Table 278 describes the message arguments.

Table 278 - System Ratio Threshold Critical Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| SystemRatioCriticalAlertTh reshold | string | A string rendering of the CIM_VTLResourceUsage.SystemRatioCri ticalAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 279 describes the alerts that are associated with this message.

Table 279 - System Ratio Threshold Critical Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

EXPERIMENTAL

EXPERIMENTAL

8.3.6.80	Message: Deduplication Ratio Threshold Warning Alert

Owning Entity: SNIA

Message ID: SML80

Message Format String: The deduplication ratio has fallen below the warning level threshold

<DeduplicationRatioWarningAlertThreshold> of the <Computer System> system

#### Message: Deduplication Ratio Threshold Critical Alert

Table 280 describes the message arguments.

Table 280 - Deduplication Ratio Threshold Warning Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| DeduplicationRatioWarnin gAlertThreshold | string | A string rendering of the CIM_VTLResourceUsage.DeduplicationR atioWarningAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 281 describes the alerts that are associated with this message.

Table 281 - Deduplication Ratio Threshold Warning Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Warning |

EXPERIMENTAL

EXPERIMENTAL

8.3.6.81	Message: Deduplication Ratio Threshold Critical Alert

Owning Entity: SNIA

Message ID: SML81

Message Format String: The deduplication ratio threshold has fallen below the crititcal level threshold <DeduplicationRatioCriticalAlertThreshold> of the <Computer System> system

Table 282 describes the message arguments.

Table 282 - Deduplication Ratio Threshold Critical Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| DeduplicationRatioCritical AlertThreshold | string | A string rendering of the CIM_VTLResourceUsage.DeduplicationR atioCriticalAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Message: Replication Traffic Threshold Warning Alert

Table 283 describes the alerts that are associated with this message.

Table 283 - Deduplication Ratio Threshold Critical Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

EXPERIMENTAL

EXPERIMENTAL

8.3.6.82	Message: Replication Traffic Threshold Warning Alert

Owning Entity: SNIA

Message ID: SML82

Message Format String: The replication traffic threshold has exceeded the warning level <ReplicationTrafficWarningAlertThreshold> of the <Computer System> system

Table 284 describes the message arguments.

Table 284 - Replication Traffic Threshold Warning Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| ReplicationTrafficWarning AlertThreshold | string | A string rendering of the CIM_VTLResourceUsage.ReplicationTraff icWarningAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 285 describes the alerts that are associated with this message.

Table 285 - Replication Traffic Threshold Warning Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 3 | Warning |

EXPERIMENTAL

Message: Replication Traffic Threshold Critical Alert

EXPERIMENTAL

8.3.6.83	Message: Replication Traffic Threshold Critical Alert

Owning Entity: SNIA

Message ID: SML83

Message Format String: The replication traffic threshold has exceeded the critical level <ReplicationTrafficCriticalAlertThreshold> of the <Computer System> system

Table 286 describes the message arguments.

Table 286 - Replication Traffic Threshold Critical Alert Message Arguments

| Message Argument | Data Type | Description | Possible Values |
| --- | --- | --- | --- |
| ReplicationTrafficCriticalAl ertThreshold | string | A string rendering of the CIM_VTLResourceUsage.ReplicationTraff icCriticalAlertThreshold property. | A number between 0 and 1 |
| Computer System | string | The Name property of the Virtual Library System or Virtual Tape Library | The Name of a Virtual Library System |
|  |  |  | The Name of a Virtual Tape Library System |

Table 287 describes the alerts that are associated with this message.

Table 287 - Replication Traffic Threshold Critical Alert Alert Information

| Name | Req | Value | Description |
| --- | --- | --- | --- |
| ALERTING_MANAGED_ELEMENT | Y |  | CIM_ComputerSystem |
| ALERT_TYPE | Y |  | Device Alert |
| PERCEIVED_SEVERITY | Y | 6 | Critical |

EXPERIMENTAL

# 1	9  Service Discovery

## 2	9.1	Objectives

3 Service discovery in the context of SMI-S refers to the discovery of dedicated SMI-S servers, general 4 purpose SMI-S servers, and directory servers, and the functions they offer in an SMI-S managed 5 environment. The specific objectives to be addressed by the discovery architecture are:

6	1)	Provide a mechanism that allows SMI-S clients to discover the SMI-S constituents in a storage network envi7	ronment so that they may communicate with these constituents using CIM Operations over HTTP protocol.

- This includes:

- • Finding the address for the SMI-S constituent;

- • Finding the capabilities of the server, including communications capabilities, security capabilities, CIM

- operational capabilities and the functional capabilities (CQL, Batch operations support, etc.);

- 2)	Provide a mechanism that is efficient in the amount of information exchanged with minimal 13	exchanges to acquire the information;

14 3) Provide a mechanism that accurately defines the services in the network, independent of whether or 15 not those services are currently available;

- 4)	Provide a mechanism that provides information on namespaces provided and the CIM Schema sup-

- ported;

- 5)	Provide a mechanism that allows SMI-S clients the profile(s) supported by agents and object man-

- agers;

- 6)	Provide a mechanism that scales to enterprise environments;

- 7)	Utilize existing standard mechanisms to effect the SMI-S service discovery to enable rapid deploy-

- ment;

- 8)	Provide a mechanism that allows SMI-S clients to determine the level of (SMI-S) support provided 24	by the constituents (e.g., R1, R2, etc.)

## 25	9.2	Overview

26 SMI-S uses the Service Location Protocol Version 2 (SLPv2), as defined by IETF RFC 2608, for its basic 27 discovery mechanism. SLPv2 is used to locate constituents (agents, object managers, etc.), but complete 28 discovery of all the services offered involves traversing the interoperability model for the SMI-S profile 29 supported. This clause of the SMI-S specification deals primarily with the information discovered using 30 SLPv2. There are references to information discovered by traversing the interoperability model, but 31 details on this are provided in 11.3.

32 NOTE  SLPv1 is not supported in SMI-S as discovery mechanism. SMI-S requires capabilities that were introduced in SLPv2 in 33 order to support the discovery of SMI-S agents and object managers.

- SLPv2 defines discovery protocols among three constituents:

- User Agent (UA): A process that attempts to establish contact with one or more services. A User Agent 36 retrieves service information from Service Agents or Directory Agents. In SMI-S, a “user agent” would be 37 part of an SMI-S Client.

38 Service Agent (SA): A process working on behalf of one or more services to advertise the services. In 39 SMI-S, a “service agent” would be supported by SMI-S dedicated or general purpose servers.

40 Directory Agent (DA): A process that caches SLP service advertisements registered by Service Agents 41 and forwards the service advertisements to User Agents on demand. In SMI-S, the SLP “Directory agent” 42 is defined as the main function of the “directory server” role in the SMI-S Reference Model. SMI-S allows 43	multiple Directory Agents to be used for purposes including load sharing and availability. These Directory 44	Agents may have the same scope, as allowed by SLPv2.

45 SLPv2 provides a framework for client applications, represented by User Agents, to find and utilize 46 services, represented by Service Agents. The Directory Agents represent an optional part that enhances 47 the performance and scalability of the protocol by acting as a cache for all services that have been 48 advertised. Directory Agents also reduce the load on Service Agents, making simpler implementations of 49 Service Agents possible. User Agents can then query the Directory Agents for services. Service Agents 50 register with Directory Agents and are required to re-register as the registrations expire. If no Directory 51 Agents are present, User Agents may request service information directly from the Service Agents.

52	Using SLPv2, a client can discover SMI-S servers and SLPv2 Directory Agents in the storage network. In 53	the case of SMI-S servers, the basic information discovered is the profiles supported and the URL of the 54	service. Details on the specific services provided with the profile are then found by traversing the service 55	structure modeled for the profile.

56 Using SLPv2, a “service agent” advertises its services. These advertisements have an expiration time 57 period. To avoid getting an advertisement deleted, a service agent shall reregister before the time period 58 expires. SMI-S servers may deregister as part of a graceful shutdown.

- A service advertisement consists of file components:

- • Service type name – describes the general type of service being advertised (ex. Printing, faxing, etc.). The 61 working assumption is that DMTF wants “WBEM Servers” advertised with the service type WBEM. This is 62 used by SMI-S servers (both dedicated and general purpose servers).

63 • Attributes – The collection of attributes describes the particular instance of the service in more detail. For 64 SMI-S, these would be the attributes defined by the service type template for WBEM. The attributes are 65 defined in 9.5.3.

66 • Service Access point – the service access point defines the point of connection that the software client of the 67	UA uses to connect to the service over the network.

68 • Scopes – These are administrative groupings of services. The default value (“default”) should be used for 69 SMI-S servers. Other scopes may be defined by the customer, but care must be taken when this is done. The 70 administrator shall do this correctly or SMI-S servers will not be visible.

71

• Language – Services advertisements contain human readable strings. These are provided in English, but 72 may also be in other languages.

73

IMPLEMENTED

SLPv2 provides for authentication of service URLs and service attributes. This provides user agents 74 (UAs) and directory agents (DAs) with assurances of the integrity of service URLs and attributes included 75 in SLP messages. The only systems which can generate digital signatures are those which have been 76 configured by administrators in advance. Agents that verify signed data may assume it is trustworthy 77 inasmuch as administrators have assured trustworthiness through the cryptographic keying of SAs and 78 DAs. The SLPv2 security model assumes that service information is public, and therefore does not 79 require confidentiality. 80

Clause 2.5 of RFC 3723, Securing Block Storage Protocols over IP, states that the SA advertisements as 81 well as UA requests and/or responses are vulnerable to these security threats:

82

1) An attacker could insert or alter service agent (SA) advertisements or responses to a UA requests in order to 83 masquerade as the real peer or launch a denial of service attack.

84

85	2)	An attacker could gain knowledge about an SA or a UA through sniffing, and launch an attack 86	against the peer.

- 3)	An attacker could spoof DA advertisements and thereby cause UAs and SAs to use a rogue DA.

- Clause 2.5 of RFC 3723 also outlines the capabilities required to address these threats, but notes that 89 SLP (as defined in RFC 2608) does not satisfy these security requirements. SLPv2 only provides end-to90 end authentication (i.e., does not support confidentiality), but with this authentication, there is no way to 91 authenticate zero result responses. Thus an attacker could mount a denial of service attack by sending 92 UAs a zero results Service Reply (SrvRply) or Attribute Reply (AttrRply) with a source address 93 corresponding to a legitimate DA advertisement.

94 The RFC 3723 mitigation strategies include reliance on digital signatures for authentication of service 95 URLs and attributes as well as IPsec. For SMI-S environments that require security in conjunction with 96 the use of SLPv2, the major RFC 3723 recommendations are not necessary as long as the SLP 97 messages are not fully trusted and SSL or TLS with server certificates are used. Additional security 98 guidance is provided in the sections associated with UAs and SAs.

IMPLEMENTED

## 99	9.3	SLP Messages

- SLP v2 divides the base set of SLP messages into required and optional subsets.

- NOTE  SLP v2 also includes a new feature, an extension format. Extension messages are attached to base messages. SMI-S does 102 not use extensions. The discussion of messages introduces terms that define the SLP services:

- • Attribute Reply (AttrRply): A reply to an Attribute Request. (optional)

- • Attribute Request (AttrRqst): A request for attributes of a given type of service or attributes of a given 105	service. (optional)

106	• DA Advertisements (DAAdvert): A solicited (unicast) or unsolicited (multicast) advertisement of Directory 107	Agent availability.

108 • SA Advertisement (SAAdvert): Information describing a service that consists of the Service Type, Service 109 Access Point, lifetime, and Attributes.

- • Service Acknowledgement (SrvAck): A reply to a SrvReg request.

- • Service Deregister (SrvDereg): A request to deregister a service or some attributes of a service. (optional) 112	• Service Register (SrvReg): A request to register a service or some attributes of a service.

- • Service Reply (SrvRply): A reply to a Service Request.

- • Service Request (SrvRqst): A request for a service on the network.

- • Service Type Reply (SrvTypeRply): A reply to a Service Type Request. (optional)

- • Service Type Request (SrvTypeRqst): A request for all types of service on the network. (optional)

- Service Agents (SAs) and User Agents (UAs) shall support Service Request, Service Reply, and 118 DAAdvertisement message types. Service Agents shall additionally support Service Registration, SA 119 Advertisement, and Service Acknowledgment message types. The remaining message types may be 120 supported by Service Agents and User Agents. Directory Agents (DAs) shall support all message types 121 with the exception of SA Advertisement. Table 288 lists each base message type, its abbreviation, 122 function code, and required/optional status.

Table 288 - Message Types

| Message Type | Abbreviation |  | Function Code |  | Required (R)/ Optional (O) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DAs | SAs |  | UAs |
| Service Request | SrvRqst | 1 |  | R | R | R |  |
| Service Reply | SrvRply | 2 |  | R | R | R |  |
| Service Registration | SrvReg | 3 |  | R | R | O |  |
| Service Deregistration | SrvDereg | 4 |  | R | O | O |  |
| Service Acknowledgement | SrvAck | 5 |  | R | R | O |  |
| Attribute Request | AttrRqst | 6 |  | R | R | R |  |
| Attribute Reply | AttrRply | 7 |  | R | R | R |  |
| DA Advertisement | DAAdvert | 8 |  | R | R | R |  |
| Service Type Request | SrvTypeRqst | 9 |  | R | O | O |  |
| Service Type Reply | SrvTypeRply | 10 |  | R | O | O |  |
| SA Advertisement | SAAdvert | 11 |  | N/A | R | O |  |

123 NOTE  The requirements in Table 288 extend the requirements defined for SLP V2. SMI-S adds additional requirements for 124 AttrRqst and AttrRply beyond those defined by the RFC.

## 125	9.4	Scopes

- SLPv2 defines a scope as follows:

- Scope: A set of services, typically making up a logical administrative group.

- Scopes are sets of service instances. The primary use of Scopes is to provide the ability to create 129 administrative groupings of services. A set of services may be assigned a scope by network 130 administrators. A User Agent (UA) seeking services is configured to use one or more scopes. The UA 131 only discovers those services that have been configured for it to use. By configuring UAs and Service 132 Agents with scopes, administrators may make services available. Scopes strings are case insensitive. 133 The default SCOPE string is “DEFAULT”.

134 SMI-S does not dictate how Scopes are set. That is, scopes can be set by customers to match their 135 needs. However, SMI-S requires that SMI-S servers use the “default” scope as a means of making SMI-S 136 advertisements visible to SMI-S clients.

137 To be compliant with SMI-S, User Agents (SMI-S clients) and Service Agents (SMI-S servers) shall not 138 require scope settings that interfere with administrative use of scopes. Specifically, this means:

- • SMI-S clients and servers shall allow an administrator to set scopes to define what is to be searched, and,

- • SMI-S clients and servers shall allow an administrator to configure scopes, including turning off the “default”

- scope.

- 143

## 144	9.5	Services Definition

145

### 9.5.1	SLP Terms

146

- Services definition uses these terms defined in SLPv2:

- • Service Type Template: A formalized, computer-readable description of a Service Type. The template defines 149	the format of the service URL and attributes supported by the service type.

150

• Service URL: A Uniform Resource Locator for a service containing the service type name, network family, 151 Service Access Point, and any other information needed to contact the service.

152

Services are defined by two components: the Service URL and the Service Type Template. The Service

153

URL defines an access point for the service and identifies a unique resource in the network. Service 154 URLs may be either existing generic URLs or URLs from the service: URL scheme.

155

The second component in a Service definition is a Service Type Template. Service Type Templates define the attributes associated with a service. These attributes, through inclusion in registrations and queries,

156 allow clients to differentiate between similar services.

157

158 SMI-S servers use a Service Type Template defined by DMTF for advertising “WBEM Servers”. The template name for WBEM Servers is “WBEM”.

159

### 160	9.5.2	Service Type

161

Service Type: The class of a network service represented by a unique string (for example a namespace 162 assigned by IANA).

163

The service type describes a class of services that share the same attributes (e.g., the service printer or 164 the service “WBEM”). 165

- The basic function of SLP discovery is the identification of the service offered by a constituent. In the case of SMI-S, the service type advertised by all constituents is “WBEM.” This follows a DMTF proposal

- for advertising WBEM Servers. The only exception to this is the Directory Server, which advertises itself as a “directory-agent.” That is, SMI-S uses a standard SLP directory service. SMI-S does not require a 168 unique SMI-S directory server. 169

For other roles (SMI-S servers) the role advertises its services as a WBEM services (e.g., “WBEM”). 170

### 9.5.3	Service Attributes

171

Attributes: A collection of tags and values describing the characteristics of a service.

172

The attributes are defined in the DMTF WBEM SLP Template (DSP0206) 1.0.

173

## 9.6	User Agents (UA)

174

A User Agent is a Client process working on the user’s behalf to establish contact with some service. A

175

User Agent retrieves service information from Service Agents (9.7) or Directory Agents (9.8). Further 176 description of a Client and its role may be found in 11.2, "SMI-S Client".

177

The only required feature of a User Agent is that it can issue SrvRqsts and interpret DAAdverts, 178 SAAdverts and SrvRply messages. If Directory Agents exist, User Agents shall issue requests as 179 Directory Agents are discovered.

180 An SMI-S Client should act as an SLP user agent (UA) using the query functions of SLP V2 to determine 181 location and other attributes of the “WBEM” SLP Service Type Template defined in , "".

182

The basic search methodology for SMI-S clients is to search for directory agents and service agents

183 within their scope. If all SMI-S servers are supported by a directory agent, then the search yields nothing

184

185	but directory agents. The client can then obtain a list of services (and their URLs) for management of the 186	SMI-S servers.

187 If any Service agents are not covered by a directory agent (i.e., are not within its scope), then the client 188 obtains service replies from those service agents.

189

190 An client would typically search for all service types available in their scope(s). This returns a list of service types available in the network. However, an SMI-S client can be assumed to be searching for 191 “WBEM” service types. If a client only manages selected devices (e.g., switches or arrays), the SMI-S 192 client can issue a request for the specific services by using predicates on the

193	“RegisteredProfilesSupported” attribute.

194

#### 195	IMPLEMENTED

196	When a SMI-S client uses SLPv2 and security is an issue, the following should be considered:

197

• SSL and TLS should be used with a certificate-based cipher suite along with a certificate installed on each 198 SMI-S server (SA) for communications with discovered SAs (SMI-S servers).

199

- • SLPv2 Service Agents (SA) and Directory Agents (DA) may advertise (SAAdverts and DAAdverts,

- respectively) their presence on the network, using multicast; however, SMI-S clients should treat these advertisements as advisory (i.e., identity shall be verified as described in 9.7 and 9.8).

202

203 • SMI-S clients should maintain and use a negative authentication cache to avoid repeatedly contacting an 204 SMI-S server that fails to authenticate as part of the SSL or TLS handshake.

#### 205	IMPLEMENTED

206

207

## 208	9.7	Service Agents (SAs)

209	A Service Agent supports an SMI-S server process working on behalf of one or more services to advertise 210	the services. 211	See Clause 11 SMI-S Roles for further description of SMI-S servers.

212

- Service Agents shall accept multicast service requests and unicast service requests. SAs may accept

- other requests (Attribute and Service Type Requests). An SA shall reply to appropriate SrvRqsts with SrvRply or SAAdvert messages. The SA shall also register with all DAs as they are discovered.

215

216 To provide for SMI-S Client discovery of SMI-S servers, a WBEM Server shall act as a Service agent (SA) for the IETF Service Level Protocol (SLP) V2 as defined in IETF RFC 2608. The service shall correspond 217 to V2 of SLP (IETF RFC 2608 and 2609) and shall use the Service Templates defined in  of this 218 specification for advertisements. An SMI-S server acting as an SA shall provide a separate SLP 219 advertisement for each address/port that the WBEM Server advertises.

220

#### 221	IMPLEMENTED

222

- When a SMI-S server uses SLPv2 and security is an issue, the following should be considered:

- • SMI-S servers should accept SSL and TLS unicast connections from SMI-S clients as well as selecting a 225 certificate-based cipher suite. 226

• SMI-S servers that advertise their existence as SLPv2 SAs (SAAdverts) should minimize leakage of 227 information, by minimizing the information that is contained in the multicast advertisements.

228

229

230 • SMI-S servers, functioning as SAs, should register with all discovered DAs, which advertise any of its 231 configured scopes and establish connections with these DAs over unicast.

232

233 • When SMI-S servers are also functioning as clients (e.g., cascading), they should follow the security guidance provided in 9.6 User Agents (UA).

234

#### 235	IMPLEMENTED

236

## 237	9.8	Directory Agents (DAs)

238 SMI-S supports existing SLPv2 Directory Agents (without modification). That is, SMI-S makes no 239 assumptions on Directory Agents that are not made by SLPv2. Note that this cannot quite be said for

240 User Agents, which are looking for SMI-S specific services, or Service Agents, which are advertising SMIS specific services.

## 9.9	Service Agent Server (SA Server)

241

### 9.9.1	General Information

- The reserved listening port for SLP is 427, the destination port for all SLP messages. Service Agents

- (SAs) are required to listen for both unicast and multicast requests. A Directory Agent (DA) shall listen for unicast request and specific multicast DA discovery service requests. SAs and User Agents (UAs) that

- perform passive DA discovery shall listen for multicast DA Advertisements (DAAdverts).

245

246	TCP/IP requires that a single server process per network interface control all incoming messages to a

port. That requirement necessitates a mechanism to share the SLP port (427).

247

248 Sharing the SLP port (427) is accomplished with a Service Agent Server (SA Server) process that ‘owns’ the port on behalf of all SAs, UAs and optional DA that are listening for SLP messages. The SA Server listens for incoming messages that request advertisement information and either answer each request or forward it to the appropriate SA. The SA Server also performs passive DA discovery and distribute the DA addresses and scopes to the SAs and UAs that it serves.

249

- A SA Server may also function as a DA if the SA Server is implemented so that it answers requests for

- advertisement information rather than forwarding each request to the appropriate SA. The combined DA/

SA Server is acting as an intermediary between a SA that registered an advertisement and a UA 252 requesting information about the advertisement.

### 253 9.9.2 SA Server (SAS) Implementation

254

255 IETF RFC 2614 describes APIs for both the C and Java languages. Both APIs are designed for standardized access to the Service Location Protocol (SLP).

256

257	The goals of the C API are: 258

259 • Directly reflect the structure of SLP messages in API calls and return types as character buffers and other simple data structures.

260

- Simplify memory management to reduce API client requirements.

- Provide API coverage of just the SLP protocol operations to reduce complexity.

261 • Allow incremental and asynchronous access to return values, so small memory implementations are possible.

262

263 • Support multithreaded library calls on platforms where thread packages are available.

264

265

##### Description

266	The Java API goals are: 267

• Provide complete coverage of all protocol features, including service type templates, through a programmatic

268	interface.

269

- Encourage modularity so that implementations can omit parts of the protocol that are not needed.

- In conformance with Java’s object-oriented nature, reflect the important SLP entities as objects and make the API itself object-oriented. 270

- Use flexible collection data types consistently in the API to simplify construction of parameters and analysis of

271	results.

272

273 • Designed for small code size to help reduce download time in networked computers.

274

### 9.9.3	SA Server (SAS) Clients

275

9.9.3.1	Description

276	An SAS Client is a Service Agent (SA), User Agent (UA), or Directory Agent (DA) that is associated with a 277	SA Server. The SA Server listens on the SLP port (427) and appropriately handle all incoming messages 278	for each SAS Client. A DA acting as a SAS Client is separately configured on the same host as the SA 279	Server.

280

9.9.3.2	SAS Client Requests – SA Server Responses

- A SA Server responds when appropriate, to incoming unicast and multicast messages from SAS Clients.

- The SA Server may answer with the appropriate advertisement, if available, or forward the request on to the appropriate SAS Client. If the SA Server is also functioning as a DA, it discards a multicast SrvRqst of

283

“service:directory-agent” that has either a missing scope list or the scope list does not contain a scope

284 the Service Agent Server/DA is configured with.

285

286

### 9.9.4	SA Server Configuration

287

- 9.9.4.1	Overview

- SA Servers may be configured via an individual SLP configuration file, programmatically, or a 290 combination of the two. DHCP may also be used obtain the scope list for a SA Server. Figure 18 291 illustrates the various means of configuring a SA Server.

- 9.9.4.2	SLP Configuration File

- If a SA Server is also functioning as a DA, the DA configuration properties shown in Table 289 shall be 294	set:

295	Table 289 - Required Configuration Properties for SA as DA

| Keyword | Data Type | Value |
| --- | --- | --- |
| net.slp.isDA | boolean | true |
| net.slp.DAAttributes | string | (SA-Server=true) |

296

297

298

299

- The DA attribute/value pair of “SA-Server=true” allows a query to be used when a SA Server/DA needs to

- be identified. In addition, when the SA Server/DA responds to a SrvRqst message with a DAAdvert message, the DA attribute/value pair is included.

302

303

Programmatic Configuration

- The remaining DA configuration property, net.slp.DAHeartBeat, with a default of 10,800 seconds, may be

- set as appropriate. If a SA Server is not functioning as a DA, the SA configuration property in Table 290 shall be set:

306

Table 290 - Required Configuration Properties for SA

307

| Keyword | Data Type | Value |
| --- | --- | --- |
| net.slp.SAAttributes | string | (SA-Server=true) |

308

309

310

- 9.9.4.3	Programmatic Configuration

Both the C and Java language API’s provide access to SLP properties contained in the SLP configuration

- file. The actual SLP configuration file is not accessed or modified via the interfaces. Once the file is loaded into memory at the start of execution, the configuration property accessors work on the in-memory 313	representation. 314

315	The C language API provides the SLPGetProperty() and SLPSetProperty() functions. The 316 SLPGetProperty() function allows read access to the SLP configuration properties while the 317 SLPSetProperty() function allows modification of the configuration properties. The SLPSetProperty() function has the following prototype:

318

- void SLPSetProperty(const char *pcName, const char *pcValue);

- The SLPSetProperty() function takes two string parameters: pcName and pcValue. The pcName 321 parameter contains the property name and pcValue contains the property value. The following example 322 uses the SLPSetProperty() function to configure a SA Server that is not functioning as a DA:

- void setSAAttributes() {

- char value[80]; /* A buffer for storing the attribute string. */ value = “SA Server=true”;

325

SLPSetProperty(“net.slp.SAAttributes”, value);

326

}

327

328	9.9.4.4	DHCP Configuration

If the Service Agent Server is also functioning as a DA, its scope list may be obtained via DHCP. Scopes 329 discovered via DHCP take precedence over the net.slp.useScopes property in the SLP configuration file. 330

331	9.9.4.5	Scope

A Service Agent Server is configured with a minimum scope of DEFAULT. If a Service Agent Server is not functioning as a DA, DEFAULT is the only scope configured. If a Service Agent Server is functioning as a DA, it may have additional scopes configured. Use of the DEFAULT scope enables the associated SAS Clients (UAs, SAs and DA) to actively discover the Service Agent Server using a well-known value for scope.

332

333

334

335

336

Figure 18 - SA Server Configuration

SA Server

DHCP

Server

1

2

C or Java API

SLP

Configuration

File

3

337

338

339

340

341

342

343

344

345

346

347

348

349	1)	The SA Server may obtain specific configuration values via an individual SLP Configuration file.

350

2)	The C or Java API provides programmatic access to the configuration file properties.

351

352	3)	The SA Server may obtain its scope values from a DHCP Server.

353

### 9.9.5	SA Server Discovery

354

355	“Discovery” of a SA Server by its SAS Clients is accomplished by successfully establishing the required communication link between the two entities. There is no need for active or passive discovery as 356	described by SLP since both the SA Server and SAS Clients reside on the same host system.

### 357	9.9.6	SAS Client Registration

358

Service Agents (SAs) that are SAS Clients register and deregister with the local SA Server using the 359 SrvReg/SrvDereg messages. The SA Server responds with a Service Acknowledgement (SrvAck) 360 message. The SA Server store a service advertisement until either its lifetime expires or a SrvDereg 361 message is received.

362 If the SA Server is also functioning as a DA, the DA registration requirement is also met. The SA server 363 also forwards any SA registration to other DAs that have the same scope as the SA.

364

## 9.10	Configurations

### 9.10.1	Overview

There are three network configurations (9.10.2, 9.10.3, 9.10.4) showing SMI-S clients and servers. The routing of SLP's multicast messages effect the SMI-S discovery process. SMI-S clients and servers shall be able to be configured to work in these environments.

### 9.10.2	Multicast Configurations

This is the simplest environment and is shown in Figure 19. This network allows multicast messages to be delivered to all the components of a SMI-S management system. As defined in IETF RFC 2608 - 8.1,

Scope

the client uses multicast SLP messages to contact the SLP Service Agent (SA) associated with each SMIS server. Then, each SA sends replies directly back to the client.

Because of the possible size of the reply, servers shall support TCP/IP (as well as UDP) to send the reply. The server shall also support the SLP oversize bit to tell the client large TCP/IP messages shall be used. When a client sees the overflow bit in a UDP response, it should retry using a TCP request.

Client Application

System

SLP Service Agent

(

SA

)

CIMOM and Provider

SLP Service Agent

(

SA

)

SMI Implementation

Routing Switch

Allows Multicast

Traffic

365

366

367

368

Figure 19 - Multicast Configuration

369

370

### 371	9.10.3	No Multicast configuration

In this configuration, shown in Figure 20, the network doesn't allow the use of multicast messages. All 372 communication shall use TCP/IP point to point connections. First, a SLP directory agent should be used. 373 Each SA shall be configurable by the user. The user will configure the SA by setting the address of the 374 SLP directory agent (DA). At startup each SA shall use a temporary registration to tell the DA its SLP 375 information (IETF RFC 2608 - 8.3). The SAs shall renew the registration before it expires (IETF RFC 2608 376 - 8.3). The registration timeout should be about 5-10 min.

377

378

379

380

381

382

383

384

385

386

387

388 The client shall also be configurable by the user. The user will configure the client by setting the DA 389 address. The client will use this address to send SLP messages to the DA (IETF RFC 2608 - 8.1). The DA 390 will satisfy the requests using information provided by the SAs.

Client Application

System

SLP Service Agent

SA

(

)

SMI Implementation

SLP Service Agent

(

SA

)

SMI Implementation

Routing Switch

Blocks Multicast

Traffic

System with

Directory Agent (DA)

Figure 20 - No Multicast configuration

### 391	9.10.4	Multicast Islands

392 Networks that allow for multicast messages to reach parts of the system, require the use of both 393 techniques described. The client should use the multicast process and the no multicast method. It should 394 be able to combine the information found each way into a single set of discovery information. The SAs 395 shall support both methods at the same time as shown in Figure 21.

396

397

Scope

398

399

Client Application System.; SLP Service Agent (SA). SMI Implementation... ( Multicast Boundary ) System with Directory Agent (DA).; SLP Service Agent (SA). SMI Implementation...

400

Figure 21 - Multicast Islands

401

402

403

404

405

# 1 10  Indications

## 10.1 Indications profile

2

### 3	10.1.1	Profile Support

4

In this version of the SMI-S specification, the only supported profile is the DMTF Indications Profile, DSP 5 1054, version 1.2.2.

6

### 10.1.2	Creating a client defined indication and subscription

7

Table 291 identifies the elements of the use case to create an indication filter and subscribe to it. 8

Table 291 - Create an IndicationFilter and subscribe to it

| Use Case Element | Description |
| --- | --- |
| Summary | Given a top level system of an autonomous profile and a URI for an indication listener create a client defined indication and subscribe to it. |
| Basic Course of Events | 1. Get the IndicationConfigurationService if it exists 1b. If not try to do a CreateInstance for the IndicationFilter (if it succeeds, then continue) 2. If the service exists, get the IndicationConfigurationCapabilities to find out if the implementation supports client defined IndicationFilters 3. If the capability exists, then do a CreateAndSubscribe for the indication 3b. Do a CreateInstance on the ListenerDestination and another CreateInstance on the IndicationSubscription |
| Exception Paths | None |
| Triggers | The administrator wants to listen for a specific indication of his/her choosing. |
| Assumptions | The implementation supports client defined IndicationFilters |
| Preconditions | The top level system of the profile and a listener destination for the application to get the indications. |
| Postconditions | The IndicationFilter is created and a subscription to it is recorded in the WBEM Server. |

9 10

11

12

13

14

15

16

17

18

19

20

21

22

### 23	10.1.3	ListenerDestination

For CIM-XML (required by this version of the standard), Listener Destinations shall use the subclass 24 CIM_ListenerDestinationCIMXML.

25

## 10.2	Indication Filter Strings

26

### 10.2.1	Definition Syntax

27

WBEM indications are defined using filter strings. The filter strings are expressed in a query language 28 that includes the type of indication and related CIM elements.

29 For versions of this standard starting with 1.3.0, new indication filters shall be defined using CQL (see 30 DMTF DSP0202). WQL is no longer supported in the current version of this standard.

Although CQL supports complex filter strings, the filters used in SMI-S are very simple and may be 31 expressed as a few patterns – literal text containing a limited number of variables representing CIM 32 elements. The patterns are defined in the following simple grammar:

33 • literal text does not include curly brackets (“{” and “}”)

• variables are surrounded by curly brackets; the usage of variables is explained in the “Semantic” sub-section following each filter string

##### Filter String

### 34	10.2.2	Instance Creation

35	10.2.2.1	Filter String

SELECT * FROM CIM_InstCreation WHERE

36

SourceInstance ISA {class-name}

37

- 10.2.2.2	Semantic

- An instance of a class is instantiated. {class-name} is the name of a class (or one of its subclasses) of the instance created.

40

### 41	10.2.3	Instance Deletion

42

10.2.3.1	Filter String

43

SELECT * FROM CIM_InstDeletion WHERE

44

SourceInstance ISA {class-name}

45

10.2.3.2	Semantic

46	An instance of a class is deleted. {class-name} is the name of a class (or one of its subclasses) of the 47	instance deleted.

48

### 10.2.4	Modification of any value in an array property

49

10.2.4.1	Filter string

50	SELECT * FROM CIM_InstModification WHERE

SourceInstance ISA {class-name} AND

51

SourceInstance.{class-name}::{property-name} <>

52

PreviousInstance.{class-name}::{property-name}

53

- 10.2.4.2	Semantic

One of the values of the array property {property-name} in class {class-name} (or one of its subclasses)

- has been modified, or an additional value is added to {property-name} or a value is removed from 56 {property-name}.

57

### 10.2.5 Modification to either of Two Specific values in an Array Property

58 59 10.2.5.1 Filter string

60

SELECT * FROM CIM_InstModification

61

WHERE SourceInstance ISA {class-name}

- AND ANY

SourceInstance.{class-name}::{property-name}[*] = {value1}

- AND ANY

64

SourceInstance.{class-name}::{property-name}[*] = {value2}

65

- 10.2.5.2	Semantic

The array property {property-name} in class {class-name} (or one of its subclasses) has been modified

- resulting in one of the entries in the array having a value of {value1} and another of the entries having a value of {value2}. Either {value1} or {value2} shall be a new value for an existing entry or is the value of a 68 newly added entry.

69

70

71

Filter String

### 10.2.6	Alert

10.2.6.1	Filter String

TSELECT * FROM CIM_AlertIndication WHERE OwningEntity=’SNIA’    AND MessageID='{message-id}'

10.2.6.2	Semantic

An alert indication referencing the standard message with message ID {message-id}. Note that the message ID is a concatenation of the name of the appropriate SNIA registry and message number. For example, the {message-id} for the first message in the FC registry is ‘FC1’.

Semantic

# 1	11  SMI-S Roles

## 2	11.1	Introduction

3 As shown in Figure 22, the complete reference model shows the roles for the various entities of the 4 management system. Any given host, network device or storage device may implement one or more of 5 these roles as described later in this clause.

Device or Subsystem

CIM-XML / WS-Management

TCP/IP

SLP

TCP/IP

SA

Lock Manager

0

...n

Directory Server

...n

0

Directory

Agent

SMI-S Client

...n

0

User

Agent

SA

SMI-S Server

...n

0

SA

SMI-S Server

0

...n

0

...n

Provider

SMI-S Server

Device or Subsystem

Prop

rietar

y

SA

Device or Subsystem

Prop

rietar

y

Proxy Model

Embedded Model

proprietary

proprietary

Figure 22 - SMI-S Roles

This profile presents a concise definition of each of these roles and the requirements on implementations

- of these roles in a management system. For each of these roles, specific functions are required to be

- implemented in one or more functional areas:

8

a)	SLP Discovery Functions – the required discovery capabilities that the role performs in the overall man-

9 agement system;

10

- Generic Operations – the management model operations that the role performs;

11

- Security – the security requirements that the role is expected to satisfy;

12

- Lock Management Operations – the locking operations that the role is expected to perform.

13 The detail of these responsibilities for each of the roles is described in this profile.

14

## 15	11.2	SMI-S Client

### 16	11.2.1	Overview

17 The SMI-S Client role in the overall management system is performed by software that is capable of 18 performing management operations on the resources under management. This includes monitoring, 19 configuration, and control of the operations on the resources. Typical clients include user interface 20 consoles, complete management frameworks, and higher-level management applications and services 21 such as policy based management systems.

22 There can be zero or more SMI-S clients in the overall management system. These clients can all coexist 23 simultaneously and can perform independent or overlapping operations in the management system. It is 24 outside the scope of this specification to specify client cooperation with other clients in any way. The 25 semantics of the described management system is that the last successful client operation is valid and 26 persists in the absence of any other client operations (last write wins).

27 It is expected that development kits for the management system will provide code for the required 28 functions implemented in clients. Consoles, frameworks and management applications can then use this 29 common code in order to comply with this specification. The specification of an API for this client code, 30 and specific language bindings for applications is outside the scope of this specification, but is a 31 candidate for follow-on work.

### 32	11.2.2	SLP Functions

33 The SMI-S Client role is required to implement SLP User Agent (UA) functionality as specified in 9.6, 34 "User Agents (UA)". The Client discovers all SMI-S servers within its configured scope that are required 35 for its operations by querying for service specific attributes that match the criteria for those operations.

### 36	11.2.3	Generic Operations

37 The SMI-S Client role shall implement client functionality as specified by the relevant WBEM protocol 38 standard and should implement asynchronous notification functionality as specified by that standard.

### 39	11.2.4	Security Considerations

40 The SMI-S Client role shall implement security as specified in 13.1.2, "General Requirements for HTTP 41 Implementations".

### 42	11.2.5	Lock Management Functions

43	Not defined in this standard.

## 44	11.3	Dedicated SMI-S Server

### 45	11.3.1	Overview

46 The intention of the SMI-S server role in a management system is to provide device management support 47 in the absence of any other role. A simple management system could consist of just a SMI-S Client and a 48 SMI-S Server and all management functions can be performed on the underlying resource. This means 49 that a vendor can offer complete management for the resource by shipping a standalone client for the 50 resource and not depend on any other management infrastructure. Although, at the same time, the SMI-S 51 Server can participate in a more complex management environment through the use of the standard 52 mechanisms described here.

53 • Embedded SMI-S Server – the SMI-S Server functions are incorporated into the resource directly and do not 54	involve separate installation steps to become operational.

55 • Proxy SMI-S Server – the SMI-S Server is hosted on a system separate from the resource and 56 communicates with the resource via either a standard or proprietary remote protocol. This typically involves

General

- an installation operation for the SMI-S Server and configuration for, or independent discovery of, the desired

- resource.

- In order to minimize the footprint on the resource or proxy hosts, the required functions of the SMI-S 60 Server role have purposely been scaled back from those of a typical general purpose WBEM Server

61	running on host with more significant resources. These required functions are described in 11.3.2 and

11.3.3.

62

### 11.3.2	SLP Functions

63

64 The SMI-S Server role is required to implement SLP Service Agent (SA) functionality as specified in 9.7, 65 "Service Agents (SAs)". Optionally, it should implement Service Agent Server functionality or use an 66 existing SA Server if one exists. The SMI-S server shall advertise service-specific attributes that allow the 67 client to locate it based on its profile (see 9.5).

### 68	11.3.3	Generic Operations

- 11.3.3.1	General

- The SMI-S Server role shall implement the server functionality as specified by the relevant WBEM 71 Protocol standard.

- 11.3.3.2	Required Operations

- SMI-S Servers shall support the following generic operations:

- • GetInstance

- • DeleteInstance

- • ModifyInstance

- • CreateInstance

- • OpenEnumerateInstances

- • OpenAssociators

- • OpenReferences

- • PullInstancesWithPath

- • CloseEnumeration

- • InvokeMethod

- • InvokeStaticMethod

85

DEPRECATED

86 The following operations are deprecated. These operations shall still be supported in SMI-S 1.x versions, 87 but will be removed in the future.

88

- EnumerateInstances

89

- EnumerateInstanceNames

90

- Associators

91

- AssociatorNames

#### Required Model Support

- References

- ReferenceNames

92

- OpenEnumerateInstancePaths

93

- OpenAssociatorPaths

94

- OpenReferencePaths

95

- PullInstancePaths

96

#### 97	DEPRECATED

98

11.3.3.3	Required Model Support

99 The SMI-S Server shall implement the Server Profile as detailed in Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4 35 Server Profile.

### 11.3.4	Security Considerations

100	The SMI-S Server role shall implement security as specified in 13 Security.

101

### 102	11.3.5	Lock Management Functions

Not defined in this standard.

103

## 11.4	General Purpose SMI-S Server

104

### 11.4.1	Overview

105

The General Purpose SMI-S Server role in an overall management system is intended to reduce the

- number of network connections needed by a Client to manage large numbers of resources. It is also

- envisioned as a convenient place to perform operations across multiple resources, further off-loading these from the Client as well. 108

In addition, the General Purpose SMI-S Server role can provide a hosting environment for the plug-in 109 instrumentation of host-based resources and management proxies for resources with remote 110 management protocols. These plug-ins are called providers and considered sub roles of the General 111 Purpose SMI-S Server.

112

A General Purpose SMI-S Server is not required in a management system, but is expected to be 113 deployed at least as a common infrastructure for host-based resources. In any large storage network, 114 there may be several General Purpose SMI-S Servers (as many as one per host). Communication 115 between General Purpose SMI-S Servers may be standardized in the future, but this capability is outside

116 the scope of this specification. General Purpose SMI-S Servers may act as a point of aggregation for multiple SMI-S Profiles as described in 35 Server Profile using existing standard mechanisms as 117 specified here. 118

119 As General Purpose SMI-S Servers are expected to be deployed on hosts with more resources and less 120 footprint concerns than other managed resources, the required functions, specified in 11.4.2, 11.4.3, and 121 11.4.4, are more extensive that of an Dedicated SMI-S Server.

122

### 123	11.4.2	SLP Functions

124 The General Purpose SMI-S Server role is required to implement SLP Service Agent (SA) functionality as 125 specified in 9.7, "Service Agents (SAs)". The General Purpose SMI-S Server shall advertise service

126 specific attributes that allow the Client to locate it based on the profiles it supports, as defined in 11.4.3.1, "General".

General

### 127	11.4.3	Generic Operations

128

11.4.3.1	General

129

130 The General Purpose SMI-S Server role shall implement WBEM Server functionality as specified by the 131 Generic Operations standard.

132	11.4.3.2	Required Operations

The General Purpose SMI-S Server is required to implement the minimum profile as specified in the 133 Generic Operations standard. In addition, it shall implement the intrinsic methods needed to support the 134 Profiles that it supports.

135

11.4.3.3	Required Model Support

136 The General Purpose SMI-S Server shall implement the Server Profile as detailed in Storage Management 137 Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4 35 Server Profile.

138

139	11.4.3.4	Security Considerations

The General Purpose SMI-S Server role shall implement security as specified inStorage Management 140 Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4 35 Server Profile.

141

### 142	11.4.4	Lock Management Functions

Not defined in this standard.

143

### 144	11.4.5	Provider Sub-role

145

11.4.5.1	Overview

- A sub-role within a General Purpose SMI-S Server that can be used to provide management support for

- the resource, especially useful when the resource is host-based (i.e., HBA or Host Software) and the platform provides a WBEM Server as part of its operating system.

148

11.4.5.2	Required Model Support

149 The Provider shall implement the Provider Profile as detailed in the object model shown in Storage 150 Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4 35 Server Profile).

151

## 11.5	Directory Server

152

The Directory Server role is used to facilitate Discovery of instances of the various roles in a management 153 system, but may also be used by management systems to store common configurations, user credentials 154 and management policies. Functions outside of Discovery are outside the scope of this specification. The 155 Directory Server role is optional for a compliant management system.

### 156	11.5.1	SLP Functions

157 The Directory Server role is required to implement SLP Directory Agent (DA) functionality as specified in 158 9.8, "Directory Agents (DAs)". The Directory registers all Agents and Object Managers within its 159 configured scope and allows queries for their respective service specific attributes.

160

### 11.5.2	Generic Operations

161

There are no additional requirements for this role.

162

### 163	11.5.3	Security Considerations

164

There are no additional security requirements for this role.

##### SLP Functions

### 165	11.5.4	Lock Management Functions

166	Not defined in this standard.

## 167	11.6	Combined Roles on a Single System

### 168	11.6.1	Overview

- As mentioned previously, the various roles of the management system can be deployed in different combinations to different systems throughout the managed environment. In general, there are no

- restrictions on what roles can be deployed on any given system, but some examples are given to illustrate typical situations. 171

### 11.6.2	General Purpose SMI-S Server as a Profile Aggregator

172

- 11.6.2.1	SLP Functions

- The General Purpose SMI-S Server role may implement SLP User Agent (UA) functionality as specified in 175 9.6, "User Agents (UA)". The General Purpose SMI-S Server discovers all Profiles within its configured

176 scope that are aggregated by querying for service specific attributes that match the criteria for those aggregations. 177

11.6.2.2	Generic Operations

178

The General Purpose SMI-S Server role may implement CIM Client functionality as specified by Generic 179 Operations standard and may implement CIM listener functionality as specified by the applicable WBEM 180 Protocol standard. A General Purpose SMI-S Server may reflect instances and classes from the 181 aggregated Profiles (perhaps by delegating operations to the Dedicated SMI-S Servers), but is not

182	required to do so. The Profile’s Model instances should be reflected in the advertised default namespace of the General Purpose SMI-S Server. The hierarchy of General Purpose SMI-S Servers and Dedicated 183	SMI-S Servers in a multi-level system needs to be reflected in the model such that it can be 184	administrated.

185

186 11.6.2.3 Security Considerations 187 There are no requirements for security for this role.

188

189	11.6.2.4	Lock Manager Functions

190

There are no requirements for locking in this release of the specification.

191

192

193

194

195

# 1	12  Installation and Upgrade

## 2	12.1	Introduction

3 The interoperability of the management communications in a storage network gives customers a choice in 4 vendors of their management solutions, but it also can introduce ease-of-use problems when these 5 different vendors each supply different components. In order to supply a complete management solution, 6 many management vendors provide not only WBEM Clients, Providers and other Management Interfaces, 7 but also software components that provide other pieces of the management infrastructure (e.g., Directory 8 Services, WBEM Services, Database Management). Problems are possible when multiple vendors install 9 or remove these components in the same configuration and conflicts can arise. One of the goals of 10 creating management interoperability is to reduce the time and expense end-users apply to the 11 management of their SANs. Thus, SAN management should be easy to install, easy to upgrade, and easy 12 to reconfigure. Mature management products using SMI-S technology should experience seamless and 13 almost completely automated installation, upgrade, and reconfiguration.

14 This clause deals with issues in installation, upgrade and uninstallation of products using SMI-S 15 technology, and recommends some steps that vendors should take to minimize the problems, leading to 16 better customer satisfaction with the overall management solution.

## 17	12.2	Role of the Administrator

18 Ultimately, a vendor’s installation software cannot make perfect decisions when the conflicts referenced 19 in 12.1 arise, since there may be valid reasons why a customer has deployed software of similar function 20 from multiple vendors. In the situation where two software components are both installed that perform the 21 same shared function, and only one can reasonably operate without conflicts, the administrator must be 22 able to resolve these conflicts and remove or disable the redundant component(s).

23 Installation software should, however, make a best effort to detect any conflicts and notify the 24 administrator of possible conflicts during its installation and initialization. A vendor’s installation software 25 should allow the administrator to install and uninstall the various infrastructure components on an 26 individual basis should such a conflict arise. The implications of this are that vendors are motivated to 27 support interoperation with other vendor’s components. The advantage to the vendor is that a customer is 28 more likely to install a component that can demonstrate the most interoperability with other components.

## 29	12.3	Goals

### 30	12.3.1	Non-Disruptive Installation and De-installation

31 WBEM Clients & Services, Providers, and Directory Services may be capable of being installed and de32 installed without disrupting the operation of other constituents in a SMI-S management environment. As 33 SANs are often deployed in mission critical environments the up-time of the solution is critical and thus, 34 the uptime of the management backbone as a key component of the solution is equally critical. 35 Additionally, the installation and de-installation of SMI-S interface constituents should not compromise 36 the availability of mission critical applications.

### 37	12.3.2	Plug-and-Play

38 The ultimate goal of management interoperability is zero administration of the management system itself. 39 A customer should be able to install new storage hardware and software and have the new component 40 become part of the management system automatically. Use of the Service Discovery process (see Clause 41 9 Service Discovery), the discovery-related aspects of the SMI-S Role definitions (see Clause 11 SMI-S 42 Roles), and the Server Profile (see Storage Management Technical Specification, Part 3 Common Profiles, 43 1.8.0 Rev 4 35 Server Profile) are intended to assist in achieving this goal.

44 During the reconfiguration of the management system, the schema that Clients see should remain 45 consistent (Schema forward compatibility is ensured via CIM standard).

###### Agents and Servers

## 46	12.4	Server Deployment

### 47	12.4.1	General

48 Manufacturers of storage hardware and software typically install their product and the accompanying 49 management support at the same time. Conflicts are possible between Agents if multiple vendors attempt 50 to install support for the same device. Also, when a device vendor needs to upgrade an Agent or Provider 51 for a device, the installation software needs to determine all of the locations of the previous installations 52 to insure there are not duplicate management paths to the device and thus, insure reliable on-going 53 operation of the device. SMI-S agent implementations must first determine the environment they will run 54 in and how they will be installed into that environment. These environments include:

- • Controlled environment (12.4.2)

- • Multiple WBEM Server environment (12.4.3)

- • Shared WBEM Server environment (12.4.4)

- For each of these issue areas, this clause provides requirements to authors of SMI-S agents and CIM59 based management software. These practices are designed to maximize interoperability.

### 60	12.4.2	Controlled Environment

61 A Controlled environment is either embedded in the system being managed or a dedicated management 62 processor that limits the software a user can install on it. Agents in controlled environments shall be 63 exempt from the requirements in 12.4.3 and 12.4.4.

### 64	12.4.3	Multiple WBEM Server systems

65

12.4.3.1	Agents and Servers

66

67 A system supporting multiple CIM agents may require multiple WBEM Servers. Because the SMI-S agent can’t control when multiple WBEM Servers are required, all SMI-S agents other than controlled 68 environments shall implement the Multiple WBEM Server requirements.

69

12.4.3.2	Determine Multiple WBEM Servers

70

71 Installation software for devices shall be able to locate existing WBEM Servers that may control the 72 device in order to offer an administrator a choice in management constituents for the device. In addition, 73 the installation software should locate existing Agents and Providers that provide device support in order 74 to reliably upgrade that support. For these reasons, an installation software program may act as a SMI-S

75	Client during installation. This will allow it to employ the Service Discovery (see Clause 9 Service

Discovery) to locate the appropriate functions, and to make the automated decisions that eliminate the 76 need for an administrator to manually configure or adjust certain aspects of the management system. 77

12.4.3.3	Ports

78

79 SMI-S uses TCP/IP, HTTP/HTTPS, and CIM-XML or WS-Management protocols. These protocols require 80 the use of TCP/IP ports. SMI-S defines the way a client discovers the server ports in 9 Service Discovery.

Any SMI-S agent (WBEM Server) that may be installed in an environment with other agents shall support 81 the use of alternate port addresses. The agent shall support user configured port addresses. 82

12.4.3.4	SLP

83

84 SMI-S requires the use of SLP for agent discovery (see Clause 9 Service Discovery). The SLP standard 85 requires the use of a “well known port” that may not be shared. Therefore, a computer system can only

- have one instance of a SLP service agent running on a system. All SMI agents on the system shall register with the common SLP service agent or provide user documentation that allows a user to manually register the agent and its profiles.

Directories

- 12.4.3.5	Directories

- Some environments require multiple copies of the same WBEM Server to be installed. This may be done 89 to solve version compatibility issues. SMI agents shall be coded to allow user settable directory names to 90 be used. Installation programs for SMI agents should find all instances of compatible WBEM Servers and 91 allow the user to select the WBEM Server installed into. The installation shall then install the agent in 92 directories relative to that WBEM Server.

- 12.4.3.6	Miscellaneous

- Conflicts are possible between Agents if multiple vendors attempt to install support for the same device. 95 Also, when a device vendor needs to upgrade an Agent or Provider for a device, the installation software 96 shall determine all of the locations of the previous installations to insure there is not duplicate 97 management paths to the device and thus, insure reliable on-going operation of the device.

- 12.4.3.7	Tools

- Utilities needed to manage the WBEM Server (e.g., users, configuration) shall be able to find the WBEM 100 Server and allow the user to select a WBEM Server if more than one is found.

### 101	12.4.4	Shared WBEM Server

102

12.4.4.1	Definition

103 A shared WBEM Server environment is when two or more unrelated providers share a single WBEM Server.

104

105

12.4.4.2	Namespaces

106

- In the case of shared WBEM Servers, namespaces help isolate implementations and reduce provider

- interaction. The device model should be implemented in a vendor specific namespace. A single vendor may choose to put multiple implementations in its own namespace. Vendor namespace names should be

- chosen to reduce any chance of conflict. The namespace name should include the vendor’s company

name or stock symbol.

110

111

12.4.4.3	Trivial sub classes

112

“CIM” classes should not be implemented directly. They should be subclassed using a name prefix unique

113	to their company. This sub classing prevents interaction between provides. Instances in the “interop”

namespace shall be subclassed.

114

115

12.4.4.4	“interop” namespace

116

117 The profile registration profile shall be implemented in a namespace named “interop”. The profile contains two parts. First part is a model of the WBEM Server. This section shall be implemented by the software 118 package that installs the WBEM Server. All other implementations shall extend the profile registration 119 profile with instances that define the profile they support.

120

- Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4, 36.3.4 The SMI-S

- Registered Profile shows what device support is already installed, and installation software should consult this schema before installing new software. If the installation software is changing the device

- support from one configuration to another, the installation software needs to uninstall or disable the

previous software support elements.

124

125

12.4.4.5	SLP support

126	Tenant providers shall extend the “Profile Registration Profile” and shall extend the SLP registered

profiles as required in the SMI-S discover clause.

127

128

###### Version/Change control

- 12.4.4.6	Version/Change control

- It is the responsibility of the SMI agent installation to protect the WBEM Server. The installation process 131 shall determine if a compatible WBEM Server is installed before becoming a tenant.

- 12.4.4.7	Base Server Profile

- Some profiles can extend the “Base Server Profile”. New providers should look for a “Base Server Profile” 134 to extend before installing its own.

135

### 136	12.4.5	Uninstallation

137 During the uninstallation of a device, the installation/uninstallation software (if available) should 138 automatically detect existing management support software for the device in order to shut down and

139 remove it in a consistent manner. This detection process need to be cognizant that SMI-S Clients may be actively using the device and that the device may need to be disabled for new management operations 140 and administrated through an orderly shutdown procedure prior to uninstallation. The implementation of 141 such procedures and any order dependency is outside the scope of this specification, but may need to be 142 considered by implementers.

143

### 144	12.4.6	Update

145 During the update of device support software, installation software should automatically detect any 146 existing device support software in order to successfully complete the upgrade. This device support may 147 exist on multiple hosts, but that situation is not specified in this version. If the update includes installing a

148 new provider, the installation software needs to use the provider installation/upgrade method that is supported by the existing Object Manager. When a software update involves a major schema version 149 upgrade (e.g., 2.x to 3.x), the installation software needs to be cognizant of the effect of the schema 150 upgrade on existing clients. For example, it may choose to simultaneously support both versions for some 151 period of time.

152

### 153	12.4.7	Reconfiguration

- When device support update requires an update of an agent or provider, the device support installation software should configure the new provider with the same subscriptions that exist in the old agent or

- provider before removing it, unless those subscriptions are specifically defined as being periodically cleaned up. This can be done via the instances of the subscriptions in the agent or object manager that 156	currently exist.

157

## 158	12.5	WBEM Service Support & Related Functions

159

### 12.5.1	Installation

160

Customers are increasingly sensitive to the size of the memory footprint for management software. The

- goal is to minimize the impact on hosts that are not dedicated to running management software by making

- appropriate choices during installation and giving the administrator control over these issues.

163

It is recommended that vendors take advantage of an existing Object Manager where one exists, by

- installing a provider that communicated with that Object Manager for device support. Additional support

- for such “multi-tenant” Object Managers will be included in a future version of this document.

- If an object manager does not exist, or the device support does not work with the existing object manager

167

(e.g., due to interface requirements) it is recommended that the vendor supply a Agent that is lightweight

- for device support. Another option is to offer to install an Object Manager that the vendor does have

- provider support for, allowing other vendors to further leverage that installation.

170

Providers that use an in-band connection to devices have an issue where zoning may alter the

- management path to the device from a provider or agent. In this case, the device support may need to be

- installed on multiple hosts in the network and the vendor needs to provide some way to coordinate which

- provider or agent is responsible for a particular device.

Base Server Profile

- Vendors should install their providers in a unique namespace for isolation and qualification reasons. The

- installer should employ the Service Discovery process (see Clause 9 Service Discovery), and/or the

Server Profile (see Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4 35

- Server Profile) to discover the existing namespaces and insure that the one created for the new device is

truly unique.

177

178

### 12.5.2	Multiple WBEM Servers on a Single Server System

179 At installation and setup, a user interface should be provided by the WBEM Server installation utility that 180 allows an administrator to manually set the TCP port number in a persistent fashion.

181

- To support discovery, the SLP Service Agent (see 9.7, "Service Agents (SAs)") associated with a newly-

- installed WBEM Server should register its TCP port number along with all the other necessary discovery information with the Discovery Service. This requirement applies to both automated port selection as well

- as manually configured installations. Clients, working through their SLP User Agent, described in 9.6, "User Agents (UA)", then use this information to establish contact with the WBEM Server.

185

186

### 12.5.3	Uninstallation/Upgrade

187

- An Object Manager may be upgraded without needing to change the Providers that it supports.

Depending on the Object Manager, the Providers may have to be reinstalled and reconfigured following

- such an upgrade. In this case, an administrator may need to re-run the device support installation

software and such software should be able to restore the previous configuration.

190

191

### 12.5.4	Reconfiguration

192	Device Support Reconfiguration (see 12.3.2, "Plug-and-Play") identifies issues that may also be

applicable to Object Managers.

193

194

### 12.5.5	Failure

195

- Temporary failure of an object manager (for example, a host being powered off) can result in bad installation decisions for installation software. In this case, it is advisable that the installation software

- provide for manual input of the characteristics of additional components of the management system that the installation process needs to consider.

198

## 12.6	Client

199

200

### 12.6.1	Uninstallation

201

When Client software is removed, the uninstallation software should ensure that all client-defined

202	information (settings, policies etc.), and any subscriptions for that client that exist in any agent or object

manager, are also removed.

203

204

### 12.6.2	Reconfiguration

205

Client software can include a Listener that is configured to listen on a specific port. When this port is

206 reconfigured, the client should redirect any Indication Handlers in existing agent and object managers as a result.

207

## 12.7	Directory Service

208

209

### 12.7.1	Installation

210

211 The installation of more than one Directory Agent—-addressed in 9.6, "User Agents (UA)"—or Service 212 Agent Server—addressed in 9.7, "Service Agents (SAs)"—providing a Directory Service in a management 213 system does not impose a significant burden for management clients and adds to the overall availability. Vendors should recommend to administrators of their products that one or more SA Servers or Directory

###### Base Server Profile

214	Agents should be deployed in the management system. This may also be done for network or system

management reasons.

215

216

### 12.7.2	Uninstallation/Failure

217	SLP Clients are defined to handle failure and uninstallation of DAs as per the specification (see Clause 9 218	Service Discovery).

219

## 12.8	Issues with Discovery Mechanisms

220

Experience with existing SMI-S installations has indicated that some sites have policies that can impact 221	the Service Discovery process (see Clause 9 Service Discovery). This subject will be addressed in 222	greater detail in a future revision of this document, but two specific items of guidance are given here, as 223	follows:

224 a) Where the site policy has caused multicast to be disabled, the DHCP option for SLP defined in IETF 225 RFC 2610 is recommended as an alternate method of locating Service Agent Servers or Directory 226 Agents. Also note that the shipping configuration of many network routers has multicast disabled.

227

b) Where the site policy has caused support for SLP itself to be disabled, an out of band method of providing a list of IP addresses for WBEM Servers is recommended, after which the Server Profile (see Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4 35 Server Profile) should be used to obtain the information about Registered Profiles usually retrieved via SLP.

#### IMPLEMENTED

# 1	13  Security

Security in the context of SMI-S refers to the protective measures employed in the management of 2 storage. The specific objectives to be addressed by security include:

1) Provide a mechanism that assures that the communications between a SMI-S client and server can3 not be read or modified by a third party (confidentiality and integrity).

4	2)	Provide a mechanism that allows SMI-S clients and servers to provide an assurance of their identity (authentication). 5

6 3) Provide a mechanism that allows control of the actions a SMI-S client is permitted to perform on a SMI-S server (authorization). 7

8 4) Provide a mechanism for records to be generated for actions performed by a SMI-S client on a SMI-S server (auditing). 9

10 5) Provide a mechanism that allows SMI-S clients to discover the SMI-S constituents in a storage network environment so that they may communicate with these constituents using CIM Operations over 11

HTTP protocol.

12

## 13	13.1	Requirements

14

### 13.1.1	Overview

15

Security requirements can be divided into five major categories:

16

- 1)	Authentication - verifying the identity of an entity (client or server)

- 2)	Authorization - deciding if an entity is allowed to perform a given operation

- 3)	Confidentiality - restricting information to only those intended recipients

- 4)	Integrity - guaranteeing that information, passed between entities, has not been modified

- 5) Non-repudiation - the ability to prove an action or event has taken place, so that this event or action cannot be denied or disavowed later. 22

SMI-S security primarily addresses authentication, confidentiality of communications, and authorization 23 to a lesser degree. Integrity has been left for future work, and non-repudiation is not currently identified 24 as a need for SMI-S.

- Security concerns occur in three areas of an SMI-S implementation:

- 1)	First, an SMI-S Server may also be a client of other services (sometimes conceptualized as a device). Those services, or devices, may require a login before discovery or operations are allowed 27 to be performed. The information needed to perform this login is generically referred to as "creden28 tials" (or in the case of devices as "device credentials"). An SMI-S server or provider needs to obtain

29 these credentials in order to talk to the service, and they should be provided confidentially.

30

2) Second, an SMI-S Server may need to authenticate an SMI-S Client. Not all Clients may be allowed 31 to query the object model, and not all Clients may be allowed to perform operations on objects in the

- model. The SMI-S Server is responsible for the process of authenticating credentials received from

- an SMI-S Client. Successful authentication establishes a trust relationship, which is represented on 34	the SMI-S Server by an authenticated Identity. Authenticating the client is the first step in determin35	ing what that Client is allowed to do.

36	3)	Thirdly, should implementers of an SMI-S Server be unaware of secure development practices, 37	attackers may be able to exploit resulting flaws in implementations.

38

### 39	13.1.2	 General Requirements for HTTP Implementations

40

The security requirements for HTTP implementations apply to both SMI-S servers and clients. An SMI-S

41 client shall comply with all security requirements for HTTP that are applicable to clients. The following are 42 general requirements for the support of security when using HTTP.

43

1)	SMI-S Servers and Clients shall conform to DMTF DSP0200 CIM Operations over HTTP.

44	2)	HTTP Basic Authentication shall be implemented. HTTP Digest Authentication should be imple45	mented. See 13.2.3.2 "User Authentication".

46

3)	To minimize compromising user identities, and credentials such as passwords, implementations

47 should use HTTP Basic Authentication ONLY in conjunction with TLS.

48

4)	Where TLS is not used, implementers should utilize HTTP Digest Authentication. See 13.2.3.2 "User

49	Authentication".

50

5) A user identity and credential used with one type of HTTP Authentication (i.e., Basic or Digest) shall 51 not ever be subsequently used with the other type of HTTP Authentication. To avoid compromising

52	the integrity of a stronger scheme, established good security practices avoid the reuse of identity & credential information across schemes of different strengths. See 13.2.3.2 "User Authentication".

53

54	6)	TLS as specified in SNIA TLS Specification for Storage Systems shall be implemented and should be used. 55

56	7)	Clients that fail to contact an SMI-S server via HTTP over TLS on TCP port 5989 should retry with 57	HTTP on TCP port 5988 if their security policy allows it.

58

8)	In order for Clients and Servers to communicate, they need to be using a consistent approach to

59 security. It is possible for properly configured Clients and Servers to fail to communicate if one is 60	relying upon port 5989 and the other on port 5988.

61

9)	Servers can accelerate discovery that a secure channel is needed by responding to HTTP contacts

- on TCP port 5988 with an HTTP REDIRECT to the appropriate HTTPS: URL (HTTP over TLS on

- TCP port 5989) to avoid the need for clients to timeout the HTTP contact attempt. Clients should honor such redirects in this situation.

64

## 65	13.2	Description of SMI-S Security

66

### 13.2.1	Security Scope

67

SMI-S security is primarily focused on securing the underlying network transport, authenticating users,

68 and securely interacting with IT infrastructure.

69

70

71

72

Overview

### 73	13.2.2	Transport Security

- For most SMI-S implementations, the Hypertext Transfer Protocol (HTTP) is the underlying

- communications protocol used to transfer SMI-S messages, but it is possible that other transports like Web Services for System Management (WS-Management) may be used. A major element of SMI-S s

76

CIM over HTTP is the mandatory transport mechanism for this version of SMI-S and the specific 77 requirements areecurity is focused on securing these underlying transports.

78

- derived from DMTF DSP0200, (Specification for CIM Operations over HTTP), which describes the

- requirements for CIM clients and servers. It is important to note that HTTP by itself offers no confidentiality or integrity protections.

SMI-S also includes a mechanism to secure HTTP communications such that data sent between the clients and servers are encrypted before being sent out over the network. This security is achieved by 81 transmitting HTTP over SSL/TLS (also known as HTTPS); the URL of a secure connection will begin with 82 https:// instead of http://. It is also important to note that an SMI-S Client communicates with an SMI-S 83 server via HTTPS on TCP port 5989 (TCP port 5988 is used for HTTP).

84

When TLS is used to secure HTTP, the client and server typically perform some form of entity 85 authentication. However, the specific nature of this entity authentication is dependent on the cipher suite 86 negotiated; a cipher suite specifies the encryption algorithm and digest algorithm to use on a SSL/TLS 87 connection. A very common scenario involves the use of server-side certificates, which the client trusts, 88 as the basis for unidirectional, entity authentication. It is possible that no authentication will occur (e.g.,

89 anonymous authentication) or on the other extreme, mutual authentication involving both client-side and server-side certificates may be required.

90

##### 91	STABLE

92

93

94 The specific requirements and options associated with the implementation of TLS within conformant 95 SMI-S systems are specified in SNIA TLS Specification for Storage Systems. The use of these TLS 96 features is strongly encouraged.

### 13.2.3	Authentication

13.2.3.1	Overview

97

At a basic level, authentication is the process used to identify a user (or entity) through the verification of 98 supplied information (i.e., verify a declared identity). This information is often a secret (e.g., a password), 99 but it may also be accomplished by possessing something (e.g., a smart card) or be something that you are (e.g., biometrics); combining multiple forms (or factors) of authentication credentials is known as 100 multi-factor authentication. Increasingly, strong (multi-factor) authentication is required for privileged 101 users or any remote access (including vendor access). It is also important to note that some of these

- credentials are static (i.e., indefinite use period) while others have expiration periods or may be one-time-

- use.

104

105 Within SMI-S, the dominant form of authentication is for users, but entity authentication does occur. In 106 addition, the SMI-S Servers frequently employ local authentication, but external authentication is an 107 option.

108

##### 109	STABLE

110

111	13.2.3.2	User Authentication

SMI-S Clients are responsible for initiating user authentication for each SMI-S Server that is accessed by a user. HTTP Basic Authentication shall be implemented and HTTP Digest Authentication should be implemented; HTTP Digest Authentication is a required contingency when authentication credentials Entity Authentication

have to be secured, but appropriate SSL or TLS protections cannot be negotiated. For both forms of HTTP authentication, the SMI-S Server functions as the authenticator and it receives the user credentials from the HTTP authentication operations.

112

Established good security practices avoid the reuse of identity & credential information across schemes 113 of different strengths. Thus, a SMI-S user identity and credential used with one type of HTTP 114 Authentication (i.e., Basic or Digest) shall not ever be subsequently used with the other type of HTTP 115 Authentication.

116

- Section 4.4 of DSP0200 defines additional requirements for HTTP authentication, above those found in

- IETF RFC 2616 or IETF RFC 2617. HTTP authentication generally starts with an HTTP client request, such as "GET Request-URI" (where Request-URI is the resource requested). If the client request does 119 not include an "Authorization" header line and authentication is required, the server responds with a "401 120 unauthorized" status code, and a "WWW-Authenticate" header line. The HTTP client shall then respond 121 with the appropriate "Authorization" header line in a subsequent request. The format of the "WWW122 Authenticate" and "Authorization" header lines varies depending on the type of authentication required:

basic authentication or digest authentication. If the authentication is successful, the HTTP server will

123 respond with a status code of "200 OK". 124

125 Basic authentication involves sending the user name and password in the clear, and should only be used 126 on a secure network, or in conjunction with a mechanism that ensures confidentiality, such as TLS. Digest 127 authentication sends a secure digest of the user name and password (and other information including a 128 nonce value), so that the password is not revealed. "401Unauthorized" responses should not include a 129 choice of authentication.

130

131	Client authentication to the SMI-S Server is based on an authentication service (local and/or external).

Differing authentication schemes may be supported, including host-based authentication, Kerberos, PKI, 132 or other; the authentication service is out of scope of this specification.

133

##### 134	STABLE

135

136 13.2.3.3	Entity Authentication

137

- Entity authentication is the process by which an agent in a distributed system gains confidence in the

- identity of a communication partner. More often than not, the entity authentication process is coupled with the distribution of a "session key" which the partners can later use for message confidentiality, integrity, or whatever else.

Within SMI-S, entity authentication is typically performed whenever SSL/TLS is used and it is accomplished using digital certificates. An SMI-S server may also use a form of entity authentication for 140 certain types of third-party authentications services. For example, RADIUS employs a shared secret to 141 protect certain user credentials. 142

### 143	13.2.4	Service Discovery

144

Service discovery protocols are network protocols which allow automatic detection of devices and 145 services offered by these devices on a computer network. Within the context of SMI-S (see Clause 9 146 Service Discovery), service discovery refers to the discovery of dedicated SMI-S servers, general 147 purpose SMI-S servers, and directory servers as well as the functions they offer in an SMI-S managed

148 environment. This release of SMI-S uses the Service Location Protocol Version 2 (SLPv2), as defined by IETF RFC 2608, for its basic discovery mechanism.

SLP is a packet-oriented protocol. Most packets are transmitted using UDP, but TCP can also be used for the transmission of longer packets. Because of the potential unreliability of UDP, SLP repeats all multicasts several times in increasing intervals until an answer has been received. All devices are required to listen on port 427 for UDP packets, SAs and DAs should also listen for TCP on the same port.

Entity Authentication

Multicasting is used extensively by SLP, especially by devices that join a network and need to find other devices.

The operation of SLP differs considerably, depending on whether a Directory Agent (DA) is in the network 149 or not. When a client first joins a network, it multicasts a query for DAs on the network. If no DA answers, 150 the client will assume that it is in a network without DAs. It is also possible to add DAs later, as they 151 multicast a "heartbeat" packet in a predefined interval that will be received by all other devices. When a 152 SA discovers a DA, it is required to register all services at the DA. When a service disappears the SA 153 should notify the DA and un-register it.

154 The SLPv2 security model assumes that service information is public, and therefore does not require 155 confidentiality. SLPv2 provides for authentication of service URLs and service attributes, thus providing 156 integrity assurances for service URLs and attributes included in SLP messages. For SMI-S environments 157 that require security in conjunction with the use of SLPv2, the major threat mitigation strategies (see RFC 158 3723) are not necessary as long as the SLP messages are not fully trusted and SSL/TLS with server 159 certificates is used. Additional security guidance is provided in 9 Service Discovery.

160

##### 161	IMPLEMENTED

162

163

164

165

166

167

168

169

170

171

172

173

174

175

176

177

178

179

180

181

182

183

184

185

186

187

188

189

190

191

Entity Authentication

# 1	Annex A (informative) SMI-S Information Model

2 This standard is based on DMTF’s CIM schema, Version 2.51. The DMTF schema is available in the 3 machine-readable Managed Object Format (MOF) format. DMTF MOFs are simultaneously released both 4 as an "Experimental" and a "Final" version of the schema. This provides developers with early access to 5 experimental parts of the models. Both versions are available at http://www.dmtf.org/standards/cim.

6

Content marked as “Experimental” or “Implemented” may be based on DMTF’s Experimental MOFs.

7

8

9

# 1	Annex B (normative) Compliance with the SNIA SMI Specification

- B.1	Compliance Statement

- The declaration of SMI-S compliance of a given CIM Instance within a WBEM Server also declares that 4 any CIM Instance associated, directly or indirectly, to the first CIM Instance will also be SMI-S compliant 5 if SMI-S itself declares compliance rules for either CIM Instance or instances of their superclasses. The 6 declaration of SMI-S compliance also declares that the implementation shall also conform to the SMI-S 7 architecture as defined in Storage Management Technical Specification, Part 2 Common Architecture.

- B.2	How Compliance of the Architecture is Declared

- An agent indicates which version of SMI-S it conforms to using “the SMI-S registered profile” as defined

- in the Profile Registration Profile (see Storage Management Technical Specification, Part 3 Common Profiles,

- 1.8.0 Rev 4 36.3.4 "The SMI-S Registered Profile"). The agent shall conform to all the provisions of the 12 versions of Storage Management Technical Specification, Part 2 Common Architecture where it 13 instantiates an instance of RegisteredProfile with a matching RegisteredVersion value.

- B.3	How Compliance of the Model Is Declared

- • The declaration of SMI-S compliance is made through the use of the Server Profile and the declaration of 16	supported profiles.

- • Direct association between CIM Instances is made through instance of a CIM Association.

- • Indirect association between CIM Instance is made through more than one CIM Association.

- • SMI-S Compliance is assessed against CIM Instances that are directly or indirectly associated to the CIM 20 Instance declared as part of the declaration of supported registered profiles. These CIM Instances comprise 21 the compliance test set.

22 • All CIM Instances / CIM Classes included in the compliance test set for whom compliance rules are defined in 23 SMI-S or for superclasses thereof shall be themselves be compliant to the rules defined in SMI-S.

24 • Compliance tests on a superclass of a given CIM Instance are limited to the attributes and behaviors defined 25	for the superclass.

26	B.4	The Server Profile and Compliance

27

B.4.1	General Guidelines

28

- Compliance is declared by the implementation of the Server Profile. All profiles require the Server Profile.

- The Server Profile defines the means by which a SMI-S Client determines the profiles  supported and the ComputerSystems associated. (see Storage Management Technical Specification, Part 3 Common Profiles, 31 1.8.0 Rev 4 35 "Server Profile" for more details.)

32

B.4.2	Example

33

34 A CIM Agent for Vendor X declares compliance to the Array Profile and the Pool Manipulation 35 Capabilities, and Setting profile through the Server Profile. Once the association (via the 36 ElementConformsToProfile association) is made to from the Array Profile declaration to the 37 ComputerSystem that realizes the Array Profile, then compliance tests begin testing compliance. Vendor

- X decided to extend the StorageVolume class with additional properties. StorageVolume is associated to

- the 	ComputerSystem 	via 	SystemDevice 	association. 	ComputerSystem, 	StorageVolume, 	and

SystemDevice are defined in SMI-S as required CIM elements (see Storage Management Technical

40

Specification, Part 4 Block Devices, 1.8.0 Rev 4 Table 2, “CIM Elements for Array”).

41

42

43 In implementing FCPort, Vendor X decided to not provide ElementName but did provide the rest of the 44 required properties. Vendor X decided to not use to WWN and instead used a vendor specific value for 45 the PermanentAddress (see 7 Correlatable and Durable Names) Additionally, Vendor X added FRUStatus 46 to their subclass of FCPort. Vendor X also decided to model the back-end fibre channel, but not use an 47 SMI-S model to do so. These back-end FCPorts are associated to the ComputerSystem via the

48	ConsumedSystemDevice association, a subclass of SystemDevice without properties overridden. These back-end fibre channel ports where modeled using a Vendor X specific class, BackendFCPorts, that is not 49	derived from FCPort. This BackendFCPorts were associated to the ComputerSystem with the 50	ConsumedSystemDevice.PartComponent role.

51

52 The compliance test includes FCPort because compliance declaration identified a particular 53 ComputerSystem the entry point into compliant CIM instantiation of the Array Profile. the compliance test

54 includes FCPorts as part of the test set because the SystemDevice association, also defined as part of the profile, includes the FCPort realized in that implementation. The compliance test also includes 55 BackendFCPorts because the ConsumedSystemDevice association to the ComputerSystem for these 56 instances is a SystemDevice association.

57

- The compliance test locates the StorageConfigurationService, StoragePools including a Primordial

- StoragePool, and StorageCapabilities associated to the ComputerSystem. Vendor X's implementation supports the creation of a StoragePool. The test attempts to create a StoragePool given one of the sizes

- reported by the Primordial StoragePool.getSupportedSizes() method using the Primordial StoragePool

reference and a StorageSetting generated from one of the StorageCapabilities.

61

- The compliance test for Vendor X's Array Profile implementation fails because:

- • FCPort.PermanentName property has a noncompliance value. Specifically, the FCPort.PermanentAddress is required to be WWN, 16 unseparated uppercase hex digits;

64

65 • ElementName property was not provided (i.e., was null);

66

• the SystemDevice associations contained references to BackendFCPort in the PartComponent property. CIM 67	defined that the PartComponent is a LogicalDevice. Since BackendFCPort is not a LogicalDevice, then the 68	test failed;

- • The “Size not supported” return code was returned from CreateOrModifyStoragePool even though one of the supported sizes was used verbatim.

- The compliance test for Vendor X's Array Profile implementation did not fail because:

71

    - StorageVolume was extended;

72

    - SystemDevice was extended.

73

74

B.5	Backward Compatibility

75

- B.5.1	Goals

- Backward compatibility between versions of SMI-S profiles is a requirement with very few exceptions. 78 The goals of backwards compatibility include:

79	a)	New profile implementations that are deployed in a customer environment work with existing SMI-S Clients. This includes:

80

- 1)	SMI-S operations, including CTP, continue to work against the new profile implementation;

- 2)	 SMI-S Clients can support a given profile version and above (later minor version numbers);

83

b) No guarantee of backwards compatibility is implied between major version numbers (i.e., 1.x to 2.x);

- c)	If a profile in a newer version of SMI-S cannot maintain backward compatibility, it shall be

- renamed (and the old profile deprecated). Otherwise the client may assume that the newer profile is backwards compatible and that all operations in the earlier version will continue to work in

- this newer version.

87

d)	It shall be possible for SMI-S provider and client implementations to support older versions of an

88	incompatible profile.

89

- e)	Content marked experimental is not standard in this version of the specification. Future versions

- of the specification may not be backwards compatible to content marked experimental in this version. Content marked experimental in this version of the specification may be removed in a

- future version. 93	B.5.2	Overview

94

95 SMI-S backward compatibility is necessary to ensure that customer environments are minimally disrupted 96 by newer implementations of SMI-S. Deployment of several concurrent implementations of multiple minor

97	versions of SMI-S shall be possible in a customer environment. Compatibility is required from both the

Client side and from the provider side. Compatibility also has aspects both in the specification of newer 98 functionality via SMI-S and in the implementation of both providers and clients.

99

Figure B.1 shows the interaction between a Client coded to an older minor version of SMI-S (M.m) acting against a later minor version (M.n) provider implementation

Figure B.1 - Provider Migration 100

101 As shown in Figure B.1, the newer implementation shall support all of the old operations from the 102 previous minor version of SMI-S in order to maintain compatibility. The Client will not be able to take 103 advantage of any newer features that have been added in the later version of the specification, but will

104 still be able to accomplish all of the functions it was coded for in the previous version. This allows minimum disruption to the customer environment.

105

106 Clients shall be written to take advantage of the functionality of implementations that are currently 107 shipping and that are or will soon be deployed in customer environments. This client functionality needs 108 to be careful in how it makes use of each SMI-S version's new features. Any client code that uses a 109 specific version's features shall also include a version check against the profile version in the 110 RegisteredProfile instance for that functionality. This version check shall verify that the functionality is at 111 a specific minor version and above (up to the next major release). If a client were only to check for a 112 specific version, it would not be able to use newer implementations of that functionality. A client will, over 113 time, contain multiple such code blocks as newer versions are supported. Each piece of code will be

114 written to the functionality introduced in a specific version and continue to work against that functionality in later minor releases.

115

B.5.3	Requirements

116

- In order to maintain backwards compatibility with older minor versions of the specification, profile authors

- have followed specific rules in developing the specification. The requirements that were followed in profile versioning and shall be followed by subsequent implementations include:

119

120 • Support for required classes: A newer minor version of an SMI-S profile shall support all required classes of the previous minor version of the profile and shall continue to require them.

121

122 • Support for conditional classes: A newer minor version of an SMI-S profile shall support all conditional 123 classes of the previous minor version of the profile and shall continue to require them as specified in the 124 conditions of the previous minor version. But the newer minor version may add other conditions under which

125 the class will be required. In addition, conditional classes in a previous minor version may be promoted to required in a newer minor version. 126

127 • Support for optional classes: A newer minor version of an SMI-S profile may promote a class to Conditional or Mandatory any class that was optional in the previous minor version.

128

- • Deprecation of classes: A newer minor version of an SMI-S profile may deprecate or include deprecated

- (via the CIM schema) classes introduced in previous minor version(s), but shall continue to require their implementation. 131

132 • Support for required properties: A newer minor version of an SMI-S profile shall support all required properties of classes in the previous minor version(s) of the profile and shall continue to require them.

133

134 • Support for conditional properties: A newer minor version of an SMI-S profile shall support all conditional 135 properties of classes in the previous minor version(s) of the profile and shall continue to require them as 136 specified by the conditions of the previous minor version. But the newer minor version may add other

137 conditions under which the property will be required. In addition, conditional properties in a previous minor version may be promoted to required in a newer minor version.

138

139 • Support for optional classes: A newer minor version of an SMI-S profile may promote a class to Conditional or Mandatory any class that was optional in the previous minor version.

140

- • Deprecation of properties: A newer minor version of an SMI-S profile may deprecate or include deprecated

- (via the CIM schema) properties of classes introduced in previous minor version(s), but shall continue to require their implementation. 143

144 • Support for component profiles: A newer minor version of an SMI-S profile shall support the functionality of 145 all component profiles of the previous minor version(s) of the profile and shall continue to require them if they 146 were required in the previous version. A newer minor version of an SMI-S profile may require a component 147 profile that was optional or conditional in the previous minor version, but shall not make optional or 148 conditional a component profile that was required in a previous minor version. If a newer minor version of an 149 SMI-S profile does not have component profiles by the same name as previous minor version(s), it shall still

150 require implementation of the Registered (Sub)Profile with the previous version information such that the client will be able to find and use the subsumed functionality.

151

- • A newer minor version of an SMI-S profile shall support all conditional component profiles of the previous

- minor version of the profile and shall continue to require them as specified in the conditions of the previous minor version. But the newer minor version may add other conditions under which the component profile will be required.

- Profile renaming: A newer minor version of an SMI-S profile that cannot remain backwards compatible shall 155	either become a major revision of the profile or shall be renamed to a different profile name such that a client 156	will not find newer, incompatible, versions of that functionality.

- B.5.4	Implementation Considerations

- Even in the case of a newer minor version of an SMI-S profile that was unable to retain backward 159 compatibility, an implementation may support clients with a separate implementation of the previous 160 minor version's functionality. Implementations shall not implement these earlier versions in such a way 161 that a client of the previous minor version would become confused or break when accessing this 162 functionality. This may happen if the previous version's functionality is implemented in the same 163 namespace as the later version, but a careful evaluation needs to be done by the implementer to 164 determine this.

165

B.6	Rules for Combining (Autonomous) Profiles

166

B.6.1	General

167

SMI-S specifies the behavior of (autonomous) profiles. The rules for compliance and backward 168 compatibility are defined in the context of a profile (an Autonomous Profile). This subclause defines the 169 rules that shall be applied when a device (or program) wishes to support the behavior of multiple 170 (autonomous) profiles.

171

The guiding principles in such support are:

172

• Maintain Compliance (see B.1 through B.4)

173

Combining (autonomous) profiles shall not break compliance rules for any of the combined individual

174	profiles.

175

• Maintain Backward Compatibility (see B.5)

176

Combining (autonomous) profiles shall not break backward compatibility for any of the combined individual

177	profiles.

178

B.6.2	Backward Compatibility Rules for combining profiles

179

The backward compatibility rules apply to combined profiles in that combined profile implementations that

180	are deployed in a customer environment shall work with SMI-S clients of any one of the profiles that were 181	combined:

182

- Support for required classes: A combination of SMI-S profiles shall support all required classes of the 183 individual profiles that have been combined and shall continue to require them. If a class is required in one 184 individual profile, it shall be required in the combination profile.

185

- Support for conditional classes: A combination of SMI-S profiles shall support all conditional classes of the 186 individual profiles that have been combined and shall continue to require them as specified in the conditions 187 of individual profiles that have been combined. If a class is conditional in one or more of the individual profiles 188 (and not required in any other individual profile) then it shall be conditional in the combination profile. If a 189 class is conditional in multiple individual profiles, but with different conditions, then all conditions shall yield 190 the existence of the class. 191

- Deprecation of classes: A combination of SMI-S profiles shall include any deprecated (via the CIM schema) 192 classes introduced by any one of the individual profiles that are combined, and shall continue to require their 193 implementation. Similarly, conditions for deprecated conditional classes shall apply (as stated in the support 194 for conditional classes).

195

196 Support for required properties: A combination of SMI-S profiles shall support all required properties of 197 classes in any one of the individual profiles that are combined and shall continue to require them. If a

- property is required in any of the individual profiles, then the property will be required in the combined profile.

- • Support for conditional properties: A combination of SMI-S profiles shall support all conditional properties 200 of classes in the individual profiles that are combined and shall continue to require them as specified by the 201 conditions of the individual profiles that are combined. If a property is conditional in one or more of the 202 individual profiles (and not required in any other individual profile) then it shall be conditional in the 203 combination profile. If a property is conditional in multiple individual profiles, but with different conditions, then 204 all conditions shall yield the existence of the class.

205 • Deprecation of properties: A combination of SMI-S profiles may include deprecated (via the CIM schema) 206 properties of classes introduced in any one of the individual profiles that are combined, and shall continue to 207 require their implementation. Similarly, conditions for deprecated conditional properties shall apply (as stated 208 in the support for conditional properties).

209 • Support for component profiles: A combination of SMI-S profiles shall support the functionality of all 210 component profiles of all of the individual profiles that are combined and shall continue to require them if they 211 were required in any one of the individual profiles that are combined. If a combination of SMI-S profiles 212 results in two references to a component profile by the same name from multiple individual profiles that were 213 combined, the combined profile may require multiple implementations if the scomponent profiles in question 214 have different major version numbers. And if the component profiles have different minor version numbers,

215 then the higher version number shall be implemented (since it provides backward compatibility to the earlier component profile). 216

217 If a component profile is required in any one of the individual profiles then it will be required in the combined profile. 218

219 If a component profile is not required in any of the individual profiles, but is conditional in at least one of the 220 individual profiles, then it will be conditional in the combined profile. If a component profile is conditional in

221 multiple individual profiles (that are being combined) then all conditions shall yield existence of the component profile.

222

B.6.3	Conditions for a New Profile

223

224 If any of the conditions outlined in B.6 Rules for Combining (Autonomous) Profiles cannot be satisfied, then a new profile shall be defined that represents the desired semantic of the device (or program) in question.

##### 225	EXPERIMENTAL

226

B.7	Rules for Vendor Extensions

227

228 SMI-S is intended to be extended by vendor implementations to cover vendor function that is not covered 229 by SMI-S. Such extensions allow clients to exploit vendor functions that are not covered by SMI-S, when

230 the client has awareness of the specific functions of the implementation. However, the extensions need to be done in such a way that they do not cause clients that support the functions in SMI-S to fail. This 231 clause describes the rules for doing vendor unique extensions to SMI-S.

- B.7.1	Objectives for Vendor Extension Rules

- The basic objectives for the rules associated with vendor extensions are:

- • Vendor extensions shall follow the compliance rules (as defined in B.3).

- • Vendor extensions shall follow the backward compatibility rules (as defined in B.5).

- Vendor extensions shall avoid extensions that nullify the existing SMI-S.

- • Vendor extensions shall avoid extensions that would confuse clients.

- B.7.2	Vendor Extensions and Compliance Rules

- When implementing a vendor extension, the following rules shall be followed:

240

  - When an implementation claims compliance to an SMI-S profile (RegisteredOrganization=”SNIA”) the 241 implementation shall honor the behavior for CIM Elements and methods as outlined in the Profile.

242

  - All CIM Elements (Classes, properties and methods) defined by the SNIA profile shall be honored, including 243	mandatory and conditional elements.

244

For example, if an SMI-S Profile defines a StorageVolume class with mandatory properties, a vendor 245 extension to the profile may not define a StorageVolume that has fewer mandatory properties.

246

  - Similarly, if the StorageVolume class is mandatory, a vendor extension may not render the use of the class 247 as conditional (or optional).

248 • Instances of CIM associations between CIM Instances shall exist as defined by the profile.

249

• A vendor extension may add associations, but the mandatory and conditional associations between 250 instances of a class specified by the profile shall exist.

251

252	For example, if an SMI-S profile defines a mandatory DeviceSAPImplementation association between a

ProtocolEndpoint and a LogicalPort, a vendor extension that adds a new ProtocolEndpoint shall also have 253 the DeviceSAPImplementation association.

254

• If this is not reasonable for the extension, for whatever reason, the vendor extension should consider using

255	different classes.

256

- When a vendor extension uses a superclass of a given CIM class used in the SMI-S profile, the extension 257 shall honor the attributes and behaviors defined for the superclass.

258

- If a vendor extension uses “System” in an SMI-S Profile that defines “ComputerSystem” classes, the 259 extension shall honor properties and associations of the inherited from System defined in the SMI-S profile.

260	B.7.3	Vendor Extensions and Backward Compatibility Rules

261

When the backward compatibility rules are applied to vendor extended SMI-S profiles, the following rules 262	shall apply: 263

• Vendor extensions that are deployed in a customer environment shall work with existing SMI-S Clients and 264 SMI-S Clients that are not aware of the extensions.

- This includes:

- • SMI-S operations continue to work against the extended profile implementation.

- • SMI-S Clients can support a given profile version or extended versions of a given profile.

- • A vendor extended implementation of an SMI-S Profile shall be backward compatible to the SMI-S profile.

269

- • If an extended version of an SMI-S profile cannot maintain backward compatibility to the SMI-S profile, it shall

- be defined as a different profile (e.g., RegisteredOrganization=”VendorID”). Otherwise the client may assume that the extended profile is backwards compatible and that all operations on the SMI-S profile will continue to 272	work in the extended version of the profile.

273

274

SNIA Technical Position

275	B.7.4	Vendor Extensions and SMI-S Nullification

276

Vendor extensions shall avoid extensions that nullify an existing SMI-S profile. This includes, but is not 277	limited to: 278

• Adding Classes that the implementation considers mandatory such that a client will fail if it does not establish 279 instances of the class. 280

This does not mean that such classes cannot be provided by the provider implementation. But it cannot expect 281	an SMI-S client to overtly create such class instances.

282

283 • Adding class properties that an implementation considers mandatory such that a client will fail if it does not set values for such properties 284

- For example, if a SettingData is used as a parameter of an SMI-S extrinsic method, a vendor extension cannot extend the SettingData with a mandatory property such that it will fail the client if it does not set the extended

- property.

- An implementation may extend a SettingData class that is used in an SMI-S extrinsic method if the 288 implementation supports a default value for the property. • A vendor extension shall not extend the method signature of any SMI-S extrinsic method.

289

290 If a vendor wants to define an extended version of an SMI-S method, it should define a new method with the extended parameter list. 291

292 Ideally, the vendor extension should support the SMI-S method, however, this is not required if the profile has a capabilities class that identifies whether or not the method in question is supported.

293

- • A vendor extension should not extend the definition of a property unless the SMI-S profile makes provisions for “vendor extensions” to the property.

- For example, properties that are enumerations with a “vendor extension” range that is formally recognized in 296 the SMI-S specification may be extended (vendor extension enumerations added).

297	However, if a property that is an enumeration, but SMI-S does not formally recognize that a “vendor extension” range of the enumeration, then the vendor extension should not use the property for this.

298

B.7.5	Vendor Extensions that Avoid Client Confusion

299

Vendor extensions shall avoid extensions that would confuse clients. This includes, but is not limited to:

300

301 • Reuse of Classes used by SMI-S profiles should be avoided (e.g., vendor extended usages).

302

For example, if an SMI-S autonomous profile defines four uses of StorageExtent, it would be unfortunate if a 303 vendor extension defined a fifth usage of StorageExtent. The general SMI-S client will be looking for four 304 different uses of StorageExtent and will likely get confused by the fifth usage. To avoid this, the vendor extension should define a new class (with all the StorageExtent properties that apply).

305

• Use of CIM properties not specified by SMI-S

306

- There are several properties “not specified” by SMI-S, but are included in CIM MOFs. An example might be

- Caption on ManagedElement. Vendor extensions should avoid using CIM properties for a specific purpose defined by the vendor extension. 309

Future versions of SMI-S may, in fact, define a specific use of the CIM property and the chances that it

310 matches the use in every vendor extension is somewhat unlikely. 311 It is safer if the vendor extension defines its own unique properties rather than attempt to re-use CIM properties. 312

#### EXPERIMENTAL

.FICON™ is an example of a suitable product available commercially. This information is given for the convenience of users of this standard and does not constitute an endorsement of this product by SNIA or any standards organization.