Storage Management Technical Specification, Part 6 Fabric

Version 1.8.0, Revision 5

Abstract: This SNIA Techncial Position defines an interface between WBEM-capable clients and servers for the secure, extensible, and interoperable management of networked storage.

This document has been released and approved by the SNIA. The SNIA believes that the ideas, methodologies and technologies described in this document accurately represent the SNIA goals and are appropriate for widespread distribution. Suggestions for revision should be directed to http://www.snia.org/feedback/.

SNIA Technical Position

23 March 2020

2

- USAGE

- Copyright © 2018 SNIA. All rights reserved. All other trademarks or registered trademarks are the property of their 3	respective owners. 4

The SNIA hereby grants permission for individuals to use this document for personal use only, and for corporations

5 and other business entities to use this document for internal use only (including internal copying, distribution, and display) provided that:

6

- 1)	Any text, diagram, chart, table or definition reproduced shall be reproduced in its entirety with no alter-

- ation, and,

9

- 2) Any document, printed or electronic, in which material from this document (or any portion hereof) is reproduced shall acknowledge the SNIA copyright on that material, and shall credit the SNIA for granting

- permission for its reuse.

- Other than as explicitly provided above, you may not make any commercial use of this document or any portion 13 thereof, or distribute this document to third parties. All rights not explicitly granted are expressly reserved to SNIA.

14 Permission to use this document for purposes other than those enumerated above may be requested by e-mailing 15 tcmd@snia.org. Please include the identity of the requesting individual and/or company and a brief description of 16 the purpose, nature, and scope of the requested use.

17

All code fragments, scripts, data tables, and sample code in this SNIA document are made available under the 18	following license:

19	BSD 3-Clause Software License

20

21	Copyright (c) 2018, The Storage Networking Industry Association.

22

Redistribution and use in source and binary forms, with or without modification, are permitted provided that the 23 following conditions are met: 24

25 • Redistributions of source code must retain the above copyright notice, this list of conditions and the following disclaimer. 26

27 • Redistributions in binary form must reproduce the above copyright notice, this list of conditions and the 28 following disclaimer in the documentation and/or other materials provided with the distribution.

29 • Neither the name of The Storage Networking Industry Association (SNIA) nor the names of its contributors may 30 be used to endorse or promote products derived from this software without specific prior written permission.

31

32	THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS" AND ANY

EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF 33 MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL 34 THE COPYRIGHT OWNER OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, 35 SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO,

36 PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN

37

CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

38

39

40

41

42

43

44

45

SNIA Technical Position 	3

46	DISCLAIMER

47

48 The information contained in this publication is subject to change without notice. The SNIA makes no 49 warranty of any kind with regard to this specification, including, but not limited to, the implied warranties 50 of merchantability and fitness for a particular purpose. The SNIA shall not be liable for errors contained 51 herein or for incidental or consequential damages in connection with the furnishing, performance, or use 52 of this specification.

53	Suggestions for revisions should be directed to http://www.snia.org/feedback/.

54

Copyright © 2003-2020 SNIA. All rights reserved. All other trademarks or registered trademarks are the

55 property of their respective owners. 56

- Portions of the CIM Schema are used in this document with the permission of the Distributed

- Management Task Force (DMTF). The CIM classes that are documented have been developed and reviewed by both the SNIA and DMTF Technical Working Groups. However, the schema is still in

- development and review in the DMTF Working Groups and Technical Committee, and subject to change.

60

61

62

63

64

65

66

67

68

69

70

71

72

73

74

75

4

# REVISION HISTORY

Revision 1

Date

April 14, 2017

SCRs Incorporated and other changes

Annex A (informative) SMI-S Information Model

- This standard is now based on DMTF’s CIM schema Version 2.50.

Fabric Profile

SMIS-180-Draft-SCR00002.html

- Added the ability to discover and control peer zoning

SMIS-180-Draft-SCR00003.html)

- Added additional FC Port Speeds

Zone Control Profile (SMIS-180-Draft-SCR00002.html)

- Added the ability to discover and control peer zoning

Comments

Editorial notes and DRAFT material are displayed.

Revision 2

Date

March 1, 2018

SCRs Incorporated and other changes

Annex A (informative) SMI-S Information Model

- This standard is now based on DMTF’s CIM schema Version 2.51.

Fabric Profile (SMI TWG Reviews)

- Corrected the Related Profile for FabricVirtualFabrics to be Virtual Fabrics

- Fixed the version numbers on the Related Profiles to match what the profiles claim

- Deleted the MemberOfCollections in the CIM Elements for the filter collections that were deleted

- Defined the condition for Peer Zoning in CIM_ZoneSettingData (ZoneMembershipSettingData to Zone)

Enhanced Zoning and Enhanced Zoning Control Profile (SMI TWG Reviews)

- Fixed the version numbers on the Related Profiles to match what the profiles claim

Zone Control Profile (SMI TWG Reviews)

- Changed the version of the Profile to be 1.8.0, since we added two methods

FDMI Profile (SMI TWG Reviews)

- Changed the version of the Profile to be 1.8.0, since we expanded the Speed enumerations

Fabric Views Profile (SMI TWG Reviews)

- In FCSwitchView changed OperationalStatus to SwitchOperationalStatus and FCPortEnabledState to PortEnabledState

- In ConcreteComponentView changed Antecedent and Dependent to GroupComponent and PartComponent

- Changed the version of the profile to 1.7.0

Virtual Fabrics Profile (SMI TWG Reviews)

- Changed the name of the profile to "Virtual Fabrics" to make the spec readable

Switch Profile (SMI TWG Reviews)

- Fixed the version numbers on the Related Profiles to match what the profiles claim

- Changed the version of the profile to 1.8.0, do to changes to the enumeration of the Speed property

- Removed the Switch Configuration Data profile from Related Profiles table, since it has been removed from the spec

- Changed the name of the FabricSwitchPartitioning Profile to Switch Partitioning in the Related Profiles table

- Changed the Requirement for CIM_ElementSettingData (FCSwitchSettings to ComputerSystem) to Mandatory, since FCSwitchSettings and the Switch are Mandatory.

Blades Profile (SMI TWG Reviews)

- Added descriptions for References in CIM_ProductPhysicalComponent- Changed the version to 1.7.0

Switch Partitioning Profile (SMI TWG Reviews)

- Changed the name of the profile to "Switch Partitioning" to make the spec readable- Changed “must” to “shall” in a number of CIM Element tables

N Port Virtualizer Profile (SMI TWG Reviews)

- Fixed the version numbers on the Related Profiles to match what the profiles claim

Inter Fabric Routing Profile (SMI TWG Reviews)

- Added a SystemDevice between the IFR Switch and the IFR FCPort- Changed the Profile version to 1.7.0

Revision 3

Date

August 14, 2018

SCRs Incorporated and other changes

FCoE Fabric (TSG-SMIS-SCR00331)

- Reworked the profile to be a component profile of the Fabric Profile

- Removed the classes associated with experimental indications

- Added a definition of CIM_EthernetPort (which was missing) - Fixed a number of mifgen warnings

Fabric Views Profile (TSG-SMIS-SCR00333)

- Changed the Central Class from FCTopologyView to CIM_ViewCapabilities

Switch Partitioning Profile (TSG-SMIS-SCR00333)

- Changed the Central Class from ComputerSystem to CIM_ComputerSystem (Partitioning)

N Port Virtualizer Profile (TSG-SMIS-SCR00333)

- Changed the Central Class from FCPort to CIM_FCPort (Fabric NPIV)

Inter Fabric Routing Profile (TSG-SMIS-SCR00333)

- Changed the Central Class from ComputerSystem to CIM_ComputerSystem (IFR Switch)

Annex B (Informative) Structure of Fabric Profiles (TSG-SMIS-SCR00331)

- Updated Figure B.1 to show how FCoE Fabrics fits into the structure of Fabric profiles

Comments None.

Revision 4

Date

October 24, 2019

SCRs Incorporated and other changes None.

Comments

Minor editorial clean up to create corrected technical position.

Revision 5

Date

March 23, 2020

SCRs Incorporated and other changes None.

Comments

Revision number updated to keep all books in sync after a minor update to the architecture book.

Suggestion for changes or modifications to this document should be sent to the SNIA Storage Management

Initiative Technical Work Group (SMI TWG) at http://www.snia.org/feedback/

- INTENDED AUDIENCE

- This document is intended for use by individuals and companies engaged in developing, deploying, and 3	promoting interoperable multi-vendor SANs through the Storage Networking Industry Association (SNIA) 4	organization.

- CHANGES TO THE SPECIFICATION

- Each publication of this specification is uniquely identified by a three-level identifier, comprised of a 7 version number, a release number and an update number. The current identifier for this specification is 8 version 1.8.0. Future publications of this specification are subject to specific constraints on the scope of 9 change that is permissible from one publication to the next and the degree of interoperability and

10 backward compatibility that should be assumed between products designed to different publications of this standard. The SNIA has defined three levels of change to a specification:

11

• Major Revision: A major revision of the specification represents a substantial change to the underlying scope 12 or architecture of the SMI-S API. A major revision results in an increase in the version number of the version

13 identifier (e.g., from version 1.x.x to version 2.x.x). There is no assurance of interoperability or backward compatibility between releases with different version numbers.

14

15 • Minor Revision: A minor revision of the specification represents a technical change to existing content or an 16 adjustment to the scope of the SMI-S API. A minor revision results in an increase in the release number of

17 the specification’s identifier (e.g., from x.1.x to x.2.x). Minor revisions with the same version number preserve interoperability and backward compatibility.

18

• Update: An update to the specification is limited to minor corrections or clarifications of existing specification

19 content. An update will result in an increase in the third component of the release identifier (e.g., from x.x.1 to

20

x.x.2). Updates with the same version and minor release levels preserve interoperability and backward

21 compatibility.

22

- TYPOGRAPHICAL CONVENTIONS

- Maturity Level 25

26 In addition to informative and normative content, this specification includes guidance about the maturity 27 of emerging material that has completed a rigorous design review but has limited implementation in

28 commercial products. This material is clearly delineated as described in the following sections. The typographical convention is intended to provide a sense of the maturity of the affected material, without 29 altering its normative content. By recognizing the relative maturity of different sections of the standard, an 30 implementer should be able to make more informed decisions about the adoption and deployment of 31 different portions of the standard in a commercial product.

32

This specification has been structured to convey both the formal requirements and assumptions of the 33 SMI-S API and its emerging implementation and deployment lifecycle. Over time, the intent is that all 34 content in the specification will represent a mature and stable design, be verified by extensive 35 implementation experience, assure consistent support for backward compatibility, and rely solely on

36 content material that has reached a similar level of maturity. Unless explicitly labeled with one of the subordinate maturity levels defined for this specification, content is assumed to satisfy these 37 requirements and is referred to as “Finalized”. Since much of the evolving specification

38 content in any given release will not have matured to that level, this specification defines three 39 subordinate levels of implementation maturity that identify important aspects of the content’s increasing 40 maturity and stability. Each subordinate maturity level is defined by its level of implementation 41 experience, its stability and its reliance on other emerging standards. Each subordinate maturity level is 42 identified by a unique typographical tagging convention that clearly distinguishes content at one maturity 43 model from content at another level.

44

45

- Experimental Maturity Level

- No material is included in this document unless its initial architecture has been completed and reviewed. 48 Some content included in this document has complete and reviewed design, but lacks implementation 49 experience and the maturity gained through implementation experience. This content is included in order 50 to gain wider review and to gain implementation experience. This material is referred to as 51 “Experimental”. It is presented here as an aid to implementers who are interested in likely future

52 developments within the SMI specification. The contents of an Experimental profile may change as implementation experience is gained. There is a high likelihood that the changed content will be included 53 in an upcoming revision of the specification. Experimental material can advance to a higher maturity level

54 as soon as implementations are available. Figure 1 is a sample of the typographical convention for 55

Experimental content.

56

64

EXPERIMENTAL

Experimental content appears here.

EXPERIMENTAL

Figure 1 - Experimental Maturity Level Tag

57

58

59

60

61

62

63

65

66

67	Implemented Maturity Level

68

Profiles for which initial implementations have been completed are classified as “Implemented”. This

69 indicates that at least two different vendors have implemented the profile, including at least one provider implementation. At this maturity level, the underlying architecture and modeling are stable, and changes in future revisions will be limited to the correction of deficiencies identified through additional implementation experience. Should the material become obsolete in the future, it must be deprecated in a minor revision of the specification prior to its removal from subsequent releases. Figure 2 is a sample of the typographical convention for Implemented content.

72

IMPLEMENTED

Implemented content appears here.

IMPLEMENTED

Figure 2 - Implemented Maturity Level Tag

70

71

73

74

75	Stable Maturity Level

Once content at the Implemented maturity level has garnered additional implementation experience, it can be tagged at the Stable maturity level. Material at this maturity level has been implemented by three different vendors, including both a provider and a client. Should material that has reached this maturity level become obsolete, it may only be deprecated as part of a minor revision to the specification. Material at this maturity level that has been deprecated may only be removed from the specification as part of a major revision. A profile that has reached this maturity level is guaranteed to preserve backward compatibility from one minor specification revision to the next. As a result, Profiles at or above the Stable 76 maturity level shall not rely on any content that is Experimental. Figure 3 is a sample of the typographical 77 convention for Implemented content.

STABLE

Stable content appears here.

STABLE

Figure 3 - Stable Maturity Level Tag

- Finalized Maturity Level

- Content that has reached the highest maturity level is referred to as “Finalized.” In addition to satisfying 80 the requirements for the Stable maturity level, content at the Finalized maturity level must solely depend 81 upon or refine material that has also reached the Finalized level. If specification content depends upon 82 material that is not under the control of the SNIA, and therefore not subject to its maturity level 83 definitions, then the external content is evaluated by the SNIA to assure that it has achieved a 84 comparable level of completion, stability, and implementation experience. Should material that has 85 reached this maturity level become obsolete, it may only be deprecated as part of a major revision to the 86 specification. A profile that has reached this maturity level is guaranteed to preserve backward

- compatibility from one minor specification revision to the next. Over time, it is hoped that all specification content will attain this maturity level. Accordingly, there is no special typographical convention, as there is with the other, subordinate maturity levels. Unless content in the specification is marked with one of the typographical conventions defined for the subordinate maturity levels, it should be assumed to have reached the Finalized maturity level.

Deprecated Material

Non-Experimental material can be deprecated in a subsequent revision of the specification. Sections identified as “Deprecated” contain material that is obsolete and not recommended for use in new development efforts. Existing and new implementations may still use this material, but shall move to the newer approach as soon as possible. The maturity level of the material being deprecated determines how long it will continue to appear in the specification. Implemented content shall be retained at least until the next revision of the specialization, while Stable and Finalized material shall be retained until the next

- major revision of the specification. Providers shall implement the deprecated elements as long as it 89 appears in the specification in order to achieve backward compatibility. Clients may rely on deprecated 90 elements, but are encouraged to use non-deprecated alternatives when possible.

91

92 Deprecated sections are documented with a reference to the last published version to include the 93 deprecated section as normative material and to the section in the current specification with the 94 replacement. Figure 4 contains a sample of the typographical convention for deprecated content.

95

DEPRECATED

Content that has been deprecated appears here.

DEPRECATED

96

97

98

99

100

101

102

103	Figure 4 - Deprecated Tag

104

105

106

107

108

109

110

111

112

113

114

115

116

117

118

119

120

121

122

123

124

125

126

127

128

129

130

131

Contents

Revision History	5

List of Figures	17

List of Tables	19

Foreword	25

1 Scope	27

2 Normative References	29

2.1 Approved references	29

2.2 References under development	29

2.3 Other references	29

3 Terms, Definitions, Symbols, Abbreviations, and Conventions	31

4 Fabric Profile	33

4.1 Synopsis	33

4.2 Description	34

4.3 Health and Fault Management	40

4.4 Cascading Considerations	40

4.5 Methods of this Profile	40

4.6 Use Cases	44

4.7 CIM Elements	46

5 Enhanced Zoning and Enhanced Zone Control Profile	73

5.1 Synopsis	73

5.2 Description	73

5.3 Health and Fault Management	73

5.4 Cascading Considerations	73

5.5 Methods of this Profile	73

5.6 Use Cases	74

5.7 CIM Elements	74

6 Zone Control Profile	77

6.1 Synopsis	77

6.2 Description	77

6.3 Durable Names and Correlatable IDs of the Profile	77

6.4 Instrumentation Requirements	77

6.5 Health and Fault Management	77

6.6 Cascading Considerations	77

6.7 Methods of this Profile	78

6.8 Use Cases	83

6.9 CIM Elements	83

7 FDMI Profile	87

7.1 Synopsis	87

7.2 Description	87

7.3 Health and Fault Management	88

7.4 Cascading Considerations	88

7.5 Methods of this Profile	88

7.6 Use Cases	88

7.7 CIM Elements	89

8 Fabric Views Profile	99

8.1 Description	99

8.2 Health and Fault Management Consideration	102

8.3 Cascading Considerations	103

8.4 Methods of the Profile	103

8.5 Use Cases	103

8.6 CIM Elements	103

9 Virtual Fabrics	109

9.1 Synopsis	109

9.2 Description	109

9.3 Health and Fault Management Consideration	112

9.4 Cascading Considerations	112

9.5 Methods of the Profile	112

9.6 Use Cases	112

9.7 CIM Elements	113

10 Switch Profile	115

10.1 Synopsis	115

10.2 Description	115

10.3 Health and Fault Management	120

10.4 Cascading Considerations	120

10.5 Methods of this Profile	120

10.6 Use Cases	120

10.7 CIM Elements	121

11 Blades Profile	139

11.1 Synopsis	139

11.2 Description	139

11.3 Instance Diagram	139

11.4 Health and Fault Management	139

11.5 Cascading Considerations	140

11.6 Methods of this Profile	140

11.7 Use Cases	140

11.8 CIM Elements	140

12 Switch Partitioning	145

12.1 Synopsis	145

12.2 Description	145

12.3 Health and Fault Management Consideration	147

12.4 Cascading Considerations	147

12.5 Methods of the Profile	147

12.6 Use Cases	147

12.7 CIM Elements	147

13 N Port Virtualizer Profile	155

13.1 Synopsis	155

13.2 Description	155

13.3 Implementation	156

13.4 Health and Fault Management Consideration	157

13.5 Cascading Considerations	157

13.6 Methods of the Profile	157

13.7 Use Cases	157

13.8 CIM Elements	157

14 Inter Fabric Routing Profile	165

14.1 Synopsis	165

14.2 Description	165

14.3 Health and Fault Management Consideration	168

14.4 Cascading Considerations	168

14.5 Methods of the Profile	168

14.6 Use Cases	168

14.7 CIM Elements	169

15 FCoE Fabric	177

15.1 Synopsis	177

15.2 Description	177

15.3 Health and Fault Management Consideration	178

15.4 Methods of the Profile	178

15.5 Use Cases	178

15.6 CIM Elements	179

Annex A (informative) SMI-S Information Model	189

Annex B (Informative) Structure of Fabric Profiles	191

# LIST OF FIGURES

Figure 1  -  Experimental Maturity Level Tag.......................................................................................10 Figure 2  -  Implemented Maturity Level Tag .......................................................................................10 Figure 3  -  Stable Maturity Level Tag..................................................................................................11 Figure 4  -  Deprecated Tag.................................................................................................................11 Figure 5  -  Fabric Instance..................................................................................................................34

Figure 6  -  NPIV Instance....................................................................................................................36 Figure 7  -  Zoning Instance (AdminDomain).......................................................................................37 Figure 8  -  Zoning Instance (ComputerSystem)..................................................................................38 Figure 9  -  Zoning Instance (Peer Zoning)..........................................................................................39 Figure 10  -  FDMI Instance .................................................................................................................88 Figure 11  -  Class Diagram for Fabric View Classes ........................................................................100 Figure 12  -  Fabric View Class Capabilities ......................................................................................101 Figure 13  -  FCTopology View Class ................................................................................................102 Figure 14  -  FCSwitch View Class ....................................................................................................102 Figure 15  -  RegisteredProfile, AdminDomain, and ComputerSystem Relationships.......................110 Figure 16  -  Two Virtual Fabric and Two Partitioning Systems.........................................................111 Figure 17  -  Two Virtual Fabrics and One Partitioning System.........................................................112 Figure 18  -  Switch Instance Diagram...............................................................................................117 Figure 19  -  Trunking Instance Diagram ...........................................................................................118 Figure 20  -  Switch Blade Instance ...................................................................................................139 Figure 21  -  Switch ComputerSystem and Partitioning System ........................................................145 Figure 22  -  Switch and Partitioning System and Partitioning Ports..................................................146 Figure 23  -  Underlying System Port Settings and Capabilities ........................................................146 Figure 24  -  N Port Virtualizer ...........................................................................................................156 Figure 25  -  IFR Switch Topology .....................................................................................................166

Figure 26  -  Inter Fabric Routing Ports..............................................................................................168 Figure 27  -  FCoE Fabric Instance Diagram .....................................................................................178

Figure B.1 The structure of the Fabric Part Profiles 191

Figure B.2 AdminDomain Properties 194

Figure B.3 Associations on AdminDomains 196

Figure B.4 Properties of various Switches 198

Figure B.5 Associations on Switches 200

Figure B.6 General ComputerSystem Properties 202

Figure B.7 General ComputerSystem Associations 204

Figure B.8 Switch FCPort Properties 206

Figure B.9 Switch FCPort Associations 208

Figure B.10 Non-Switch Port Properties 210

Figure B.11 Non-Switch Port Associations 212

# LIST OF TABLES

| Table 1 - | Supported Profiles for Fabric...............................................................................................................33 |
| --- | --- |
| Table 2 - | Port OperationalStatus........................................................................................................................45 |
| Table 3 - | OperationalStatus for ComputerSystem..............................................................................................45 |
| Table 4 - | CIM Elements for Fabric......................................................................................................................46 |
| Table 5 - | SMI Referenced Properties/Methods for CIM_ActiveConnection........................................................48 |
| Table 6 - | SMI Referenced Properties/Methods for CIM_AdminDomain (Fabric)................................................49 |
| Table 7 - | SMI Referenced Properties/Methods for CIM_AdminDomain (SAN) ..................................................49 |
| Table 8 - | SMI Referenced Properties/Methods for CIM_Component (Platform to Fabric) .................................50 |
| Table 9 - | SMI Referenced Properties/Methods for CIM_Component (Switch to Fabric)....................................50 |

Table 10  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Host Platform)...............................51 Table 11  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioned Switch)........................51

Table 12  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Storage Platform) .........................52 Table 13  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Switch)..........................................53

Table 14  - SMI Referenced Properties/Methods for CIM_ConnectivityCollection ................................................53

Table 15  - SMI Referenced Properties/Methods for CIM_ContainedDomain.......................................................54 Table 16  - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (Non-Switch to FCPort)...54 Table 17  - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (Switch to FCPort) ..........55 Table 18  - SMI Referenced Properties/Methods for CIM_ElementCapabilities (ZoneCapabilities to Fabric.)......55 Table 19  - SMI Referenced Properties/Methods for CIM_ElementCapabilities (ZoneCapabilities to Switch.).....55

Table 20  - SMI Referenced Properties/Methods for CIM_ElementSettingData (ZoneMembershipSettingData to Zone)56

Table 21  - SMI Referenced Properties/Methods for CIM_FCActiveConnection...................................................56

Table 22  - SMI Referenced Properties/Methods for CIM_FCPort (Host FCPort).................................................57

Table 23  - SMI Referenced Properties/Methods for CIM_FCPort (Host NPIV FCPort)........................................58

Table 24  - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned Switch FCPort)............................59 Table 25  - SMI Referenced Properties/Methods for CIM_FCPort (Storage FCPort)............................................60

Table 26  - SMI Referenced Properties/Methods for CIM_FCPort (Switch FCPort)..............................................61

Table 27  - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (AdminDomain to ProtocolEndpoint)62

Table 28  - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (ComputerSystem to ProtocolEndpoint)63

Table 29  - SMI Referenced Properties/Methods for CIM_HostedCollection (Fabric to ConnectivityCollection)...63 Table 30  - SMI Referenced Properties/Methods for CIM_HostedCollection (System to LogicalPortGroup)........63 Table 31  - SMI Referenced Properties/Methods for CIM_HostedCollection (Zones or ZoneSets to Fabric) .......64 Table 32  - SMI Referenced Properties/Methods for CIM_HostedCollection (Zones or ZoneSets to Switch).......64 Table 33  - SMI Referenced Properties/Methods for CIM_HostedDependency....................................................65 Table 34  - SMI Referenced Properties/Methods for CIM_LogicalPortGroup........................................................65

Table 35  - SMI Referenced Properties/Methods for CIM_MemberOfCollection (ConnectivityCollection to ProtocolEndpoint)65

Table 36  - SMI Referenced Properties/Methods for CIM_MemberOfCollection (LogicalPortGroup to FCPort)...66 Table 37  - SMI Referenced Properties/Methods for CIM_MemberOfCollection (ZoneSet to Zone).....................66 Table 38  - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint ........................................................66 Table 39  - SMI Referenced Properties/Methods for CIM_SystemDevice (Non-Switch FCPort to Fabric) ...........67

Table 40  - SMI Referenced Properties/Methods for CIM_SystemDevice (Non-Switch FCPort to Platform)........67 Table 41  - SMI Referenced Properties/Methods for CIM_SystemDevice (Switch FCPort to Switch)...................68 Table 42  - SMI Referenced Properties/Methods for CIM_Zone (Active) ..............................................................68 Table 43  - SMI Referenced Properties/Methods for CIM_Zone (Inactive)............................................................68 Table 44  - SMI Referenced Properties/Methods for CIM_ZoneCapabilities.........................................................69 Table 45  - SMI Referenced Properties/Methods for CIM_ZoneMembershipSettingData.....................................70 Table 46  - SMI Referenced Properties/Methods for CIM_ZoneSet (Active).........................................................70 Table 47  - SMI Referenced Properties/Methods for CIM_ZoneSet (Inactive) ......................................................71

Table 48  - SMI Referenced Properties/Methods for CIM_ZoneSettingData (ZoneMembershipSettingData to Zone)71

Table 49  - Supported Profiles for Enhanced Zoning and Enhanced Zoning Control............................................73

Table 50  - CIM Elements for Enhanced Zoning and Enhanced Zoning Control...................................................74

Table 51  - SMI Referenced Properties/Methods for CIM_ElementSettingData (ZoneMembershipSettingData to NamedAddressCollection)74

Table 52  - SMI Referenced Properties/Methods for CIM_HostedCollection (AdminDomain to Collection)..........75 Table 53  - SMI Referenced Properties/Methods for CIM_HostedCollection (ComputerSystem to Collection) ....75 Table 54  - SMI Referenced Properties/Methods for CIM_MemberOfCollection...................................................75 Table 55  - SMI Referenced Properties/Methods for CIM_NamedAddressCollection...........................................76 Table 56  - SMI Referenced Properties/Methods for CIM_ZoneService (Zone Service).......................................76 Table 57  - CIM Elements for Zone Control...........................................................................................................83

Table 58  - SMI Referenced Properties/Methods for CIM_HostedService (Fabric (AdminDomain) to ZoneService)84

Table 59  - SMI Referenced Properties/Methods for CIM_HostedService (Switch (ComputerSystem) to ZoneService)84

Table 60  - SMI Referenced Properties/Methods for CIM_ZoneService (Zone Service).......................................84

Table 61  - CIM Elements for FDMI.......................................................................................................................89 Table 62  - SMI Referenced Properties/Methods for CIM_Component (Host Server to Fabric)............................90 Table 63  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Host Server) .................................90 Table 64  - SMI Referenced Properties/Methods for CIM_ControlledBy...............................................................90 Table 65  - SMI Referenced Properties/Methods for CIM_ElementSoftwareIdentity.............................................91

Table 66  - SMI Referenced Properties/Methods for CIM_FCPort (Host FCPort).................................................91 Table 67  - SMI Referenced Properties/Methods for CIM_HostedCollection (System to LogicalPortGroup)........92 Table 68  - SMI Referenced Properties/Methods for CIM_InstalledSoftwareIdentity ............................................93 Table 69  - SMI Referenced Properties/Methods for CIM_MemberOfCollection (LogicalPortGroup to FCPort)...93 Table 70  - SMI Referenced Properties/Methods for CIM_PhysicalPackage (HBA Package) ..............................93 Table 71  - SMI Referenced Properties/Methods for CIM_PortController .............................................................94 Table 72  - SMI Referenced Properties/Methods for CIM_Product (HBA Product)...............................................94 Table 73  - SMI Referenced Properties/Methods for CIM_ProductPhysicalComponent .......................................95 Table 74  - SMI Referenced Properties/Methods for CIM_Realizes......................................................................95 Table 75  - SMI Referenced Properties/Methods for CIM_SoftwareIdentity (Driver).............................................96 Table 76  - SMI Referenced Properties/Methods for CIM_SoftwareIdentity (Firmware) .......................................96

Table 77  - SMI Referenced Properties/Methods for CIM_SoftwareIdentity (Option ROM) ..................................97 Table 78  - SMI Referenced Properties/Methods for CIM_SystemDevice (ComputerSystem to FCPort).............97 Table 79  - SMI Referenced Properties/Methods for CIM_SystemDevice (ComputerSystem to PortController) ..98

Table 80  - CIM Elements for Fabric Views.........................................................................................................103 Table 81  - SMI Referenced Properties/Methods for CIM_ElementCapabilities (View Capabilities)...................104 Table 82  - SMI Referenced Properties/Methods for CIM_ElementView (View to FC Port)................................104 Table 83  - SMI Referenced Properties/Methods for CIM_ElementView (View to Switch)..................................104 Table 84  - SMI Referenced Properties/Methods for CIM_ViewCapabilities .......................................................105

Table 85  - SMI Referenced Properties/Methods for CIM_ConcreteComponentView (FCSwitchView to Fabric)105

Table 86  - SMI Referenced Properties/Methods for FC_ConcreteComponentView (FCTopologyView to Fabric)106

Table 87  - SMI Referenced Properties/Methods for CIM_FCSwitchView ..........................................................106

Table 88  - SMI Referenced Properties/Methods for CIM_FCTopologyView ......................................................108 Table 89  - Supported Profiles for Virtual Fabrics................................................................................................109 Table 90  - CIM Elements for FabricVirtualFabrics..............................................................................................113

Table 91  - SMI Referenced Properties/Methods for CIM_Component (AdminDomain to Partitioning CS)........113

Table 92  - SMI Referenced Properties/Methods for CIM_ElementConformsToProfile (SAN AdminDomain to Virtual Fabrics RegisteredProfile)114

Table 93  - Supported Profiles for Switch............................................................................................................115

Table 94  - DetailedPortState for FCPort.............................................................................................................119 Table 95  - PortAvailability for FCPort .................................................................................................................119 Table 96  - CIM Elements for Switch ...................................................................................................................121

Table 97  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioned Switch)......................122 Table 98  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Switch)........................................123 Table 99  - SMI Referenced Properties/Methods for CIM_ComputerSystemPackage........................................125

Table 100  - SMI Referenced Properties/Methods for CIM_ElementCapabilities (FCPort to FCPortCapabilities)125

Table 101  - SMI Referenced Properties/Methods for CIM_ElementCapabilities (System to FCSwitchCapabilities)125

Table 102  - SMI Referenced Properties/Methods for CIM_ElementSettingData (FCPortSettings to FCPort).....126

Table 103  - SMI Referenced Properties/Methods for CIM_ElementSettingData (FCSwitchSettings to ComputerSystem)126

Table 104  - SMI Referenced Properties/Methods for CIM_ElementStatisticalData (FCPortRateStatistics to FCPort)126

Table 105  - SMI Referenced Properties/Methods for CIM_ElementStatisticalData (FCPortStatistics to FCPort)127

Table 106  - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned Switch FCPort).........................127 Table 107  - SMI Referenced Properties/Methods for CIM_FCPort (Switch FCPort)...........................................128 Table 108  - SMI Referenced Properties/Methods for CIM_FCPortCapabilities...................................................130 Table 109  - SMI Referenced Properties/Methods for CIM_FCPortRateStatistics ...............................................131 Table 110  - SMI Referenced Properties/Methods for CIM_FCPortSettings ........................................................131 Table 111  - SMI Referenced Properties/Methods for CIM_FCPortStatistics.......................................................132 Table 112  - SMI Referenced Properties/Methods for CIM_FCSwitchCapabilities...............................................134 Table 113  - SMI Referenced Properties/Methods for CIM_FCSwitchSettings ....................................................134 Table 114  - SMI Referenced Properties/Methods for CIM_HostedCollection (Redundancy Set)........................135 Table 115  - SMI Referenced Properties/Methods for CIM_HostedCollection (Statistics Collection)...................135

Table 116  - SMI Referenced Properties/Methods for CIM_MemberOfCollection (FCPort to RedundancySet)...136

Table 117  - SMI Referenced Properties/Methods for CIM_MemberOfCollection (NetworkPortStatistics to StatisticalCollection)136

Table 118  - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint .....................................................136 Table 119  - SMI Referenced Properties/Methods for CIM_RedundancySet.......................................................137 Table 120  - SMI Referenced Properties/Methods for CIM_StatisticsCollection...................................................137 Table 121  - SMI Referenced Properties/Methods for CIM_SystemDevice..........................................................138 Table 122  - CIM Elements for Blades..................................................................................................................140 Table 123  - SMI Referenced Properties/Methods for CIM_LogicalModule..........................................................141 Table 124  - SMI Referenced Properties/Methods for CIM_ModulePort ..............................................................141 Table 125  - SMI Referenced Properties/Methods for CIM_PhysicalPackage (Logical Module)..........................141 Table 126  - SMI Referenced Properties/Methods for CIM_Product (Blade)........................................................142 Table 127  - SMI Referenced Properties/Methods for CIM_ProductPhysicalComponent ....................................142 Table 128  - SMI Referenced Properties/Methods for CIM_Realizes (Logical Module Package) ........................143 Table 129  - SMI Referenced Properties/Methods for CIM_SystemDevice (Logical Module)..............................143 Table 130  - Supported Profiles for SwitchPartitioning .........................................................................................145 Table 131  - CIM Elements for SwitchPartitioning ................................................................................................147 Table 132  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioned) ................................148 Table 133  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioning)................................149

Table 134  - SMI Referenced Properties/Methods for CIM_ElementCapabilities (Association between NetworkPort and NetworkPortCapabilities)149

Table 135  - SMI Referenced Properties/Methods for CIM_ElementConformsToProfile (Partitioning ComputerSystem to Switch Partitioning RegisteredProfile)149

Table 136  - SMI Referenced Properties/Methods for CIM_ElementSettingData (Association between NetworkPort and NetworkPortSettings)150

Table 137  - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned).................................................150

Table 138  - SMI Referenced Properties/Methods for CIM_HostedDependency (NetworkPort to FCPort)..........151

Table 139  - SMI Referenced Properties/Methods for CIM_HostedDependency (Partitioning CS to Partitioned CS)151

Table 140  - SMI Referenced Properties/Methods for CIM_NetworkPort (Partitioning)........................................152 Table 141  - SMI Referenced Properties/Methods for CIM_NetworkPortCapabilities ..........................................152

Table 142  - SMI Referenced Properties/Methods for CIM_NetworkPortSettings................................................153

Table 143  - SMI Referenced Properties/Methods for CIM_SystemDevice (FCPort to Partitioned ComputerSystem)153

Table 144  - SMI Referenced Properties/Methods for CIM_SystemDevice (NetworkPort to ComputerSystem)..153

Table 145  - Supported Profiles for N Port Virtualizer...........................................................................................155 Table 146  - CIM Elements for N Port Virtualizer..................................................................................................157 Table 147  - SMI Referenced Properties/Methods for CIM_Component (N Port Virtualizer to Fabric).................158

Table 148  - SMI Referenced Properties/Methods for CIM_ComputerSystem (N Port Virtualizer) ......................158

Table 149  - SMI Referenced Properties/Methods for CIM_ComputerSystemPackage (N Port Virtualizer to Physical Package)159

Table 150  - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (ProtocolEndpoint to Gateway FCPort)159

Table 151  - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (ProtocolEndpoint to NPIV FCPort)159

Table 152  - SMI Referenced Properties/Methods for CIM_FCActiveConnection (Gateway) ..............................160 Table 153  - SMI Referenced Properties/Methods for CIM_FCActiveConnection (N Port Virtualiztion)...............160 Table 154  - SMI Referenced Properties/Methods for CIM_FCPort (Fabric NPIV)...............................................161 Table 155  - SMI Referenced Properties/Methods for CIM_FCPort (Gateway)....................................................162

Table 156  - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (N Port Virtualizer System to ProtocolEndpoint)163

Table 157  - SMI Referenced Properties/Methods for CIM_LogicalIdentity (NPIV Port to Switch Port)...............163

Table 158  - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint (N Port Virtualizer).......................163

Table 159  - SMI Referenced Properties/Methods for CIM_SystemDevice (N Port Virtualizer Gateway FCPort to Gateway System)164

Table 160  - SMI Referenced Properties/Methods for CIM_SystemDevice (N Port Virtualizer NPIV FCPort to Gateway System)164

Table 161  - CIM Elements for Inter Fabric Routing .............................................................................................169 Table 162  - SMI Referenced Properties/Methods for CIM_Component (Backbone Switch to Fabric)................169 Table 163  - SMI Referenced Properties/Methods for CIM_Component (IFR Switch to Fabric) ..........................170 Table 164  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Backbone Switch)......................170 Table 165  - SMI Referenced Properties/Methods for CIM_ComputerSystem (IFR Switch)................................171 Table 166  - SMI Referenced Properties/Methods for CIM_FCActiveConnection................................................172 Table 167  - SMI Referenced Properties/Methods for CIM_FCPort (IFR FCPort)................................................173 Table 168  - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint .....................................................174 Table 169  - SMI Referenced Properties/Methods for CIM_SystemDevice..........................................................175 Table 170  - CIM Elements for FCoE Fabric.........................................................................................................179 Table 171  - SMI Referenced Properties/Methods for CIM_AdminDomain (Ethernet Network)...........................180 Table 172  - SMI Referenced Properties/Methods for CIM_Component (Switch to Ethernet Network) ...............180

Table 173  - SMI Referenced Properties/Methods for CIM_ComputerSystem (Ethernet Switch) ........................181

Table 174  - SMI Referenced Properties/Methods for CIM_ConnectivityCollection (LANEndpoints to EthernetNetwork)181

Table 175  - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (Ethernet Switch EthernetPort to LANEndpoint)182

Table 176  - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (Non-Switch EthernetPort to LANEndpoint)182

Table 177  - SMI Referenced Properties/Methods for CIM_ActiveConnection (Links transported over Ethernet (FCoE))182

Table 178  - SMI Referenced Properties/Methods for CIM_EthernetPort (Host EthernetPort).............................183 Table 179  - SMI Referenced Properties/Methods for CIM_EthernetPort (Storage EthernetPort))......................183

Table 180  - SMI Referenced Properties/Methods for CIM_EthernetPort (Switch EthernetPort))........................184

Table 181  - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (AdminDomain to LANEndpoint)184

Table 182  - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (ComputerSystem to LANEndpoint)185

Table 183  - SMI Referenced Properties/Methods for CIM_HostedCollection (Ethernet Network AdminDomain to ConnectivityCollection)185

Table 184  - SMI Referenced Properties/Methods for CIM_HostedDependency (ComputerSystem (Ethernet Switch) to Partitioning ComputerSystem)186

Table 185  - SMI Referenced Properties/Methods for CIM_HostedDependency (FCPort to EthernetPort).........186

Table 186  - SMI Referenced Properties/Methods for CIM_LANEndpoint (Ethernet ProtocolEndpoint)..............186

Table 187  - SMI Referenced Properties/Methods for CIM_MemberOfCollection (ConnectivityCollection to LANEndpoint)187

Table 188  - SMI Referenced Properties/Methods for CIM_SystemDevice (Switch EthernetPort to Switch).......187

Table 189  - SMI Referenced Properties/Methods for CIM_SystemDevice (non-Switch EthernetPort to Ethernet AdminDomain)188

Table B.1  AdminDomain Properties 193

Table B.2  AdminDomain Associations 194

Table B.3  Switch ComputerSystem Properties 197 Table B.4  Switch ComputerSystem Associations 198

Table B.5  Non-Switch ComputerSystem Properties 201

Table B.6  Non-Switch ComputerSystem Associations 202

Table B.7  Switch FCPort Properties 205

Table B.8  Switch FCPort Associations 206

Table B.9  Non-Switch Port Properties 209

Table B.10  Non-Switch Port Associations 210

# FOREWORD

The Fabric Part of the Storage Management Technical Specification defines management profiles for

Autonomous (top level) profiles for programs and devices whose central function is providing support for storage networking. The Fabric Part includes fabric management including topology and device

- management for switches. The Fabric part also provides management of extenders that pass fibre

- channel frames over other protocols as well as a gateway that maps and translates iSCSI to Fibre 7

Channel. As part of fabric management, this specification also has controls for fibre channel zoning and 8 fibre channel security.

9

Parts of this Standard

10

This standard is subdivided in the following parts:

11

  - Storage Management Technical Specification, Part 1 Overview, 1.8.0 Rev 4

12

  - Storage Management Technical Specification, Part 2 Common Architecture, 1.8.0 Rev 4

13

  - Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4

14

  - Storage Management Technical Specification, Part 4 Block Devices, 1.8.0 Rev 4

15

  - Storage Management Technical Specification, Part 5 Filesystems, 1.8.0 Rev 4

16

  - Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4

17

  - Storage Management Technical Specification, Part 7 Host Elements, 1.8.0 Rev 4

18

  - Storage Management Technical Specification, Part 8 Media Libraries, 1.8.0 Rev 4

19

SNIA Web Site

20

Current SNIA practice is to make updates and other information available through their web site at 21 http://www.snia.org

22

SNIA Address

23

Requests for interpretation, suggestions for improvement and addenda, or defect reports are welcome. 24

They should be sent via the SNIA Feedback Portal at http://www.snia.org/feedback/ or by mail to the

25

Storage Networking Industry Association, 4360 ArrowsWest Drive, Colorado Springs, Colorado 80907, 26

U.S.A.

# 1  Scope

The Storage Management Technical Specification, Part 6 Fabric defines management profiles for

Autonomous (top level) profiles for programs and devices whose central function is providing support for storage networking. This version of Storage Management Technical Specification, Part 6 Fabric includes 5 these autonomous profiles:

• Fabric

6

This profile defines the model and functions of a storage network including topology and zoning control.

- • Switch

- This profile defines the model and functions of a Fibre Channel Switch including state, status, and control of the device and it’s connections and product information,

9

• Extender

10

11 This profile defines the model and functions of a networking device that allows for fibre channel to be extended over other networks, and specifically over IP (FCIP).

12

13

14

15

16

17

# 2  Normative References

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

## 2.1	Approved references

5	ISO/IEC 14776-452 SCSI Primary Commands - 3 (SPC-3) [ANSI INCITS 408-2005]

6

## 2.2	References under development

- Storage Management Technical Specification, Part 2 Common Architecture, 1.8.0 Rev 4

- Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4

## 9	2.3	Other references

10	DMTF DSP1054 Indications Profile 1.2.2 http://www.dmtf.org/sites/default/files/standards/documents/DSP1054_1.2.2.pdf

11

12

13

14

# 3  Terms, Definitions, Symbols, Abbreviations, and Conventions

For the purposes of this document, the terms, definitions, symbols, abbreviations, and conventions given in Storage Management Technical Specification, Part 2 Common Architecture, 1.8.0 Rev 4 apply.

4

STABLE

# 1	4  Fabric Profile

## 4.1	Synopsis

2	Profile Name: Fabric (Autonomous Profile)

3

Version: 1.8.0

4	Organization: SNIA

5

Central Class: AdminDomain

Scoping Class: AdminDomain

Related Profiles: Table 1 describes the supported profiles for Fabric.

Table 1 - Supported Profiles for Fabric

| Profile Name | Organization | Version | Requirement | Description |
| --- | --- | --- | --- | --- |
| Zone Control | SNIA | 1.8.0 | Optional |  |
| Enhanced Zoning and Enhanced Zoning Control | SNIA | 1.4.0 | Optional |  |
| Virtual Fabrics | SNIA | 1.5.0 | Optional | Experimental. |
| N Port Virtualizer | SNIA | 1.7.0 | Optional | Experimental. |
| FDMI | SNIA | 1.8.0 | Optional |  |
| Fabric Views | SNIA | 1.7.0 | Optional | Experimental. |
| FCoE Fabric | SNIA | 1.8.0 | Optional | Experimental. |
| Launch In Context | DMTF | 1.0.0 | Optional | Experimental. See DSP1102, version 1.0.0 |
| Indications | DMTF | 1.2.2 | Mandatory | See DSP1054, version 1.2.2 |

6

7

8

9 10

11

12

13

## 14	4.2	Description

15

4.2.1	Instance Diagram

Figure 7: "Fabric Instance" illustrates the basic elements of the Fabric Profile.

16

43	Figure 7 - Fabric Instance

44

4.2.2	SANS and Fabrics as AdminDomains

45

A SAN and Fabric are represented in CIM by AdminDomain. A SAN contains one or more Fabrics, which

46	are modeled as AdminDomains. The “containment” of Fabrics to SANs is through the association

ContainedDomain. AdminDomain is sub-classed from System. This is significant because a SAN and a 47

Fabric can be considered a group of components that operate together as a single system and should be/ 48 are managed as such. The relationship of the Fabrics in a SAN is intended to be able to aggregate all the 49

Fabrics that are in the same physical topology. The possible use cases include: Fibre Channel Virtual 50

Fabrics which have multiple Virtual Fabrics in the same underlying physical topology, Fibre Channel Inter-

51

Fabric Routing which routes Fibre Channel frames between Fabrics, and redundant Fabrics where two 52 physically separate networks connect the same set of hosts and storage.

53

An AdminDomain in CIM is keyed by the property Name with an associated optional property NameFormat. 54

The naming convention for SANs are left up to the implementation, but the Name shall be correlatable

55

(the same) when multiple agents are managing the same SAN for Virtual Fabric (VF) and Inter-Fabric 56

Routing (IFR).

57

- For Fibre Channel Fabrics, the identifier (AdminDomain.Name) is the Fabric WWN as reported by the

- fibre channel fabric services. The AdminDomain for the Fibre Channel Fabric shall have a NameFormat of

- WWN.

61

62	4.2.3	Fabrics and Topology

63

A Fabric in CIM minimally contains a ConnectivityCollection and its component systems. They are 64 associated to the Fabric by the association Component. For the purposes of this discussion, it is 65 assumed one models both.

66

ConnectivityCollection represents the foundation necessary for routing (and is the reason it is defined in the

67

Network model). A ConnectivityCollection groups a set of ProtocolEndpoints that are able to communicate

68 with each other directly. The ProtocolEndpoint is associated to the ConnectivityCollection by 69 MemberOfCollection. A link is represented by the association ActiveConnection, which associates two

- ProtocolEndpoints, defined as a connection that is currently carrying traffic or is configured to carry traffic.

- It is important at this point to clarify the relationship (or use) of the ProtocolEndpoint versus the use of

- FCPort (discussed later). A NetworkPort (from which FCPort is subclassed) is the device that is used to represent the logical aspects of the link and data layers. The ProtocolEndpoint is used to represent the

- higher network layers for routing. This is best understood when thinking about Ethernet and IP, but applies to fibre channel also. When two ProtocolEndpoints are capable of communicating, the association ActiveConnection is used to represent the capability to communicate and completes the picture of the topology.

- One can ultimately represent multiple ConnectivityCollections (e.g., FC, IP (over FC), and IP (FC encapsulated in IP) for the same fibre channel fabric. 75

76	The minimum set of requirements for this profile is based on FC-GS-6.

77

4.2.4	Systems and NetworkPorts

As discussed in section 4.2.3, a Port is associated to a device to represent the link layer. A NetworkPort is associated to the ProtocolEndpoint by DeviceSAPImplementation and “joins” the System and Device model to the Network model. Instantiation of DeviceSAPImplementation, ProtocolEndpoint, and ActiveConnection is not necessary if the transceiver is not installed or the cable connecting the port to another port is not installed since the device is not capable of communicating.

Systems, or in this case ComputerSystem, represent the fabric elements that contain Ports. These are typically Hosts, Switches and Storage Systems. In Fibre Channel, these are called Platforms and Interconnect Elements. The property Dedicated in ComputerSystem allows these fabric elements to be identified. For a host, Dedicated is set to “Not Dedicated”, for a switch, Dedicated is set to “Switch”, and for a storage system, Dedicated is set to “Storage”. The Ports on a System are associated by SystemDevice.

Discovery from the viewpoint of the fabric includes the end device, but often times the information available is minimal or not available. In the case of Fibre Channel, this occurs if the platform database is not populated. If this is the case, then discovery cannot tell whether a Fibre Channel Node is contained within the same platform or not. When this occurs, ComputerSystem is not instantiated and the LogicalPortGroup representing the Node and the FCPort are associated to the AdminDomain representing the Fabric.

The instrumentation needs to respond to physical fabric changes by adding or removing Logical elements to the AdminDomain. Adding an element to the fabric is straightforward, however it is not always clear when an element has been removed. The device may have been reset, or temporarily shut down, in which case it would be an element in the fabric with an “unknown” status. The lifetime of objects that can no longer be discovered is implementation specific.

78 If the instrumentation is unable to determine the type of platform discovered (defined in FC-GS-6), then 79 the agent shall set the ComputerSystem.Dedicated property to “Unknown”.

Additional identification information about Switch ComputerSystem (e.g., DomainID) is placed in OtherIdentifyingInfo property.

EXPERIMENTAL

4.2.5	N Port Virtualization (NPIV)

Within a fabric, a host or an N Port Virtualizer can "virtualize" one or more fibre channel ports for each "physical" fibre channel port, known as NPIV. To model NPIV, Figure 7: "Fabric Instance" is extended as shown in Figure 8: "NPIV Instance" with multiple ActiveConnections to the same switch port. The association, HostedDependency, is used to associate the FCPorts representing the "physical" fibre channel port or hosting FCPort with the "virtual" fibre channel ports with the "virtual" port or dependent hosted FCPort.

ProtocolEndpoint

ProtocolType="Fibre

Channel"

DeviceSAP

Implementation

ComputerSystem

Dedicated=Not Dedicated

FCPort

PortType=N

System

Device

ProtocolEndpoint

ProtocolType="Fibre

Channel"

DeviceSAP

Implementation

ComputerSystem

Dedicated=Switch

FCPort

PortType=F

System

Device

Active

Connection

ProtocolEndpoint

ProtocolType="Fibre

Channel"

DeviceSAP

Implementation

FCPort

(

Virtual

)

PortType=N

PortDisciminator=NPIV

System

Device

Hosted

Dependency

Figure 8 - NPIV Instance

EXPERIMENTAL

Basic Model for Zoning

80	4.2.6	Zoning

81

4.2.6.1	Basic Model for Zoning

Figure 9 illustrates the basic model for zoning

Zone

Active=FALSE

ZoneSet

Active=FALSE

ZoneService

ZoneCapabilities

Hosted

Collection

Element

Capabilities

MemberOf

Collection

ZoneMembership

SettingData

NamedAddress

Collection

(

See Enhanced Zoning and

Enhanced Zone Control)

Hosted

Collection

Hosted

Service

MemberOf

Collection

AdminDomain

ZoneSet

Active=TRUE

Zone

Active=TRUE

ZoneMembership

SettingData

MemberOf

Collection

Element

SettingData

Element

SettingData

Element

SettingData

Figure 9 - Zoning Instance (AdminDomain)

82

83

84

EXPERIMENTAL

85

The zoning model is based on FC-GS-7.

86

87	EXPERIMENTAL

88

This model represents the management model for defining Zone Sets, Zones, and Zone Members and

89

“activation” of a Zone Set for a fabric. In the following discussion it may be helpful to also define the 90 following:

91

92 • Active ZoneSet: the Zone Set currently enforced by the Fabric.

93

• Zone Set Database: The database of the Zone Sets not enforced by the Fabric. Referred to in this document 94 as the Inactive Zone Sets.

95

96

Basic Model for Zoning

97 • Zoning Definitions: a generic term used to indicate both the above concepts.

98

- The zoning model refers to a Zone Set as ZoneSet, a Zone as Zone, ZoneAlias as a NamedAddressCollection,

- and Zone Member as ZoneMembershipSettingData. ZoneSets shall only contain Zones associated by

- MemberOfCollection. Zones shall only contain ZoneMembershipSettingData associated by ElementSettingData

- (except for peer zones, see 4.2.6.2 Peer Zoning for more information)  or NamedAddressCollections associated by MemberOfCollection. For more information with regards to NamedAddressCollection, see 5 Enhanced Zoning and Enhanced Zone Control Profile.

The class ZoneMembershipSettingData has two properties that indicate how the device was identified to be “zoned”. They are ConnectivityMemberType (e.g., PermanentAddress for WWN, NetworkAddress for FCID, etc.) and ConnectivityMemberID which contains the actual device identifier.

The Active Zone Set, defined by an instance of ZoneSet with the Active property set to TRUE, shall only be hosted on the AdminDomain representing the Fabric. The Inactive Zone Sets, defined by an instance of ZoneSet with the Active property set to FALSE, shall be hosted on either the AdminDomain representing the Fabric as shown in

Figure 9: "Zoning Instance (AdminDomain)" or the ComputerSystem representing the switch as shown in Figure

10: "Zoning Instance (ComputerSystem)". It is allowed to have no ZoneSets (active or inactive), only an active ZoneSet, only an inactive ZoneSet(s), or both an inactive ZoneSet(s) and an active ZoneSet.

Figure 10 - Zoning Instance (ComputerSystem)

- The ZoneService and ZoneCapabilities are also associated to the same System (AdminDomain or

- ComputerSystem) as the Inactive Zone Sets using the association HostedService or ElementCapabilities, 111 respectively.

- ZoneService provides the configuration methods to control create ZoneSets, Zones, Zone Aliases, and

- Zone Members, as well as activation of the Zone Set. This service and its methods are described in the 5 Enhanced Zoning and Enhanced Zone Control Profile.

114

115

EXPERIMENTAL

116

117

4.2.6.2	Peer Zoning

- For peer zones, there are zone members identified as either a principal member or peer member. The

- semantics of a peer zone is that peer members are allowed to communicate with the principal member; and Peer members are not allowed to communicate among themselves (unless allowed by other Zones in

- the Zone Set). The addition in the model to for peer zones is that ZoneMembershipSettingData is associated to the Zone with the association ZoneSettingData which has an addition property, ZoneMembershipRole, to identify whether the member is a principal or peer.

Traditionally, SAN zoning has been wholly owned and controlled by switches in the fabric and administered through a management interface (e.g. SMI). There is an additional mechanism where a target can, in band, create peer zones, commit, and enable them in the fabric. For the purposes of this document, these peer zones will be called peer target zones. Otherwise (those create through a 121 management interface), will be call peer user zones. The property ZoneType in the class Zone is used to 122 identify the mechanism it was created with.

123

Figure 11 illustrates peer zoning.

124

Zone

ElementName=Zone1

ZoneType=4 (Peer User)

ZoneMembership

SettingData

ConnectivityMemberType=2 (WWN)

ConnectivityMemberID=1

ZoneSe

ttingData

eer

P

ZoneMembership

SettingData

ConnectivityMemberType=2 (WWN)

ConnectivityMemberID=2

tingData

ZoneSet

cipal

Prin

ZoneMembership

SettingData

ConnectivityMemberType=2 (WWN)

ConnectivityMemberID=3

tingData

ZoneSet

cipal

Prin

Zone

ElementName=Zone2

ZoneType=5 (Peer Target)

ZoneMembership

SettingData

ConnectivityMemberType=2 (WWN)

ConnectivityMemberID=4

ZoneSet

tingData

P

eer

Zone

ElementName=Zone3

ZoneType=2 (Regular)

ElementS

ettingData

125

126

127

128

129

Figure 11 - Zoning Instance (Peer Zoning)

EXPERIMENTAL

EXPERIMENTAL

4.2.7	Conditional Classes and Properties

Table 4 contains some classes which are conditional on implementation of other profiles in the Fabric Book:

- CIM_ComputerSystem (Partitioned Switch) is conditional on the implementation of the Virtual Fabrics component profile • CIM_FCPort (Partitioned Switch FCPort) is conditional on the implementation of the Switch Partitioning component profile

In addition, some of the mandatory classes of the Fabric Profile include properties that are conditional on the implementation of other profiles in the Fabric book.

- Some properties in CIM_ComputerSystem (Switch) are conditional on the implementation of the Switch

Profile.

- Some properties in CIM_FCPort (Switch FCPort) are conditional on the implementation of the Switch Profile

EXPERIMENTAL

## 4.3	Health and Fault Management

The following classes report possible Health and Fault information through LifeCycle indications:

- ComputerSystem,

- FCPort

These LifeCycle indications are more fully described in Table 4, “CIM Elements for Fabric,”.

Also in Table 4, “CIM Elements for Fabric,” is a list of AlertIndications which may also be indicators for Health and Fault Management.

## 4.4	Cascading Considerations

Not defined in this document

## 130	4.5	Methods of this Profile

131

132	4.5.1	Extrinsic Methods of the Profile

133

All of the Fabric Profile extrinsic methods return one of the following status codes. Depending on the error 134 condition, a method may return additional error codes and/or throw an appropriate exception to indicate 135 the error encountered.

136

0: Success

137

138	1: Not Supported

139

140	2: Unspecified Error

141

3: Timeout

4: Failed

5:Invalid Parameter

- 6: Access_Denied

- 7: Not_Found (not applicable for SessionControl)

- 8: Already_Exists (Not applicable for ActivateZoneSet and SessionControl)

- 9: Insufficient_Resources

146

4.5.2	CreateZoneSet

- This method creates a ZoneSet and the association HostedCollection. The newly created association,

- HostedCollection, associates the Zone to the same AdminDomain (Fabric) that the ZoneService is hosted to. For the newly created ZoneSet, the Active property is always set to false.

- CreateZoneSet(

[Required, IN, Description (

- "A user-friendly name for the ZoneSet that is "

"unique within the AdminDomain." ),

151

ModelCorrespondence { "CIM_ZoneSet.ElementName" }]

- string ZoneSetName,

- [IN, OUT, Description (

"A reference to the newly created Zone." )]       CIM_ZoneSet REF ZoneSet);

4.5.3	CreateZone

This method creates a Zone and the association HostedCollection. The newly created association, HostedCollection, associates the Zone to the same AdminDomain (Fabric) that the ZoneService is hosted to. For the newly created Zone, the Active property is always set to false.

CreateZone(

[Required, IN, Description (

"A user-friendly name for the Zone that is unique "

"within the AdminDomain." ),

ModelCorrespondence { "CIM_Zone.ElementName" }]    string ZoneName,

[IN, Description ( "The type of zoning to be enforced." ),

ValueMap { "2", "3" },

Values { "Default", "Protocol" },

ModelCorrespondence { "CIM_Zone.ZoneType" }]    uint16 ZoneType,

[IN, Description (

"Specific clarification to be used when the "

"ZoneType has an enumeration that requires it. "

"Specifically, if ZoneType is \"Protocol\", then "

"SubType is the Fibre Channel FC4 type." ),        ValueMap { "2", "3", "4" },

- Values { "SCSI", "VI", "IP" },

ModelCorrespondence { "CIM_Zone.ZoneSubType" }]

- uint16 ZoneSubType,

[IN, OUT, Description (           "A reference to the newly created Zone." )]

156

CIM_Zone REF Zone)

157	4.5.4	CreateZoneAlias

This method creates a ZoneAlias and the association HostedCollection. The newly created association,

158

HostedCollection, associates the ZoneAlias to the same AdminDomain (Fabric) that the ZoneService is 159 hosted to. For the newly created ZoneAlias, the Active property is always set to false.

160

- CreateZoneAlias(

[Required, IN, Description (

- "A name (alias) for the ZoneAlias that is unique in "163	          "the context of the hosting System." ),

- ModelCorrespondence { "CIM_ZoneAlias.CollectionAlias" }]   string CollectionAlias,

[IN, OUT, Description (

"A reference to the newly created ZoneAlias." )]    CIM_NamedAddressCollection REF ZoneAlias);

4.5.5	CreateZoneMembershipSettingData

This method creates a ZoneMembershipSettingData instance and adds it to the specified Zone or ZoneAlias by creating a MemberOfCollection association.

CreateZoneMembershipSettingData(

[Required, IN, Description (

- "Identifies the type of identification placed in "

- "ConnectivityMemberID." ),

- ValueMap { "0", "1", "2", "3", "4", "5", "..", 168	          "0x8000.." },

Values { "Unknown", "Other", "PermanentAddress",

"NetworkAddress", "SwitchPortID",

"LogicalPortGroup", "DMTF_Reserved",

"Vendor_Reserved" },

ModelCorrespondence {

"CIM_ZoneMembershipSettingData.ZoneMemberType" }]    uint16 ConnectivityMemberType,

[Required, IN, Description (

"This property specifies the type of identification "

"used in the ConnectivityMemberID field. For Fibre "

"Channel: \n"

"* A ConnectivityMemberType of \'PermanentAddress\', "

"the ConnectivityMemberID is the NxPort WWN; \n"

"* A ConnectivityMemberType of \'NetworkAddress\', "

"the ConnectivityMemberID is the NXPort Address ID; \n"

"* A ConnectivityMemberType of \'SwitchPortID\', "

"the ConnectivityMemberID is \'Domain:PortNumber\'." ),

ModelCorrespondence {

"CIM_ZoneMembershipSettingData.ConnectivityMemberID" }]    string ConnectivityMemberID,

[Required, IN, Description (

"The collection that the ZoneMembershipSettingData "

"should be added to. The collection is either a "

"Zone or ZoneAlias." )]

CIM_SystemSpecificCollection REF SystemSpecificCollection,

[IN, OUT, Description (

"A reference to the newly created ZoneMembershipSettingData."

)]

CIM_ZoneMembershipSettingData REF ZoneMembershipSettingData)

4.5.6	AddZone

- This method adds to the specified ZoneSet the specified Zone. Adding a Zone to a ZoneSet, extends the zone enforcement definition of the ZoneSet to include the members of that Zone. If adding the Zone is

- successful, the Zone should be associated with the ZoneSet by MemberOfCollection.

171

- AddZone(

- [Required, IN, Description (

- "A reference to the ZoneSet that the Zone is added to."

- )]

- CIM_ZoneSet REF ZoneSet,

- [IN, Description (

- "A reference to the Zone that is to be added to the ZoneSet."

- )]

- CIM_Zone REF Zone)

181

4.5.7	AddZoneMembershipSettingData

182

- This method adds to the specified Zone or ZoneAlias, the specified ZoneMembershipSettingData. If

- adding the ZoneMembershipSettingData is successful, an ElementSettingData association will be created

between the ZoneMembershipSettingData and either the Zone or ZoneAlias.

185

- AddZoneMembershipSettingData(

- [Required, IN, Description (

- "A reference to the Zone or ZoneAlias that the "

- "ZoneMembershipSettingData is to be added to." )]

- CIM_SystemSpecificCollection REF SystemSpecificCollection, 191	      [IN, Description (

- "A reference to the ZoneMembershipSettingData that "

- "is to be added to the Zone or ZoneAlias." )]

- CIM_ZoneMembershipSettingData REF ZoneMembershipSettingData)

195

- 4.5.8	AddZoneAlias

- This method adds to the specified Zone the specified NamedAddressCollection (ZoneAlias). If

- adding the NamedAddressCollection (ZoneAlias) is successful, an ElementSettingData association will 199 be created between the NamedAddressCollection and either the Zone or ZoneAlias. 200

AddZoneAlias(

201

[Required, IN, Description (

202

"A reference to the Zone that the ZoneAlias is to be added to."

203

)]

204

CIM_Zone REF Zone,

205

[IN, Description (

206

"A reference to the ZoneAlias that is to be added to the Zone."

207

)]

208	   CIM_NamedAddressCollection REF ZoneAlias)

209

4.5.9	ActivateZoneSet

210

- This method activates the specified ZoneSet. After the ZoneSet is activated, the ZoneSet, associated

- Zone, NamedAddressCollection (ZoneAliases), and ZoneMembershipSettingData instances will have the active flag set to true.

213

- ActivateZoneSet(

- [Required, IN, Description (

"A reference to the ZoneSet to be activated." )]

- CIM_ZoneSet REF ZoneSet,

- [Required, IN, Description (

- "Activate indicates whether the references to "

- "ZoneSet should be activated (Active=true) or "

- "deactivated (Active=false)." )]

- boolean Activate)

222

- 4.5.10	SessionControl

- This method requests that a session start (value=2), end (value=3), or terminate (value=4). 225 SessionControl is used in zoning to group a set of zoning tasks (CreateZoneSet, CreateZone,

226 CreateZoneAlias, CreateZoneMembershipSettingData, AddZone, AddZoneMemberSettingData, AddZoneAlias) together as a transaction (start) and when completed (end) to be applied (end) or can be dropped (terminated).

227

- uint32 SessionControl(

- [IN, Description (

"RequestedSessionState is an integer enumeration "

- "that indicates whether the ZoneService session has "

- "been requested to start (value=2), end (value=3), "

- "or terminate (value=4)." ),

ValueMap { "2", "3", "4" },

233

Values { "Start", "End", "Terminate" },

- ModelCorrespondence {

- "CIM_ZoneService.RequestedSessionState" }]   uint16 RequestedSessionState)

236

## 4.6	Use Cases

- 4.6.1	Fabric Identifier

The client needs to consider that the fabric identifier is not durable but is correlatable and may change

- over time. See Storage Management Technical Specification, Part 2 Common Architecture, 1.8.0 Rev 4 7 Correlatable and Durable Names.

- 240

241 242

243

244

245

246	4.6.2	FCPort OperationalStatus

OperationalStatus is the property to indicate status and state for the FCPort (both Platform and Switch 247 ports). The FCPort instance has one of the following Operational Statuses.

248

249	Table 2 - Port OperationalStatus

| OperationalStatus |  | Description |
| --- | --- | --- |
| OK | Port is online |  |
| Error | Port has a failure |  |
| Stopped | Port is disabled |  |
| InService | Port is in Self Test |  |
| Unknown |  |  |

250

251 252

253 254

255

256

4.6.3	Switch ComputerSystem OperationalStatus

257

OperationalStatus is the property to indicate status and state for switch ComputerSystems. The switch 258	ComputerSystem instance has one of the following Operational Statuses and possibly one of the 259	Subsidiary statuses. 260 Table 3 - OperationalStatus for ComputerSystem

261

| Operational Status | Possible Subsidiary Operational Status | Description |
| --- | --- | --- |
| OK |  | The system has a good status |
| OK | Stressed | The system is stressed, for example the temperature is over limit or there is too much IO in progress |
| OK | Predictive Failure | The system will probably will fail sometime soon |
| Degraded |  | The system is operational but not at 100% redundancy. A component has suffered a failure or something is running slow |
| Error |  | An error has occurred causing the system to stop. This error may be recoverable with operator intervention. |
| Error | Non-recoverable error | A severe error has occurred. Operator intervention is unlikely to fix it |
| Error | Supporting entity in error | A modeled element has failed |
| InService |  | Switch is in Self Test. |
| No contact |  | The provider knows about the array but has not talked to it since last reboot |
| Lost communication |  | The provider used to be able to communicate with the array, but has now lost contact. |
| Starting |  | The system is starting up |
| Stopping |  | The system is shutting down. |
| Stopped |  | The data path is OK but shut down, the management channel is still working. |

262

263

264 265 266

267 268

269

270

271

272

273 274

275

276

277 278

279

280

281

## 282	4.7	CIM Elements

283

284	4.7.1	Overview

Table 4 describes the CIM elements for Fabric. 285

Table 4 - CIM Elements for Fabric

| Element Name | Requirement | Description |
| --- | --- | --- |
| 4.7.2 CIM_ActiveConnection | Mandatory | The association between ProtocolEndpoints representing the links between devices. |
| 4.7.3 CIM_AdminDomain (Fabric) | Mandatory | AdminDomain representing the Fabric. Associated to RegisteredProfile. |
| 4.7.4 CIM_AdminDomain (SAN) | Mandatory | AdminDomain representing the SAN. |
| 4.7.5 CIM_Component (Platform to Fabric) | Optional | Aggregates Hosts and Arrays (Platforms) in the AdminDomain that represents the Fabric. |
| 4.7.6 CIM_Component (Switch to Fabric) | Mandatory | Aggregates Switches in the AdminDomain that represents the Fabric. |
| 4.7.7 CIM_ComputerSystem (Host Platform) | Mandatory | The ComputerSystem representing the Host Platform. |
| 4.7.8 CIM_ComputerSystem (Partitioned Switch) | Conditional | Conditional requirement: Required if the Virtual Fabrics profile is implemented. The ComputerSystem representing a Partitioned Switch. |
| 4.7.9 CIM_ComputerSystem (Storage Platform) | Mandatory | The ComputerSystem representing the Storage Platform (e.g.an Array). |
| 4.7.10 CIM_ComputerSystem (Switch) | Mandatory | The ComputerSystem representing the Switch. |
| 4.7.11 CIM_ConnectivityCollection | Mandatory | Collects the ProtocolEndpoints of the fabric. |
| 4.7.12 CIM_ContainedDomain | Mandatory | Associates a Fabric to a SAN. |
| 4.7.13 CIM_DeviceSAPImplementation (Non-Switch to FCPort) | Mandatory | Associates the Non-Switch (Host or Storage) FCPort to the ProtocolEndpoint. |
| 4.7.14 CIM_DeviceSAPImplementation (Switch to FCPort) | Mandatory | Associates the Switch FCPort to the ProtocolEndpoint. |
| 4.7.15 CIM_ElementCapabilities (ZoneCapabilities to Fabric.) | Mandatory | Associates ZoneCapabilities to a Fabric. |
| 4.7.16 CIM_ElementCapabilities (ZoneCapabilities to Switch.) | Mandatory | Associates ZoneCapabilities to a switch system. |
| 4.7.17 CIM_ElementSettingData (ZoneMembershipSettingData to Zone) | Mandatory | Associates ZoneMembershipSettingData to the Zone. |
| 4.7.18 CIM_FCActiveConnection | Optional | Experimental. The association between ProtocolEndpoints representing the links between fibre channel devices (including ISLs). |
| 4.7.19 CIM_FCPort (Host FCPort) | Mandatory | Fibre Channel Port for Host Platforms. |
| 4.7.20 CIM_FCPort (Host NPIV FCPort) | Optional | Experimental. A Host NPIV Fibre Channel Port for Host Platforms. |
| 4.7.21 CIM_FCPort (Partitioned Switch FCPort) | Conditional | Conditional requirement: Support for the Switch Partitioning profile. Fibre Channel Port for Partitioned Switch. |
| 4.7.22 CIM_FCPort (Storage FCPort) | Mandatory | Fibre Channel Port for Devices (Storage Platforms). |
| 4.7.23 CIM_FCPort (Switch FCPort) | Mandatory | Fibre Channel Port for Switch. |

286

287 288

289 290 291

292 293

294 295

296 297 298

299 300

301 302 303

304 305

306

307 308 309

310

311

312

313

314

315

316

317 318

319

320

- Table 4 - CIM Elements for Fabric

| Element Name | Requirement | Description |
| --- | --- | --- |
| 4.7.24 CIM_HostedAccessPoint (AdminDomain to ProtocolEndpoint) | Optional | Associates the ProtocolEndpoint to a Fabric AdminDomain. |
| 4.7.25 CIM_HostedAccessPoint (ComputerSystem to ProtocolEndpoint) | Mandatory | Associates the ProtocolEndpoint to the hosting ComputerSystem. |
| 4.7.26 CIM_HostedCollection (Fabric to ConnectivityCollection) | Mandatory | Associates the ConnectivityCollection to the AdminDomain representing the Fabric. |
| 4.7.27 CIM_HostedCollection (System to LogicalPortGroup) | Mandatory | Associates the LogicalPortGroup to the ComputerSystem representing the platform (host or array) or the Fabric (if platforms are not implemented). |
| 4.7.28 CIM_HostedCollection (Zones or ZoneSets to Fabric) | Mandatory | Associates the ZoneSets and Zones to the AdminDomain representing the Fabric. |
| 4.7.29 CIM_HostedCollection (Zones or ZoneSets to Switch) | Mandatory | Associates the ZoneSets and Zones to the hosting System (the ComputerSystem representing the switch). |
| 4.7.30 CIM_HostedDependency | Optional | Experimental. The association representing the relationship of the FCPort that hosts the FCPorts that are virtualized. |
| 4.7.31 CIM_LogicalPortGroup | Mandatory | Fibre Channel Node. |
| 4.7.32 CIM_MemberOfCollection (ConnectivityCollection to ProtocolEndpoint) | Mandatory | Associates ConnectivityCollection to ProtocolEndpoint. |
| 4.7.33 CIM_MemberOfCollection (LogicalPortGroup to FCPort) | Mandatory | Associates a LogicalPortGroup to its platform FC Ports. |
| 4.7.34 CIM_MemberOfCollection (ZoneSet to Zone) | Mandatory | Associates a ZoneSet to its Zones. |
| 4.7.35 CIM_ProtocolEndpoint | Mandatory | The endpoint of a link (ActiveConnection). |
| 4.7.36 CIM_SystemDevice (Non-Switch FCPort to Fabric) | Optional | Associates non-Switch FCPorts to the System (Fabric). |
| 4.7.37 CIM_SystemDevice (Non-Switch FCPort to Platform) | Optional | Associates non-Switch FCPorts to the ComputerSystem (Platform). |
| 4.7.38 CIM_SystemDevice (Switch FCPort to Switch) | Mandatory | Associates Switch FCPorts to the ComputerSystem (Switch). |
| 4.7.39 CIM_Zone (Active) | Mandatory | The active Zones being enforced by the Fabric. |
| 4.7.40 CIM_Zone (Inactive) | Mandatory | The inactive Zones being enforced by the Fabric. |
| 4.7.41 CIM_ZoneCapabilities | Mandatory | The Zoning Capabilities of the associated Fabric (or Switch). |
| 4.7.42 CIM_ZoneMembershipSettingData | Mandatory | Defines the zone member. |
| 4.7.43 CIM_ZoneSet (Active) | Mandatory | The active ZoneSets being enforced by the Fabric. |
| 4.7.44 CIM_ZoneSet (Inactive) | Mandatory | The inactive ZoneSet. |
| 4.7.45 CIM_ZoneSettingData (ZoneMembershipSettingData to Zone) | Conditional | Experimental. Conditional requirement: Required if the CIM_ZoneCapabilities.PeerZoningSupported='4' (Peer User) or '5' (Peer Target).Associates ZoneMembershipSettingData to the Zone with the peer zone role. |
| SELECT * FROM CIM_InstCreation WHERE SourceInstance ISA CIM_ComputerSystem | Mandatory | Creation of a ComputerSystem instance.. |
| SELECT * FROM CIM_InstDeletion WHERE SourceInstance ISA CIM_ComputerSystem | Mandatory | Deletion of a ComputerSystem instance. |

- 323

324 325 326

327 328

329

330

331 332

333

334

335

336 337

338

339

340

341

342

343

344

345

346

347

348

349

350

351

352

353	Table 4 - CIM Elements for Fabric

| Element Name | Requirement | Description |
| --- | --- | --- |
| SELECT * FROM CIM_InstCreation WHERE SourceInstance ISA CIM_FCPort | Mandatory | Creation of a FC Port instance. |
| SELECT * FROM CIM_InstDeletion WHERE SourceInstance ISA CIM_FCPort | Mandatory | Deletion of a FC Port instance. |
| SELECT * FROM CIM_InstModification WHERE SourceInstance ISA CIM_FCPort AND SourceInstance.CIM_FCPort::OperationalStatus <> PreviousInstance.CIM_FCPort::OperationalStatus | Mandatory | CQL -Modification of OperationalStatus of a FC Port instance. |
| SELECT * FROM CIM_InstModification WHERE SourceInstance ISA CIM_ComputerSystem AND SourceInstance.CIM_ComputerSystem::OperationalStatu s <> PreviousInstance.CIM_ComputerSystem::Operationalstat us | Mandatory | CQL -Modification of OperationalStatus of a ComputerSystem instance. |
| SELECT * FROM CIM_AlertIndication WHERE OwningEntity='SNIA' and MessageID='FC1' | Mandatory | CQL -Modification of Zone Database. |
| SELECT * FROM CIM_AlertIndication WHERE OwningEntity='SNIA' and MessageID='FC2' | Mandatory | CQL -ZoneSet Activated. |
| SELECT * FROM CIM_AlertIndication WHERE OwningEntity='SNIA' and MessageID='FC5' | Mandatory | Experimental. Switch Status Changed. |
| SELECT * FROM CIM_AlertIndication WHERE OwningEntity='SNIA' and MessageID='FC6' | Mandatory | Experimental. Fabric Merge/Segmentation. |
| SELECT * FROM CIM_AlertIndication WHERE OwningEntity='SNIA' and MessageID='FC7' | Mandatory | Experimental. Switch Added/Removed. |
| SELECT * FROM CIM_AlertIndication WHERE OwningEntity='SNIA' and MessageID='FC8' | Mandatory | Experimental. Fabric Added/Removed. |

354

4.7.2	CIM_ActiveConnection

The association between ProtocolEndpoints representing the links between devices (including ISLs). For loops, multiple ActiveConnections are instantiated as one to many relationships.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 5 describes class CIM_ActiveConnection.

Table 5 - SMI Referenced Properties/Methods for CIM_ActiveConnection

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The reference to the ProtocolEndpoint for one end of the link. |
| Dependent |  | Mandatory | The reference to the ProtocolEndpoint for the other end of the link. |

4.7.3	CIM_AdminDomain (Fabric)

AdminDomain representing the fabric. This is a logical entity and can represent virtual fabrics.

Note that if the Virtual Fabrics Profile is implemented, OtherIndentifyInfo shall contain a Virtual Fabric ID with a 'SNIA:VF_ID' in the corresponding index of IdentifyingDescriptions.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Shall 	be 	associated 	to 	RegisteredProfile 	using 	ElementConformsToProfile 	association. The RegisteredProfile instance shall have RegisteredName set to 'Fabric', RegisteredOrganization set to 'SNIA', and RegisteredVersion set to '1.8.0'.

Table 6 describes class CIM_AdminDomain (Fabric).

### Table 6 - SMI Referenced Properties/Methods for CIM_AdminDomain (Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | C | Mandatory | WWN of Fabric. |
| NameFormat |  | Mandatory | Shall be 'WWN'. |
| ElementName |  | Optional | A user friendly name for the Fabric (implementation dependent). |
| OtherIdentifyingInfo |  | Mandatory | For a Fabric AdminDomain this property shall contain the value 'Fabric'. For Virtual Fabrics, one of the indices shall contain the value of the Virtual Fabric ID and in the corresponding index for OtherIdentifyingDescription the value 'SNIA:VF_ID'. |
| IdentifyingDescriptions |  | Mandatory | For a Fabric AdminDomain this property shall contain the value 'SNIA:DetailedType' in the index for the OtherIdentifyingInfo of 'Fabric'. For Virtual Fabrics, One of the indices shall contain the value 'SNIA:VF_ID' and in the corresponding index for OtherIdentifyingInfo the value of the Virtual Fabric ID. |

4.7.4	CIM_AdminDomain (SAN)

AdminDomain representing the SAN.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 7 describes class CIM_AdminDomain (SAN).

### Table 7 - SMI Referenced Properties/Methods for CIM_AdminDomain (SAN)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name |  | Mandatory | An arbitrary name (implementation dependent). |
| NameFormat |  | Mandatory | Dependent on the arbitrary name chosen. |

### Table 7 - SMI Referenced Properties/Methods for CIM_AdminDomain (SAN)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| ElementName |  | Optional | A user friendly name for the SAN (implementation dependent). |
| OtherIdentifyingInfo |  | Mandatory | For a SAN AdminDomain this property shall contain the value 'SAN'. |
| IdentifyingDescriptions |  | Mandatory | For a SAN AdminDomain this property shall contain the value 'SNIA:DetailedType' in the index for the OtherIdentifyingInfo of 'SAN'. |

4.7.5	CIM_Component (Platform to Fabric)

Aggregates Hosts and Arrays (Platforms) in the AdminDomain that represents the Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 8 describes class CIM_Component (Platform to Fabric).

### Table 8 - SMI Referenced Properties/Methods for CIM_Component (Platform to Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | Reference to a Storage Platform or Host Platform ComputerSystem. |
| GroupComponent |  | Mandatory | Reference to the AdminDomain representing the Fabric (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

4.7.6	CIM_Component (Switch to Fabric)

Aggregates Switches in the AdminDomain that represents the Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 9 describes class CIM_Component (Switch to Fabric).

### Table 9 - SMI Referenced Properties/Methods for CIM_Component (Switch to Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | Reference to Switch (a ComputerSystem with Dedicated='5'). |
| GroupComponent |  | Mandatory | Reference to the AdminDomain representing the Fabric (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

4.7.7	CIM_ComputerSystem (Host Platform)

The ComputerSystem representing the Host Platform. This class is typically instantiated if the end device has populated the Fibre Channel Platform Database or FDMI.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 10 describes class CIM_ComputerSystem (Host Platform).

### Table 10 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Host Platform)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | C | Mandatory | The Platform Name or FDMI Host Name. |
| ElementName |  | Mandatory | The Platform Label. |
| NameFormat |  | Mandatory |  |
| Dedicated |  | Mandatory | For a FC-GS Platform Type of Host, this shall be 'Not Dedicated' (0) or 'Unknown' (1). |

4.7.8	CIM_ComputerSystem (Partitioned Switch)

The ComputerSystem representing a Partitioned Switch. This is a constrained case of the Switch ComputerSystem. Instance of a Partitioned Switch shall have an entry of 'Virtual Switch' and an entry for the Virtual Fabric ID in the OtherIdentifyingInfo array property (as well as the DomainID for the Switch).

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Required if the Virtual Fabrics profile is implemented.

Table 11 describes class CIM_ComputerSystem (Partitioned Switch).

### Table 11 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioned Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | See the CreationClassName definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| Name | C | Mandatory | See the Name definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| ElementName |  | Mandatory | See the ElementName definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| NameFormat |  | Mandatory | See the NameFormat definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| OperationalStatus |  | Mandatory | See the OperationalStatus definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| OtherIdentifyingInfo |  | Mandatory | One element shall contain the DomainID stored in decimal format. Another value corresponding to 'SNIA:DetailedType" shall contain 'Virtual Switch'. Yet another element shall contain a Virtual Fabric ID corresponding to 'SNIA:VF_ID'. In addition, other 'SNIA:DetailedType' entries may also be in this property (e.g., 'Front Domain', 'Translate Domain' or 'Backbone'). |
| Dedicated |  | Mandatory | See the Dedicated definition in section 4.7.10 CIM_ComputerSystem (Switch). |

### Table 11 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioned Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| IdentifyingDescriptions |  | Mandatory | One of the indices shall contain the value 'DomainID' and in the corresponding index for OtherIdentifyingInfo the value of the Domain ID. One of the other indices shall contain the value 'SNIA:DetailedType' and in the corresponding index for OtherIdentifyingInfo a value of 'Virtual Switch' for a Partitioned Switch. Yet another element shall contain 'SNIA:VF_ID' in the element that corresponds to the virtual fabric ID in OtherIdentifyingInfo. |
| EnabledState |  | Mandatory | Experimental. See the EnabledState definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| RequestedState |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestedState definition in section 10.7.3 CIM_ComputerSystem (Switch) 10.7.3 CIM_ComputerSystem (Switch). |
| EnabledDefault |  | Optional | See the EnabledDefault definition in section 10.7.3 CIM_ComputerSystem (Switch) |
| RequestStateChange() |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestStateChange definition in section 10.7.3 CIM_ComputerSystem (Switch). |

4.7.9	CIM_ComputerSystem (Storage Platform)

The ComputerSystem representing the Storage Platform (e.g. an Array). This class is typically instantiated if the end device has populated the Fibre Channel Platform Database.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 12 describes class CIM_ComputerSystem (Storage Platform).

### Table 12 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Storage Platform)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | C | Mandatory | The Platform Name. |
| ElementName |  | Mandatory | The Platform Label. |
| NameFormat |  | Mandatory |  |
| Dedicated |  | Mandatory | For a FC-GS Platform Type for storage subsystems, 'Storage' (3) or 'Unknown' (1). |

4.7.10	CIM_ComputerSystem (Switch)

The ComputerSystem representing the Switch.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 13 describes class CIM_ComputerSystem (Switch).

Table 13 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | C | Mandatory | The Switch WWN. |
| ElementName |  | Mandatory | The Switch Symbolic Name. |
| NameFormat |  | Mandatory | Shall be 'WWN'. |
| OperationalStatus |  | Mandatory | One of the defined values (2¦3¦6¦8¦9¦10¦11¦12¦13) shall be present in the array value. |
| OtherIdentifyingInfo |  | Mandatory | One element shall contain the DomainID stored in decimal format. Another value corresponding to 'SNIA:DetailedType" shall contain 'Front Domain', 'Translate Domain', 'Virtual Switch' or 'None'. |
| Dedicated |  | Mandatory | Shall be 5 (Switch). |
| IdentifyingDescriptions |  | Mandatory | One of the indices shall contain the value 'DomainID' and in the corresponding index for OtherIdentifyingInfo the value of the Domain ID. One of the other indices shall contain the value 'SNIA:DetailedType' and in the corresponding index for OtherIdentifyingInfo a value of 'Front Domain' for an Inter-Fabric Routing (IFR) Front Domain, 'Translate Domain' for an Inter-Fabric Routing (IFR) Translate Domain, 'Virtual Switch' for a Virtual Fabric Partitioned Switch, or 'None' when the other values don't apply. |
| EnabledState |  | Mandatory | Experimental. This property shall be 5 ('Not Applicable') when an implementation does not support the Switch Profile for this switch. |
| RequestedState |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestedState definition in section 10.7.3 CIM_ComputerSystem (Switch). |
| EnabledDefault |  | Optional | See the EnabledDefault definition in section 10.7.3 CIM_ComputerSystem (Switch). |
| RequestStateChange() |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestStateChange definition in section 10.7.3 CIM_ComputerSystem (Switch). |

4.7.11	CIM_ConnectivityCollection

Collects the ProtocolEndpoints of the fabric.

Created By: Static

Modified By: Static

355

Deleted By: Static

356

Requirement: Mandatory

357

358

Table 14 describes class CIM_ConnectivityCollection.

359

360	Table 14 - SMI Referenced Properties/Methods for CIM_ConnectivityCollection

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Optional | Not required, can be the Fabric WWN. |

361

- 4.7.12	CIM_ContainedDomain

Associates one or more Fabrics to a SAN.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

- Table 15 describes class CIM_ContainedDomain. 364

Table 15 - SMI Referenced Properties/Methods for CIM_ContainedDomain

365

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to the AdminDomain representing the Fabric (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |
| GroupComponent |  | Mandatory | The reference to the AdminDomain representing the SAN (OtherIdentifyingInfo contains 'SAN' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

366

367

368

369

370

371

372

4.7.13	CIM_DeviceSAPImplementation (Non-Switch to FCPort) Associates the Non-Switch (Host or Storage) FCPort to the ProtocolEndpoint. 373

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 16 describes class CIM_DeviceSAPImplementation (Non-Switch to FCPort).

Table 16 - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (Non-Switch to FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ProtocolEndpoint. |
| Antecedent |  | Mandatory | Reference to the Non-Switch FCPort (Host or Storage). |

4.7.14	CIM_DeviceSAPImplementation (Switch to FCPort) Associates the Switch FCPort to the ProtocolEndpoint.

Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

377

378

379

- Table 17 describes class CIM_DeviceSAPImplementation (Switch to FCPort).

Table 17 - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (Switch to FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ProtocolEndpoint. |
| Antecedent |  | Mandatory | Reference to the Switch FCPort or Virtual Swtich FCPort. |

4.7.15	CIM_ElementCapabilities (ZoneCapabilities to Fabric.)

Associates the ZoneCapabilities to a System. The system is the AdminDomain representing the Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

- Requirement: Mandatory

382

383	Table 18 describes class CIM_ElementCapabilities (ZoneCapabilities to Fabric.).

384

Table 18 - SMI Referenced Properties/Methods for CIM_ElementCapabilities (ZoneCapabilities to Fabric.)

385

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Capabilities |  | Mandatory | Reference to ZoneCapabilities. |
| ManagedElement |  | Mandatory | Reference to a Fabric AdminDomain (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

386

387

4.7.16	CIM_ElementCapabilities (ZoneCapabilities to Switch.)

Associates the ZoneCapabilities to a System. The system normally is the AdminDomain representing the Fabric, but in some cases where the Zone Database is not a fabric entity, it maybe hosted on a ComputerSystem representing the Switch.

Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

391

- Table 19 describes class CIM_ElementCapabilities (ZoneCapabilities to Switch.).

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Capabilities |  | Mandatory | Reference to ZoneCapabilities. |
| ManagedElement |  | Mandatory | Reference to a Switch ComputerSystem. |

- Table 19 - SMI Referenced Properties/Methods for CIM_ElementCapabilities (ZoneCapabilities to Switch.) 394

4.7.17	CIM_ElementSettingData (ZoneMembershipSettingData to Zone) Associates ZoneMembershipSettingData to the Zone.

Created By: Extrinsic: AddZoneMemberSettingData

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

398

399	Table 20 describes class CIM_ElementSettingData (ZoneMembershipSettingData to Zone).

400

Table 20 - SMI Referenced Properties/Methods for CIM_ElementSettingData (ZoneMembershipSettingData 401 to Zone)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SettingData |  | Mandatory | Reference to a ZoneMembershipSettingData. |
| ManagedElement |  | Mandatory | Reference to the Zone. |

402

4.7.18	CIM_FCActiveConnection

Experimental. The association between ProtocolEndpoints representing the links between fibre channel devices (including ISLs). For loops and NPIV, multiple ActiveConnections are instantiated as one to many relationships.

Created By: Static

Modified By: Static

Deleted By: Static

403	Requirement: Optional

404

- Table 21 describes class CIM_FCActiveConnection.

- Table 21 - SMI Referenced Properties/Methods for CIM_FCActiveConnection

407

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Discriminator |  | Optional | An array property enumeration used to discriminate the context in which the ActiveConnection is instantiated. Values supported by this standard include '0' (Unknown), '2' (None), '3' (NPIV), '4' (Chassis), '5' (Virtual Fabric) or '6' (IFR), or ‘10’ (FCoE). |
| Antecedent |  | Mandatory | The reference to the ProtocolEndpoint for one end of the link. |
| Dependent |  | Mandatory | The reference to the ProtocolEndpoint for the other end of the link. |

408

409

410

411

4.7.19	CIM_FCPort (Host FCPort)

Fibre Channel Port for Host Platforms.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 22 describes class CIM_FCPort (Host FCPort).

Table 22 - SMI Referenced Properties/Methods for CIM_FCPort (Host FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Optional | Port Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |
| PermanentAddress | CD | Mandatory | Fibre Channel Port WWN. Expressed as 16 unseparated upper case hex digits. |
| NetworkAddresses | C | Mandatory | Fibre Channel ID (FCID). Expressed as 8 un-separated upper case hex digits. |
| OperationalStatus |  | Mandatory | One of the defined values (0¦2¦6¦10¦11) shall be present in the array value. |
| PortType |  | Mandatory | The specific port type (0¦1¦10¦11¦12¦13¦14¦15¦16¦17¦18) currently enabled (from FC-GS Port.Type). |
| LinkTechnology |  | Mandatory | Shall be 4 ('FC'). |
| SupportedFC4Types |  | Optional | An array of integers indicating the Fibre Channel FC-4 protocols supported. EXPERIMENTAL ¦When supporting FC-SB, this property shall be either '27' (FC-SB-2 Channel) for Host Ports or '28' (FC-SB-2 Control Unit) for targets.¦ EXPERIMENTAL. |
| SupportedCOS |  | Optional | An array of integers indicating the Fibre Channel Classes of Service that are supported. EXPERIMENTAL ¦When supporting FC-SB, this property shall be either '2' or '3' for FC-SB-2 Channel or FC-SB-2 Control Unit ports, respectively.¦ EXPERIMENTAL. |

412

413

414

415	4.7.20	CIM_FCPort (Host NPIV FCPort)

416

Experimental. A Host NPIV Fibre Channel Port for Host Platforms.

417

- Created By: Static

Modified By: Static

- Deleted By: Static

Requirement: Optional

Table 23 describes class CIM_FCPort (Host NPIV FCPort).

Table 23 - SMI Referenced Properties/Methods for CIM_FCPort (Host NPIV FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassNam e |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Optional | Port Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |
| PermanentAddress | CD | Mandatory | Fibre Channel Port WWN. Expressed as 16 un-separated upper case hex digits. |
| NetworkAddresses | C | Mandatory | Fibre Channel ID (FCID). Expressed as 8 un-separated upper case hex digits. |
| OperationalStatus |  | Mandatory | One of the defined values (0¦2¦6¦10¦11) shall be present in the array value. |
| PortType |  | Mandatory | The specific port type (0¦1¦10¦11¦12¦13¦14¦15¦16¦17¦18) currently enabled (from FC-GS Port.Type). |
| LinkTechnology |  | Mandatory | Shall be 4 ('FC'). |
| PortDiscriminator |  | Mandatory | Experimental. For NPIV Host FCPorts this shall contain '7' (NPIV). |
| SupportedFC4Types |  | Optional | An array of integers indicating the Fibre Channel FC-4 protocols supported. EXPERIMENTAL ¦When supporting FC-SB, this property shall be either '27' (FC-SB-2 Channel) for Host Ports or '28' (FC-SB-2 Control Unit) for targets.¦ EXPERIMENTAL. |
| SupportedCOS |  | Optional | An array of integers indicating the Fibre Channel Classes of Service that are supported. EXPERIMENTAL ¦When supporting FC-SB, this property shall be either '2' or '3' for FC-SB-2 Channel or FC-SB-2 Control Unit ports, respectively.¦ EXPERIMENTAL. |

420

421

422

423

424

425

426

4.7.21	CIM_FCPort (Partitioned Switch FCPort) Fibre Channel Port for Partitioned Switch.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Support for the Switch Partitioning profile.

427	Table 24 describes class CIM_FCPort (Partitioned Switch FCPort).

428

Table 24 - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned Switch FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassNam e |  | Mandatory | See the SystemCreationClassName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| SystemName |  | Mandatory | See the SystemName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| CreationClassName |  | Mandatory | See the CreationClassName definition in section 4.7.23 CIM_FCPort (Switch FCPort). |
| DeviceID |  | Mandatory | See the DeviceID definition in section 4.7.23 CIM_FCPort (Switch FCPort). |
| ElementName |  | Mandatory | See the ElementName definition in section 4.7.23 CIM_FCPort (Switch FCPort). |
| PermanentAddress | CD | Mandatory | See the PermanentAddress definition in section 4.7.23 CIM_FCPort (Switch FCPort). |
| Speed |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the Speed definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| OperationalStatus |  | Mandatory | See the OperationalStatus definition in section 4.7.23 CIM_FCPort (Switch FCPort). |
| PortType |  | Mandatory | See the PortType definition in section 4.7.23 CIM_FCPort (Switch FCPort). |
| LinkTechnology |  | Mandatory | See the LinkTechnology definition in section 4.7.23 CIM_FCPort (Switch FCPort). |
| EnabledState |  | Mandatory | Experimental. See the EnabledState definition in section 4.7.23 CIM_FCPort (Switch FCPort). |
| DetailedPortState |  | Conditional | Experimental. Conditional requirement: Required if the Switch profile is implemented. See Table 94 - DetailedPortState for FCPort. |
| PortAvailability |  | Optional | Experimental. See Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 Table 95 - PortAvailability for FCPort. |
| RequestedState |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestedState definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| EnabledDefault |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the EnabledDefault definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| MaxSpeed |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the MaxSpeed definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| PortNumber |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the PortNumber definition in section 10.7.12 CIM_FCPort (Switch FCPort). |

429

430

431

432

433

434

435

436

437

438

439

440

441

442

443

444

445

446

447	Table 24 - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned Switch FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PortDiscriminator |  | Mandatory | Experimental. This array property identifies the context in which this FCPort is instantiated. For a Partitioned Switch FCPort, the values may be '3' (VF),''6' (IFR (Virtual)) or '7' (NPIV). |
| RequestStateChange() |  | Conditional | Conditional requirement: Support for the Switch Profile and a non-null value in FCPortCapabilities.RequestedStatesSupported. See the RequestStateChange definition in section 10.7.12 CIM_FCPort (Switch FCPort). |

448

449

4.7.22	CIM_FCPort (Storage FCPort)

450

Fibre Channel Port for Storage Platforms.

451

- Created By: Static

- Modified By: Static

Deleted By: Static

- Requirement: Mandatory

Table 25 describes class CIM_FCPort (Storage FCPort).

Table 25 - SMI Referenced Properties/Methods for CIM_FCPort (Storage FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Optional | Port Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |
| PermanentAddress | CD | Mandatory | Fibre Channel Port WWN. Expressed as 16 un-separated upper case hex digits. |
| NetworkAddresses | C | Mandatory | Fibre Channel ID (FCID). Expressed as 8 un-separated upper case hex digits. |
| OperationalStatus |  | Mandatory | One of the defined values (0¦2¦6¦10¦11) shall be present in the array value. |
| PortType |  | Mandatory | The specific port type (0¦1¦10¦11¦12¦13¦14¦15¦16¦17¦18) currently enabled (from FC-GS Port.Type). |
| LinkTechnology |  | Mandatory | Shall be 4 ('FC'). |
| SupportedFC4Types |  | Optional | An array of integers indicating the Fibre Channel FC-4 protocols supported. EXPERIMENTAL ¦When supporting FC-SB, this property shall be either '27' (FC-SB-2 Channel) for Host Ports or '28' (FC-SB-2 Control Unit) for targets.¦ EXPERIMENTAL. |
| SupportedCOS |  | Optional | An array of integers indicating the Fibre Channel Classes of Service that are supported. EXPERIMENTAL ¦When supporting FC-SB, this property shall be either '2' or '3' for FC-SB-2 Channel or FC-SB-2 Control Unit ports, respectively.¦ EXPERIMENTAL. |

455

456

457

458

459

460

461

462

463

464

465

466

467

468	4.7.23	CIM_FCPort (Switch FCPort)

Fibre Channel Port for Switch.

469

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 26 describes class CIM_FCPort (Switch FCPort).

Table 26 - SMI Referenced Properties/Methods for CIM_FCPort (Switch FCPort)

470

| Properties | Flag s | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | Port Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |
| PermanentAddress | CD | Mandatory | Fibre Channel Port WWN. |
| Speed |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the Speed definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| OperationalStatus |  | Mandatory | One of the defined values (0¦2¦6¦10¦11) shall be present in the array value. |
| PortType |  | Mandatory | The specific port type (0¦1¦10¦11¦12¦13¦14¦15¦16¦17¦18) currently enabled (from FC-GS Port.Type). |
| LinkTechnology |  | Mandatory | This shall be 4 ('FC'). |
| EnabledState |  | Mandatory | Experimental. This property shall be 5 ('Not Applicable') when an implementation does not support the Switch Profile for the switch this port is on. |
| DetailedPortState |  | Conditional | Experimental. Conditional requirement: Required if the Switch profile is implemented. See Table 94 - DetailedPortState for FCPort. |
| PortAvailability |  | Optional | Experimental. See Table 95 - PortAvailability for FCPort. |
| RequestedState |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestedState definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| EnabledDefault |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the EnabledDefault definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| MaxSpeed |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the MaxSpeed definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| PortNumber |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the PortNumber definition in section 10.7.12 CIM_FCPort (Switch FCPort). |

471

472

473

474

475

476

477

478

479

480

481

482

483

484

485	Table 26 - SMI Referenced Properties/Methods for CIM_FCPort (Switch FCPort)

| Properties | Flag s | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PortDiscriminator |  | Optional | Experimental. This array property identifies the context in which this FCPort is instantiated. For this version of the standard, the values may be '0' (Unknown), '2' (Not applicable), '3' (VF), '4' (FCIP), '5' (IFR), '6' (IFR (Virtual)), '7' (NPIV), '8' (Internal) or '9' (Chassis). |
| RequestStateChange() |  | Conditional | Conditional requirement: Support for the Switch Profile and a non-null value in FCPortCapabilities.RequestedStatesSupported. See the RequestStateChange definition in section 10.7.12 CIM_FCPort (Switch FCPort). |

486

487

488

489

490

491

4.7.24	CIM_HostedAccessPoint (AdminDomain to ProtocolEndpoint)

Associates the ProtocolEndpoint to the Fabric AdminDomain for those systems not registered in the Platform Database or discovered through FDMI.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 27 describes class CIM_HostedAccessPoint (AdminDomain to ProtocolEndpoint).

### Table 27 - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (AdminDomain to ProtocolEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ProtocolEndpoint. |
| Antecedent |  | Mandatory | Reference to the Fabric AdminDomain (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

4.7.25	CIM_HostedAccessPoint (ComputerSystem to ProtocolEndpoint)

Associates the ProtocolEndpoint to the hosting System. The hosting System is a ComputerSystem for the Switch or Platform.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

492

493

494

495

496

497

498	Table 28 describes class CIM_HostedAccessPoint (ComputerSystem to ProtocolEndpoint).

### Table 28 - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (ComputerSystem to ProtocolEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ProtocolEndpoint. |
| Antecedent |  | Mandatory | Reference to the Switch, Storage Platform or Host Platform ComputerSystem. |

4.7.26	CIM_HostedCollection (Fabric to ConnectivityCollection)

Associates the ConnectivityCollection to the AdminDomain representing the Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 29 describes class CIM_HostedCollection (Fabric to ConnectivityCollection).

### Table 29 - SMI Referenced Properties/Methods for CIM_HostedCollection (Fabric to ConnectivityCollection)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ConnectivityCollection. |
| Antecedent |  | Mandatory | Reference to the Fabric AdminDomain (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

4.7.27	CIM_HostedCollection (System to LogicalPortGroup)

Associates the LogicalPortGroup to the ComputerSystem representing the platform (host or array) or the Fabric (if platforms are not implemented).

Created By: Static

499	Modified By: Static

Deleted By: Static

500

Requirement: Mandatory

501

502

Table 30 describes class CIM_HostedCollection (System to LogicalPortGroup).

503

- Table 30 - SMI Referenced Properties/Methods for CIM_HostedCollection (System to LogicalPortGroup)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the LogicalPortGroup. |
| Antecedent |  | Mandatory | Reference to the Platform ComputerSystem or Fabric AdminDomain. |

4.7.28	CIM_HostedCollection (Zones or ZoneSets to Fabric)

Associates the ZoneSets and Zones to the AdminDomain representing the Fabric.

- Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 31 describes class CIM_HostedCollection (Zones or ZoneSets to Fabric).

Table 31 - SMI Referenced Properties/Methods for CIM_HostedCollection (Zones or ZoneSets to Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the Zone or ZoneSet. |
| Antecedent |  | Mandatory | Reference to the Fabric AdminDomain (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

4.7.29	CIM_HostedCollection (Zones or ZoneSets to Switch)

Associates the ZoneSets and Zones to the hosting System (the ComputerSystem representing the switch).

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 32 describes class CIM_HostedCollection (Zones or ZoneSets to Switch).

Table 32 - SMI Referenced Properties/Methods for CIM_HostedCollection (Zones or ZoneSets to Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the Zone or ZoneSet. |
| Antecedent |  | Mandatory | Reference to the Switch ComputerSystem. |

4.7.30	CIM_HostedDependency

Experimental. The association representing the relationship of the FCPort that hosts the FCPorts that are virtualized. This association is required for all FCPorts that are instantiated as part of NPIV.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 33 describes class CIM_HostedDependency.

Table 33 - SMI Referenced Properties/Methods for CIM_HostedDependency

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | A reference to the hosting Host FCPort (“physical? fibre channel port). |
| Dependent |  | Mandatory | A reference to the hosted Host NPIV FCPort (virtualized fibre channel port with PortDiscriminator='7'). |

4.7.31	CIM_LogicalPortGroup

Represents the Fibre Channel Node. Associated to the host system by the HostedCollection Association. The hosting System is either a ComputerSystem representing the Platform or the AdminDomain representing the fabric in the case for those systems not registered in the Platform Database or discovered through FDMI (but available through the Name Server/Management Server).

Created By: Static

Modified By: Static

Deleted By: Static

- Requirement: Mandatory

- Table 34 describes class CIM_LogicalPortGroup.

508

509	Table 34 - SMI Referenced Properties/Methods for CIM_LogicalPortGroup

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| Name | CD | Mandatory | Fibre Channel Node WWN. |
| NameFormat |  | Mandatory | Shall be 'WWN'. |
| ElementName | N | Mandatory | Node Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |

510

511

512

4.7.32	CIM_MemberOfCollection (ConnectivityCollection to ProtocolEndpoint)

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 35 describes class CIM_MemberOfCollection (ConnectivityCollection to ProtocolEndpoint).

Table 35 - SMI Referenced Properties/Methods for CIM_MemberOfCollection (ConnectivityCollection to

### ProtocolEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Collection |  | Mandatory | Reference to the ConnectivityCollection. |
| Member |  | Mandatory | Reference to the ProtocolEndpoint. |

4.7.33	CIM_MemberOfCollection (LogicalPortGroup to FCPort)

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 36 describes class CIM_MemberOfCollection (LogicalPortGroup to FCPort).

Table 36 - SMI Referenced Properties/Methods for CIM_MemberOfCollection (LogicalPortGroup to FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Collection |  | Mandatory | Reference to the LogicalPortGroup. |
| Member |  | Mandatory | Reference to the Non-Switch (Host or Storage) FCPort. |

513

514

4.7.34	CIM_MemberOfCollection (ZoneSet to Zone)

515

Created By: Static 516

Modified By: Static 517

Deleted By: Static

518

Requirement: Mandatory

519

Table 37 describes class CIM_MemberOfCollection (ZoneSet to Zone).

Table 37 - SMI Referenced Properties/Methods for CIM_MemberOfCollection (ZoneSet to Zone)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Collection |  | Mandatory | Reference to the ZoneSet. |
| Member |  | Mandatory | Reference to the Zone. |

4.7.35	CIM_ProtocolEndpoint

The endpoint of a link (ActiveConnection). ProtocolEndpoint shall be implemented when an ActiveConnection exists. It may be implemented if no ActiveConnections exist.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 38 describes class CIM_ProtocolEndpoint.

Table 38 - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | CD | Mandatory | The Fibre Channel Port WWN. |

Table 38 - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| NameFormat |  | Mandatory | 'WWN'. |
| ProtocolIFType |  | Mandatory | Shall be 56(Fibre channel). |

4.7.36	CIM_SystemDevice (Non-Switch FCPort to Fabric) Associates non-Switch FCPorts to the System (Fabric).

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 39 describes class CIM_SystemDevice (Non-Switch FCPort to Fabric).

Table 39 - SMI Referenced Properties/Methods for CIM_SystemDevice (Non-Switch FCPort to Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to a Host or Storage FCPort. |
| GroupComponent |  | Mandatory | The reference to the Fabric (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

4.7.37	CIM_SystemDevice (Non-Switch FCPort to Platform)

Associates non-switch FCPorts to the Platform ComputerSystem.

Created By: Static

Modified By: Static

Deleted By: Static Requirement: Optional

520

521

Table 40 describes class CIM_SystemDevice (Non-Switch FCPort to Platform).

522

523	Table 40 - SMI Referenced Properties/Methods for CIM_SystemDevice (Non-Switch FCPort to Platform)

524

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to a Host or Storage FcPort. |
| GroupComponent |  | Mandatory | The reference to the Platform (Storage or Host) System. |

525

4.7.38	CIM_SystemDevice (Switch FCPort to Switch)

Associates the Switch FCPort to the ComputerSystem (Switch).

Created By: Static

Modified By: Static Deleted By: Static

526

Requirement: Mandatory 527

528	Table 41 describes class CIM_SystemDevice (Switch FCPort to Switch).

Table 41 - SMI Referenced Properties/Methods for CIM_SystemDevice (Switch FCPort to Switch) 529

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to the switch FCPort. |
| GroupComponent |  | Mandatory | The reference to the Switch ComputerSystem. |

4.7.39	CIM_Zone (Active)

The active Zones being enforced by the Fabric.

Created By: Extrinsic: ActivateZoneSet

Modified By: Static

530	Deleted By: Static

Requirement: Mandatory

531

532

Table 42 describes class CIM_Zone (Active).

- Table 42 - SMI Referenced Properties/Methods for CIM_Zone (Active)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | The Zone Name. |
| ZoneType |  | Mandatory | The Zone Type. |
| Active |  | Mandatory | Shall be TRUE. Indicates that this ZoneSet is active. |

4.7.40	CIM_Zone (Inactive)

The inactive Zones being enforced by the Fabric.

- Created By: Extrinsic: CreateZone

535

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

539

540	Table 43 describes class CIM_Zone (Inactive).

Table 43 - SMI Referenced Properties/Methods for CIM_Zone (Inactive)

541

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | The Zone Name. |
| ZoneType |  | Mandatory | The Zone Type. |
| Active |  | Mandatory | Shall be FALSE. Indicates that this ZoneSet is inactive. |

4.7.41	CIM_ZoneCapabilities

The Zoning Capabilities of the associated Fabric (or Switch).

542 ZoneCapabilities exposes the capabilities of the AdminDomain representing the Fabric for active zoning and the capabilities of the ComputerSystem representing the Switch or AdminDomain representing the 543

Fabric for Zone Set Database.

544

545	If a ZoneCapability property is not applicable or does not explicitly exists (e.g. the capability is limited

only by a memory size), the property is NULL.

546

- Created By: Static

Modified By: Static

- Deleted By: Static

Requirement: Mandatory

Table 44 describes class CIM_ZoneCapabilities.

Table 44 - SMI Referenced Properties/Methods for CIM_ZoneCapabilities

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| MaxNumZoneSets |  | Optional | The maximum number of ZoneSets in the Zone Set Database. NULL should be returned in such cases when the property is not applicable or the number is not limited explicitly. |
| MaxNumZone |  | Optional | The maximum number of Zones in the Zone Set Database. NULL should be returned in such cases when the property is not applicable or the number is not limited explicitly. |
| MaxNumZoneMembers |  | Optional | The maximum number of ZoneMembers in the Zone Set Database. All ZoneMembers included in both Zones and ZoneAliases are counted, while the same ZoneMember included in multiple Zones or ZoneAliases is counted only once. NULL should be returned in such cases when the property is not applicable or the number is not limited explicitly. |
| MaxNumZoneAliases |  | Optional | The maximum number of ZoneAliases in the Zone Set Database NULL should be returned in such cases when the property is not applicable or the number is not limited explicitly. |
| ZoneNameMaxLen |  | Mandatory | The maximum length for the name of a ZoneAlias (NamedAddressCollection.ElementName), Zone (Zone.ElementName) or ZoneSet (ZoneSet.ElementName) the Fabric (or Switch) are capable of supporting. |
| ZoneNameFormat |  | Mandatory | The name format of a ZoneAlias *NamedAddressCollection.ElementName), Zone (Zone.ElementName) or ZoneSet (ZoneSet.ElementName) supported by either the Fabric (or the Switch). |
| MaxNumZonesPerZoneSet |  | Optional | The maximum number of Zones per ZoneSet. NULL should be returned in such cases when the property is not applicable or the number is not limited explicitly. |
| SupportedConnectivityMember Types |  | Mandatory | An array containing the supported connectivity member types supported which include Permanent Address (WWN), Switch Port ID (Domain:Port in base10), Network Address (FCID), Logical Port Group (Node WWN). |
| PeerZoningSupported |  | Optional | Experimental. Specifies whether any peer zoning is supported and which types of peer zoning is supported. |

549

550

551

552

553

554

555

556

557

558

559	4.7.42	CIM_ZoneMembershipSettingData

560

Defines the zone member.

Created By: Extrinsic: AddZoneMemberSettingData

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 45 describes class CIM_ZoneMembershipSettingData.

Table 45 - SMI Referenced Properties/Methods for CIM_ZoneMembershipSettingData

561

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ConnectivityMemberType |  | Mandatory | Permanent Address (WWN), Switch Port ID (Domain:Port in base10), Network Address (FCID). |
| ConnectivityMemberID | C | Mandatory | The value of the WWN, Domain/Port, or FCID. |

562

563

564

565

566

567	4.7.43	CIM_ZoneSet (Active)

The active ZoneSet being enforced by the Fabric.

Created By: Extrinsic: ActivateZoneSet

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 46 describes class CIM_ZoneSet (Active).

Table 46 - SMI Referenced Properties/Methods for CIM_ZoneSet (Active)

568

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | The ZoneSet name. |
| Active |  | Mandatory | shall be TRUE. Indicates that this ZoneSet is active and members cannot be changed. |

569

570

571

572

4.7.44	CIM_ZoneSet (Inactive) The inactive ZoneSets.

Created By: Extrinsic: CreateZoneSet

Modified By: Static

Deleted By: Static Requirement: Mandatory

573

574

575

576

Table 47 describes class CIM_ZoneSet (Inactive).

### Table 47 - SMI Referenced Properties/Methods for CIM_ZoneSet (Inactive)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | The ZoneSet name. |
| Active |  | Mandatory | Shall be FALSE. Indicates that this ZoneSet is inactive. |

STABLE

EXPERIMENTAL

4.7.45	CIM_ZoneSettingData (ZoneMembershipSettingData to Zone) Associates ZoneMembershipSettingData to the Zone.

Created By: Extrinsic: AddZoneMemberSettingDataWithRole

Modified By: Static

Deleted By: Static

Requirement: Required if the CIM_ZoneCapabilities.PeerZoningSupported='4' (Peer User) or '5' (Peer Target).

Figure 48 describes class CIM_ZoneSettingData (ZoneMembershipSettingData to Zone)

Table 48 - SMI Referenced Properties/Methods for CIM_ZoneSettingData (ZoneMembershipSettingData to

Zone)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SettingData |  | Mandatory | Reference to a ZoneMembershipSettingData. |
| ManagedElement |  | Mandatory | Reference to Zone. |
| ZoneMembershipRole |  | Mandatory | Specifies whether peer zoning is supported and which types of peer zoning is supported.. |

EXPERIMENTAL

STABLE

# 1	5  Enhanced Zoning and Enhanced Zone Control Profile

## 2	5.1	Synopsis

3	Profile Name: Enhanced Zoning and Enhanced Zoning Control (Component Profile)

4

- Version: 1.4.0

- Organization: SNIA

- Central Class: ZoneService

- Scoping Class: an AdminDomain in a referencing autonomous profile

- Related Profiles: Table 49 describes the supported profiles for Enhanced Zoning and Enhanced Zoning Control.

10

11	Table 49 - Supported Profiles for Enhanced Zoning and Enhanced Zoning Control

| Profile Name | Organization | Version | Requirement | Description |
| --- | --- | --- | --- | --- |
| Zone Control | SNIA | 1.8.0 | Mandatory |  |

## 5.2	Description

This profile describes the additional zoning functions for enhanced zoning. Note that Sessions are normally part of enhanced zoning, but are included in the base fabric profile to address the various types of zoning operations into a single object model. In this profile, then only Zone Alias is added.

## 12	5.3	Health and Fault Management

13	Not defined in this document

## 14	5.4	Cascading Considerations

15	Not defined in this document

16

## 17	5.5	Methods of this Profile

18

5.5.1	CreateZoneAlias

19

The method creates a ZoneAlias and the association HostedCollection. The newly created association, 20 HostedCollection, associates the ZoneAlias to the same AdminDomain the ZoneService is hosted to. For 21 the newly created ZoneAlias, the Active property is always set to false. 22

CreateZoneAlias(

23

[IN] string CollectionAlias,

24	[OUT] CIM_NamedAddressCollection ref ZoneAlias);

25

5.5.2	AddZoneAlias

26

Adds to the Zone the specified ZoneAlias.

27

AddZoneAlias(

- [IN] CIM_Zone ref Zone,

- [IN] CIM_NamedAddressCollection ref ZoneAlias);

30

## 31	5.6	Use Cases

32	Not defined in this document.

33

## 34	5.7	CIM Elements

35

5.7.1	Overview

36

37	Table 50 describes the CIM elements for Enhanced Zoning and Enhanced Zoning Control.

38

Table 50 - CIM Elements for Enhanced Zoning and Enhanced Zoning Control

| Element Name | Requirement | Description |
| --- | --- | --- |
| 5.7.2 CIM_ElementSettingData (ZoneMembershipSettingData to NamedAddressCollection) | Mandatory | Associates ZoneMembershipSettingData to the NamedAddressCollection representing the ZoneAlias. |
| 5.7.3 CIM_HostedCollection (AdminDomain to Collection) | Mandatory | Associates the NameAddressCollection representing the Zone Alias to the AdminDomain. |
| 5.7.4 CIM_HostedCollection (ComputerSystem to Collection) | Mandatory | Associates the NameAddressCollection representing the Zone Alias to the System. |
| 5.7.5 CIM_MemberOfCollection | Mandatory | Associates NamedAddressCollection with Zone. |
| 5.7.6 CIM_NamedAddressCollection | Mandatory | The Zone Alias. |
| 5.7.7 CIM_ZoneService (Zone Service) | Mandatory | The service that allows for all of the zoning configuration changes. |

39 40 41

42 43 44

45 46 47

48 49

50

51

5.7.2	CIM_ElementSettingData (ZoneMembershipSettingData to NamedAddressCollection)

52

53	Associates ZoneMembershipSettingData to the NamedAddressCollection representing the ZoneAlias. 54

Created By: Static Modified By: Static

55

Deleted By: Static

56

Requirement: Mandatory

57

58

Table 51 describes class CIM_ElementSettingData (ZoneMembershipSettingData to NamedAddressCollection). 59

- Table 51 - SMI Referenced Properties/Methods for CIM_ElementSettingData (ZoneMembershipSettingData

- to NamedAddressCollection)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SettingData |  | Mandatory |  |
| ManagedElement |  | Mandatory |  |

62

63

64 65

66

5.7.3	CIM_HostedCollection (AdminDomain to Collection)

67 Associates the NamedAddressCollection representing the Zone Alias to the AdminDomain representing 68 the Fabric. 69

Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

- Table 52 describes class CIM_HostedCollection (AdminDomain to Collection).

74

75	Table 52 - SMI Referenced Properties/Methods for CIM_HostedCollection (AdminDomain to Collection)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | Reference to the Fabric AdminDomain (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |
| Dependent |  | Mandatory | Reference to the NamedAddressCollection representing the Zone Alias. |

76

77 78

79

80

5.7.4	CIM_HostedCollection (ComputerSystem to Collection)

81

Associates the NamedAddressCollection representing the Zone Alias to the ComputerSystem 82	representing the switch.

83	Created By: Static

Modified By: Static Deleted By: Static

84

Requirement: Mandatory

85

86

Table 53 describes class CIM_HostedCollection (ComputerSystem to Collection).

87

- Table 53 - SMI Referenced Properties/Methods for CIM_HostedCollection (ComputerSystem to Collection)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | A reference to a Switch ComputerSystem. |
| Dependent |  | Mandatory | Reference to the NamedAddressCollection representing the Zone Alias. |

- 90

91

- 5.7.5	CIM_MemberOfCollection

- Associates NamedAddressCollection with Zone.

94

Created By: Static 95

Modified By: Static 96

Deleted By: Static

97

Requirement: Mandatory

98

99

100	Table 54 describes class CIM_MemberOfCollection. 101

Table 54 - SMI Referenced Properties/Methods for CIM_MemberOfCollection

102

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Collection |  | Mandatory |  |
| Member |  | Mandatory |  |

103

104

105

106

107

108	5.7.6	CIM_NamedAddressCollection 109	The Zone Alias.

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

114

115

Table 55 describes class CIM_NamedAddressCollection.

116

Table 55 - SMI Referenced Properties/Methods for CIM_NamedAddressCollection

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| CollectionAlias |  | Mandatory | The Zone Alias Name. |

117 118 119

120

121

5.7.7	CIM_ZoneService (Zone Service)

- The service that allows for all of the zoning configuration changes. The definition in this profile adds

- additional methods to the same service defined in Zone Control.

124

Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

128

129

Table 56 describes class CIM_ZoneService (Zone Service).

130

131	Table 56 - SMI Referenced Properties/Methods for CIM_ZoneService (Zone Service)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | The Class Name. |
| Name |  | Mandatory | Opaque. |
| CreateZoneAlias() |  | Mandatory |  |
| AddZoneAlias() |  | Mandatory |  |

132

133 134 135

136 137

138

139

140

141

142

143

144	STABLE

145

146

147

148 149

150

STABLE

# 1	6  Zone Control Profile

## 2	6.1	Synopsis

3	Profile Name: Zone Control (Component Profile)

4

5	Version: 1.8.0

6

Organization: SNIA

7 Central Class: ZoneService

8

9	Scoping Class: AdminDomain

10

Related Profiles: Not defined in this document.

11

## 12	6.2	Description

13 This profile includes extrinsic methods for creating Zone Sets, Zones, and Zone Members and adding 14 Zones to Zone Sets and Zone Members to Zones. Additionally SMI-S defines intrinsic methods for the 15 removing of Zone Members from Zones and Zone Aliases, Zones from Zone Sets, and deleting Zone 16 Members, Zones, and Zone Sets. The profile also includes methods to allow a client to request or release 17 a lock on the fabric for zoning configuration changes.

18

When an Inactive ZoneSet is “Activated”, new instances representing the Active Zone Set and Active 19 Zones are generated from the Inactive Zone Set definition (where a switch may prune the referenced 20 Zone Set collapsing aliases, removes empty zones, etc.).

21

When a new Zone Set is “Activated”, the instances representing the previous active Zone Set no longer 22	exists.

23 This profile also includes the property ConfigurationID on the ZoneService. The ConfigurationID is used to identify a unique point in time identifier of the zoning configuration. It may be a timestamp, a 24 generation identifier, or a checksum. The ConfigurationID shall change whenever the zoning

25	configuration changes. The value of the property is used as an input to ActivateZoneSet() and

SessionControl() to allow the client to confirm that the change requested are consistent with the last 26 discovery the client made of the zoning configuration.

- In the case where the Inactive Zone Sets are hosted on a switch, the client cannot know which Inactive

Zone Set was used to define the current Active Zone Set. Also if two Inactive Zone Sets with the same

- name are hosted on two different switches, the definitions maybe completely different.

29

## 6.3	Durable Names and Correlatable IDs of the Profile

30

Not defined in this document

31

## 6.4	Instrumentation Requirements

The agent shall support the use case defined in 6.8.

## 6.5	Health and Fault Management

32	Not defined in this document

## 33	6.6	Cascading Considerations

34	Not defined in this document

35

## 36	6.7	Methods of this Profile

37

38	6.7.1	CreateZoneSet

The method creates a ZoneSet and associates it to the System (AdminDomain representing the Fabric or the ComputerSystem representing the Switch) that the ZoneService is hosted on.

39

CreateZoneSet (

40

[IN] string ZoneSetName,

41

[OUT] CIM_ZoneSet ref ZoneSet);

42

- 6.7.2	CreateZone

- The method creates a Zone and associates it to System (AdminDomain representing the Fabric or the 45 ComputerSystem representing the Switch) that the ZoneService is hosted on. 46

CreateZone (

[IN] string ZoneName,

47	[IN] uint16 ZoneType,

[IN] uint16 ZoneSubType,

48

[OUT] CIM_Zone ref Zone);

49

50

- 6.7.3	CreateZoneMembershipSettingData

- The method creates a ZoneMembershipSettingData (a zone member) and adds it to the specified Zone or 53	NamedAddressCollection representing a Zone Alias. The ConnectivityMemberID is dependent upon the 54	ConnectivityMemberType.

55

For Fibre Channel, the ConnectivityMemberType of “PermanentAddress”, the ConnectivityMemberID is 56 the NxPort WWN; for ConnectivityMemberType of “NetworkAddress”, the ConnectivityMemberID is the 57 NXPort Address ID; for ConnectivityMemberType of “SwitchPortID”, the ConnectivityMemberID is 58 “Domain:PortNumber”.

59

CreateZoneMembershipSettingData (

[IN] uint16 ConnectivityMemberType,

60	[IN] string ConnectivityMemberID,

[IN] CIM_SystemSpecificCollection ref SystemSpecificCollection,

61

[OUT] CIM_ZoneMembershipSettingData ref ZoneMembershipSettingData);

62

63

- 6.7.4	AddZone

- The method adds to the specified ZoneSet the specified Zone. Adding a Zone to a ZoneSet, extends the

- zone enforcement definition of the ZoneSet to include the members of that Zone. If adding the Zone is, successful, the Zone should be associated to the ZoneSet by MemberOfCollection.

- AddZone (

[IN] CIM_ZoneSet ref ZoneSet,

68

[IN] CIM_Zone ref Zone);

69

70

- 6.7.5	AddZoneMembershipSettingData

- The method adds to the specified Zone or NamedAddessCollection representing the Zone Alias the specified ZoneMembershipSettingData (a zone member).

AddZoneMembershipSettingData (

- [IN] CIM_SystemSpecificCollection ref SystemSpecificCollection,

[IN] CIM_ZoneMembershipSettingData ref ZoneMembershipSettingData); 74

75	EXPERIMENTAL

76

77	6.7.6	ActivateZoneSetWithJob

78

This method activates the specified ZoneSet with the expectation that a job will be created. It has an

- optional input, ConfigurationID, which is used to confirm that the ZoneSet being activated matches the

- definition the application previously discovered or changed. If the ConfigurationID passed in matches the

81

ConfigurationID associated to the current zone definitions, then the fabric attempts to activate the

82 ZoneSet. If the ConfigurationIDs do not match, the method exits without activating the ZoneSet. Once a 83 ZoneSet is activated, a ZoneSet with the property Active set to true, its associated Zones with the 84 property Active set to true, and the Zone’s associated ZoneMembershipSettingData are instantiated.

85

This method may run asynchronously due to the length of the operation. This typically occurs when the 86 session is ended and the changes are committed. When the method is run asynchronously, the value

87 “Method parameters checked - job started” is returned and the output parameter Job is populated with a reference to the job instance. 88

ActivateZoneSet shall be supported outside of a session. ActivateZoneSet being called within a session 89 is implementation specific.

90

Calling ActivateZoneSet outside of a session while a session is open is implementation specific.

91

- uint32 ActivateZoneSetWithJob (

- [IN] CIM_ZoneSet ref ZoneSet,

- [IN] boolean Activate,

[IN,OUT] string ConfigurationID,

[OUT] CIM_ConcreteJob REF Job,

[IN] datetime TimeoutPeriod) 95

EXPERIMENTAL

96

97

6.7.7	ActivateZoneSet

98

99 This method activates the specified ZoneSet without a Job. ActivateZoneSetWithJob replaces this method. It is supported for backwards compatibility and should not be used in future implementations.

100

101	Once a ZoneSet is activated, a ZoneSet with the property Active set to true, its associated Zones with the property Active set to true, and the Zone’s associated ZoneMembershipSettingData are instantiated.

102

ActivateZoneSet shall be supported outside of a session. ActivateZoneSet being called within a session 103 is implementation specific.

104

Calling ActivateZoneSet outside of a session while a session is open is implementation specific.

105 uint32 ActivateZoneSet (

[IN] CIM_ZoneSet ref ZoneSet,

[IN] boolean Activate )

106

107

108

109

110

111

112	EXPERIMENTAL

113

6.7.8	SessionControlWithJob

114

115 This method allows a client to request or release a lock on the fabric for zoning configuration changes. 116 The method has an optional input, ConfigurationID, which is used to confirm that the ZoneSet being

117 activated matches the definition the application previously discovered or changed. If the ConfigurationID passed in matches the ConfigurationID associated to the current zone definitions, then the fabric 118 attempts to grant the lock. If the ConfigurationIDs do not match, the method exits without attempting to 119 have the fabric grant the lock.

120

121 As described in FC-GS, in the context of Enhanced Zoning Management, management actions to a Zone 122 Server (e.g., write access to the Zoning Database) shall occur only inside a GS session. Clients 123 executing zoning management operations shall use fabric sessions cooperatively if the SMI-S agent 124 supports it. (If the value of SessionState is 4 (“Not Applicable”) then no cooperative session usage is 125 possible). 126

Before a client executes zoning management operations (intrinsic or extrinsic methods), the client shall 127 request a new session and wait for the request to be granted. To request a new session, first wait until the

128 property “SessionState” of the fabric’s ZoneService is 3 (“Ended”) and the property 129 “RequestedSessionState” is 5 “No Change”. Then call SessionControl with RequestedSessionState = 2 130 (“Started”). Once zoning management operations are completed, the client shall release the session to 131 enable the provider to propagate changes to the fabric, and to allow other clients to perform management 132 operations. To end a session and commit the changes, call SessionControl with RequestedSessionState

133 = 3 (“Ended”). To abort a sequence of zoning management operations without updating the fabric, call SessionControl with RequestedSessionState = 4 (“Terminated”).

134

135 SMI-S agents shall block on calls to SessionControl until the request is fulfilled. For example, an error may occur while committing changes to a fabric, i.e., after a call to SessionControl with 136 RequestedSessionState = 3 (“Ended”). The method cannot return until the session has ended, so that a 137 CIM error can be returned if a problem occurs. While the method is in progress, another client may read 138 the value of the RequestedSessionState property and see the value set by the method currently in

139 progress. Once the request is fulfilled, the RequestedSessionState property is set to value 5 “No Change”, regardless of the value in the setInstance operation.

140

141 Sessions can timeout. The session timeout behavior and settings are defined by FC-SW in the section 142 discussing mapping GS sessions for Enhanced Zoning Management.

143

This method may run asynchronously due to the length of the operation. This shall occur when the 144 session is “Ended” to commit the changes. It shall not be used for any other case. When the method is 145 run asynchronously, the value “Method parameters checked - job started” is returned and the output 146 parameter Job is populated with a reference to the job instance.

147

A SMIS agent may raise an error if these client cooperation rules are not followed. For the purposes of a 148 SMIS agent, a series of requests from the same authenticated entity are considered to be from a single 149 client. An agent may verify that such a series corresponds to the sequence described above and raise the 150 error CIM_ERR_FAILED at any time if the sequence is violated. 151 uint32 SessionControlWithJob (

[IN,

ValueMap {"2", "3", "4"},

152	 Values {"Started", "Ended", "Terminated"}]

uint16 RequestedSessionState,

153

[IN,OUT] string ConfigurationID, 154

- [OUT] CIM_ConcreteJob REF Job,

- [IN] datetime TimeoutPeriod)

- EXPERIMENTAL

158

159

160	6.7.9	SessionControl

161

The method enables a client to request a lock of the fabric to begin zoning configuration changes. It is 162 supported for legacy implementations. SessionControlWithJob replaces this method. It is supported for 163 backwards compatibility only and should not be used in future implementations.

164

165 This method allows a client to request or release a lock on the fabric for zoning configuration changes. As 166 described in FC-GS, in the context of Enhanced Zoning Management, management actions to a Zone 167 Server (e.g., write access to the Zoning Database) shall occur only inside a GS session. Clients 168 executing zoning management operations shall use fabric sessions cooperatively if the SMI-S agent 169 supports it. (If the value of SessionState is 4 (“Not Applicable”) then no cooperative session usage is 170 possible).

171 Before a client executes zoning management operations (intrinsic or extrinsic methods), the client shall 172 request a new session and wait for the request to be granted. To request a new session, first wait until the

173 property “SessionState” of the fabric’s ZoneService is 3 (“Ended”) and the property 174 “RequestedSessionState” is 5 “No Change”. Then call SessionControl with RequestedSessionState = 2 175 (“Started”). Once zoning management operations are completed, the client shall release the session to 176 enable the provider to propagate changes to the fabric, and to allow other clients to perform management

177	operations. To end a session and commit the changes, call SessionControl with RequestedSessionState

= 3 (“Ended”). To abort a sequence of zoning management operations without updating the fabric, call 178 SessionControl with RequestedSessionState = 4 (“Terminated”).

179

SMIS agents shall block on calls to SessionControl until the request is fulfilled. For example, an error may 180 occur while committing changes to a fabric, i.e., after a call to SessionControl with 181 RequestedSessionState = 3 (“Ended”). The method cannot return until the session has ended, so that a 182 CIM error can be returned if a problem occurs. While the method is in progress, another client may read

183 the value of the RequestedSessionState property and see the value set by the method currently in progress. Once the request is fulfilled, the RequestedSessionState property is set to value 5 “No

184

Change”, regardless of the value in the setInstance operation.

185

186	Sessions can timeout. The session timeout behavior and settings are defined by INCITS FC-SW in the

section discussing mapping GS sessions for Enhanced Zoning Management.

187

188 A SMIS agent may raise an error if these client cooperation rules are not followed. For the purposes of a SMIS agent, a series of requests from the same authenticated entity are considered to be from a single client. An agent may verify that such a series corresponds to the sequence described above and raise the error CIM_ERR_FAILED at any time if the sequence is violated.

189

uint32 SessionControl (

190	[IN,

191

ValueMap {"2", "3", "4"},

192

Values {"Started", "Ended", "Terminated"}]

193 uint16 RequestedSessionState;}; 194

6.7.10	Intrinsics for removing a zone from a zone set

195

- As shown in Figure 9: "Zoning Instance (AdminDomain)" in the Fabric Profile, a zone is a member of a

- zone set if there is a “CIM_MemberOfCollection” association from the zone set to the zone. To remove a zone from a zone set, delete the instance of the association “CIM_MemberOfCollection” using the intrinsic operation deleteInstance.

- 6.7.11	Intrinsics for removing a zone alias from a zone

- A zone alias is a member of a zone if there is a “CIM_MemberOfCollection” association from the zone to 200 the zone alias. To remove a zone alias from a zone set, delete the instance of the association 201 “CIM_MemberOfCollection” using the intrinsic operation deleteInstance.

202

6.7.12	Intrinsics for removing a zone member from a zone or zone alias

203

204	Zone members are represented by CIM_ZoneMembershipSettingData instances. No instance of

CIM_ZoneMembershipSettingData exists unless it is associated to a zone or zone alias by a 205 CIM_ElementSettingData association. However, an instance of CIM_ZoneMembershipSettingData may 206 be associated to more than one zone or zone alias.

207

Removing a zone member from a zone or zone alias is equivalent to deleting the instance of the 208 CIM_ElementSettingData association. Delete the instance using the intrinsic operation deleteInstance.

209	If 	this 	is 	the 	last 	instance 	of 	a 	CIM_ElementSettingData 	association 	for 	a 	particular 210	CIM_ZoneMembershipSettingData, do not delete the instance of CIM_ZoneMembershipSettingData; it is 211	the provider's responsibility to clean up these structures.

212

6.7.13	Intrinsic for deleting a zone member

213

- Zone members are represented by CIM_ZoneMembershipSettingData instances associated to zones or zone aliases via CIM_ElementSettingData associations. To delete a zone member (and remove it from

- any zones or zone aliases from which it is a member) use the CIM operation deleteInstance to delete the instance of CIM_ZoneMembershipSettingData.

216

217 Do not delete the corresponding instances of the CIM_ElementSettingData; it is the provider's 218 responsibility to clean up these structures.

219	6.7.14	Intrinsic for deleting a zone, zone alias, or zone set

220

221 Use the intrinsic operation deleteInstance to delete a zone, zone alias or zone set. Client are allowed to 222 delete zones or zone aliases that are members of collections (zones or zone sets). Clients are allowed to 223 delete the last member of a zone or zone set, leaving the collection empty.

224	A zone set or zone cannot be deleted if it is currently active (the error would be CIM_ERR_FAILED).

Some implementations may prohibit deleting zonesets, zones or zone aliases that still have members (the 225 error would be CIM_ERR_FAILED). When a zone, zone alias or zone set is deleted, the client does not 226 have to delete the corresponding instances of CIM_MemberOfCollection or CIM_HostedCollection; it is 227 the provider's responsibility to clean up these structures.

228

229

230	EXPERIMENTAL

231

232	6.7.15	AddZoneMembershipSettingDataWithRole

233

- This method is used when defining peer zones adding to the specified Zone or ZoneAlias, the specified

- ZoneMembershipSettingData 	along 	with 	its 	role 	in 	the 	peer 	zone. 	If 	adding 	the

ZoneMembershipSettingData is successful, an ElementSettingData association will be created between 236 the ZoneMembershipSettingData and either the Zone or ZoneAlias.

- AddZoneMembershipSettingDataWithRole(

- [IN] CIM_SystemSpecificCollection REF SystemSpecificCollection,

- [IN] CIM_ZoneMembershipSettingData REF ZoneMembershipSettingData,240	         [IN] uint16 ZoneMembershipRole);

241 242

243	6.7.16	CreateZoneMembershipSettingDataWithRole

244

This method creates a ZoneMembershipSettingData instance for a peer zone with its role and adds it to 245 the specified Zone or ZoneAlias by creating a MemberOfCollection association. 246

CreateZoneMembershipSettingDataWithRole(

247

[IN] uint16 ConnectivityMemberType,

248

[IN] uint16 ConnectivityMemberRole,

249

[IN] string ConnectivityMemberID,

250

[IN] CIM_SystemSpecificCollection REF SystemSpecificCollection,

251

[IN] CIM_ZoneMembershipSettingData REF ZoneMembershipSettingData); 252

EXPERIMENTAL

253

## 254	6.8	Use Cases

255

Many agent implementations do not allow Zone, a ZoneAlias or a Zone Set to be defined empty. Since the

256 methods defined in SMI-S do not support creating a Zone Set with a Zone and a Zone with a Zone

Member, the SessionControl method should be used to build a Zone Definition that is interoperable. This 257 is done by calling ZoneSession() to “Start” defining or updating the Zone Definition. The client then calls 258 the appropriate methods as necessary to build the desired Zone Definition. For example, calling

- CreateZoneSet() to create a new Zone Set, CreateZone() to create a new Zone, AddZoneToZoneSet() to add the newly created Zone to the newly created Zone Set, and CreateZoneMembershipSettingData() to create and add a new Zone Member to the newly created Zone. Upon completion of the new zoning

- definition, ZoneControl is called again to “End” the session. The changes to the Zone Definition would 261 then be applied to the Zone Set Database. This set of calls would create a Zone Definition where the 262 Zone and ZoneSet are not empty and would be interoperable across all agent implementations. No recipes are defined in this version of the standard.

263

## 264	6.9	CIM Elements

265

6.9.1	Overview

266	Table 57 describes the CIM elements for Zone Control. 267

Table 57 - CIM Elements for Zone Control

| Element Name | Requirement | Description |
| --- | --- | --- |
| 6.9.2 CIM_HostedService (Fabric (AdminDomain) to ZoneService) | Optional | Associates the ZoneService to the AdminDomain representing the fabric. |
| 6.9.3 CIM_HostedService (Switch (ComputerSystem) to ZoneService) | Optional | Associates the ZoneService to the ComputerSystem representing the switch. |
| 6.9.4 CIM_ZoneService (Zone Service) | Mandatory | The service that allows for all of the zoning configuration changes. |

268

269

270

271

272

273

- 6.9.2	CIM_HostedService (Fabric (AdminDomain) to ZoneService)

- Associates the ZoneService to the AdminDomain representing the fabric.

276

Created By: Static Modified By: Static

277 Deleted By: Static

278

- Requirement: Optional

- Table 58 describes class CIM_HostedService (Fabric (AdminDomain) to ZoneService).

281

Table 58 - SMI Referenced Properties/Methods for CIM_HostedService (Fabric (AdminDomain) to ZoneService)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The reference to the AdminDomain representing the fabric (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |
| Dependent |  | Mandatory | The reference to the ZoneService. |

282 283

284

285

286

6.9.3	CIM_HostedService (Switch (ComputerSystem) to ZoneService)

287

Associates the ZoneService to the ComputerSystem representing the switch. 288

- Created By: Static

Modified By: Static

- Deleted By: Static

291

Requirement: Optional

292

Table 59 describes class CIM_HostedService (Switch (ComputerSystem) to ZoneService).

293

Table 59 - SMI Referenced Properties/Methods for CIM_HostedService (Switch (ComputerSystem) to Zone-

294	Service)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The reference to the ComputerSystem representing the switch. |
| Dependent |  | Mandatory | The reference to the ZoneService. |

295

296

297

298

299

6.9.4	CIM_ZoneService (Zone Service)

300

The service that allows for all of the zoning configuration changes.

301

Created By: Static

302

Modified By: Static

303

Deleted By: Static

304

Requirement: Mandatory

305 Table 60 describes class CIM_ZoneService (Zone Service).

306

Table 60 - SMI Referenced Properties/Methods for CIM_ZoneService (Zone Service)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | The Class Name. |
| Name |  | Mandatory | Opaque. |
| OperationalStatus |  | Mandatory | Status of Zoning Service. |
| SessionState |  | Mandatory | State of session. Valid values are "Starting", "Ended". |

307 308

309 310

311

312

313

314 315

Table 60 - SMI Referenced Properties/Methods for CIM_ZoneService (Zone Service)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| RequestedSessionState |  | Mandatory | The requested session state from the client. The valid values that can be set are "Start", "End", and "Terminate". |
| DefaultZoningState |  | Mandatory |  |
| CreateZoneSet() |  | Mandatory | The method creates a ZoneSet and associates it to the System (AdminDomain representing the Fabric or the ComputerSystem representing the Switch) that the ZoneService is hosted on. |
| CreateZone() |  | Mandatory | The method creates a Zone and associates it to System (AdminDomain representing the Fabric or the ComputerSystem representing the Switch) that the ZoneService is hosted on. |
| CreateZoneMembershipSettingData() |  | Mandatory | The method creates a ZoneMembershipSettingData (a zone member) and adds it to the specified Zone or NamedAddressCollection representing a Fibre Channel Node. |
| AddZone() |  | Mandatory | The method adds to the specified ZoneSet the specified Zone. |
| AddZoneMembershipSettingData() |  | Mandatory | The method adds to the specified Zone or NamedAddessCollection representing the Fibre Channel Node the specified ZoneMembershipSettingData (a zone member). |
| ActivateZoneSet() |  | Mandatory | The method activates the specified ZoneSet. |
| SessionControl() |  | Mandatory | The method enables a client to request a lock of the fabric to begin zoning configuration changes. |
| AddZoneMembershipSettingDataWith Role() |  | Optional | Experimental. This method is used when defining peer zones adding to the specified Zone or ZoneAlias, the specified ZoneMembershipSettingData along with its role in the peer zone. |
| CreateZoneMembershipSettingDataWi thRole() | Optional | Optional | Experimental. This method creates a ZoneMembershipSettingData instance for a peer zone with its role and adds it to the specified Zone or ZoneAlias by creating a ZoneSettingData association. |

IMPLEMENTED

# 1	7  FDMI Profile

## 7.1	Synopsis

2	Profile Name: FDMI (Component Profile)

3

4	Version: 1.8.0

5

6	Organization: SNIA

7

Central Class: ComputerSystem

8

9	Scoping Class: an AdminDomain in a referencing autonomous profile

10

11	Related Profiles: Not defined in this document.

12

## 13	7.2	Description

14 The Fabric-Device Management Interface (FDMI) enables the management of devices such as HBAs 15 through the Fabric. The FDMI complements data in the Fabric Profile. It allows for any entity in the Fabric 16 to expose through SMI the HBA information without having an agent resident on the Host containing the 17 HBA. Figure 12 shows an FDMI instance.

18

This profile only addresses HBA type devices. The HBA Management Interface defined by FDMI is a subset of interface defined by the Fibre Channel HBA API specification, as exposed by Storage 19 Management Technical Specification, Part 7 Host Elements, 1.8.0 Rev 4 5 FC HBA Profile.

20

21

22

23

24

25

Product (HBA Product)

ComputerSystem (Host Server)

Dedicated=

PortController

Installed

Software

Identity

)

(

Driver

SoftwareIdentity

FCPort (Host FCPort)

Element

Statistical

Data

FCPortStatistics

(

see referencing profile

)

LogicalPortGroup

MemberOf

Collection

ControlledBy

FCode/BIOS

(

)

SoftwareIdentityt

(

)

Firmware

SoftwareIdentity

(

e.g. Card

)

PhysicalPackage (HBA Package)

ProductPhysicalComponent

System

Device

System

Device

Hosted

Collection

Realizes

Element

Software

Identity

Element

Software

Identity

Element

Software

Identity

AdminDomain (Fabric)

)

see referencing profile

(

Component

26

Figure 12 - FDMI Instance

## 7.3	Health and Fault Management

Not defined in this document

27

## 28	7.4	Cascading Considerations

Not defined in this document

29

## 30	7.5	Methods of this Profile

Not defined in this document

31

## 32	7.6	Use Cases

Not defined in this document.

33

34

## 35	7.7	CIM Elements

36

7.7.1	Overview

Table 61 describes the CIM elements for FDMI.

37	Table 61 - CIM Elements for FDMI

| Element Name | Requirement | Description |
| --- | --- | --- |
| 7.7.2 CIM_Component (Host Server to Fabric) | Mandatory | Aggregates Hosts (Platforms) in the AdminDomain that represents the Fabric. |
| 7.7.3 CIM_ComputerSystem (Host Server) | Mandatory | The System the HBA is within. |
| 7.7.4 CIM_ControlledBy | Mandatory | Associates the FCPorts of the Host Server with the PortController. |
| 7.7.5 CIM_ElementSoftwareIdentity | Mandatory | Associates the SoftwareIdentity to the HBA. |
| 7.7.6 CIM_FCPort (Host FCPort) | Mandatory | The HBA Fibre Channel Port. |
| 7.7.7 CIM_HostedCollection (System to LogicalPortGroup) | Mandatory | Associates the LogicalPortGroup (Fibre Channel Node) to the hosting System. |
| 7.7.8 CIM_InstalledSoftwareIdentity | Mandatory | Associates the SoftwareIdentity representing the driver to the System it is installed on. |
| 7.7.9 CIM_MemberOfCollection (LogicalPortGroup to FCPort) | Mandatory | Associates FCPort to the LogicalPortGroup. |
| 7.7.10 CIM_PhysicalPackage (HBA Package) | Mandatory | The physical package that the HBA is contained in. |
| 7.7.11 CIM_PortController | Mandatory | The HBA. |
| 7.7.12 CIM_Product (HBA Product) | Mandatory | The product information for the HBA. |
| 7.7.13 CIM_ProductPhysicalComponent | Mandatory | Associates the Product to the PhysicalPackage. |
| 7.7.14 CIM_Realizes | Mandatory | Associates the PhysicalPackage to the PortController. |
| 7.7.15 CIM_SoftwareIdentity (Driver) | Mandatory | The software for the driver. |
| 7.7.16 CIM_SoftwareIdentity (Firmware) | Mandatory | The software for the firmware. |
| 7.7.17 CIM_SoftwareIdentity (Option ROM) | Mandatory | The software for the Option ROM. |
| 7.7.18 CIM_SystemDevice (ComputerSystem to FCPort) | Mandatory | Associates the FCPort to the Host Server ComputerSystem. |
| 7.7.19 CIM_SystemDevice (ComputerSystem to PortController) | Mandatory | Associates the Host Server ComputerSystem with the PortController. |

38

39

40

7.7.2	CIM_Component (Host Server to Fabric)

Aggregates Hosts (Platforms) in the AdminDomain that represents the Fabric.

Created By: Static

Modified By: Static Deleted By: Static

41

Requirement: Mandatory

42

43

44

45

46	Table 62 describes class CIM_Component (Host Server to Fabric).

Table 62 - SMI Referenced Properties/Methods for CIM_Component (Host Server to Fabric)

47

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | Reference to a Host Server ComputerSystem. |
| GroupComponent |  | Mandatory | Reference to the AdminDomain representing the Fabric. |

7.7.3	CIM_ComputerSystem (Host Server)

The system the HBA is within. It is identified using Host Name from the FDMI interface.

48

Created By: Static

49

Modified By: Static

50

Deleted By: Static

51

Requirement: Mandatory

52

53

Table 63 describes class CIM_ComputerSystem (Host Server).

54	Table 63 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Host Server)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name |  | Mandatory | The FDMI name of the host containing the Device. The key identifier helping in discovery to determine which HBAs are in the same host. |
| ElementName |  | Mandatory | The Platform Label. |
| NameFormat |  | Mandatory |  |
| Dedicated |  | Mandatory | For a FC-GS Platform Type of Host, this shall be 'Not Dedicated' (0). |

7.7.4	CIM_ControlledBy

Associates the FCPorts of the Host Server with the PortController.

55

56	Created By: Static

Modified By: Static

57

Deleted By: Static

58

Requirement: Mandatory

59

60

Table 64 describes class CIM_ControlledBy.

- Table 64 - SMI Referenced Properties/Methods for CIM_ControlledBy

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | A reference to an instance of a Host Server FCPort. |
| Antecedent |  | Mandatory | The reference to the PortController. |

- 7.7.5	CIM_ElementSoftwareIdentity

- Associates the SoftwareIdentities representing the various software for the HBA to the PortController 64 representing the HBA.

- Created By: Static

- Modified By: Static

- Deleted By: Static

Requirement: Mandatory

68

Table 65 describes class CIM_ElementSoftwareIdentity.

Table 65 - SMI Referenced Properties/Methods for CIM_ElementSoftwareIdentity

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | A reference to an instance of software (Firmware, Driver or Option ROM). |
| Dependent |  | Mandatory | A reference to the HBA PortController. |

69

70	7.7.6	CIM_FCPort (Host FCPort) 71	The HBA Fibre Channel Port.

72

Created By: Static

73

Modified By: Static

74

Deleted By: Static Requirement: Mandatory

75

Table 66 describes class CIM_FCPort (Host FCPort).

Table 66 - SMI Referenced Properties/Methods for CIM_FCPort (Host FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | Port Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |
| LinkTechnology |  | Mandatory | 'FC'. |
| PermanentAddress |  | Mandatory | Fibre Channel Port WWN. |
| NetworkAddresses | C | Mandatory | Fibre Channel ID (FCID). Expressed as 8 un-separated upper case hex digits. |
| ActiveFC4Types |  | Mandatory | The active Fibre Channel FC-4 protocol. |

Table 66 - SMI Referenced Properties/Methods for CIM_FCPort (Host FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SupportedFC4Types |  | Optional | An array of integers indicating the Fibre Channel FC-4 protocols supported EXPERIMENTAL When supporting FC-SB, this property shall be either '27' (FC-SB-2 Channel) for Host Ports or '28' (FC-SB-2 Control Unit) for targets. |
| SupportedCOS |  | Optional | An array of integers indicating the Fibre Channel Classes of Service that are supported. |
| PortType |  | Mandatory | The specific port type currently enabled (from FC-GS Port.Type). |
| Speed |  | Optional | Experimental. Speed of zero represents a link not established. - GFC is 1062500000 bps - GFC is 2125000000 bps 4 GFC is 4250000000 bps 8 GFC = 8500000000 bps 10 GFC single channel variants are 10518750000 bps 10 GFC four channel variants are 12750000000 bps 16 GFC = 14025000000 bps 32 GFC = 28500000000 bps 128 GFC = 114000000000 bps This is the raw bit rate. |

76

77

78

79

80

81

82

7.7.7	CIM_HostedCollection (System to LogicalPortGroup)

Associates the LogicalPortGroup (Fibre Channel Node) to the hosting System.

Created By: Static

- Modified By: Static

- Deleted By: Static

Requirement: Mandatory

85

86

Table 67 describes class CIM_HostedCollection (System to LogicalPortGroup).

87

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The Host Server ComputerSystem. |
| Dependent |  | Mandatory | The LogicalPortGroup. |

88 Table 67 - SMI Referenced Properties/Methods for CIM_HostedCollection (System to LogicalPortGroup) 89

7.7.8	CIM_InstalledSoftwareIdentity

Associates the SoftwareIdentity representing the driver to the System it is installed on.

Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

93

- Table 68 describes class CIM_InstalledSoftwareIdentity.

- Table 68 - SMI Referenced Properties/Methods for CIM_InstalledSoftwareIdentity

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstalledSoftware |  | Mandatory | A reference to Driver Software. |
| System |  | Mandatory | A reference to the Host Server ComputerSystem. |

96

7.7.9	CIM_MemberOfCollection (LogicalPortGroup to FCPort) Associates FCPort to the LogicalPortGroup.

Created By: Static

97

Modified By: Static

98	Deleted By: Static

99

Requirement: Mandatory

100

101

Table 69 describes class CIM_MemberOfCollection (LogicalPortGroup to FCPort).

102

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Collection |  | Mandatory | A reference to a LogicalPortGroup on the Host Server. |
| Member |  | Mandatory | A reference to an FCPort on the Host Server. |

103 Table 69 - SMI Referenced Properties/Methods for CIM_MemberOfCollection (LogicalPortGroup to FCPort) 104

7.7.10	CIM_PhysicalPackage (HBA Package)

The physical package that the HBA is contained by. It can be simply a PhysicalPackage that the system and HBA is contained within. If it is known that the HBA is on a separate board, Card (a subclass of PhysicalPackage) can be used.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 70 describes class CIM_PhysicalPackage (HBA Package).

Table 70 - SMI Referenced Properties/Methods for CIM_PhysicalPackage (HBA Package)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Tag |  | Mandatory | An arbitrary string that uniquely identifies the PhysicalPackage. |
| ElementName |  | Optional | User Friendly name. This property is OPTIONAL. |

105

106

107

108

109	Table 70 - SMI Referenced Properties/Methods for CIM_PhysicalPackage (HBA Package)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Name |  | Optional |  |
| Manufacturer |  | Mandatory |  |
| Model |  | Mandatory |  |
| SerialNumber |  | Optional |  |
| Version |  | Optional |  |
| PartNumber |  | Optional |  |

110

111

7.7.11	CIM_PortController

The HBA. The HBA may have logical operations that can apply to it (e.g. OperationalStatus).

Created By: Static Modified By: Static 112

Deleted By: Static

113

Requirement: Mandatory

114

115	Table 71 describes class CIM_PortController.

116

117	Table 71 - SMI Referenced Properties/Methods for CIM_PortController

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory |  |
| SystemName |  | Mandatory |  |
| CreationClassName |  | Mandatory |  |
| DeviceID |  | Mandatory |  |
| ControllerType |  | Mandatory |  |

118

7.7.12	CIM_Product (HBA Product) The product information for the HBA.

Created By: Static

- Modified By: Static

- Deleted By: Static

Requirement: Mandatory

121

122

Table 72 describes class CIM_Product (HBA Product).

123

- Table 72 - SMI Referenced Properties/Methods for CIM_Product (HBA Product)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Name |  | Mandatory | Commonly used Product name. |
| IdentifyingNumber |  | Mandatory | Product identification such as a serial number. |
| Vendor |  | Mandatory | The manufacturer or the OEM. |

- Table 72 - SMI Referenced Properties/Methods for CIM_Product (HBA Product)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Version |  | Mandatory | Product version information. |
| ElementName |  | Mandatory | User Friendly name. Suggested use is Vendor, Version and product name. |

7.7.13	CIM_ProductPhysicalComponent

Associates the Product to the PhysicalPackage. This is necessary to link the Product information to the 126

HBA.

127

Created By: Static 128

Modified By: Static 129

Deleted By: Static

130

Requirement: Mandatory

131

Table 73 describes class CIM_ProductPhysicalComponent.

132

Table 73 - SMI Referenced Properties/Methods for CIM_ProductPhysicalComponent

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| GroupComponent |  | Mandatory | The product information (CIM_Product) for the HBA Package. |
| PartComponent |  | Mandatory | The Physical Package for the HBA. |

7.7.14	CIM_Realizes

133 Associates the PhysicalPackage to the PortController.

134

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

- Table 74 describes class CIM_Realizes.

Table 74 - SMI Referenced Properties/Methods for CIM_Realizes

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The Physical Package for the HBA. |
| Dependent |  | Mandatory | The PortController of the HBA. |

7.7.15	CIM_SoftwareIdentity (Driver) The software for the driver.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

- Table 75 describes class CIM_SoftwareIdentity (Driver). 141

Table 75 - SMI Referenced Properties/Methods for CIM_SoftwareIdentity (Driver)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory |  |
| VersionString |  | Mandatory |  |
| Manufacturer |  | Mandatory |  |
| BuildNumber |  | Optional |  |
| MajorVersion |  | Optional |  |
| RevisionNumber |  | Optional |  |
| MinorVersion |  | Optional |  |
| Classifications |  | Mandatory |  |

142

143

144

145

146

7.7.16	CIM_SoftwareIdentity (Firmware) The software for the firmware.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

147

Table 76 describes class CIM_SoftwareIdentity (Firmware).

148

Table 76 - SMI Referenced Properties/Methods for CIM_SoftwareIdentity (Firmware)

149

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory |  |
| VersionString |  | Mandatory |  |
| Manufacturer |  | Mandatory |  |
| BuildNumber |  | Optional |  |
| MajorVersion |  | Optional |  |
| RevisionNumber |  | Optional |  |
| MinorVersion |  | Optional |  |
| Classifications |  | Mandatory |  |

150

151

152

153

7.7.17	CIM_SoftwareIdentity (Option ROM) The software for the Option ROM.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

154	Table 77 describes class CIM_SoftwareIdentity (Option ROM).

155

Table 77 - SMI Referenced Properties/Methods for CIM_SoftwareIdentity (Option ROM)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory |  |
| VersionString |  | Mandatory |  |
| Manufacturer |  | Mandatory |  |
| BuildNumber |  | Optional |  |
| MajorVersion |  | Optional |  |
| RevisionNumber |  | Optional |  |
| MinorVersion |  | Optional |  |
| Classifications |  | Mandatory |  |

156

157

158

159

160

- 7.7.18	CIM_SystemDevice (ComputerSystem to FCPort)

- Associates the FCPort to the Host Server ComputerSystem.

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

- Table 78 describes class CIM_SystemDevice (ComputerSystem to FCPort).

Table 78 - SMI Referenced Properties/Methods for CIM_SystemDevice (ComputerSystem to FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | A reference to a Host FCPort. |
| GroupComponent |  | Mandatory | A reference to the Host Server ComputerSystem. |

7.7.19	CIM_SystemDevice (ComputerSystem to PortController)

Associates the Host Server ComputerSystem with the PortController.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 79 describes class CIM_SystemDevice (ComputerSystem to PortController).

Table 79 - SMI Referenced Properties/Methods for CIM_SystemDevice (ComputerSystem to PortController)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | A reference to the HBA PortController. |
| GroupComponent |  | Mandatory | A reference to the Host Server ComputerSystem. |

IMPLEMENTED

EXPERIMENTAL

# 1	8  Fabric Views Profile

## 8.1	Description

2

- 8.1.1	Synopsis

- Profile Name: Fabric Views (Component Profile)

5

6	Version: 1.7.0

7

8	Organization: SNIA

9

Central Class: CIM_ViewCapabilities

10

Scoping Class: a ComputerSystem in a referencing autonomous profile

11

Related Profiles: Not defined in this document.

12

8.1.2	Overview

- This Profile specifies View Classes for the Fabric Profiles.

- View classes provide an optimization of retrieval of information provided by multiple (associated) instances in a Profile. There is no support for update of view classes instances. Update of a view class

- instance can only be accomplished by updating the base class instances from which the view is derived.

16

17

8.1.3	Goals of View Classes

18

8.1.3.1	Intended Goals

19

Goals that view classes are intended to address are:

20

• Get more data in one call to CIM Server.

21

The CIM model for fabrics involve a number of classes and associations. The objective is to allow discovery of 22	the fabric model using view classes with a reduction in the number of association traversals required.

23 • Allow providers to optimize the Request.

24

In many cases, the data represented by a view class is actually kept (and returned) by a device as one entity.

- When the "normalized" CIM model is traversed many calls are made to retrieve that one entity. The provider takes the data from the one entity and carves it up for each CIM request. In many cases this involves retrieving

- the same entity multiple times. The objective is to allow a Provider to return the single entity in one SMI-S

- request (for data that is typically kept together by the device).

28

29

8.1.3.2	Additional Goals

30

• Do more things in one call to CIM Server.

31

An example would be retrieval or discovery of model information with fewer calls. However, this goal also

32 extends to updating the CIM model (e.g., configuration actions). The view classes are not intended to help in the latter case. However, view classes should facilitate access to underlying classes in support of configuration 33 operations.

34

- It is important to note that the view classes proposal was based directly on experiences relating to the

- scalability and performance of SMI-S real-world implementations. The focus is on improving performance in large configurations (e.g., thousands of volumes and thousands of disk drives).

Pre-defined View Classes

- 8.1.4	Specific Requirements and Objectives of View Classes

38

39	8.1.4.1	Pre-defined View Classes

In order to gain the desired performance advantage, it is felt that view classes would have to be pre40 defined (in SMI-S) to allow provider optimization of the requested information.

41 • Enable Associator Calls to View Class instances.

42

It should be possible to retrieve a view class by an associators call to the class.

43 However, it is desired that the association should be clearly distinguished from existing associations on the 44 base classes.

45

• Enable Associator Calls from View Class instances.

46

It should be possible to get related classes (e.g., base classes) from the view class by using associator calls.

- Again, the associations used should be clearly distinguished from existing associations on the base classes.

- • Allow View Classes to be used where other classes are used.

- This certainly includes "read" intrinsics and as parameters of Extrinsics

- However, at this time "Write" intrinsic support is deferred and use in Extrinsics (as IN or OUT parameters) is not 51 covered in this release of SMI-S.

52

8.1.4.2	Support Life Cycle Indications on View Classes

53	This requirement is being deferred for consideration in a future release of SMI-S.

54

8.1.5	Class Diagram for Fabric Views View Classes

55

Figure 13 illustrates the class diagram for the Fabric view classes.

56

CIM_ConcreteComponentView

CIM_ManagedElement

CIM_ViewCapabilities

CIM_FCSwitchView

CIM_Capabilities

CIM_AbstractComponent

CIM_FCTopologyView

CIM_View

57

58

### Figure 13 - Class Diagram for Fabric View Classes

The ViewCapabilities inherits from CIM_Capabilities. The FCSwitchView class inherits from CIM_View.

The TopologyView class is a top level association class. The Fabric Views profile has one subclass of 59 CIM_AbstractComponent: CIM_ConcreteComponentView. This associates the view to its scoping system.

60

61

- 8.1.6	View Class Capabilities

- The implementation shall identify which view classes are implemented using a set of conditions. The 64 model for determining whether or not the Fabric Views Profile is supported and which views are 65 supported is illustrated in Figure 14: "Fabric View Class Capabilities".

|  |  |  |  |  |  | Adm inDom ain: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RegisteredProfile |  |  |  |  |  |  |  |  |  |  |
| RegisteredNam e = Fabricس |  |  |  |  |  | O therIdentifyingInfo= Fabric IdentifyingDescriptions= SNIA:DetailedType |  |  |  |  |
|  |  |  |  | Elem entConform sToProfile |  |  |  |  |  |  |
| Referenc |  | edProfile |  |  |  | Elem entC | apabiliities |  |  |  |
|  | RegisteredProfile |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ViewCapabilities |  |  |  |  |  |
|  | RegisteredNam e = Fabric Views |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | SupportedViews[] = SNIA:TopologyView س￼ SNIA:FCSwitchView |  |  |  |  |  |

### Figure 14 - Fabric View Class Capabilities

66 First a client may determine whether or not a profile implementation has implemented any view classes 67 by looking for a RegistereProfile with a RegisteredName of “Fabric Views”. If this RegisteredProfile exists 68 then the profile supports some number of view classes.

69 Next a client would be able to determine which view classes are supported by an implementation by 70 following the ElementConformsToProfile to the Fabric and then following the ElementCapabilities from the 71 Fabric to the CIM_ViewCapabilities instance. There shall be one instance of the CIM_ViewCapabilities 72 class for each Fabric if the profile supports the Fabric Views Profile. The CIM_ViewCapabilities instance 73 shall have an array of strings (SupportedViews) that identify the view classes that are supported. For 74 example, if the SupportedViews array includes the “SNIA:TopologyView” string, then the TopologyView 75 class shall be supported.

- 8.1.7	Topology View

- Discovering a topology for a large fibre channel fabric (thousands of ports) time-wise is expensive. The 78 existing fabric profile requires traversals of three associations and two classes for each link. Using the

- view class reduces this to one enumeration of the view class itself for all links. The CIM_FCTopologyView, shown in Figure 15: "FCTopology View Class", provides the properties to be able

- to build a topology without having to do associators from the ProtocolEndpoint to the FCPort to the 81 FCNode or FCSwitch.

AdminDomain

)

Fabric Profile

(

ComputerSystem

)

Fabric Profile

(

FCPort

)

Fabric Profile

(

Component

SystemDevice

FCTopologyView

)

(

Antecendent

)

Antecendent

(

(

)

Dependent

(

)

Dependent

ComponentView

### Figure 15 - FCTopology View Class

- 8.1.8	FCSwitch View

- Discovering a fibre channel switch and its associated ports is expensive. The CIM_FCSwitchView, shown 84 in Figure 16: "FCSwitch View Class", provides the properties to be able to discover all the switch 85 properties for a Fibre Channel Switch and Ports without having to do associators from the 86 ComputerSystem representing the Switch to FCPort, LogicalModeule, Product, PhysicalPackage, 87 SoftwareIdentity, FCSwitchCapabilities, FCPortCapabilities, AccessPoint, and Location.

AdminDomain

)

Fabric Profile

(

ComputerSystem

)

Fabric Profile

(

FCPort

)

Fabric Profile

(

Component

SystemDevice

FCSwitchView

ElementView

ElementView

ComponentView

### Figure 16 - FCSwitch View Class

## 88	8.2	Health and Fault Management Consideration

89	Not defined in this document

## 90	8.3	Cascading Considerations

91	Not defined in this document

## 92	8.4	Methods of the Profile

93	Not defined in this document

## 8.5	Use Cases

Not defined in this document.

## 94	8.6	CIM Elements

- 8.6.1	Overview

- Table 80 describes the CIM elements for Fabric Views. 97

Table 80 - CIM Elements for Fabric Views

| Element Name | Requirement | Description |
| --- | --- | --- |
| 8.6.2 CIM_ElementCapabilities (View Capabilities) | Mandatory | Associates the Fabric AdminDomain to the CIM_ViewCapabilities supported by the implementation. |
| 8.6.3 CIM_ElementView (View to FC Port) | Conditional | Conditional requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:FCSwitchView'. Associates the CIM_FCSwitchView instances to corresponding the FC Port instance. |
| 8.6.4 CIM_ElementView (View to Switch) | Conditional | Conditional requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:FCSwitchView'. Associates the CIM_FCSwitchView instances to corresponding the Fibre Channel Switch instance. |
| 8.6.5 CIM_ViewCapabilities | Mandatory | The CIM_ViewCapabilities identifies the capabilities of the implementation of view classes. |
| 8.6.6 CIM_ConcreteComponentView (FCSwitchView to Fabric) | Conditional | Conditional requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:FCSwitchView'. Associates the Fabric AdminDomain to the CIM_FCSwitchView. |
| 8.6.7 CIM_ConcreteComponentView (FCTopologyView to Fabric) | Conditional | Conditional requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:TopologyView'. Associates the Fabric AdminDomain to the CIM_FCTopologyView. |
| 8.6.8 CIM_FCSwitchView | Conditional | Conditional requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:FCSwitchView'. The CIM_FCSwitchView provides the properties for a Fibre Channel Switch. |
| 8.6.9 CIM_FCTopologyView | Conditional | Conditional requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:TopologyView'. The CIM_FCTopologyView provides the properties to be able to build a topology for a Fibre Channel Fabric. |

98

99

100

101

102

8.6.2	CIM_ElementCapabilities (View Capabilities)

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 81 describes class CIM_ElementCapabilities (View Capabilities).

Table 81 - SMI Referenced Properties/Methods for CIM_ElementCapabilities (View Capabilities)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Capabilities |  | Mandatory | The ViewCapabilities. |
| ManagedElement |  | Mandatory | The Fabric AdminDomain that has the ViewCapabilities. |

- 8.6.3	CIM_ElementView (View to FC Port)

Associates the CIM_FCSwitchView instances to corresponding the FC Port instance.

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:FCSwitchView'.

108

Table 82 describes class CIM_ElementView (View to FC Port).

Table 82 - SMI Referenced Properties/Methods for CIM_ElementView (View to FC Port)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The base CIM_FCPort instance on which the CIM_FCSwitchView instance is based. |
| Dependent |  | Mandatory | The CIM_FCSwitchView instance that is based on the CIM_FCPort instance. |

109

110

111	8.6.4	CIM_ElementView (View to Switch)

112

Associates the CIM_FCSwitchView instances to corresponding the Fibre Channel Switch instance.

113

- Created By: Static

- Modified By: Static

Deleted By: Static

- Requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:FCSwitchView'.

Table 83 describes class CIM_ElementView (View to Switch).

Table 83 - SMI Referenced Properties/Methods for CIM_ElementView (View to Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The base CIM_ComputerSystem (Switch) instance on which the CIM_FCSwitchView instance is based. |
| Dependent |  | Mandatory | The CIM_FCSwitchView instance that is based on the CIM_ComputerSystem (Switch) instance. |

117

118

- 8.6.5	CIM_ViewCapabilities

- The CIM_ViewCapabilities instance defines the capabilities of an implementation support for view 121 classes. The CIM_ViewCapabilities is subclassed from CIM_Capabilities.

122

Created By: Static 123

Modified By: Static Deleted By: Static

124

Requirement: Mandatory

Table 84 describes class CIM_ViewCapabilities.

Table 84 - SMI Referenced Properties/Methods for CIM_ViewCapabilities

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | An opaque, unique id for the view class capability of an implementation. |
| ElementName |  | Optional | A provider supplied user-Friendly Name for this CIM_ViewCapabilities element. |
| SupportedViews |  | Mandatory | This array of strings lists the view classes that are supported by the implementation. Supported string values are 'SNIA:TopologyView' or 'SNIA:FCSwitchView'. |

125

126

127

128

129

8.6.6	CIM_ConcreteComponentView (FCSwitchView to Fabric)

130

- Associates the Fabric AdminDomain to the CIM_FCSwitchView.

Created By: Static

- Modified By: Static

Deleted By: Static

Requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:FCSwitchView'.

Table 85 describes class CIM_ConcreteComponentView (FCSwitchView to Fabric).

### Table 85 - SMI Referenced Properties/Methods for CIM_ConcreteComponentView (FCSwitchView to Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| GroupComponent |  | Mandatory | The Fabric the FCSwitchView instance belongs to. |
| PartComponent |  | Mandatory | The FCSwitchView instance. |

133

134

135	8.6.7	CIM_ConcreteComponentView (FCTopologyView to Fabric)

136

Associates the Fabric AdminDomain to the CIM_FCTopologyView. NOTE: This is an association 137 (CIM_ConcreteComponentView) to an association (CIM_FCTopologyView).

138

139	Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:TopologyView'.

140

Table 86 describes class CIM_ConcreteComponentView (TopologyView to Fabric).

### Table 86 - SMI Referenced Properties/Methods for FC_ConcreteComponentView (FCTopologyView to Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| GroupComponent |  | Mandatory | The Fabric the FCTopologyView instance belongs to. |
| PartComponent |  | Mandatory | The FCTopologyView instance. |

141

142

- 8.6.8	CIM_FCSwitchView

- The CIM_FCSwitchView provides the properties to be able to discover all the switch properties for a Fibre

- Channel Switch and Ports without having to do associators from the ComputerSystem representing the

Switch to FCPort, FCSwitchCapabilities, FCPortCapabilities, LogicalModule, Product, PhysicalPackage,

146

SoftwareIdentity, AccessPoint, and Location.

147

- Created By: Static

Modified By: Static

- Deleted By: Static

Requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:FCSwitchView'.

Table 87 describes class CIM_FCSwitchView.

Table 87 - SMI Referenced Properties/Methods for CIM_FCSwitchView

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| SwitchWWN | C | Mandatory | A Name_Identifier that is associated with a Fibre Channel switch. |
| DomainID |  | Mandatory | The Domain ID of the Switch as identified in the OtherIdentifyingInfo property of the underlying Switch ComputerSystem instance. |
| ElementName |  | Optional | The Symbolic Name of the Switch. |
| SwitchOperationalStatus |  | Mandatory | The operational status of the Switch. |
| SwitchEnabledState |  | Mandatory | EnabledState indicates the enabled and disabled states of the Fibre Channel Switch as defined in the EnabledState property of the underlying Switch ComputerSystem instance. |
| SwitchNPIVSupported |  | Optional | Flag indicating whether NPIV is support on the fibre channel switch as defined in the FCSwitchCapabilities.NPIVSupported property. |
| FCPortWWN | C | Mandatory | The FC Port Name Identifier (FC Port WWN). |
| FCPortElementName |  | Mandatory | The user friendly name of the FCPort. |
| FCPortType |  | Mandatory | The PortType of the FCPort.). |
| FCPortOperationalStatus |  | Mandatory | The operational status of the FCPort. |
| PortEnabledState |  | Mandatory | EnabledState indicates the enabled and disabled states of the Fibre Channel Port. |
| PortDiscriminator |  | Optional | An array property that identifies the context in which this FCPort is instantiated as defined in the underlying FCPort. |

150

151

152

153

154

155

156

157

158

159

### Table 87 - SMI Referenced Properties/Methods for CIM_FCSwitchView

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PortNumber |  | Mandatory | The port number labeled for an administrative purposes relative to the logical module. This is typically what is exposed through the devices element manager or CLI. |
| LogicalModulePortNumber |  | Optional | The port number labeled for an administrative purposes relative to the logical module. |
| DomainPortNumber |  | Optional | The port number used for domain id and physical port zone members. |
| ModuleNumber |  | Conditional | Conditional requirement: Required if the Blades profile is implemented. Logical modules are often named by the physical or logical slot that they occupy within the Switch. |
| PortNPIVSupported |  | Optional | Flag indicating whether NPIV is support on the fibre channel port as defined in the FCPortCapabilities for the FCPort. |
| MaxPortNPIVLogins |  | Optional | Maximum number of NPIV logins supported on the fibre channel port as defined in the FCPortCapabilities for the FCPort. |
| Vendor |  | Mandatory | The name of the Product's supplier of the Switch. |
| ProductName |  | Mandatory | Commonly used Product name. |
| VersionString |  | Mandatory | A string representing the complete software version of the Switch. |
| Model |  | Mandatory | The model of the switch as defined in the PhysicalPackage for the Switch. |
| SerialNumber |  | Optional | The SerialNumber of the switch as defined in the PhysicalPackage for the Switch. |
| PartNumber |  | Optional | The PartNumber of the switch as defined in the PhysicalPackage for the Switch. |
| OEM |  | Mandatory | The Vendor property in the Product associated to the switch for the OEM. |
| OEMProductName |  | Mandatory | The Name property in the Product associated to the switch for the OEM. |
| OEMIdentifyingNumber |  | Mandatory | The IdentifyingNumber property in the Product associated to the switch for the OEM. |
| AccessInfo |  | Conditional | Conditional requirement: Required if the Access Points profile is implemented.The management address as reported in the AccessInfo property of the CIM_RemoteServiceAccessPoint instance for the switch. |
| LocationName |  | Conditional | Conditional requirement: Required if the Location profile is implemented.A free-form string defining a label for the Location as defined by the Name property of the CIM_Location for the switch. |
| LocationPhysicalPosition |  | Optional | A free-form string indicating a street, building or other type of address for the switch's Location. |

8.6.9	CIM_FCTopologyView

The CIM_FCTopologyView provides the properties to be able to build a topology for a Fibre Channel fabric without having to do associators from the ProtocolEndpoint to the FCPort to the FCNode or FCSwitch.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Required if the array property CIM_ViewCapabilities.SupportedViews contains the string 'SNIA:TopologyView'.

Table 88 describes class CIM_FCTopologyView.

Table 88 - SMI Referenced Properties/Methods for CIM_FCTopologyView

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| AntecedentFCPortWWN | C | Mandatory | The FC Port Name Identifier (FC Port WWN) of the FCPort the antecedent reference is associated to. |
| AntecedentElementWWN | C | Mandatory | The FC Switch Name Identifier (FC Switch WWN) or FC Node Name (FC Node WNN) the AntecedentFCPort is associated to. |
| AntecedentFCPortElement Name |  | Mandatory | The user friendly name of the FCPort the antecedent reference is associated to. |
| AntecedentFCPortType |  | Mandatory | The PortType of the FCPort the antecedent reference is associated to.). |
| DependentFCPortWWN | C | Mandatory | The FC Port Name Identifier (FC Port WWN) of the FCPort the Dependent reference is associated to. |
| DependentElementWWN | C | Mandatory | The FC Switch Name Identifier (FC Switch WWN) or FC Node Name (FC Node WNN) the DependentFCPort is associated to. |
| DependentFCPortElement Name |  | Mandatory | The user friendly name of the FCPort the dependent reference is associated to. |
| DependentFCPortType |  | Mandatory | The PortType of the FCPort the dependent reference is associated to.). |
| AntecedentFCPort |  | Mandatory | The reference to the antecedent FCPort that is communicating with the dependent FCPort. |
| AntecedentSystem |  | Mandatory | The reference to the system of the antecedent FCPort in the AntecedentFCPort SystemDevice association. |
| DependentFCPort |  | Mandatory | The reference to the dependent FCPort that is communicating with the antecedent FCPort. |
| DependentSystem |  | Mandatory | The reference to the system of the Dependent FCPort in the DependentFCPort SystemDevice association. |

160

161

162

163

164

165

166

167

168

EXPERIMENTAL

EXPERIMENTAL

# 1	9  Virtual Fabrics

## 9.1	Synopsis

2	Profile Name: Virtual Fabrics (Component Profile)

3

4	Version: 1.5.0

5

6	Organization: SNIA

7

Central Class: AdminDomain

8

Scoping Class: an AdminDomain in a referencing autonomous profile

9	Related Profiles: Table 89 describes the supported profiles for Virtual Fabrics.

10

11	Table 89 - Supported Profiles for Virtual Fabrics

| Profile Name | Organization | Version | Requirement | Description |
| --- | --- | --- | --- | --- |
| SwitchPartitioning | SNIA | 1.5.0 | Mandatory |  |

12

13

14

15

16

## 17	9.2	Description

18 Technologies have recently been implemented for Fibre Channel SANs to logically separate the hardware into multiple fabrics yet keep them physically interconnected. The term for this technology defined by

19

ANSI T11 is "Virtual Fabrics". ANSI T11 calls the underlying hardware “Core Switches”. To be consistent

20 with a more broader use model and DMTF schematics, this profile will call these the partitioning systems.

21

ANSI T11 calls the switching construct that resides in the partitioning system, a virtual switch. Again for

22 the broader use model and consistency with the DMTF, this system will be call the partitioned system. The Fabric Profile already provides the discovery of the “virtual fabrics” and the “partitioned switches”. This profile allows for the discovery of the underlying partitioning system. The Switch Partitioning Profile provides the mechanism for configuring the partitioning system.

All Virtual Fabrics shall be associated to the same SAN for the same underlying “physical” topology. Also the Registered Profile associates to the SAN object to allow the client to arrive at the proper SAN object without traversing upwards from the fabric instance. In Figure 17: "RegisteredProfile, AdminDomain, and ComputerSystem Relationships", the relationship is shown. Note also that the partitioning ComputerSystems are associated with the SAN.

23

24

Figure 17 - RegisteredProfile, AdminDomain, and ComputerSystem Relationships

In Figure 17 the only thing added to the model beyond those elements in the Fabric Profile and the Switch Partitioning Profile are the ElementConformsToProfile association from the SAN AdminDomain (in the Fabric Profile) to the Virtual Fabrics RegisteredProfile and the Component association from the Partitioning ComputerSystem (from the Switch Partitioning Profile) to the same SAN AdminDomain. The Virtual Fabrics Profile is a component profile of the Fabric Profile and it requires implementation of the Switch Partitioning Profile.

In Figure 18: "Two Virtual Fabric and Two Partitioning Systems", the partitioning systems are shown presenting two “virtual fabric” with only one link (ActiveConnection) in each fabric. See section 4.2.3 for more information with regards to fabric topology using ActiveConnection. In this configuration each fabric, partitioned switch, and port are uniquely identified. Note that the new instances to support this profile are outlined in a bolded line.

33

34

35

36

Figure 18 - Two Virtual Fabric and Two Partitioning Systems

In Figure 18: "Two Virtual Fabric and Two Partitioning Systems", a single partitioning system is creating 37 two virtual fabrics and in Figure 19: "Two Virtual Fabrics and One Partitioning System", the system is

38 partitioning the network port. When the NetworkPort is partitioned, each port is not guaranteed to be uniquely identified (though the key to the instance is) and the client needs to map it to the fabric it is 39 participating in to maintain the ports identity.

40

41

42

43

44

45

46

47

48

49

50

Figure 19 - Two Virtual Fabrics and One Partitioning System

In all cases you can have a one-to-one or many-to-one relationship between partitioned switches and the partitioning switch. The same is true between the partitioned FCPorts and the NetworkPorts.

To determine the end-to-end path of a fabric, the client follows the rules outlined in the Fabric Profile (see

63 4.2.3) with the exception that when entering a switch, if there exists a hosted dependency on the switch port, the partitioning system shall be traversed instead of the partitioned system. So in Figure 19: "Two

64

Virtual Fabrics and One Partitioning System", when the client traverses across ActiveConnection to the 65

66 ProtocolEndpoint to the FCPort and down the HostedDependency, it realizes that both ends of the link are 67 actually in the same partitioned system.

68

## 9.3	Health and Fault Management Consideration

69

The possible Health and Fault information through LifeCycle indications are addressed in the Fabric 70 Profile.

71

## 9.4	Cascading Considerations

Not defined in this document

## 9.5	Methods of the Profile

Not defined in this document

## 9.6	Use Cases

Not defined in this document.

## 72	9.7	CIM Elements

9.7.1	Overview

Table 90 describes the CIM elements for FabricVirtualFabrics.

### Table 90 - CIM Elements for FabricVirtualFabrics

| Element Name | Requirement | Description |
| --- | --- | --- |
| 9.7.2 CIM_Component (AdminDomain to Partitioning CS) | Mandatory | Associates the partitioning ComputerSystems representing the underlying physical switches to the AdminDomain representing the SAN. |
| 9.7.3 CIM_ElementConformsToProfile (SAN AdminDomain to Virtual Fabrics RegisteredProfile) | Mandatory | Ties the SAN AdminDomain to the registered profile for Virtual Fabrics. |

73

74

75

76	9.7.2	CIM_Component (AdminDomain to Partitioning CS)

77

Associates the partitioning ComputerSystems representing the underlying physical switches to the

78 AdminDomain representing the SAN.This allows the client another means to determine which ComputerSystems are part of the Fabric versus those that are part of the underlying physical topology. 79 ComputerSystems representing Fibre Channel switches are associated to the AdminDomain representing 80 the fabric (see the Fabric Profile).

Created By: External

Modified By: Static

Deleted By: External

Requirement: Mandatory

Table 91 describes class CIM_Component (AdminDomain to Partitioning CS).

Table 91 - SMI Referenced Properties/Methods for CIM_Component (AdminDomain to Partitioning CS)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| GroupComponent |  | Mandatory | The reference to the AdminDomain representing the SAN (OtherIdentifyingInfo contains 'SAN' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |
| PartComponent |  | Mandatory | The reference to the partitioning ComputerSystem. |

9.7.3	CIM_ElementConformsToProfile (SAN AdminDomain to Virtual Fabrics RegisteredProfile)

The CIM_ElementConformsToProfile ties SAN AdminDomain to the registered profile for Virtual Fabrics.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 92 describes class CIM_ElementConformsToProfile (SAN AdminDomain to Virtual Fabrics RegisteredProfile).

Table 92 - SMI Referenced Properties/Methods for CIM_ElementConformsToProfile (SAN AdminDomain to

### Virtual Fabrics RegisteredProfile)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| ManagedElement |  | Mandatory | A SAN AdminDomain (OtherIdentifyingInfo contains 'SAN' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions') that represents the Virtual Fabric. |
| ConformantStandard |  | Mandatory | RegisteredProfile instance describing the Virtual Fabrics profile. |

EXPERIMENTAL

STABLE

# 1	10  Switch Profile

## 10.1	Synopsis

2	Profile Name: Switch (Autonomous Profile)

3

4	Version: 1.8.0

5

6	Organization: SNIA

7

Central Class: ComputerSystem

8

Scoping Class: ComputerSystem

Related Profiles: Table 93 describes the supported profiles for Switch.

9

10	Table 93 - Supported Profiles for Switch

| Profile Name | Organization | Version | Requirement | Description |
| --- | --- | --- | --- | --- |
| Blades | SNIA | 1.8.0 | Optional |  |
| Access Points | SNIA | 1.3.0 | Optional |  |
| Physical Package | SNIA | 1.5.0 | Mandatory |  |
| Software | SNIA | 1.4.0 | Mandatory |  |
| SwitchPartitioning | SNIA | 1.5.0 | Optional | Experimental. |
| Location | SNIA | 1.4.0 | Optional |  |
| Power Supply | SNIA | 1.0.1 | Optional | Experimental. |
| Fan | SNIA | 1.5.0 | Optional | Experimental. |
| RecordLog | DMTF | 1.0.1 | Optional |  |
| Launch In Context | DMTF | 1.0.0 | Optional | Experimental. See DSP1102, version 1.0.0 |
| Indications | DMTF | 1.2.2 | Mandatory | See DSP1054, version 1.2.2 |

11

12

13

14

15

16

17

18

19

20

21

22

23

24

## 10.2	Description

25

26

10.2.1	Switch Profile

27

- The Switch Profile models logical and physical aspects of a Fibre Channel switch. The ComputerSystem

- class constitutes the center of the switch model (and is the top level object which the profile registration

- points to). An instance of a ComputerSystem is identified as a switch by the property Dedicated set to “switch”.

31

32 This profile includes discovery components including ports, port statistics, product information, software, 33 and chassis information. It also includes configuration of the switch including switch and port state 34 change, port speed, switch name, symbolic names, and DomainID.

35 Both the Switch and Port have a capabilities class, FCSwitchCapabilities and FCPortCapabilities, 36 respectively, defining which configuration options are supported by the switch. The capabilities define 37 what components are configurable and any restrictions that apply. Except for state change, an associated 38 settings class is defined for both the switch and port, FCSwitchSettings and FCPortSettings, which the 39 client uses to request configuration changes to the Switch or Port, respectively. A setting does not 40 necessarily result in a change to the underlying Switch or Port. The client can determine whether the 41 setting was applied by looking at the associated property in the Switch or Port class.

42

The model for configuration is made up of three components, capabilities, settings, and the 43 ManagedElements, ComputerSystem and FCPort, as shown in Figure 20. The capabilities define what 44 components are configurable and any restrictions that apply, the settings define what the client requests, 45 and the ManagedElements expose the actual changes that were applied.

46 The ComputerSystem (Dedicated as Switch) and FCPort classes have the method RequestStateChange() 47 for requesting that the state be changed and an associated property RequestedState on the classes

48 which indicates the current state change that has been requested. The property EnabledState can be examined to determine whether the device has completed the state change.

If a switch is modular, for instance if the switch is comprised of multiple blades on a backplane, LogicalModule can optionally be used to model each sub-module, and as an aggregation point for the switch ports. This is described in the Blade Profile.

FCPort describes the logical aspects of the port link and the data layers. PhysicalConnector models the physical aspects of a port. An instance of the FCPortStatistics class is expected for each instance of the FCPort class. FCPortStatistics expose real time port health and traffic information.

If the instrumentation is embedded in a switch, it shall provide a switch profile implementation for the hosting switch, and it may proxy a switch profile implementation for other switches reported in the Fabric Profile.

49

50

51

52

53

54

55

85	Figure 20 - Switch Instance Diagram

86

10.2.2	FC Port Settings and Capabilities

Capabilities describe the possible features that a ManagedElement supports. Settings are used to describe the requested configuration. The ManagedElement itself describes what settings have been applied and operating.

- For FC Port Type, there are settings that are not in the actual ManagedElement, FCPort.Types. These are settings that allow a subrange of possible port types. They are:

- A G_Port is a Switch Port that is capable of either operating as an E_Port or F_Port. A G_Port determines through Port Initialization whether it operates as an E_Port or as an F_Port.

- A GL_Port is a G_Port that is also capable of operating as an FL_Port.

- A Fx_Port is a switch port capable of operating as an F_Port or FL_Port.

The actual FCPort when operating shall run one of the port types as per FC-GS. In most cases a switch has a default setting to autonegotiate, which in most cases equates to GL or G being set in FCPortSetting.RequestedType. It is required that this setting, FCPortSetting.RequestedType, be shown regardless of whether it was set administratively or is the default behavior of the switch.

FCPortSetting.RequestedType represents a setting that the administrator can understand and clearly

87 identify why a switch port ends up running a particular port type. If the switch does not support setting the port type, the RequestedTypesSupported array will be empty. It is valid to have a port type of “Unknown” 88 until the link has been established. In 10.7.15 CIM_FCPortSettings, Figure 110 is showing the 89 relationship of particular port types and how the capabilities relate to the setting for these port types.

90

91 The same concept applies for FCPort settings for speed except there is a separate property indicating 92 auto negotiate, FCPortSettings.AutoSenseSpeed (LogicalPortSettings.AutoSenseSpeed). Note that this

93 setting may have been previously set through some other administrative interface (e.g., CLI), but should still be reported in FCPortSettings.RequestedSpeed. If FCPortSetting.AutoSenseSpeed is true, then the 94 value of FCPortSettings.RequestedSpeed is ignored and the speed will be negotiated by the hardware. If 95 it is disabled, the port will operate at the speed configured in FCPortSettings.RequestedSpeed.

FCPortSettings.RequestedSpeed allows the port speed to be administratively set (WRITE qualifier). It also indicates to the client that the port has been administratively set (now or at a previous time). This property can only be set administratively if FCPortCapabilities.RequestedSpeedsSupported[] is not empty, and may only be set to one of the values in FCPortCapabilities.RequestedSpeedsSupported[].

FCPortCapabilities.RequestedSpeedsSupported indicates whether the device allows the speed to be administratively set. For instance, a 4Gb port may allow 1, 2, and 4 Gb. FCPort.Speed (LogicalPort.Speed) represents the actual speed the port is running and a speed of zero represents that the link has not been established.

10.2.3	Trunking

Trunking describes from a switch perspective which ports are working together passing frames using the class RedundancySet. The RedundancySet has a property TypeOfSet which is used to identify what type of redundancy or trunking is occurring among the switch ports associated to the RedundancySet using MemberOfCollection. Figure 21 shows a Trunking instance.

Figure 21 - Trunking Instance Diagram

ComputerSystem

Dedicated="switch"

FCPort

FCPort

System

Device

.

.

.

RedundancySet

MemberOf

Collection

Hosted

Collection

96

97

EXPERIMENTAL

- 10.2.4	DetailedPortState, PortAvailability, OperationalStatus, and EnabledState

- Table 2 - "Port OperationalStatus" is the starting point for the status of FCPort. It is a mandatory property of 4 Fabric Profile. EnabledState is also mandatory in Clause 4: Fabric Profile, but only as the value Not 100 Applicable. EnabledState has additional values when 10 Switch Profile is implemented. Those values are

101	shown in Table 94 which also shows how EnabledState relates to OperationalStatus. Table 94 and Table

95 contain specifics about the expected values of the properties for FCPorts DetailedPortState and

102

PortAvailability and their relationship to OperationalStatus and EnabledState.

103

104 In Table 94 are specifics about the expected values of the properties for FCPorts DetailedPortState and 105 its relationship to OperationalStatus, EnabledState and PortAvailability.

106	Table 94 - DetailedPortState for FCPort

| DetailedPortState | OperationalStatus | EnabledState | PortAvailability |
| --- | --- | --- | --- |
| Transceiver Validation | Starting | Starting | Available |
| Locking To Reference Signal | Starting | Starting | Available |
| Synchronized | OK | Enabled | Available |
| Invalid Transceiver | Error | Enabled, but offline | Available |
| No Light (1) | OK | Enabled, but offline | Available |
| Cannot Synchronize | Error | Disabled | Available |
| Laser Fault | Error | Disabled | Available |
| Port Fault | Error | Disabled | Available |
| Diagnostic Fault | Error | Disabled | Available |
| Unknown | Unknown | Disabled | Any value in Table 95 |
| (1) When there is no light, this only indicates that the transceiver is not receiving light and the transceiver is operational. Typically this case occurs when there is no cable attached, but can also be caused by a broken/defective cable. |  |  |  |

107

108

109

110

111

112

113

114

115

116

117

118	In Table 95 are specifics about the expected values of the properties for PortAvailability and its

relationship to OperationalStatus, EnabledState and DetailedPortState.

119

120

Table 95 - PortAvailability for FCPort

| PortAvailability | OperationalStatus | EnabledState | DetailedPortState |
| --- | --- | --- | --- |
| Available | Any value in Table 2, “Port OperationalStatus,” | Any value in Table 94 except Unknown | Any value in Table 94 |
| Not Installed | Stopped | Disabled | Unknown |
| No Transceiver | Stopped | Disabled | Unknown |
| Not Licensed | Stopped | Disabled | Unknown |

EXPERIMENTAL

EXPERIMENTAL

10.2.5	Conditional Classes and Properties

Table 96 contains classes that are conditional on the implementation of other profiles: • CIM_ComputerSystem (Partitioned Switch) is conditional on the implementation of the Switch Partitioning Profile • CIM_FCPort (Partitioned Switch FCPort) is conditional on the implementation of the Switch Partitioning Profile

In addition, some properties of the Switch classes are conditional on implementation of other profiles:

- The 	ElementName, 	OtherIdentifyingInfo, 	IdentifyingDescriptions 	and 	EnabledState 	of

CIM_ComputerSystem (Switch) are required if the Fabric Profile is implemented.

- EnabledState of CIM_FCPort (Switch FCPort) is required if the Fabric Profile is implemented

- PortDiscriminator of CIM_FCPort (Switch FCPort) is required if the Switch Partitioning Profile is implemented.

EXPERIMENTAL

## 10.3	Health and Fault Management

The following classes report possible Health and Fault information through LifeCycle indications:

- ComputerSystem

- FCPort

These LifeCycle indications are more fully described in Storage Management Technical Specification, Part 3 Common Profiles, 1.8.0 Rev 4 Table 224.

Also in Table 96, “CIM Elements for Switch” is a list of AlertIndications which may also be indicators for Health and Fault Management.

## 10.4	Cascading Considerations

121

Not defined in this document

122

123

## 10.5	Methods of this Profile

124

125	Not defined in this document

126

## 10.6	Use Cases

127	Not defined in this document.

128

129

130

131

132

133

134

## 135	10.7	CIM Elements

- 10.7.1	Overview

- Table 96 describes the CIM elements for Switch. 138

Table 96 - CIM Elements for Switch

139

| Element Name | Requirement | Description |
| --- | --- | --- |
| 10.7.2 CIM_ComputerSystem (Partitioned Switch) | Conditional | Conditional requirement: Support for the Switch Partitioning profile. Represents the Partitioned Switch. |
| 10.7.3 CIM_ComputerSystem (Switch) | Mandatory | Represents the Switch Associated to RegisteredProfile. |
| 10.7.4 CIM_ComputerSystemPackage | Mandatory | Associates PhysicalPackage to the ComputerSystem (Switch). |
| 10.7.5 CIM_ElementCapabilities (FCPort to FCPortCapabilities) | Mandatory | Associates FCPort to the FCPortCapabilities. |
| 10.7.6 CIM_ElementCapabilities (System to FCSwitchCapabilities) | Mandatory | Associates FCSwitchCapabilities to the ComputerSystem (Switch). |
| 10.7.7 CIM_ElementSettingData (FCPortSettings to FCPort) | Optional | Associates FCPortSettings to FCPort. |
| 10.7.8 CIM_ElementSettingData (FCSwitchSettings to ComputerSystem) | Mandatory | Associates FCSwitchSettings to ComputerSystem (Switch). |
| 10.7.9 CIM_ElementStatisticalData (FCPortRateStatistics to FCPort) | Optional | Associates FCPortRateStatistics to the FCPort. |
| 10.7.10 CIM_ElementStatisticalData (FCPortStatistics to FCPort) | Mandatory | Associates FCPortStatistics to the FCPort. |
| 10.7.11 CIM_FCPort (Partitioned Switch FCPort) | Conditional | Conditional requirement: Support for the Switch Partitioning profile. Fibre Channel Switch Port for a Virtual Swtich. |
| 10.7.12 CIM_FCPort (Switch FCPort) | Mandatory | Fibre Channel Switch Port. |
| 10.7.13 CIM_FCPortCapabilities | Mandatory | Switch Port Capabilities. |
| 10.7.14 CIM_FCPortRateStatistics | Optional | Fibre Channel Switch Port Rate Statistics. |
| 10.7.15 CIM_FCPortSettings | Optional | Switch Port Settings. |
| 10.7.16 CIM_FCPortStatistics | Mandatory | Fibre Channel Switch Port Statistics. |
| 10.7.17 CIM_FCSwitchCapabilities | Mandatory | Fibre Channel Switch Capabilities. |
| 10.7.18 CIM_FCSwitchSettings | Mandatory | Fibre Channel Switch Settings. |
| 10.7.19 CIM_HostedCollection (Redundancy Set) | Optional | Associates the redundancy set of FCPorts to the ComputerSystem representing the switch. |
| 10.7.20 CIM_HostedCollection (Statistics Collection) | Mandatory | Associates the Statistics Collection to the ComputerSystem representing the switch. |
| 10.7.21 CIM_MemberOfCollection (FCPort to RedundancySet) | Optional | Associates the FCPort to the RedundancySet. |
| 10.7.22 CIM_MemberOfCollection (NetworkPortStatistics to StatisticalCollection) | Mandatory | Associates the NetworkPortStatistics to the StatisticsCollection. |
| 10.7.23 CIM_ProtocolEndpoint | Optional | The endpoint of a link (ActiveConnection). |

140

141

142

143

144

145

146

147

148

149

150

151

Table 96 - CIM Elements for Switch

| Element Name | Requirement | Description |
| --- | --- | --- |
| 10.7.24 CIM_RedundancySet | Optional | The class RedundancySet along with the association MemberOfCollection in this profile is used to show port aggregation for Fibre Channel trunking. |
| 10.7.25 CIM_StatisticsCollection | Mandatory | Collection to aggregate the FCPortStatistics for each switch. |
| 10.7.26 CIM_SystemDevice | Mandatory | Associates FCPort to the ComputerSystem (Switch). |
| SELECT * FROM CIM_InstCreation WHERE SourceInstance ISA CIM_ComputerSystem | Mandatory | New Switch Instance. |
| SELECT * FROM CIM_InstDeletion WHERE SourceInstance ISA CIM_ComputerSystem | Mandatory | Deletion of Switch Instance. |
| SELECT * FROM CIM_InstModification WHERE SourceInstance ISA CIM_ComputerSystem AND SourceInstance.CIM_ComputerSystem::OperationalStatus <> PreviousInstance.CIM_ComputerSystem::OperationalStatus | Mandatory | CQL -Modification of OperationalStatus in Switch Instance. |
| SELECT * FROM CIM_InstModification WHERE SourceInstance ISA CIM_FCPort AND SourceInstance.CIM_FCPort::OperationalStatus <> PreviousInstance.CIM_FCPort::OperationalStatus | Mandatory | CQL -Modification of OperationalStatus in FC Port Instance. |

152

153

154

155

156

157

158

10.7.2	CIM_ComputerSystem (Partitioned Switch)

159

- Represents the Partitioned Switch. A Partitioned Switch shall contain 'Virtual Switch' in one of its entries

- in OtherIdentifyingInfo with a corresponding value of 'SNIA:DetailedType' in the IdentifyingDescriptions array property.

- Created By: Static

- Modified By: Static

- Deleted By: Static

165

Requirement: Support for the Switch Partitioning profile.

166

167

Table 97 describes class CIM_ComputerSystem (Partitioned Switch).

168

169	Table 97 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioned Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | See the CreationClassName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| Name | D | Mandatory | See the Name definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| ElementName |  | Conditional | Conditional requirement: Support for the Fabric profile. See the ElementName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch) User friendly name. Can be set if FCSwitchCapabilities.ElementNameEditSupported for the switch is True. |

170

171

172

173 174

175

176

177

178

179 180

- Table 97 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioned Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| NameFormat |  | Mandatory | See the NameFormat definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| OtherIdentifyingInfo | C | Mandatory | DomainID stored in decimal format, with a value between 0 and 255.Another value corresponding to 'SNIA:DetailedType" shall contain 'Virtual Switch'. Other entries in OtherIdentifyingInfo may also exist. |
| OperationalStatus |  | Mandatory | See Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 Table 3 - OperationalStatus for ComputerSystem. |
| IdentifyingDescriptions |  | Mandatory | 'DomainID' is placed into the entry that corresponds to the DomainID in OtherIdentifyingInfo and 'SNIA:DetailedType' is placed in the entry corresponding to 'Virtual Switch' in OtherIdentifyingInfo. |
| Dedicated |  | Mandatory | See the Dedicated definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| EnabledState |  | Conditional | Conditional requirement: Support for the Fabric profile. See the EnabledState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| RequestedState |  | Mandatory | See the RequestedState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| EnabledDefault |  | Optional | See the EnabledDefault definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| RequestStateChange() |  | Conditional | Conditional requirement: Support for a non-null value in FCSwitchCapabilities.RequestedStatesSupported. |

- 183

184 185 186

187 188

189 190 191

192 193 194

195 196 197

198 199 200

201 202 203

204 205 206

207

10.7.3	CIM_ComputerSystem (Switch)

208	Represents the Switch.

209

Created By: Static

210

Modified By: Static

211

Deleted By: Static

212

Requirement: Mandatory

213

Shall 	be 	associated 	to 	RegisteredProfile 	using 	ElementConformsToProfile 	association. The

214

RegisteredProfile instance shall have RegisteredName set to 'Switch', RegisteredOrganization set to

215

'SNIA', and RegisteredVersion set to '1.8.0'. 216

217	Table 98 describes class CIM_ComputerSystem (Switch). 218

Table 98 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | D | Mandatory | Switch Name (WWN). |

219 220

221 222

223

224 225

226	Table 98 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| ElementName |  | Conditional | Conditional requirement: Support for the Fabric profile. See the ElementName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch) User friendly name. Can be set if FCSwitchCapabilities.ElementNameEditSupported for the switch is True. |
| NameFormat |  | Mandatory | Shall be 'WWN'. |
| OtherIdentifyingInfo | C | Conditional | Conditional requirement: Support for the Fabric profile. See the OtherIdentifyingInfo definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch) DomainID stored in decimal format, with a value between 0 and 255. In addition, if the Fabric profile is implemented, then another value corresponding to 'SNIA:DetailedType" shall contain 'Front Domain', 'Translate Domain', 'Virtual Switch', 'Backbone' or 'None'. |
| OperationalStatus |  | Mandatory | See Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 Table 3 - OperationalStatus for ComputerSystem. |
| IdentifyingDescriptions |  | Conditional | Conditional requirement: Support for the Fabric profile. See the IdentifyingDescriptions definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch) 'DomainID' is placed into corresponding index of OtherIdentifyingInfo. In addition, if the Fabric profile is implemented, then one of the other indices shall contain the value 'SNIA:DetailedType' and in the corresponding index for OtherIdentifyingInfo a value of 'Front Domain' for an Inter-Fabric Routing (IFR) Front Domain, 'Translate Domain' for an Inter-Fabric Routing (IFR) Translate Domain, 'Virtual Switch' for a Partitioned Switch, 'Backbone' for a Backbone Switch, or 'None' when the other values don't apply. |
| Dedicated |  | Mandatory | 'Switch'. |
| EnabledState |  | Conditional | Conditional requirement: Support for the Fabric profile. See the EnabledState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch) |
| RequestedState |  | Mandatory | The Switch state requested via RequestStateChange(). Shall be of the range specified in FCSwitchCapabilities.RequestedStatesSupported if a state change has been requested. Otherwise shall be 'Not Applicable'. |
| EnabledDefault |  | Optional | Default startup for the Switch. |
| RequestStateChange() |  | Conditional | Conditional requirement: Support for a non-null value in FCSwitchCapabilities.RequestedStatesSupported. |

227

228 229 230

231

232

233

234 235 236

237 238 239

240 241 242

243 244 245

246 247 248

249 250 251

252 253

254

255 256 257

258 259

260

10.7.4	CIM_ComputerSystemPackage

261 Associates PhysicalPackage to the ComputerSystem (Switch).

262

- Created By: Static

- Modified By: Static

265

Deleted By: Static

266

Requirement: Mandatory

267

268	Table 99 describes class CIM_ComputerSystemPackage.

269

Table 99 - SMI Referenced Properties/Methods for CIM_ComputerSystemPackage

270

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The reference to the PhysicalPackage. |
| Dependent |  | Mandatory | The reference to the Switch ComputerSystem. |

271 272 273

274

275	10.7.5	CIM_ElementCapabilities (FCPort to FCPortCapabilities)

276

Created By: Static

277

Modified By: Static

278

Deleted By: Static

279

Requirement: Mandatory

280

281 Table 100 describes class CIM_ElementCapabilities (FCPort to FCPortCapabilities).

282

- Table 100 - SMI Referenced Properties/Methods for CIM_ElementCapabilities (FCPort to FCPortCapabili-

- ties)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| ManagedElement |  | Mandatory | The reference to the switch FCPort. |
| Capabilities |  | Mandatory | The reference to the FCPortCapabilities. |

285

286 287 288

289

10.7.6	CIM_ElementCapabilities (System to FCSwitchCapabilities)

- Created By: Static

- Modified By: Static

- Deleted By: Static

Requirement: Mandatory

293

294	Table 101 describes class CIM_ElementCapabilities (System to FCSwitchCapabilities).

295

296	Table 101 - SMI Referenced Properties/Methods for CIM_ElementCapabilities (System to FCSwitchCapabilities)

297

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| ManagedElement |  | Mandatory | The reference to the Switch ComputerSystem. |
| Capabilities |  | Mandatory | The reference to the FCSwitchCapabilities. |

298

299 300 301

302

303	10.7.7	CIM_ElementSettingData (FCPortSettings to FCPort)

304

Created By: Static

305

Modified By: Static

306

Deleted By: Static

307

Requirement: Optional

308

309

310

311	Table 102 describes class CIM_ElementSettingData (FCPortSettings to FCPort).

312

Table 102 - SMI Referenced Properties/Methods for CIM_ElementSettingData (FCPortSettings to FCPort)

313

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| ManagedElement |  | Mandatory | The reference to the switch FCPort. |
| SettingData |  | Mandatory | The reference to the FCPortSettings. |

314 315 316

317

318

- 10.7.8	CIM_ElementSettingData (FCSwitchSettings to ComputerSystem) Associates FCSwitchSettings to ComputerSystem (Switch).

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

324

325	Table 103 describes class CIM_ElementSettingData (FCSwitchSettings to ComputerSystem).

326

Table 103 - SMI Referenced Properties/Methods for CIM_ElementSettingData (FCSwitchSettings to ComputerSystem)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SettingData |  | Mandatory | The reference to the FCSwitchSettings. |
| ManagedElement |  | Mandatory | The reference to the switch ComputerSystem. |

327 328 329

330 331

332

333	10.7.9	CIM_ElementStatisticalData (FCPortRateStatistics to FCPort)

Created By: Static

334

Modified By: Static 335

Deleted By: Static

336

Requirement: Optional

337

338

339	Table 104 describes class CIM_ElementStatisticalData (FCPortRateStatistics to FCPort).

340

Table 104 - SMI Referenced Properties/Methods for CIM_ElementStatisticalData (FCPortRateStatistics to

341	FCPort)

342

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Stats |  | Mandatory | The reference to the FCPortRateStatistics. |
| ManagedElement |  | Mandatory | The reference to the switch FCPort. |

343

344 345

346

10.7.10 CIM_ElementStatisticalData (FCPortStatistics to FCPort)

347

- Created By: Static

- Modified By: Static

Deleted By: Static

- Requirement: Mandatory

- 352

Table 105 describes class CIM_ElementStatisticalData (FCPortStatistics to FCPort).

353

Table 105 - SMI Referenced Properties/Methods for CIM_ElementStatisticalData (FCPortStatistics to 354

FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Stats |  | Mandatory | The reference to the FCPortStatistics. |
| ManagedElement |  | Mandatory | The reference to the switch FCPort. |

355 356 357

358 359

360

- 10.7.11 CIM_FCPort (Partitioned Switch FCPort)

- The Fibre Channel Switch Port for a Virtual Swtich.

- Created By: Static

364

- Modified By: Extrinsic: RequestStateChange

- Deleted By: Static

- Requirement: Support for the Switch Partitioning profile.

368

369	Table 106 describes class CIM_FCPort (Partitioned Switch FCPort).

370

Table 106 - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned Switch FCPort)

371

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | See the SystemCreationClassName definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| SystemName |  | Mandatory | See the SystemName definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| CreationClassName |  | Mandatory | See the CreationClassName definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| DeviceID |  | Mandatory | See the DeviceID definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| ElementName |  | Mandatory | See the ElementName definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| OperationalStatus |  | Mandatory | See Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 Table 2 - Port OperationalStatus. |
| EnabledState |  | Conditional | Conditional requirement: Support for the Fabric profile. |
| DetailedPortState |  | Mandatory | Experimental. See Table 94 - DetailedPortState for FCPort. |
| PortAvailability |  | Optional | Experimental. See Table 95 - PortAvailability for FCPort. |
| RequestedState |  | Mandatory | See the RequestedState definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| EnabledDefault |  | Mandatory | See the EnabledDefault definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| Speed |  | Mandatory | See the Speed definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| MaxSpeed |  | Mandatory | See the MaxSpeed definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| PortType |  | Mandatory | See the PortType definition in section 10.7.12 CIM_FCPort (Switch FCPort). |

372 373 374

375 376 377

378 379 380

381

382

383

384 385 386

387 388

389 390 391

392

393

394 395

- Table 106 - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned Switch FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PortNumber |  | Mandatory | See the PortNumber definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| PermanentAddress |  | Mandatory | See the PermanentAddress definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| LinkTechnology |  | Mandatory | See the LinkTechnology definition in section 10.7.12 CIM_FCPort (Switch FCPort). |
| PortDiscriminator |  | Conditional | Experimental. Conditional requirement: Support for the Switch Partitioning profile. This array property identifies the context in which this FCPort is instantiated. For Partitioned Switch FCPorts, the values may be '3' (VF), '6' (IFR (Virtual)) or '7' (NPIV). |
| DomainPortNumber |  | Optional | Experimental. The port number used for domain id and physical port zone members. |
| LogicalModulePortNumber |  | Optional | Experimental. The port number labeled for an administrative purposes relative to the logical module. |
| RequestStateChange() |  | Conditional | Conditional requirement: Support for a non-null value in FCPortCapabilities.RequestedStatesSupported. See the RequestedStateChange definition in section 10.7.12 CIM_FCPort (Switch FCPort). |

- 398

399 400 401

402 403

404 405 406

407

408

409 410 411

412 413

414

10.7.12 CIM_FCPort (Switch FCPort)

415	The Fibre Channel Switch Port.

416

Created By: Static

417

Modified By: Extrinsic: RequestStateChange

418

Deleted By: Static

419

Requirement: Mandatory

420

421

422	Table 107 describes class CIM_FCPort (Switch FCPort). 423

Table 107 - SMI Referenced Properties/Methods for CIM_FCPort (Switch FCPort)

424

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | The Class Name. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | User friendly name. Can be set if FCPortCapabilities.ElementNameEditSupported is True. |
| OperationalStatus |  | Mandatory | See Storage Management Technical Specification, Part 6: Fabric, version 1.8.0, Table 2 - Port OperationalStatus. |
| EnabledState |  | Conditional | Conditional requirement: Support for the Fabric profile. |
| DetailedPortState |  | Mandatory | Experimental. See Table 94 - DetailedPortState for FCPort. |
| PortAvailability |  | Optional | Experimental. See Table 95 - PortAvailability for FCPort. |

425 426 427

428

429

430

431

432

433

434

435

436 437 438

439

- Table 107 - SMI Referenced Properties/Methods for CIM_FCPort (Switch FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| RequestedState |  | Mandatory | The port state requested via RequestStateChange(). Shall be of the range specified in FCPortCapabilities.RequestedStatesSupported if a state change has been requested. Otherwise shall be 'Not Applicable'. |
| EnabledDefault |  | Mandatory | Default startup for the port. Used in conjunction with RequestedState can allow for persistent disabling of a port. |
| Speed |  | Mandatory | Experimental. Speed of zero represents a link not established. - GFC is 1062500000 bps - GFC is 2125000000 bps 4 GFC is 4250000000 bps 8 GFC= 8500000000 bps 10 GFC single channel variants are 10518750000 bps 10 GFC four channel variants are 12750000000 bps 16 GFC = 14025000000 bps 32 GFC = 28500000000 bps 128 GFC = 114000000000 bps This is the raw bit rate. |
| MaxSpeed |  | Mandatory | The max speed of the Port in Bits per Second using the same algorithm as Speed. |
| PortType |  | Mandatory | FC-GS Port.Type The specific mode currently enabled for the Port. The values: 'N' = Node Port 'NL' = Node Port supporting FC arbitrated loop 'E' = Expansion Port connecting fabric elements (for example, FC switches) 'F' = Fabric (element) Port 'FL' = Fabric (element) Port supporting FC arbitrated loop 'B' = Bridge Port. PortTypes are defined in the ANSI INCITS FC-GS standards. Can be set using FCPortSettings.RequestedType. |
| PortNumber |  | Mandatory | NetworkPorts are often numbered relative to either a logical modules or a network element. |
| PermanentAddress |  | Mandatory | Fibre Channel Port WWN. |
| LinkTechnology |  | Mandatory | 'FC'. |
| PortDiscriminator |  | Optional | Experimental. This array property identifies the context in which this FCPort is instantiated. For this version of the standard, the values may be '0' (Unknown), '2' (Not applicable), '3' (VF), '4' (FCIP), '5' (IFR), '6' (IFR (Virtual)), '7' (NPIV), '8' (Internal) or '9' (Chassis). |
| DomainPortNumber |  | Optional | Experimental. The port number used for domain id and physical port zone members. |

- 442

443

444

445 446 447

448 449 450

451

452

453 454 455

456 457 458

459 460 461

462 463 464

465

466

467

468

469 470 471

472

473

474

475 476 477

478

479

480

481

- Table 107 - SMI Referenced Properties/Methods for CIM_FCPort (Switch FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| LogicalModulePortNumber |  | Optional | Experimental. The port number labeled for an administrative purposes relative to the logical module. |
| RequestStateChange() |  | Conditional | Conditional requirement: Support for a non-null value in FCPortCapabilities.RequestedStatesSupported. Method to change the port state. FCPortCapabilities.RequestedStatesSupported indicates what states can be set. |

- 484

485 486 487

488

489

490

- 10.7.13 CIM_FCPortCapabilities

- Created By: Static

- Modified By: Static

494

Deleted By: Static Requirement: Mandatory

495

496

Table 108 describes class CIM_FCPortCapabilities.

497

498	Table 108 - SMI Referenced Properties/Methods for CIM_FCPortCapabilities

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | Shall be 'FC Port Capabilities'. |
| ElementNameEditSupported |  | Mandatory | Indicates whether FCPort.ElementName is settable. |
| MaxElementNameLen |  | Mandatory | Indicates the maximum string length of FCPort.ElementName. |
| RequestedStatesSupported | N | Mandatory | Indicates the supported states for calling FCPort.RequestStateChange(). If this property is NULL it means RequestedStateChange is not supported. Otherwise the values may be '2' (Enabled), '3' (Disabled), '4' (Shutdown), '6' (Offline), '7' (Test), '8' (Defer), '9' (Quiesce), '10' (Reboot) and/or '11' (Reset). If either '2' (Enabled) or '3' (Disabled) is supported, that both '2' (Enabled) and '3' (Disabled) shall be supported. |
| RequestedSpeedsSupported |  | Mandatory | Indicates the supported speeds that can be set in FCPortSettings.RequestedSpeed. |
| AutoSenseSpeedConfigurable |  | Mandatory | Indicates whether FCPortSettings.AutoSenseSpeed can be set to auto-negotiate speed. |
| RequestedTypesSupported |  | Mandatory | Indicates the list of supported port types that can be set in FCPortSettings.RequestedType. |
| NPIVSupported |  | Optional | Experimental. |
| MaxNPIVLogins |  | Optional | Experimental. |

499

500 501 502

503 504 505

506 507

508

509

510

511 512 513

514 515

516

517

10.7.14 CIM_FCPortRateStatistics

518

Fibre Channel Switch Port Rate Statistics represent the rate per second over the SampleInterval. An 519 instance of this class can represent the statistics for the current statistics, archived and consolidated 520 statistics, or both.

521

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Optional

526

527	Table 109 describes class CIM_FCPortRateStatistics.

528

Table 109 - SMI Referenced Properties/Methods for CIM_FCPortRateStatistics

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| StatisticTime |  | Mandatory | The time the statistic was collected. |
| SampleInterval |  | Mandatory | The interval at which the rates are calculated. |
| TxFrameRate |  | Optional |  |
| RxFrameRate |  | Optional |  |
| MaxTxFrameRate |  | Optional |  |
| MaxRxFrameRate |  | Optional |  |
| TxRate |  | Mandatory |  |
| RxRate |  | Mandatory |  |
| PeakTxRate |  | Optional |  |
| PeakRxRate |  | Optional |  |

529 530

531

532

533 534 535

536 537 538

539 540 541

542 543

544

10.7.15 CIM_FCPortSettings

- Created By: Static

- Modified By: Static

- Deleted By: Static

548

Requirement: Optional

549

550

Table 110 describes class CIM_FCPortSettings.

- Table 110 - SMI Referenced Properties/Methods for CIM_FCPortSettings

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | Shall be 'FC Port Settings'. |
| RequestedSpeed | M | Mandatory | The requested value to which FCPort.Speed should be set. |
| AutoSenseSpeed | M | Mandatory | The request for the FCPort to auto sense the speed (FCPort.Speed). |
| RequestedType | M | Mandatory | The requested setting for the FCPort.PortType. |

- 553 554

555

556

557

558

559

560

561

10.7.16 CIM_FCPortStatistics

562

Snapshot of performance and error counters for the Fibre Channel Switch.

563

Created By: Static Modified By: Static 564

Deleted By: Static 565

566	Requirement: Mandatory

567

568	Table 111 describes class CIM_FCPortStatistics.

569

570	Table 111 - SMI Referenced Properties/Methods for CIM_FCPortStatistics

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| StatisticTime |  | Optional | The time the statistics were collected. If historical data is instantiated (present), this property shall be set with the time representing the time the statistic was collected. |
| ElementName |  | Optional |  |
| BytesTransmitted |  | Mandatory |  |
| BytesReceived |  | Mandatory |  |
| PacketsTransmitted |  | Mandatory |  |
| PacketsReceived |  | Mandatory |  |
| CRCErrors |  | Mandatory |  |
| LinkFailures |  | Mandatory |  |
| PrimitiveSeqProtocolErrCount |  | Mandatory |  |
| LIPCount |  | Optional |  |
| NOSCount |  | Optional |  |
| ErrorFrames |  | Optional |  |
| DumpedFrames |  | Optional |  |
| LossOfSignalCounter |  | Optional |  |
| LossOfSyncCounter |  | Optional |  |
| InvalidTransmissionWords |  | Optional |  |
| FramesTooShort |  | Optional |  |
| FramesTooLong |  | Optional |  |
| AddressErrors |  | Optional |  |
| BufferCreditNotProvided |  | Optional |  |
| BufferCreditNotReceived |  | Optional |  |
| DelimiterErrors |  | Optional |  |
| EncodingDisparityErrors |  | Optional |  |
| LinkResetsReceived |  | Optional |  |
| LinkResetsTransmitted |  | Optional |  |
| MulticastFramesReceived |  | Optional |  |
| MulticastFramesTransmitted |  | Optional |  |
| FBSYFrames |  | Optional |  |
| PBSYFrames |  | Optional |  |
| FRJTFrames |  | Optional |  |

571

572 573

574 575

576 577 578

579 580 581

582 583 584

585 586

587 588 589

590 591

592 593

594

595 596 597

598

599

600

601

602

603

604

605

606

607

- Table 111 - SMI Referenced Properties/Methods for CIM_FCPortStatistics

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PRJTFrames |  | Optional |  |
| RXClass1Frames |  | Optional |  |
| TXClass1Frames |  | Optional |  |
| Class1FBSY |  | Optional |  |
| Class1PBSY |  | Optional |  |
| Class1FRJT |  | Optional |  |
| Class1PRJT |  | Optional |  |
| RXClass2Frames |  | Optional |  |
| TXClass2Frames |  | Optional |  |
| Class2FBSY |  | Optional |  |
| Class2PBSY |  | Optional |  |
| Class2FRJT |  | Optional |  |
| Class2PRJT |  | Optional |  |
| RXClass3Frames |  | Optional |  |
| TXClass3Frames |  | Optional |  |
| Class3FramesDiscarded |  | Optional |  |
| RXBroadcastFrames |  | Optional |  |
| TXBroadcastFrames |  | Optional |  |
| FramePacingCount |  | Optional | Experimental. The number of 2.5 microsecond units that a frame transmission is blocked due to 0 BB credits. |

- 610

611 612 613

614 615

616 617 618

619 620 621

622 623 624

625 626

627

628 629 630

631 632 633

634

635	10.7.17 CIM_FCSwitchCapabilities

Fibre Channel Switch Capabilities

636

637

638	EXPERIMENTAL

639

When the attached HBA supports FC-SB, and the switch is in a cascaded configuration, 640	DomainIDConfigureable and DomainIDLockedSupported shall be TRUE.

641

642	EXPERIMENTAL

643

644	Created By: Static

645

Modified By: Static

646

Deleted By: Static

647

Requirement: Mandatory

648

649

650 651

Table 112 describes class CIM_FCSwitchCapabilities.

652

Table 112 - SMI Referenced Properties/Methods for CIM_FCSwitchCapabilities

653

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | Shall be 'FC Switch Capabilities'. |
| ElementNameEditSupported |  | Mandatory | Capability indicating whether ComputerSystem.ElementName for the switch can be set. |
| MaxElementNameLen |  | Mandatory | Capability specifying the maximum name of ComputerSystem.ElementName for the switch. |
| RequestedStatesSupported | N | Mandatory | The states the switch can support via ComputerSystem.RequestedState. If this property is NULL it means RequestedStateChange is not supported. Otherwise the values may be '2' (Enabled), '3' (Disabled), '4' (Shutdown), '6' (Offline), '7' (Test), '8' (Defer), '9' (Quiesce), '10' (Reboot) and/or '11' (Reset). |
| DomainIDConfigureable |  | Mandatory | Indicates whether the DomainID setting can be modified. |
| MinDomainID |  | Conditional | Conditional requirement: Support for DomainID configuration.Shall be set if DomainIDConfigureable is true. |
| MaxDomainID |  | Conditional | Conditional requirement: Support for DomainID configuration.Shall be set if DomainIDConfigureable is true. |
| DomainIDLockedSupported |  | Mandatory |  |
| PrincipalPrioritiesSupported |  | Mandatory |  |
| NPIVSupported |  | Optional | Experimental. |

654

655

656 657

658 659 660

661 662

663 664 665

666

667

668 669 670

671 672

673

10.7.18 CIM_FCSwitchSettings

674

Fibre Channel Switch Settings

675

676

677	EXPERIMENTAL

678

When the attached HBA supports FC-SB, and the switch is in a cascaded configuration,

679

PreferredDomainID shall be set to a unique value, and DomainIDLocked shall be true.

680

- EXPERIMENTAL

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

686

687	Table 113 describes class CIM_FCSwitchSettings. 688

Table 113 - SMI Referenced Properties/Methods for CIM_FCSwitchSettings

689

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | Shall be 'FC Switch Settings'. |

690 691 692

- Table 113 - SMI Referenced Properties/Methods for CIM_FCSwitchSettings

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PreferredDomainID | M | Conditional | Conditional requirement: Support for DomainID configuration. Required if FCSwitchCapabilities.DomainIDConfigureable is TRUE. |
| DomainIDLocked | M | Conditional | Conditional requirement: Support for DomainID locking. Required if FCSwitchCapabilities.DomainIDLockSupported is TRUE. |
| PrincipalPriority | M | Conditional | Conditional requirement: Support for Principal Priorities. Required if FCSwitchCapabilities.PrincipalPrioritiesSupported is not set to 'Not Applicable'. |

- 695

696 697 698

699 700

701

702

10.7.19 CIM_HostedCollection (Redundancy Set)

703

- Created By: Static

- Modified By: Static

Deleted By: Static

- Requirement: Optional

707

708	Table 114 describes class CIM_HostedCollection (Redundancy Set).

Table 114 - SMI Referenced Properties/Methods for CIM_HostedCollection (Redundancy Set)

709

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The switch ComputerSystem. |
| Dependent |  | Mandatory | The redundancy set of FCPorts. |

710 711 712

713

714

10.7.20 CIM_HostedCollection (Statistics Collection)

715

Created By: Static

716

Modified By: Static 717

Deleted By: Static

718

Requirement: Mandatory

719

720

721	Table 115 describes class CIM_HostedCollection (Statistics Collection).

722

Table 115 - SMI Referenced Properties/Methods for CIM_HostedCollection (Statistics Collection)

723

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The switch ComputerSystem. |
| Dependent |  | Mandatory | The Statistics Collection. |

724 725 726

727

728

729	10.7.21 CIM_MemberOfCollection (FCPort to RedundancySet)

Created By: Static Modified By: Static

730

Deleted By: Static

731

Requirement: Optional

732

733 734

735	Table 116 describes class CIM_MemberOfCollection (FCPort to RedundancySet).

736

Table 116 - SMI Referenced Properties/Methods for CIM_MemberOfCollection (FCPort to RedundancySet)

737

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Member |  | Mandatory | The reference to the switch FCPort. |
| Collection |  | Mandatory | The reference to the RedundancySet. |

738

739

740

741

742	10.7.22 CIM_MemberOfCollection (NetworkPortStatistics to StatisticalCollection)

743

Created By: Static

744

Modified By: Static

745

Deleted By: Static

746

Requirement: Mandatory

747

748 Table 117 describes class CIM_MemberOfCollection (NetworkPortStatistics to StatisticalCollection).

749

- Table 117 - SMI Referenced Properties/Methods for CIM_MemberOfCollection (NetworkPortStatistics to

- StatisticalCollection)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Member |  | Mandatory | The reference to the FCPortStatistics. |
| Collection |  | Mandatory | The reference to the StatisticsColelction. |

752

753 754

755

10.7.23 CIM_ProtocolEndpoint

756

757	The 	endpoint 	of 	a 	link 	(ActiveConnection). 	ProtocolEndpoint 	shall 	be 	implemented 	when

BroadcastReset() is supported (Force LIP). It is expected that the Fabric Profile is also implemented

758

which defines the necessary information for determining who will receive the Force LIP on the loop.

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Optional

763

764

Table 118 describes class CIM_ProtocolEndpoint.

765

766	Table 118 - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | CD | Mandatory | The Fibre Channel Port WWN. |
| NameFormat |  | Mandatory | 'WWN'. |
| ProtocolIFType |  | Mandatory | Shall be 56 (Fibre Channel). |

767

768

769 770 771

772

773

774

775

776

777	Table 118 - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| BroadcastResetSupported |  | Mandatory |  |
| BroadcastReset() |  | Optional | Sends a Force LIP to all attached Ports. Required if BroadcastResetSupported is TRUE. |

778

779 780 781

782

783	10.7.24 CIM_RedundancySet

784

Created By: Static

785

Modified By: Static

786

Deleted By: Static

787

Requirement: Optional

788

789 Table 119 describes class CIM_RedundancySet.

790

791	Table 119 - SMI Referenced Properties/Methods for CIM_RedundancySet

792

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| TypeOfSet |  | Mandatory |  |

793 794 795

796

797

10.7.25 CIM_StatisticsCollection

798

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

- Table 120 describes class CIM_StatisticsCollection.

Table 120 - SMI Referenced Properties/Methods for CIM_StatisticsCollection

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory |  |
| SampleInterval |  | Mandatory |  |
| TimeLastSampled |  | Mandatory |  |

804

805

806

807

808 809 810

811

10.7.26 CIM_SystemDevice

- Created By: Static

- Modified By: Static

814

Deleted By: Static

815

Requirement: Mandatory

816 817

818 819 820

821	Table 121 describes class CIM_SystemDevice. 822

Table 121 - SMI Referenced Properties/Methods for CIM_SystemDevice

823

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to the switch FCPort. |
| GroupComponent |  | Mandatory | The reference to the switch System. |

824

825

826

827

STABLE

828 829

830 831 832

833 834 835

836 837 838

839 840 841

842 843

844 845

846 847

848

849

850 851

852 853

854

855

856

857 858

859 860

861

862

STABLE

# 1	11  Blades Profile

## 2	11.1	Synopsis

- Profile Name: Blades (Component Profile)

- Version: 1.7.0

- Organization: SNIA

Central Class: PhysicalPackage

Scoping Class: a ComputerSystem in a referencing autonomous profile

Related Profiles: Not defined in this document.

## 11.2	Description

This profile describes how blades in a director class switch can be discovered and managed.

## 11.3	Instance Diagram

Figure 22 shows a Switch Blade instance.

Product (Blade)

ComputerSystem

Dedicated="switch"

(

See Switch Profile

)

(

See Switch Profile

)

FCPort

Module

Port

)

See Switch Profile

(

FCPort

PhysicalPackage

(

)

Logical Module

Realizes

ProductPhysical

Component

System

Device

ComputerSystem

Package

LogicalModule

(

See Physical

Package Package)

PhysicalPackage

6

7

8

9

10

11

### Figure 22 - Switch Blade Instance

12

13

## 11.4	Health and Fault Management

14

15	Not defined in this document

## 11.5	Cascading Considerations

Not defined in this document

## 11.6	Methods of this Profile

Not defined in this document

## 11.7	Use Cases

Not defined in this document.

## 11.8	CIM Elements

11.8.1	Overview

Table 122 describes the CIM elements for Blades.

Table 122 - CIM Elements for Blades

| Element Name | Requirement | Description |
| --- | --- | --- |
| 11.8.2 CIM_LogicalModule | Mandatory | The Blade. |
| 11.8.3 CIM_ModulePort | Mandatory | Associates the LogicalModule to the FCPort. |
| 11.8.4 CIM_PhysicalPackage (Logical Module) | Mandatory | The physical package within which the LogicalModule is contained. |
| 11.8.5 CIM_Product (Blade) | Optional | The product information for the Blade. |
| 11.8.6 CIM_ProductPhysicalComponent | Optional | Associates the Product to the PhysicalPackage. |
| 11.8.7 CIM_Realizes (Logical Module Package) | Optional | Associates the LogicalModule to its PhysicalPackage. |
| 11.8.8 CIM_SystemDevice (Logical Module) | Mandatory | Associates the LogicalModule to the ComputerSystem representing the Switch. |
| SELECT * FROM CIM_InstCreation WHERE SourceInstance ISA CIM_LogicalModule | Optional | Creation of a Creation LogicalModule instance. See Section |
| SELECT * FROM CIM_InstDeletion WHERE SourceInstance ISA CIM_LogicalModule | Optional | Deletion of an LogicalModule instance. |
| SELECT * FROM CIM_InstModification WHERE SourceInstance ISA CIM_LogicalModule AND SourceInstance.CIM_LogicalModule::OperationalStatus <> PreviousInstance.CIM_LogicalModule::OperationalStatus | Optional | CQL -Change in status of LogicalModule. |

16

17	11.8.2	CIM_LogicalModule

18

Created By: Static

19

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

23

24

25

26

27	Table 123 describes class CIM_LogicalModule. 28

Table 123 - SMI Referenced Properties/Methods for CIM_LogicalModule

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory |  |
| SystemName |  | Mandatory |  |
| CreationClassName |  | Mandatory |  |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory |  |
| OperationalStatus |  | Mandatory |  |
| ModuleNumber |  | Mandatory |  |

29

30

31

32

33

34

11.8.3	CIM_ModulePort

35

Created By: Static

- Modified By: Static

- Deleted By: Static

Requirement: Mandatory

38

39	Table 124 describes class CIM_ModulePort.

Table 124 - SMI Referenced Properties/Methods for CIM_ModulePort

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | A reference to a switch FCPort. |
| GroupComponent |  | Mandatory | A reference to a LogicalModule. |

40

41

42

11.8.4	CIM_PhysicalPackage (Logical Module)

43

Created By: Static

Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

46

Table 125 describes class CIM_PhysicalPackage (Logical Module).

Table 125 - SMI Referenced Properties/Methods for CIM_PhysicalPackage (Logical Module)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory |  |
| Tag |  | Mandatory |  |
| ElementName |  | Optional |  |
| Name |  | Optional |  |
| Manufacturer |  | Mandatory |  |
| Model |  | Mandatory |  |

47	Table 125 - SMI Referenced Properties/Methods for CIM_PhysicalPackage (Logical Module)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SerialNumber |  | Optional |  |
| Version |  | Optional |  |
| PartNumber |  | Optional |  |

48

11.8.5	CIM_Product (Blade)

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 126 describes class CIM_Product (Blade).

Table 126 - SMI Referenced Properties/Methods for CIM_Product (Blade)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Name |  | Mandatory | Commonly used Product name. |
| IdentifyingNumber |  | Mandatory | Product identification such as a serial number. |
| Vendor |  | Mandatory | The manufacturer or the OEM. |
| Version |  | Mandatory | Product version information. |
| ElementName |  | Mandatory | User Friendly name. Suggested use is Vendor, Version and product name. |

11.8.6	CIM_ProductPhysicalComponent

Associates the Product to the PhysicalPackage. This is necessary to link the Product information to the Blade.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 127 describes class CIM_ProductPhysicalComponent.

Table 127 - SMI Referenced Properties/Methods for CIM_ProductPhysicalComponent

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| GroupComponent |  | Mandatory | Reference to Product |
| PartComponent |  | Mandatory | Reference to PhysicalPackage |

11.8.7	CIM_Realizes (Logical Module Package)

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 128 describes class CIM_Realizes (Logical Module Package).

Table 128 - SMI Referenced Properties/Methods for CIM_Realizes (Logical Module Package)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The reference to the PhysicalPackage. |
| Dependent |  | Mandatory | The reference to the LogicalModule representing the Blade. |

11.8.8	CIM_SystemDevice (Logical Module)

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 129 describes class CIM_SystemDevice (Logical Module).

Table 129 - SMI Referenced Properties/Methods for CIM_SystemDevice (Logical Module)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| GroupComponent |  | Mandatory | A reference to a Switch ComputerSystem. |
| PartComponent |  | Mandatory | A reference to a LogicalModule. |

49

50	STABLE

51

52

53

54

55

56

57

EXPERIMENTAL

# 1	12  Switch Partitioning

## 12.1	Synopsis

2	Profile Name: Switch Partitioning (Component Profile)

3

Version: 1.5.0

Organization: SNIA

4

5	Central Class: CIM_ComputerSystem (Partitioning)

6

Scoping Class: a ComputerSystem in a referencing autonomous profile

Related Profiles: Table 130 describes the supported profiles for SwitchPartitioning.

7	Table 130 - Supported Profiles for SwitchPartitioning

8

| Profile Name | Organization | Version | Requirement | Description |
| --- | --- | --- | --- | --- |
| Physical Package | SNIA | 1.5.0 | Mandatory |  |

9 10

11

12

13

14

## 15	12.2	Description

16

The Switch Partitioning Profile is used when a “switch” actually is implementing multiple instances of a profile. The instances of the profile can be the same profiles, for example the Switch Profile, or different

17	profiles, for example the Switch Profile and the Extender Profile. For the context of further discussion, the

“switch” representing the entire set of systems will be called the Partitioning System and the systems that 18 it is “hosting” are the Partitioned System. For virtual fabrics, ANSI T11 calls the partitioning system the 19 “Core Switch” and the partitioned system the “Virtual Switch”. In other literature, a Partitioning System 20 may be referred to as a Physical System or Physical Switch and a Partitioned Switch may be called a 21 Logical Switch. In Figure 23: "Switch ComputerSystem and Partitioning System", an example is shown 22 with a Partitioning System hosting a Partitioned System running the Extender Profile and the Switch 23 Profile. The nomenclature in this profile uses blue to identify the partitioning entity. 24

25

26

ComputerSystem (Partitioned

Switch)

Dedicated="Switch"

ComputerSystem (Partition

ing)

Dedicated="Not Dedicated",

Bridge/Extender

س

Switch

ComputerSystem (Partitioned Switch)

Dedicated="Switch"

ComputerSystem (Partitioned)

Dedicated="Bridge/Extender

"

Ho

sted

Depe

ndency

27

Figure 23 - Switch ComputerSystem and Partitioning System

28 The relationship between the Partitioning System and its partitions is indicated with the 29 HostedDependency association. The Partitioning System can be determined by locating the 30 ComputerSystem that is not a dependent in a HostedDependency relationship to another 31 ComputerSystem. The Partitioning System also has the Dedicated property set to “Not Dedicated” plus 32 the Dedicated values of the partitioned systems it supports.

33 In Figure 24: "Switch and Partitioning System and Partitioning Ports", the Partitioning Ports have been 34 added. For this Profile, the NetworkPort class or a subclass is required to identify the partitioning entity.

FC Port (Partitioned Switch FCPort)

ComputerSystem

Partitioned Switch

)

(

Dedicated="Switch"

ComputerSystem

Partitioning

(

)

Dedicated="Not Dedicated" &

Switch

Network Port

Hosted

Dependency

Hosted

Dependency

System

Device

FC Port (Partitioned Switch FCPort)

ComputerSystem

(

Partitioned Switch

)

Dedicated="Switch"

Network Port

Hosted

Dependency

Hosted

Dependency

System

Device

System

Device

Figure 24 - Switch and Partitioning System and Partitioning Ports

FC Port

System

Device

ComputerSystem

Partitioned Switch 2

Dedicated=Switch

ComputerSystem

Partitioning System A

Dedicated="Not Dedicated" &

Switch

NetworkPort

System

Device

Hosted

Dependency

Hosted

Dependency

Network

PortSettings

Network

PortCapabilities

Element

Capabilities

Element

SettingData

FC Port

Figure 25 - Underlying System Port Settings and Capabilities

35 In Figure 25: "Underlying System Port Settings and Capabilities", there is the class 36 NetworkPortCapabilities with the property NetworkIDsConfigurable which indicates whether this port can 37 be assigned to a particular fibre channel fabric. If this property is true, then the property, 38 NetworkIDsFormat should be VF_ID and the array property NetworkIDs can be set to define which fibre 39 channel virtual fabrics this port can belong to. When the port connects to one of the fibre channel fabrics 40 in the setting, an FCPort instance shall be instantiated to represent the Fibre Channel Port that is active 41 in the particular fabric with the association HostedDependency showing which NetworkPort the FCPort

42 was partitioned from. If the setting contained more than one NetworkID, for every active connection to a fabric, an instance of FCPort shall be instantiated.

43

44	The FCSwitches partitioned from the partitioning ComputerSystem are implicitly created when a FCPort is connected to a virtual fabric. Currently there is no mechanism to explicitly create a partitioned switch.

45

46 The Physical Package Package is supported as part of this profile and shall be associated to the 47 partitioning system. The partitioned systems may also be associated to the Physical Package Package if 48 the partitioned system is defined in a separate profile. For example, the partitioned system in the Switch

49 Profile may associate to the same instances of PhysicalPackage and Product that the portioning system is associated to.

50

## 12.3	Health and Fault Management Consideration

51

Not defined in this document

52

## 12.4	Cascading Considerations

53

Not defined in this document

54

## 12.5	Methods of the Profile

55

Not defined in this document

## 12.6	Use Cases

Not defined in this document.

## 12.7	CIM Elements

12.7.1	Overview

Table 131 describes the CIM elements for SwitchPartitioning.

56	Table 131 - CIM Elements for SwitchPartitioning

| Element Name | Requirement | Description |
| --- | --- | --- |
| 12.7.2 CIM_ComputerSystem (Partitioned) | Mandatory | The partitioned ComputerSystem. |
| 12.7.3 CIM_ComputerSystem (Partitioning) | Mandatory | The partitioning ComputerSystem. |
| 12.7.4 CIM_ElementCapabilities (Association between NetworkPort and NetworkPortCapabilities) | Mandatory | Association between NetworkPort and NetworkPortCapabilities. |
| 12.7.5 CIM_ElementConformsToProfile (Partitioning ComputerSystem to Switch Partitioning RegisteredProfile) | Mandatory | Ties the Partitioning ComputerSystem to the registered profile for Switch Partitioning. |
| 12.7.6 CIM_ElementSettingData (Association between NetworkPort and NetworkPortSettings) | Mandatory | Association between NetworkPort and NetworkPortSettings. |
| 12.7.7 CIM_FCPort (Partitioned) | Mandatory | Fibre Channel Port on the partitioned system. |
| 12.7.8 CIM_HostedDependency (NetworkPort to FCPort) | Mandatory | Association between NetworkPort to FCPort. |
| 12.7.9 CIM_HostedDependency (Partitioning CS to Partitioned CS) | Mandatory | Association between the Partitioning ComputerSystem and Partitioned ComputerSystem. |
| 12.7.10 CIM_NetworkPort (Partitioning) | Mandatory | The partitioning port. |

57

58

59

60

61

62

63

### Table 131 - CIM Elements for SwitchPartitioning

| Element Name | Requirement | Description |
| --- | --- | --- |
| 12.7.11 CIM_NetworkPortCapabilities | Mandatory | The NetworkPort Capabilities. |
| 12.7.12 CIM_NetworkPortSettings | Mandatory | Defines the Virtual Fabrics the switch port can connect to. |
| 12.7.13 CIM_SystemDevice (FCPort to Partitioned ComputerSystem) | Mandatory | Associates the partitioned classes (FCPort to the ComputerSystem). |
| 12.7.14 CIM_SystemDevice (NetworkPort to ComputerSystem) | Mandatory | Associates the partitioning classes (NetworkPort to the ComputerSystem). |

12.7.2	CIM_ComputerSystem (Partitioned)

The ComputerSystem representing the Interconnect Element (e.g. a switch) or Platform (e.g. Host and Array).

Created By: External

Modified By: Static

Deleted By: External

Requirement: Mandatory

Table 132 describes class CIM_ComputerSystem (Partitioned).

Table 132 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioned)

64

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | C | Mandatory | The Partitioned System Name. The name shall be correlateable to the system represented (e.g., a Switch). |
| Dedicated |  | Mandatory |  |
| OtherIdentifyingInfo |  | Conditional | Conditional requirement: Required if the Virtual Fabrics profile is implemented. One of the entries shall contain the Virtual Fabric ID for the virtual fabric (if virtual fabrics is implemented). Other entries in OtherIdentifyingInfo define the partitioned system (e.g., Switch OtherIdentifyInfo if the Partitioned system is a Switch). |
| IdentifyingDescriptions |  | Conditional | Conditional requirement: Required if the Virtual Fabrics profile is implemented. One of the entries shall contain 'SNIA:VF_ID' for the virtual fabric (if virtual fabrics is implemented). Other entries in IdentifyingDescriptions define the partitioned system (e.g., Switch IdentifyDescriptions if the Partitioned system is a Switch). |

65

66

67

68

69

70

12.7.3	CIM_ComputerSystem (Partitioning)

The ComputerSystem representing the Interconnect Element (e.g. a switch) or Platform (e.g. Host and Array).

Created By: External

Modified By: Static

Deleted By: External

Requirement: Mandatory

71	Table 133 describes class CIM_ComputerSystem (Partitioning).

72

Table 133 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Partitioning)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name |  | Mandatory | The Partitioning System Name. |
| Dedicated |  | Mandatory |  |

73

74

75

76

77

12.7.4	CIM_ElementCapabilities (Association between NetworkPort and NetworkPortCapabilities) Association between NetworkPort and NetworkPortCapabilities.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

78

Table 134 describes class CIM_ElementCapabilities (Association between NetworkPort and 79	NetworkPortCapabilities). 80

Table 134 - SMI Referenced Properties/Methods for CIM_ElementCapabilities (Association between Net-

81	workPort and NetworkPortCapabilities)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| ManagedElement |  | Mandatory | The reference to the NetworkPort. |
| Capabilities |  | Mandatory | The reference to the NetworkPortCapabilities. |

82

83

84

85

12.7.5	CIM_ElementConformsToProfile (Partitioning ComputerSystem to Switch Partitioning RegisteredProfile)

The CIM_ElementConformsToProfile ties Partitioning ComputerSystem to the registered profile for Switch Partitioning.

Created By: Static

Modified By: Static

Deleted By: Static

86	Requirement: Mandatory

87

88

Table 135 describes class CIM_ElementConformsToProfile (Partitioning ComputerSystem to Switch 89	Partitioning RegisteredProfile). 90

Table 135 - SMI Referenced Properties/Methods for CIM_ElementConformsToProfile (Partitioning Comput-

91	erSystem to Switch Partitioning RegisteredProfile)

92

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| ManagedElement |  | Mandatory | A Partitioning ComputerSystem that represents the a base system supporting partitioned systems. |
| ConformantStandard |  | Mandatory | RegisteredProfile instance describing the Switch Partitioning profile. |

93	12.7.6	CIM_ElementSettingData (Association between NetworkPort and NetworkPortSettings)

94

Association between NetworkPort and NetworkPortSettings.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 136 describes class CIM_ElementSettingData (Association between NetworkPort and NetworkPortSettings).

- Table 136 - SMI Referenced Properties/Methods for CIM_ElementSettingData (Association between Net-

- workPort and NetworkPortSettings)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SettingData |  | Mandatory | The reference to the NetworkPortSettings. |
| ManagedElement |  | Mandatory | The reference to the NetworkPort. |

97

98

99

100

12.7.7	CIM_FCPort (Partitioned)

101

102	Fibre Channel Port on the partitioned system. (e.g., a switch partition).

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 137 describes class CIM_FCPort (Partitioned).

Table 137 - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned)

103

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassNam e |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Optional | Port Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |
| PermanentAddress | CD | Mandatory | Fibre Channel Port WWN. Expressed as 16 un-separated upper case hex digits. |
| NetworkAddresses | C | Mandatory | Fibre Channel ID (FCID). Expressed as 8 un-separated upper case hex digits. |
| OperationalStatus |  | Mandatory | The operational status of the FC Port. |
| PortType |  | Mandatory | The specific port type currently enabled (from FC-GS Port.Type). |
| LinkTechnology |  | Mandatory | Shall be 4 ('FC'). |

104

105

106

107

108

109

Table 137 - SMI Referenced Properties/Methods for CIM_FCPort (Partitioned)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SupportedFC4Types |  | Optional | An array of integers indicating the Fibre Channel FC-4 protocols supported. When supporting FC-SB, this property shall be either '27' (FC-SB-2 Channel) for Host Ports or '28' (FC-SB-2 Control Unit) for targets. |
| SupportedCOS |  | Optional | An array of integers indicating the Fibre Channel Classes of Service that are supported. When supporting FC-SB, this property shall be either '2' or '3' for FC-SB-2 Channel or FC-SB-2 Control Unit ports, respectively. |

12.7.8	CIM_HostedDependency (NetworkPort to FCPort)

The association between the partitioning NetworkPort and the partitioned FCPort. The Antecedent references the partitioning port and the Dependent references the partitioned port. The association can be used to determine whether the system is in the underlying SAN topology or the Fabric topology.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 138 describes class CIM_HostedDependency (NetworkPort to FCPort). Table 138 - SMI Referenced Properties/Methods for CIM_HostedDependency (NetworkPort to FCPort)

110

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | NetworkPort representing the Partitioning System. |
| Dependent |  | Mandatory | FCPort representing the Partitioned System. |

111

112

113

114

115

- 12.7.9	CIM_HostedDependency (Partitioning CS to Partitioned CS)

- The association between the Partitioning ComputerSystem and the Partitioned ComputerSystem. The Antecedent references the partitioning system and the Dependent references the partitioned system. The association can be used to determine whether the system is in the underlying SAN topology or the Fabric 118

topology.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 139 describes class CIM_HostedDependency (Partitioning CS to Partitioned CS).

119

- Table 139 - SMI Referenced Properties/Methods for CIM_HostedDependency (Partitioning CS to Parti-

- tioned CS)

122

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | ComputerSystem representing the Partitioning System. |
| Dependent |  | Mandatory | ComputerSystem representing the Partitioned System. |

123

124

125

126	12.7.10 CIM_NetworkPort (Partitioning)

The partitioning port.

127

Created By: External

Modified By: Static

Deleted By: External

Requirement: Mandatory

Table 140 describes class CIM_NetworkPort (Partitioning).

Table 140 - SMI Referenced Properties/Methods for CIM_NetworkPort (Partitioning)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassNam e |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |

128

129

130

131

132

133

134

12.7.11 CIM_NetworkPortCapabilities The NetworkPort Capabilities.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 141 describes class CIM_NetworkPortCapabilities.

Table 141 - SMI Referenced Properties/Methods for CIM_NetworkPortCapabilities

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | Shall be set to "NetworkPortCapabilities". |
| NetworkIDsConfigurable |  | Mandatory | If the switch supports configuring virtual fabrics, this property shall be TRUE. If the switch only supports discovery, this property shall be FALSE. |
| NetworkIDsFormat |  | Mandatory | For configuring virtual fabrics, this property shall be VF_ID. |

135

136

137

138

139

140

141

12.7.12 CIM_NetworkPortSettings

Defines the Virtual Fabrics the switch port can connect to. The property NetworkIDs is an array which should contain the Virtual Fabric IDs (VF_ID) that the NetworkPort will partition FCPorts for.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 142 describes class CIM_NetworkPortSettings.

Table 142 - SMI Referenced Properties/Methods for CIM_NetworkPortSettings

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Mandatory | Shall be set to "NetworkPortSettings". |
| NetworkIDs |  | Mandatory | Contains the Virtual Fabric IDs that the NetworkPort will host FCPorts for. |

142

143

144

12.7.13 CIM_SystemDevice (FCPort to Partitioned ComputerSystem)

145

Associates the FCPort to the partitioned ComputerSystem.

146

147	Created By: External 148	Modified By: Static

Deleted By: External

149	Requirement: Mandatory

Table 143 describes class CIM_SystemDevice (FCPort to Partitioned ComputerSystem).

### Table 143 - SMI Referenced Properties/Methods for CIM_SystemDevice (FCPort to Partitioned ComputerSystem)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| GroupComponent |  | Mandatory | The reference to the partitioned ComputerSystem. |
| PartComponent |  | Mandatory | The reference to the partitioned FCPort. |

150

151

12.7.14 CIM_SystemDevice (NetworkPort to ComputerSystem)

152

Associates the NetworkPort to the ComputerSystem.

153

154	Created By: External 155	Modified By: Static

Deleted By: External

156	Requirement: Mandatory

Table 144 describes class CIM_SystemDevice (NetworkPort to ComputerSystem).

Table 144 - SMI Referenced Properties/Methods for CIM_SystemDevice (NetworkPort to ComputerSystem)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| GroupComponent |  | Mandatory | The reference to the ComputerSystem. |
| PartComponent |  | Mandatory | The reference to the NetworkPort. |

157

158	EXPERIMENTAL

159

160

161

EXPERIMENTAL

# 1	13  N Port Virtualizer Profile

## 2	13.1	Synopsis

3	Profile Name: N Port Virtualizer (Component Profile)

4

Version: 1.7.0

5

Organization: SNIA

6

Central Class: CIM_FCPort (Fabric NPIV)

Scoping Class: AdminDomain

Related Profiles: Table 145 describes the supported profiles for N Port Virtualizer. Table 145 - Supported Profiles for N Port Virtualizer

| Profile Name | Organization | Version | Requirement | Description |
| --- | --- | --- | --- | --- |
| Physical Package | SNIA | 1.5.0 | Conditional | Conditional requirement: Required if the Switch profile is implemented. |
| Switch | SNIA | 1.8.0 | Optional |  |

## 13.2	Description

The N Port Virtualizer Profile is a component profile of the Fabric Profile that adds additional classes and places requirements on the FCPort class of the Fabric Profile.

7

8

9

10

11

12

13

14

15

## 16	13.3	Implementation

17	Figure 26 illustrates the main elements in modeling of an N Port Virtualizer.

### Figure 26 - N Port Virtualizer

37

Figure 26 shows elements in the Fabric Profile as they relate to the elements of the N Port Virtualizer 38 Profile. The classes of the N Port Virtualizer are in the lower left box with association to class instances in 39 the Fabric Profile. The key elements of the N Port Virtualizer are:

- An N Port Virtualizer ComputerSystem (with a Dedicated property of “Gateway”)

40

- FCPorts with an PortDiscriminator property of “NPIV”

41

- One (or more) FCPorts for connecting the N Port Virtualizer system to the Switch system

42 • ProtocolEndpoints for all the FCPorts in the N Port Virtualizer

43

44 • All the FCPorts in the N Port Virtualizer are scoped to the N Port Virtualizer ComputerSystem

- ActiveConnection associations between the ProtocolEndpoints in Platform ComputerSystems (of Fabric) and the ProtocolEndpoints for the NPIV FCPorts in the N Port Virtualizer

The ActiveConnections essentially represent connections between host system ports and the NPIV ports of the N Port Virtualizer (similar to going to switch ports when the N Port Virtualizer is not present).

NOTE  In addition to the ActiveConnections between the Platform ports and the NPIV ports, ActiveConnections are also instantiated for the actual switch port to which the NPIV port provides connectivity to. When the N Port Virtualizer is implemented it adds a constraint on the ActiveConnections between Platform FCPorts and Switch FCPorts. The ActiveConnection shall be subclassed to FCActiveConnection and contain the Discriminator property which shall contain a value of ‘3’ (NPIV) for FCActiveConnections that are effected through the N Port Virtualizer.

- LogicalIdentity associations between the NPIV FCPorts and the Switch FCPorts to represent the Switch FCPort that is supporting the NPIV FCPort.

## 45	13.4	Health and Fault Management Consideration

Not defined in this document.

## 46	13.5	Cascading Considerations

47

48	Not defined in this document.

## 13.6	Methods of the Profile

49	Not defined in this document.

## 50	13.7	Use Cases

51	Not defined in this document.

## 13.8	CIM Elements

52

- 13.8.1	Overview

Table 146 describes the CIM elements for N Port Virtualizer.

- Table 146 - CIM Elements for N Port Virtualizer

| Element Name | Requirement | Description |
| --- | --- | --- |
| 13.8.2 CIM_Component (N Port Virtualizer to Fabric) | Mandatory | Aggregates N Port Virtualizers in the AdminDomain that represents the Fabric. |
| 13.8.3 CIM_ComputerSystem (N Port Virtualizer) | Mandatory | The ComputerSystem representing the N Port Virtualizer. |
| 13.8.4 CIM_ComputerSystemPackage (N Port Virtualizer to Physical Package) | Conditional | Conditional requirement: Required if the Switch profile is implemented. Associates PhysicalPackage to the ComputerSystem (N Port Virtualizer). |
| 13.8.5 CIM_DeviceSAPImplementation (ProtocolEndpoint to Gateway FCPort) | Mandatory | Associates the N Port Virtualizer Gateway FCPort to its ProtocolEndpoint. |
| 13.8.6 CIM_DeviceSAPImplementation (ProtocolEndpoint to NPIV FCPort) | Mandatory | Associates the N Port Virtualizer NPIV FCPort to its ProtocolEndpoint. |
| 13.8.7 CIM_FCActiveConnection (Gateway) | Optional | The association between ProtocolEndpoints representing the links between fibre channel switch ports and N Port Virtualizer gateway ports that are used to effect active connections between platform and switch ports. |
| 13.8.8 CIM_FCActiveConnection (N Port Virtualiztion) | Optional | The association between ProtocolEndpoints representing the links between fibre channel platform ports and switch ports that are effected through an N Port Virtualizer. |
| 13.8.9 CIM_FCPort (Fabric NPIV) | Mandatory | NPIV Fibre Channel Ports of the N Port Virtualizer. |
| 13.8.10 CIM_FCPort (Gateway) | Mandatory | A Fibre Channel Port of the N Port Virtualizer that is used to connect to Switch Ports. |
| 13.8.11 CIM_HostedAccessPoint (N Port Virtualizer System to ProtocolEndpoint) | Mandatory | Associates the ProtocolEndpoint to the N Port Virtualizer ComputerSystem. |
| 13.8.12 CIM_LogicalIdentity (NPIV Port to Switch Port) | Mandatory | Associates ProtocolEndpoints of N Port Virtualizer NPIV FCPorts to ProtocolEnpoints of Switch FCPorts. |
| 13.8.13 CIM_ProtocolEndpoint (N Port Virtualizer) | Mandatory | The endpoint of a link (ActiveConnection) on the N Port Virtualizer. |
| 13.8.14 CIM_SystemDevice (N Port Virtualizer Gateway FCPort to Gateway System) | Optional | Associates N Port Virtualizer Gateway FCPorts to the ComputerSystem (N Port Virtualizer). |
| 13.8.15 CIM_SystemDevice (N Port Virtualizer NPIV FCPort to Gateway System) | Optional | Associates N Port Virtualizer NPIV FCPorts to the ComputerSystem (N Port Virtualizer). |

55

56

57

58

59

60

61

62

63

64

65

66

67

68

69

70

13.8.2	CIM_Component (N Port Virtualizer to Fabric)

Aggregates N Port Virtualizers in the AdminDomain that represents the Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 147 describes class CIM_Component (N Port Virtualizer to Fabric).

Table 147 - SMI Referenced Properties/Methods for CIM_Component (N Port Virtualizer to Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | Reference to an N Port Virtualizer (a ComputerSystem with Dedicated='20'). |
| GroupComponent |  | Mandatory | Reference to the AdminDomain representing the Fabric (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

13.8.3	CIM_ComputerSystem (N Port Virtualizer)

The ComputerSystem representing the N Port Virtualizer.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

71

72	Table 148 describes class CIM_ComputerSystem (N Port Virtualizer).

73

Table 148 - SMI Referenced Properties/Methods for CIM_ComputerSystem (N Port Virtualizer)

74

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | C | Mandatory | This shall be the WWN for the N Port Virtualizer gateway. |
| ElementName |  | Mandatory | The N Port Virtualizer Symbolic Name. |
| NameFormat |  | Mandatory | Shall be 'WWN'. |
| OperationalStatus |  | Mandatory | One of the defined values (2¦3¦6¦8¦9¦10¦11¦12¦13) shall be present in the array value. |
| OtherIdentifyingInfo |  | Optional |  |
| Dedicated |  | Mandatory | Shall be '20' (Gateway). |
| IdentifyingDescriptions |  | Optional |  |

75

76

77

- 13.8.4	CIM_ComputerSystemPackage (N Port Virtualizer to Physical Package)

- Associates PhysicalPackage to the ComputerSystem (N Port Virtualizer). This is required if the Switch Profile is implemented.

80

- Created By: Static

- Modified By: Static

- Deleted By: Static

Requirement: Required if the Switch profile is implemented.

84

Table 149 describes class CIM_ComputerSystemPackage (N Port Virtualizer to Physical Package).

### Table 149 - SMI Referenced Properties/Methods for CIM_ComputerSystemPackage (N Port Virtualizer to Physical Package)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The reference to the PhysicalPackage. |
| Dependent |  | Mandatory | The reference to the Switch ComputerSystem. |

13.8.5	CIM_DeviceSAPImplementation (ProtocolEndpoint to Gateway FCPort) Associates the N Port Virtualizer Gateway FCPort to its ProtocolEndpoint.

Created By: Static

Modified By: Static

Deleted By: Static Requirement: Mandatory

85

86

Table 150 describes class CIM_DeviceSAPImplementation (ProtocolEndpoint to Gateway FCPort).

87

88	Table 150 - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (ProtocolEndpoint to Gateway FCPort)

89

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ProtocolEndpoint. |
| Antecedent |  | Mandatory | Reference to the Gateway FCPort. |

90

91

13.8.6	CIM_DeviceSAPImplementation (ProtocolEndpoint to NPIV FCPort) Associates the N Port Virtualizer NPIV FCPort to its ProtocolEndpoint.

Created By: Static

Modified By: Static

Deleted By: Static

- Requirement: Mandatory

- Table 151 describes class CIM_DeviceSAPImplementation (ProtocolEndpoint to NPIV FCPort).

94

- Table 151 - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (ProtocolEndpoint to

- NPIV FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ProtocolEndpoint. |
| Antecedent |  | Mandatory | Reference to the NPIV FCPort (PortDiscriminator='7'). |

97

98	13.8.7	CIM_FCActiveConnection (Gateway)

The association between ProtocolEndpoints representing the links between fibre channel switch ports and N Port Virtualizer gateway ports that are used to effect active connections between platform and switch ports. Gateway FCActiveConnections are part of this profile.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

99

- Table 152 describes class CIM_FCActiveConnection (Gateway).

- Table 152 - SMI Referenced Properties/Methods for CIM_FCActiveConnection (Gateway)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Discriminator |  | Optional | For ActiveConnections that are instantiated to represent connections established through an N Port Virtualizer, the value shall include '3' (NPIV). |
| Antecedent |  | Mandatory | The reference to the ProtocolEndpoint in the switch. |
| Dependent |  | Mandatory | The reference to the ProtocolEndpoint in the N Port Virtualizer. |

102

103

104

105

13.8.8	CIM_FCActiveConnection (N Port Virtualiztion)

The association between ProtocolEndpoints representing the links between fibre channel platform ports and switch ports that are effected through an N Port Virtualizer. This class table is a constrain on the ActiveConnection in the Fabric Profile.

Created By: Static 106

Modified By: Static

107

108	Deleted By: Static

Requirement: Optional

109

110

Table 153 describes class CIM_FCActiveConnection (N Port Virtualiztion).

111

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Discriminator |  | Optional | For ActiveConnections that are instantiated to represent connections established through an N Port Virtualizer, the value shall include '3' (NPIV). |
| Antecedent |  | Mandatory | The reference to the ProtocolEndpoint for one end of the link. |
| Dependent |  | Mandatory | The reference to the ProtocolEndpoint for the other end of the link. |

112	Table 153 - SMI Referenced Properties/Methods for CIM_FCActiveConnection (N Port Virtualiztion) 113

13.8.9	CIM_FCPort (Fabric NPIV)

NPIV Fibre Channel Ports of the N Port Virtualizer. These are ports that support NPIV and are realized through switch ports.

- Created By: Static

- Modified By: Static

116

- Deleted By: Static

- Requirement: Mandatory

119

120	Table 154 describes class CIM_FCPort (Fabric NPIV).

Table 154 - SMI Referenced Properties/Methods for CIM_FCPort (Fabric NPIV)

121

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The N Port Virtualizer System's CreationClassName. |
| SystemName |  | Mandatory | The N Port Virtualizer System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Optional | Port Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |
| PermanentAddress | CD | Mandatory | Fibre Channel Port WWN. Expressed as 16 un-separated upper case hex digits. |
| NetworkAddresses | C | Mandatory | Fibre Channel ID (FCID). Expressed as 8 un-separated upper case hex digits. |
| OperationalStatus |  | Mandatory | One of the defined values (0¦2¦6¦10¦11) shall be present in the array value. |
| PortType |  | Mandatory | This shall be port type '10' (N). |
| LinkTechnology |  | Mandatory | Shall be 4 ('FC'). |
| SupportedFC4Types |  | Optional | An array of integers indicating the Fibre Channel FC-4 protocols supported. When supporting FC-SB, this property shall be either '27' (FC-SB2 Channel) for Host Ports or '28' (FC-SB-2 Control Unit) for targets. |
| SupportedCOS |  | Optional | An array of integers indicating the Fibre Channel Classes of Service that are supported. When supporting FC-SB, this property shall be either '2' or '3' for FC-SB-2 Channel or FC-SB-2 Control Unit ports, respectively.. |
| PortDiscriminator |  | Mandatory | This array property identifies the context in which this FCPort is instantiated. For the Fabric NPIV FC Ports, the values shall include '7' (NPIV). |

122

123

124

125

126

127

128

13.8.10 CIM_FCPort (Gateway)

A Fibre Channel Port of the N Port Virtualizer that is used to connect to Switch Ports. These are ports are the active connection between the N Port Virtualizer system and the Switch ports in the Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 155 describes class CIM_FCPort (Gateway).

Table 155 - SMI Referenced Properties/Methods for CIM_FCPort (Gateway)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The N Port Virtualizer System's CreationClassName. |
| SystemName |  | Mandatory | The N Port Virtualizer System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| DeviceID |  | Mandatory | Opaque. |
| ElementName |  | Optional | Port Symbolic Name if available. Otherwise NULL. If the underlying implementation includes characters that are illegal in CIM strings, then truncate before the first of those characters. |
| PermanentAddress | CD | Mandatory | Fibre Channel Port WWN. Expressed as 16 un-separated upper case hex digits. |
| NetworkAddresses | C | Mandatory | Fibre Channel ID (FCID). Expressed as 8 un-separated upper case hex digits. |
| OperationalStatus |  | Mandatory | One of the defined values (0¦2¦6¦10¦11) shall be present in the array value. |
| PortType |  | Mandatory | This shall be port type '10' (N). |
| LinkTechnology |  | Mandatory | Shall be 4 ('FC'). |
| SupportedFC4Types |  | Optional | An array of integers indicating the Fibre Channel FC-4 protocols supported. When supporting FC-SB, this property shall be either '27' (FC-SB-2 Channel) for Host Ports or '28' (FC-SB-2 Control Unit) for targets. |
| SupportedCOS |  | Optional | An array of integers indicating the Fibre Channel Classes of Service that are supported. When supporting FC-SB, this property shall be either '2' or '3' for FC-SB-2 Channel or FC-SB-2 Control Unit ports, respectively. |
| PortDiscriminator |  | Mandatory | This array property identifies the context in which this FCPort is instantiated. For this version of the standard, the values shall not contain '7' (NPIV). |

129

130

131

132

133

134

135

136

13.8.11 CIM_HostedAccessPoint (N Port Virtualizer System to ProtocolEndpoint) Associates the ProtocolEndpoint to the N Port Virtualizer ComputerSystem.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 156 describes class CIM_HostedAccessPoint (N Port Virtualizer System to ProtocolEndpoint).

### Table 156 - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (N Port Virtualizer System to ProtocolEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ProtocolEndpoint. |
| Antecedent |  | Mandatory | Reference to the N Port Virtualizer ComputerSystem. |

13.8.12 CIM_LogicalIdentity (NPIV Port to Switch Port)

Associates ProtocolEndpoints of N Port Virtualizer NPIV FCPorts to ProtocolEnpoints of Switch FCPorts.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

137

138	Table 157 describes class CIM_LogicalIdentity (NPIV Port to Switch Port).

139

Table 157 - SMI Referenced Properties/Methods for CIM_LogicalIdentity (NPIV Port to Switch Port)

140

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemElement |  | Mandatory | This is a reference to an N Port Virtualizer ProtocolEndpoint for an NPIV FCPort. |
| SameElement |  | Mandatory | This is a reference to Switch ProtocolEndpoint that supports the N Port Virtualizer ProtocolEndpoint. |

141

142

143

13.8.13 CIM_ProtocolEndpoint (N Port Virtualizer)

The endpoint of a link (ActiveConnection). ProtocolEndpoint shall be implemented when an ActiveConnection exists. It may be implemented if no ActiveConnections exist.

Created By: Static

Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

146

- Table 158 describes class CIM_ProtocolEndpoint (N Port Virtualizer).

- Table 158 - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint (N Port Virtualizer)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The N Port Virtualizer System's CreationClassName. |
| SystemName |  | Mandatory | The N Port Virtualizer System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | CD | Mandatory | The Fibre Channel Port WWN. |
| NameFormat |  | Mandatory | 'WWN'. |
| ProtocolIFType |  | Mandatory | Shall be 56(Fibre channel). |

149

- 13.8.14 CIM_SystemDevice (N Port Virtualizer Gateway FCPort to Gateway System)

Associates N Port Virtualizer Gateway FCPorts to the ComputerSystem (N Port Virtualizer).

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 159 describes class CIM_SystemDevice (N Port Virtualizer Gateway FCPort to Gateway System).

- Table 159 - SMI Referenced Properties/Methods for CIM_SystemDevice (N Port Virtualizer Gateway FCPort

- to Gateway System)

153

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to the Gateway FCPort. |
| GroupComponent |  | Mandatory | The reference to the N Port Virtualizer ComputerSystem. |

154

155

156

157

13.8.15 CIM_SystemDevice (N Port Virtualizer NPIV FCPort to Gateway System)

158

Associates N Port Virtualizer NPIV FCPorts to the ComputerSystem (N Port Virtualizer).

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 160 describes class CIM_SystemDevice (N Port Virtualizer NPIV FCPort to Gateway System).

### Table 160 - SMI Referenced Properties/Methods for CIM_SystemDevice (N Port Virtualizer NPIV FCPort to Gateway System)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to the Fabric NPIV FCPort. |
| GroupComponent |  | Mandatory | The reference to the N Port Virtualizer ComputerSystem. |

159

160

161	EXPERIMENTAL

162

163

164

165

EXPERIMENTAL

# 1	14  Inter Fabric Routing Profile

## 2	14.1	Synopsis

3	Profile Name: Inter Fabric Routing (Component Profile)

4

5	Version: 1.7.0

Organization: SNIA

Central Class: CIM_ComputerSystem (IFR Switch)

Scoping Class: AdminDomain

Related Profiles: Not defined in this document.

## 14.2	Description

14.2.1	Overview

This profile defines the mechanism to model Inter Fabric Routing (IFR) as defined in the T11 document FC-IFR. IFR is a fibre channel standard that allows a device in one fabric to be shared in another fabric without having to merge the two fabrics into a single fabric.

14.2.2	Switch Topology

Figure 27 illustrates the switch topology for Inter Fabric Routing.

6

### Figure 27 - IFR Switch Topology

There is a fabric, called backbone, which provides the mechanism to physically connect the two fabrics, called edge, together. In the instance diagram, two edge fabrics (A and B) are connected by a single 8 backbone fabric. The two partitioning switches, Partitioning Switch 1 and 2, make up the backbone fabric 9 and provide the routing between Edge Fabric A and B. These two partitioning switches also physically 10 connect to only one additional switch in Fabric A and B, identified as Edge Switch 1 and 2, respectively. The partitioning switches presents to each edge fabric, two partitioned switches for the front and translate domains. In this example, a device called initiator is physically in Fabric A and another device called

11 Target is physically in Fabric B, but shared to Fabric A as can be seen by device having an 12 ActiveConnection to two different partitioned switches.

13

14 This profile itself does not introduce any new classes. It extends the Fabric Profile. Without the Inter 15 Fabric Routing Profile, a client would see three separate fabrics without any knowledge that they are 16 physically inter-connected and the shared devices appearing in different fabrics might even confuse the 17 user. That is, Partitioning System 1 and 2 and the HostedDependency would not exist and only present 18 Fabric A, B, and Backbone as three separate fabrics through the Fabric Profile.

19

This profile does require new properties (in bold). Specifically it requires that the switches have the 20 property IdentifyingDescription populated with either "Front Domain", the domain that links directly to the 21 Edge Fabric, or "Translate Domain", the domain that links to the shared device.

22

- 14.2.3	Identification of Ports

- It also requires the PortDiscriminator property on FCPort to be populated on all the ports of the switches with either "IFR" or "IFR (Virtual)". The "IFR (Virtual)" FCPorts are those ports that have no underlying

- physical manifestation. Specifically, these are the port on, or connected, to the translate domain.

26

27	Figure 28 illustrates the usage of PortDiscriminators in modeling of an Inter Fabric Routing.

28

29

30

31

32

33

ComputerSystem (IFR Switch)

Dedicated="Switch"

Name=Logical Switch 1"

OtherIdentifyingInfo=Front Domain

IdentifyingDescriptions=SNIA:DetailedType

ComputerSystem (IFR Switch)

Dedicated="Switch"

Name=Logical Switch 2"

OtherIdentifyingInfo=Translate Domain

IdentifyingDescriptions=SNIA:DetailedType

ComputerSystem (Partitioning)

Dedicated="Not Dedicated" &

Switch

ComputerSystem (IFR Switch)

Dedicated="Switch"

Name=Logical Switch 1"

OtherIdentifyingInfo=Front Domain

IdentifyingDescriptions=SNIA:DetailedType

ComputerSystem (IFR Switch)

Dedicated="Switch"

Name=Logical Switch 2"

OtherIdentifyingInfo=Translate Domain

IdentifyingDescriptions=SNIA:DetailedType

Hosted

Dependency

AdminDomain (Fabric)

Name=Fabric A"

OtherIdentifyingInfo=Fabric

IdentifyingDescriptions=SNIA:DetailedType

AdminDomain (Fabric)

Name=Fabric B"

OtherIdentifyingInfo=Fabric

IdentifyingDescriptions=SNIA:DetailedType

Component

Component

FC Port

PortDiscriminator=

٭

)

IFR

(

ProtocolEndpoint

DeviceSAP

Implementation

FC Port

PortDiscriminator=

٭

(

))

IFR (Virtual

ProtocolEndpoint

DeviceSAP

Implementation

ActiveConnection

Discriminator=

س

)

IFR

(

SystemDevice

SystemDevice

Figure 28 - Inter Fabric Routing Ports

## 34	14.3	Health and Fault Management Consideration

35	Not defined in this document.

## 36	14.4	Cascading Considerations

37	Not defined in this document.

## 38	14.5	Methods of the Profile

39	Not defined in this document.

## 14.6	Use Cases

Not defined in this document.

40

41

## 42	14.7	CIM Elements

43

14.7.1	Overview

Table 161 describes the CIM elements for Inter Fabric Routing.

44

45	Table 161 - CIM Elements for Inter Fabric Routing

| Element Name | Requirement | Description |
| --- | --- | --- |
| 14.7.2 CIM_Component (Backbone Switch to Fabric) | Mandatory | Aggregates Backbone Switches in the AdminDomain that represents an interconnect Fabric. |
| 14.7.3 CIM_Component (IFR Switch to Fabric) | Mandatory | Aggregates IFR Switches in the AdminDomain that represents a connected Fabric. |
| 14.7.4 CIM_ComputerSystem (Backbone Switch) | Mandatory | The is a special case of a switch (one used to interconnect fabrics). |
| 14.7.5 CIM_ComputerSystem (IFR Switch) | Mandatory | This is an IFR special case of a Switch ComputerSystem. |
| 14.7.6 CIM_FCActiveConnection | Optional | The association between ProtocolEndpoints representing the links between fibre channel devices (including ISLs). |
| 14.7.7 CIM_FCPort (IFR FCPort) | Mandatory | Fibre Channel Port for an IFR Switch. |
| 14.7.8 CIM_ProtocolEndpoint | Mandatory | The endpoint of a link (ActiveConnection). |
| 14.7.9 CIM_SystemDevice | Mandatory | Associates an IFR FCPoret to IFR Switch (ComputerSystem) |

46

47

48

49

14.7.2	CIM_Component (Backbone Switch to Fabric)

Aggregates Backbone Switches in the AdminDomain that represents an interconnected Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

50	Table 162 describes class CIM_Component (Backbone Switch to Fabric).

51

Table 162 - SMI Referenced Properties/Methods for CIM_Component (Backbone Switch to Fabric)

| Properties | Flags | Requiremen t | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | Reference to a Backbone Switch. |
| GroupComponent |  | Mandatory | Reference to the AdminDomain representing the Fabric (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

52

53

54

55

56

14.7.3	CIM_Component (IFR Switch to Fabric)

Aggregates IFR Switches in the AdminDomain that represents a connected Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

57

- Table 163 describes class CIM_Component (IFR Switch to Fabric).

- Table 163 - SMI Referenced Properties/Methods for CIM_Component (IFR Switch to Fabric)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | Reference to an IFR Switch. |
| GroupComponent |  | Mandatory | Reference to the AdminDomain representing the Fabric (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

60

61

62

63

14.7.4	CIM_ComputerSystem (Backbone Switch)

In the Inter Fabric Routing profile, this is ComputerSystem representing switches for interconnecting fabrics.

Created By: External

Modified By: Static

Deleted By: External

64	Requirement: Mandatory

65

66	Table 164 describes class CIM_ComputerSystem (Backbone Switch).

67

Table 164 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Backbone Switch)

68

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | See the CreationClassName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| Name | C | Mandatory | See the Name definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| ElementName |  | Mandatory | See the ElementName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| NameFormat |  | Mandatory | See the NameFormat definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| OperationalStatus |  | Mandatory | See the OperationalStatus definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| Dedicated |  | Mandatory | See the Dedicated definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| OtherIdentifyingInfo | C | Mandatory | One of the indices shall contain the value 'Backbone' and in the corresponding index for OtherIdentifyingDescription the value 'SNIA:DetailedType'. |
| IdentifyingDescriptions |  | Mandatory | One of the indices shall contain the value 'SNIA:DetailedType' and in the corresponding index for OtherIdentifyingInfo a value of 'Backbone'' for an Inter-Fabric Routing (IFR) switch for interconnecting fabrics. |

69

70

Table 164 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Backbone Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| EnabledState |  | Mandatory | See the EnabledState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| RequestedState |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestedState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| EnabledDefault |  | Optional | See the EnabledDefault definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| RequestStateChange() |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestStateChange definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |

14.7.5	CIM_ComputerSystem (IFR Switch)

In the Inter Fabric Routing profile, this is ComputerSystem representing switches.

Created By: External

Modified By: Static Deleted By: External

71

Requirement: Mandatory

72

73	Table 165 describes class CIM_ComputerSystem (IFR Switch).

74

75	Table 165 - SMI Referenced Properties/Methods for CIM_ComputerSystem (IFR Switch)

| Properties | Flags | Requiremen t | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | See the CreationClassName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| Name | C | Mandatory | See the Name definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| ElementName |  | Mandatory | See the ElementName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| NameFormat |  | Mandatory | See the NameFormat definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| OperationalStatus |  | Mandatory | See the OperationalStatus definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| Dedicated |  | Mandatory | See the Dedicated definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |

76

77

Table 165 - SMI Referenced Properties/Methods for CIM_ComputerSystem (IFR Switch)

| Properties | Flags | Requiremen t | Description & Notes |
| --- | --- | --- | --- |
| OtherIdentifyingInfo | C | Mandatory | One of the indices shall contain the value 'Front Domain' or 'Translate Domain' and in the corresponding index for OtherIdentifyingDescription the value 'SNIA:DetailedType'. |
| IdentifyingDescriptions |  | Mandatory | One of the indices shall contain the value 'SNIA:DetailedType' and in the corresponding index for OtherIdentifyingInfo a value of 'Front Domain' for an Inter-Fabric Routing (IFR) Front Domain or 'Translate Domain' for an Inter-Fabric Routing (IFR) Translate Domain. |
| EnabledState |  | Mandatory | See the EnabledState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.10 CIM_ComputerSystem (Switch). |
| RequestedState |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestedState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| EnabledDefault |  | Optional | See the EnabledDefault definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |
| RequestStateChange() |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestStateChange definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.3 CIM_ComputerSystem (Switch). |

14.7.6	CIM_FCActiveConnection

78	The association between ProtocolEndpoints representing the links between fibre channel devices

(including ISLs). For loops and NPIV, multiple ActiveConnections are instantiated as one to many 79

- relationships.

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Optional

- Table 166 describes class CIM_FCActiveConnection.

Table 166 - SMI Referenced Properties/Methods for CIM_FCActiveConnection

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Discriminator |  | Mandatory | An array property enumeration used to discriminate the context in which the ActiveConnection is instantiated. For ActiveConnections between IFR Ports this property shall contain the value '6' (IFR). |
| Antecedent |  | Mandatory | The reference to the ProtocolEndpoint for one end of the link. |
| Dependent |  | Mandatory | The reference to the ProtocolEndpoint for the other end of the link. |

- 14.7.7	CIM_FCPort (IFR FCPort)

- Fibre Channel Port for an IFR Switch.

88

- Created By: Static

- Modified By: Static

- Deleted By: Static

Requirement: Mandatory

92

Table 167 describes class CIM_FCPort (IFR FCPort).

Table 167 - SMI Referenced Properties/Methods for CIM_FCPort (IFR FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | See the SystemCreationClassName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| SystemName |  | Mandatory | See the SystemName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| CreationClassName |  | Mandatory | See the CreationClassName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| DeviceID |  | Mandatory | See the DeviceID definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| ElementName |  | Mandatory | See the ElementName definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| PermanentAddress | CD | Mandatory | Fibre Channel Port WWN. |
| Speed |  | Optional | See the Speed definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.12 CIM_FCPort (Switch FCPort). |
| OperationalStatus |  | Mandatory | See the OperationalStatus definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| PortType |  | Mandatory | See the PortType definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| LinkTechnology |  | Mandatory | See the LinkTechnology definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| EnabledState |  | Mandatory | See the EnabledState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 4.7.23 CIM_FCPort (Switch FCPort). |
| DetailedPortState |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 Table 94 - DetailedPortState for FCPort. |
| PortAvailability |  | Optional | See Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 Table 95 - PortAvailability for FCPort. |
| RequestedState |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the RequestedState definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.12 CIM_FCPort (Switch FCPort). |

Table 167 - SMI Referenced Properties/Methods for CIM_FCPort (IFR FCPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| EnabledDefault |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the EnabledDefault definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.12 CIM_FCPort (Switch FCPort). |
| MaxSpeed |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the MaxSpeed definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.12 CIM_FCPort (Switch FCPort). |
| PortNumber |  | Conditional | Conditional requirement: Required if the Switch profile is implemented. See the PortNumber definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.12 CIM_FCPort (Switch FCPort). |
| PortDiscriminator |  | Mandatory | Ports that participate in IFR shall contain the value '5' [IFR] or '6' [IFR (Virtual)]. |
| RequestStateChange() |  | Conditional | Conditional requirement: Support for the Switch Profile and a non-null value in FCPortCapabilities.RequestedStatesSupported. See the RequestStateChange definition in section Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 10.7.12 CIM_FCPort (Switch FCPort). |

- 14.7.8	CIM_ProtocolEndpoint

- The endpoint of a link (ActiveConnection). ProtocolEndpoint shall be implemented when an 95	ActiveConnection exists. It may be implemented if no ActiveConnections exist.

96

Created By: Static

97

Modified By: Static

98

Deleted By: Static

99

Requirement: Mandatory

100

Table 168 describes class CIM_ProtocolEndpoint.

Table 168 - SMI Referenced Properties/Methods for CIM_ProtocolEndpoint

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | CD | Mandatory | The Fibre Channel Port WWN. |
| NameFormat |  | Mandatory | 'WWN'. |
| ProtocolIFType |  | Mandatory | Shall be 56(Fibre channel). |

EXPERIMENTAL

14.7.9	CIM_SystemDevice

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 169 describes class CIM_SystemDevice.

Table 169 - SMI Referenced Properties/Methods for CIM_SystemDevice

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to the switch FCPort.. |
| GroupComponent |  | Mandatory | The reference to the IFR switch System. |

EXPERIMENTAL

# 1	15  FCoE Fabric

## 2	15.1	Synopsis

Profile Name: FCoE Fabric (Component Profile)

3	Version: 1.8.0

4

Organization: SNIA

5	Central Class: AdminDomain

6

7	Scoping Class: AdminDomain

8

Related Profiles: Not defined in this document

9

## 10	15.2	Description

11 The FCoE Fabric Profile extends the Fabric Profile to provide the necessary information to identify that 12 the discovered Fibre Channel Fabric is running FCoE and the correlatable Ethernet components. 13 Specifically this profile identifies the Ethernet Switch and Ethernet Ports provided by a FCoE Switch as 14 well as the end devices running FCoE. The added portions of the model for the Ethernet portion.

15

- The Ethernet side, for purposes of this profile, will be called the Ethernet Network and is a mirror image in many ways of the Fibre Channel Fabric profile. For example, it has ConnectivityCollection collecting

- ProtocolEndpoints in this case LANEndpoints. ComputerSystems having the Dedicated property set to “Ethernet Switch” instead of “FC Switch” which is hosted to the “physical” ComputerSystem (see 15 "FCoE Fabric"). An AdminDomain having components of ComputerSystems being the Ethernet Switches. LANEndpoints associated to EthernetPorts as ProtocolEndpoints associated with FCPorts. Also any of the FCPorts that are hosted on EthernetPorts have the PortDescriminator property set to “FCoE”.

Figure 29 shows the FCoE Fabric instance.

Figure 29 - FCoE Fabric Instance Diagram

## 15.3	Health and Fault Management Consideration

Not defined in this document

## 15.4	Methods of the Profile

Not defined in this document

## 15.5	Use Cases

Not defined in this document.

## 18	15.6	CIM Elements

19

15.6.1	Overview

20	Table 170 describes the CIM elements for FCoE Fabric. 21

Table 170 - CIM Elements for FCoE Fabric

| Element Name | Requirement | Description |
| --- | --- | --- |
| 15.6.2 CIM_AdminDomain (Ethernet Network) | Mandatory | AdminDomain representing the Ethernet Network. |
| 15.6.3 CIM_Component (Switch to Ethernet Network) | Mandatory | Aggregates Switches in the AdminDomain that represents the Ethernet Fabric. |
| 15.6.4 CIM_ComputerSystem (Ethernet Switch) | Mandatory | The ComputerSystem representing an Ethernet Switch. |
| 15.6.5 CIM_ConnectivityCollection (LANEndpoints to EthernetNetwork) | Mandatory | Collects the LanEndpoints of the Ethernet Network. |
| 15.6.6 CIM_DeviceSAPImplementation (Ethernet Switch EthernetPort to LANEndpoint) | Mandatory | Associates the Switch EthernetPort to the LANEndpoint. |
| 15.6.7 CIM_DeviceSAPImplementation (Non-Switch EthernetPort to LANEndpoint) | Mandatory | Associates the Non-Switch (Host or Storage) EthernetPort to the LANEndpoint. |
| 15.6.8 CIM_ActiveConnection (Links transported over Ethernet (FCoE)) | Mandatory | The association between ProtocolEndpoints representing the links between fibre channel devices (including ISLs). |
| 15.6.9 CIM_EthernetPort (Host EthernetPort)) | Mandatory |  |
| 15.6.10 CIM_EthernetPort (Storage EthernetPort) | Mandatory |  |
| 15.6.11 CIM_EthernetPort (Switch EthernetPort) | Mandatory |  |
| 15.6.12 CIM_HostedAccessPoint (AdminDomain to LANEndpoint) | Mandatory | Associates the LANEndpoint to an Ethernet Fabric AdminDomain. |
| 15.6.13 CIM_HostedAccessPoint (ComputerSystem to LANEndpoint) | Mandatory | Associates the LANEndpoint to the hosting Ethernet Switch ComputerSystem. |
| 15.6.14 CIM_HostedCollection (Ethernet Network AdminDomain to ConnectivityCollection) | Mandatory | Associates the ConnectivityCollection of LANEndpoints to the AdminDomain representing the Ethernet Network. |
| 15.6.15 CIM_HostedDependency (ComputerSystem (Ethernet Switch) to Partitioning ComputerSystem) | Optional | The association representing the relationship of the ComputerSystem (Ethernet Switch) that hosts the ComputerSystems (FC Switches) that are virtualized. |
| 15.6.16 CIM_HostedDependency (FCPort to EthernetPort) | Optional | The association representing the relationship of the EthernetPort that hosts the FCPorts for FCoE. |
| 15.6.17 CIM_LANEndpoint (Ethernet ProtocolEndpoint) | Mandatory | The endpoint of a link (ActiveConnection). |
| 15.6.18 CIM_MemberOfCollection (ConnectivityCollection to LANEndpoint) | Mandatory | Associates ConnectivityCollection to LANEndpoint. |
| 15.6.19 CIM_SystemDevice (Switch EthernetPort to Switch) | Mandatory | Associates Switch EthernetPorts to the ComputerSystem (Switch). |
| 15.6.20 CIM_SystemDevice (non-Switch EthernetPort to Ethernet AdminDomain) | Mandatory | Associates non-Switch EthernetPorts to the Ethernet AdminDomain. |

22

23

24

25

26

27

28

29

15.6.2	CIM_AdminDomain (Ethernet Network)

AdminDomain representing the Ethernet Network.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 171 describes class CIM_AdminDomain (Ethernet Network).

Table 171 - SMI Referenced Properties/Methods for CIM_AdminDomain (Ethernet Network)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | Name of Class. |
| Name |  | Mandatory | An arbitrary name (implementation dependent). |
| NameFormat |  | Mandatory | Dependent on the arbitrary name chosen. |
| ElementName |  | Optional | A user friendly name for the Ethernet Network (implementation dependent). |
| OtherIdentifyingInfo |  | Mandatory | For an Ethernet Network AdminDomain this property shall contain the value 'Ethernet'. |
| IdentifyingDescriptions |  | Mandatory | For a SAN AdminDomain this property shall contain the value 'SNIA:DetailedType' in the index for the OtherIdentifyingInfo of 'Ethernet'. |

30

- 15.6.3	CIM_Component (Switch to Ethernet Network)

- Aggregates Switches in the AdminDomain that comprise the Ethernet Fabric.

33

Created By: Static

34

Modified By: Static

35

Deleted By: Static Requirement: Mandatory 36

Table 172 describes class CIM_Component (Switch to Ethernet Network).

Table 172 - SMI Referenced Properties/Methods for CIM_Component (Switch to Ethernet Network)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | Reference to an Ethernet Switch. |
| GroupComponent |  | Mandatory | Reference to the AdminDomain representing the Ethernet Network (OtherIdentifyingInfo contains 'Ethernet' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

15.6.4	CIM_ComputerSystem (Ethernet Switch)

The ComputerSystem representing an Ethernet Switch. This is a constrained case of the Switch ComputerSystem.

Created By: Static

Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

39

- Table 173 describes class CIM_ComputerSystem (Ethernet Switch).

- Table 173 - SMI Referenced Properties/Methods for CIM_ComputerSystem (Ethernet Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| CreationClassName |  | Mandatory | See the CreationClassName definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| Name | C | Mandatory | The Name serves as the key of a System instance in an enterprise environment. |
| ElementName |  | Mandatory | See the ElementName definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| NameFormat |  | Mandatory | The format of the Name property. |
| OperationalStatus |  | Mandatory | See the OperationalStatus definition in section 4.7.10 CIM_ComputerSystem (Switch). |
| Dedicated |  | Mandatory | Shall be 38 (Ethernet Switch). |
| EnabledState |  | Mandatory | See the EnabledState definition in section 4.7.10 CIM_ComputerSystem (Switch). |

42

43

44

45

46

47

48	15.6.5	CIM_ConnectivityCollection (LANEndpoints to EthernetNetwork)

49

Collects the ProtocolEndpoints of the fabric.

50

- Created By: Static

Modified By: Static

- Deleted By: Static

Requirement: Mandatory

Table 174 describes class CIM_ConnectivityCollection (LANEndpoints to EthernetNetwork).

### Table 174 - SMI Referenced Properties/Methods for CIM_ConnectivityCollection (LANEndpoints to EthernetNetwork)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| InstanceID |  | Mandatory | Opaque. |
| ElementName |  | Optional | Not required, can be the Fabric WWN. |

15.6.6	CIM_DeviceSAPImplementation (Ethernet Switch EthernetPort to LANEndpoint) Associates the Ethernet Switch EthernetPort to the LANEndpoint.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 175 describes class CIM_DeviceSAPImplementation (Ethernet Switch EthernetPort to LANEndpoint).

### Table 175 - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (Ethernet Switch EthernetPort to LANEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the LANEndpoint. |
| Antecedent |  | Mandatory | Reference to the Ethernet Switch EthernetPort. |

15.6.7	CIM_DeviceSAPImplementation (Non-Switch EthernetPort to LANEndpoint)

Associates the Non-Switch (Host or Storage) EthernetPort to the LANEndpoint.

53

Created By: Static

54

Modified By: Static

55

Deleted By: Static

56

Requirement: Mandatory

57

58

Table 176 describes class CIM_DeviceSAPImplementation (Non-Switch EthernetPort to LANEndpoint).

59	Table 176 - SMI Referenced Properties/Methods for CIM_DeviceSAPImplementation (Non-Switch EthernetPort to LANEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the LANEndpoint. |
| Antecedent |  | Mandatory | Reference to the Non-Switch EthernetPort. |

15.6.8	CIM_ActiveConnection (Links transported over Ethernet (FCoE))

The association between ProtocolEndpoints representing the links between fibre channel devices

60

(including ISLs). For loops and NPIV, multiple ActiveConnections are instantiated as one to many 61 relationships.

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

- Table 177 describes class CIM_ActiveConnection (Links transported over Ethernet (FCoE)).

### Table 177 - SMI Referenced Properties/Methods for CIM_ActiveConnection (Links transported over Ethernet (FCoE))

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | The reference to the ProtocolEndpoint for one end of the link. |
| Dependent |  | Mandatory | The reference to the ProtocolEndpoint for the other end of the link. |

67	15.6.9	CIM_EthernetPort (Host EthernetPort))

68

Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

72

Table 178 describes class CIM_EthernetPort (Host EthernetPort)..

73

Table 178 - SMI Referenced Properties/Methods for CIM_EthernetPort (Host EthernetPort).

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory |  |
| SystemName |  | Mandatory |  |
| CreationClassName |  | Mandatory |  |
| DeviceID |  | Mandatory |  |
| LinkTechnology |  | Mandatory | Shall be 2 ('Ethernet'). |
| OperationalStatus |  | Mandatory | Shall be 0 (Unknown), 2 (OK), 6 (Error), 10 (Stopped), or 11 (In Service) |
| PermanentAddress | CD | Mandatory | The MAC Address. Shall be formatted as 12 un-separated upper case hex digits. |

74

75

76

77

78

79

15.6.10 CIM_EthernetPort (Storage EthernetPort)

80

81	Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory.

Table 179 describes class CIM_EthernetPort (Storage EthernetPort).

Table 179 - SMI Referenced Properties/Methods for CIM_EthernetPort (Storage EthernetPort))

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassNa me |  | Mandatory |  |
| SystemName |  | Mandatory |  |
| CreationClassName |  | Mandatory |  |
| DeviceID |  | Mandatory |  |
| LinkTechnology |  | Mandatory | Shall be 2 (Ethernet). |
| OperationalStatus |  | Mandatory | Shall be 0 (Unknown), 2 (OK), 6 (Error), 10 (Stopped), or 11 (In Service). |
| PermanentAddress | CD | Mandatory | The MAC Address. Shall be formatted as 12 un-separated upper case hex digits. |

82

83

84

85

86

87

15.6.11 CIM_EthernetPort (Switch EthernetPort)

88	Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory.

Table 180 describes class CIM_EthernetPort (Switch EthernetPort).

Table 180 - SMI Referenced Properties/Methods for CIM_EthernetPort (Switch EthernetPort))

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassNa me |  | Mandatory |  |
| SystemName |  | Mandatory |  |
| CreationClassName |  | Mandatory |  |
| DeviceID |  | Mandatory |  |
| LinkTechnology |  | Mandatory | Shall be 2 (Ethernet). |
| OperationalStatus |  | Mandatory | Shall be 0 (Unknown), 2 (OK), 6 (Error), 10 (Stopped), or 11 (In Service). |
| PermanentAddress | CD | Mandatory | The MAC Address. Shall be formatted as 12 un-separated upper case hex digits. |

15.6.12 CIM_HostedAccessPoint (AdminDomain to LANEndpoint)

Associates the LANEndpoint to the Ethernet Fabric AdminDomain for those systems not registered in the Platform Database or discovered through FDMI.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 181 describes class CIM_HostedAccessPoint (AdminDomain to LANEndpoint).

89 Table 181 - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (AdminDomain to LANEnd90 point)

91

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the LANEndpoint. |
| Antecedent |  | Mandatory | Reference to the Fabric AdminDomain (OtherIdentifyingInfo contains 'Fabric' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

92

93

94

15.6.13 CIM_HostedAccessPoint (ComputerSystem to LANEndpoint)

Associates the LANEndpoint to the hosting System. The hosting System is a ComputerSystem for the Switch or Platform.

Created By: Static

Modified By: Static

95	Deleted By: Static

Requirement: Mandatory

Table 182 describes class CIM_HostedAccessPoint (ComputerSystem to LANEndpoint).

### Table 182 - SMI Referenced Properties/Methods for CIM_HostedAccessPoint (ComputerSystem to LANEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the LANEndpoint. |
| Antecedent |  | Mandatory | Reference to the Ethernet Switch. |

15.6.14 CIM_HostedCollection (Ethernet Network AdminDomain to ConnectivityCollection) Associates the ConnectivityCollection to the AdminDomain representing the Fabric.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 183 describes class CIM_HostedCollection (Ethernet Network AdminDomain to ConnectivityCollection).

### Table 183 - SMI Referenced Properties/Methods for CIM_HostedCollection (Ethernet Network AdminDomain to ConnectivityCollection)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Dependent |  | Mandatory | Reference to the ConnectivityCollection. |
| Antecedent |  | Mandatory | Reference to the Ethernet Network AdminDomain (OtherIdentifyingInfo contains 'Ethernet' with a corresponding 'SNIA:DetailedType' in IdentifyingDescriptions'). |

15.6.15 CIM_HostedDependency (ComputerSystem (Ethernet Switch) to Partitioning ComputerSystem)

The association representing the relationship of the ComputerSystem (Ethernet Switch) that hosts the ComputerSystems (FC Switches) that are virtualized.

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Optional

Table 184 describes class CIM_HostedDependency (ComputerSystem (Ethernet Switch) to Partitioning ComputerSystem).

### Table 184 - SMI Referenced Properties/Methods for CIM_HostedDependency (ComputerSystem (Ethernet Switch) to Partitioning ComputerSystem)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | A reference to the hosting Host ComputerSystem ('physical Ethernet switch'). |
| Dependent |  | Mandatory | A reference to the hosted fibre channel switch (a ComputerSystem with Dedicated='5'). |

15.6.16 CIM_HostedDependency (FCPort to EthernetPort)

96	The association representing the relationship of the EthernetPort that hosts the FCPorts for FCoE. This

association is required for all FCPorts at the Hosts that are instantiated as part of FCoE.

97

98

Created By: Static

99	Modified By: Static

100

Deleted By: Static

Requirement: Optional

101

Table 185 describes class CIM_HostedDependency (FCPort to EthernetPort).

Table 185 - SMI Referenced Properties/Methods for CIM_HostedDependency (FCPort to EthernetPort)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Antecedent |  | Mandatory | A reference to the hosting Host EthernetPort (“physical”). |
| Dependent |  | Mandatory | A reference to the hosted FCPort for FCoE (virtualized fibre channel port with PortDiscriminator='10'). |

102

15.6.17 CIM_LANEndpoint (Ethernet ProtocolEndpoint)

103

- The endpoint of an ethernet link.

- Created By: Static

- Modified By: Static

- Deleted By: Static

- Requirement: Mandatory

- Table 186 describes class CIM_LANEndpoint (Ethernet ProtocolEndpoint).

Table 186 - SMI Referenced Properties/Methods for CIM_LANEndpoint (Ethernet ProtocolEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| SystemCreationClassName |  | Mandatory | The scoping System's CreationClassName. |
| SystemName |  | Mandatory | The scoping System's Name. |
| CreationClassName |  | Mandatory | Name of Class. |
| Name | D | Mandatory | The MACAddress. |

- Table 186 - SMI Referenced Properties/Methods for CIM_LANEndpoint (Ethernet ProtocolEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| NameFormat |  | Mandatory | 'MACAddress'. |
| ProtocolIFType |  | Mandatory | Shall be 6(Ethernet). |

111

112

113

114

115

15.6.18 CIM_MemberOfCollection (ConnectivityCollection to LANEndpoint)

116	Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 187 describes class CIM_MemberOfCollection (ConnectivityCollection to LANEndpoint).

Table 187 - SMI Referenced Properties/Methods for CIM_MemberOfCollection (ConnectivityCollection to

### LANEndpoint)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| Collection |  | Mandatory | Reference to the ConnectivityCollection. |
| Member |  | Mandatory | Reference to the LANEndpoint. |

117

118

119

120

121

15.6.19 CIM_SystemDevice (Switch EthernetPort to Switch)

122

Associates the Switch EthernetPort to the ComputerSystem (Switch).

123	Created By: Static

124

Modified By: Static

Deleted By: Static

Requirement: Mandatory

Table 188 describes class CIM_SystemDevice (Switch EthernetPort to Switch).

Table 188 - SMI Referenced Properties/Methods for CIM_SystemDevice (Switch EthernetPort to Switch)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to the switch EthernetPort. |
| GroupComponent |  | Mandatory | The reference to the Switch ComputerSystem. |

125

126

127	15.6.20 CIM_SystemDevice (non-Switch EthernetPort to Ethernet AdminDomain)

128

Associates the non-Switch EthernetPort to the Ethernet AdminDomain..

Created By: Static

Modified By: Static

Deleted By: Static

Requirement: Mandatory

129	Table 189 describes class CIM_SystemDevice (non-Switch EthernetPort to EthernetAdminDomain).

Table 189 - SMI Referenced Properties/Methods for CIM_SystemDevice (non-Switch EthernetPort to Ethernet AdminDomain)

| Properties | Flags | Requirement | Description & Notes |
| --- | --- | --- | --- |
| PartComponent |  | Mandatory | The reference to a non-switch EthernetPort. |
| GroupComponent |  | Mandatory | The reference to the Ethernet AdminDomain. |

130

131	EXPERIMENTAL

132

133

134

135

136

137

138

139

140

141

142

143

144

145

146

147

# 1	Annex A (informative) SMI-S Information Model

2 This standard is based on DMTF’s CIM schema, Version 2.51. The DMTF schema is available in the 3 machine-readable Managed Object Format (MOF) format. DMTF MOFs are simultaneously released both 4 as an "Experimental" and a "Final" version of the schema. This provides developers with early access to 5 experimental parts of the models. Both versions are available at http://www.dmtf.org/standards/cim.

6

Content marked as “Experimental” or “Implemented” may be based on DMTF’s Experimental MOFs.

7

8

9

Annex B (Informative) Structure of Fabric Profiles

1

EXPERIMENTAL

# Annex B (Informative) Structure of Fabric Profiles

- B.1	Purpose of this Annex

- This is an informative annex to the Storage Management Technical Specification, Part 6 Fabric, 1.8.0 Rev 4 to

- explain how the main profiles in this part interact. Particular focus is put on the various definitions and 5 usages of CIM_AdminDomain, CIM_ComputerSystem and CIM_FCPort.

- B.2	Overview of Fabric Part Profiles

- Figure B.1 illustrates the profiles defined in Storage Management Technical Specification, Part 6 Fabric, 1.8.0 8	Rev 4 and how they relate.

Virtual Fabics

Zone Control

IFR Profile

Fabric Profile

Physical Package

Switch Profile

Enhanced

Zoning

FDMI

Switch

Configuration

Data

FabricSecurity

Fabric Path

Performance

Fabric Views

Launch In

Context

Component

Component

Physical Package

Blades

Access Points

Software

Power Supply

Fan

Record Log

Launch In Context

Switch Partitioning

Profile

Component

N Port Virtualizer

Stitching

)

Switch

(

Component

Physical Package

Component

Conditional on Switch

FCoE Fabric

9

10

11

### Figure B.1 - The structure of the Fabric Part Profiles

- There are two main Autonomous Profiles: the Fabric Profile and the Switch Profile. The other profiles

- shown are component profiles of either the Fabric or Switch Profiles. In the case of the Switch Partitioning Profile, it may be a component of either the Fabric Profile (when Virtual Fabrics are implemented) or the Switch Profile.

- The Fabric Profile (and its component profiles) are distinguished by the existence of instances of the

- CIM_AdminDomain class. There are two types of CIM_AdminDomain instances. One type represents a Fabric and the other represents a SAN.

Both the Fabric and Switch Profiles reference various types of Switches (represented by

- CIM_ComputerSystem) and other types of CIM_ComputerSystem instances. Between the two profiles, nine distinct variations of CIM_ComputerSystem are defined. CIM_ComputerSystem is used to represent

SMI-S 1.8.0 Revision 5	SNIA Technical Position

a Switch, a Partitioned Switch, an IFR Switch, a Backbone Switch, a Storage Platform, a Host Platform, a

.

- Gateway Switch, a "partitioning" system or a "partitioned" system.

18

19 An implementation may implement both the Fabric and Switch Profiles. When this is done, the switches in the Fabric Profile will have the same Name property as the switches in the Switch Profile. Some properties are required if the either profile is implemented (e.g., Name, NameFormat and Dedicated), 20 some properties are required if Fabric Profile is implemented and some properties are required if the 21 Switch Profile is implemented.

22

- Like CIM_ComputerSystem, both the Fabric Profile and the Switch Profile make references to instances of CIM_FCPort. There are 9 distinct types of Ports. An instance of an FCPort can be one or more of the a

- switch FCPort, a partitioned switch FCPort, an IFR FCPort, a Fabric NPIV FCPort, an FCoE FCPort, a 25 Host NPIV Port, a Gateway FCPort or a non-Switch (Platform) FCPort. In addition, a port may also be a 26 NetworkPort attached to a Partitioning System.

27 To understand which AdminDomain, ComputerSystem or FCPort a particular instance is, a client would 28 inspect certain properties of these classes. In some cases, it will be necessary to also inspect 29 associations on the instance to determine which type of instance it is.

30

- For AdminDomain, the distinguishing properties are OtherIdentifyingInfo and IdentifyingDescriptions. A

SAN 	will 	have 	an 	OtherIdentifyingInfo="SAN" 	with 	a 	corresponding

- IdentifyingDescription="SNIA:DetailedType". A fabric will have an OtherIdentifyingInfo="Fabric" with a 33 corresponding IdentifyingDescription="SNIA:DetailedType". All instances of fibre channel AdminDomain 34 shall have one of these two values. Furthermore, an instance of AdminDomain shall not specify both 35 "SAN" and "Fabric" in their OtherIdentifyingInfo values (with a corresponding

- IdentifyingDescription="SNIA:DetailedType"). In addition, if FCoE is implemented there will be another

AdminDomain for the Ethernet network. That AdminDomain shall have OtherIdentifyingInfo=”Ethernet” with a

- corresponding IdentifyingDescription=”SNA:DetailedType”.

38

39 For ComputerSystem, the distinguishing properties are Dedicated, OtherIdentifyingInfo and 40 IdentifyingDescriptions. The Fabric Profile defines 4 types of CIM_ComputerSystem instances: A Switch,

41 a Partitioned Switch, a Storage Platform and a Host Platform. In addition, the IFR component profile of the Fabric defines an IFR Switch CIM_ComputerSystem and the N Port Virtualizer defines an N Port 42

Virtualizer CIM_ComputerSystem. In addition, if the Virtual Fabrics Profile is implemented, this profile 43 introduces a "partitioning" CIM_ComputerSystem. In the case of an FCoE environment, the partitioning

44

CIM_ComputerSystem represents an Ethernet Switch. So, a complete implementation of the Fabric 45 Profile (and its component profiles) results in 8 different types of CIM_ComputerSystem.

46

The Switch Profile defines 2 types of CIM_ComputerSystems: Switch and Partitioned Switch. These are 47 similar to the Fabric definitions of Switch and Partitioned Switch ComputerSystems, but there are slight

48 differences. In addition, if the Switch Partitioning Profile is implemented, the "Partitioning"

49

CIM_ComputerSystem and "Partitioned" CIM_ComputerSystem are introduced. These are the same as

50 the partitioning and partitioned CIM_ComputerSystems defined in Virtual Fabrics.

51

For FCPort, the distinguishing property is the PortDiscriminator property of FCPort for most types of 52

FCPort. A Partitioned Switch FCPort contains a "3" in its PortDiscriminator. An IFR FCPort contains either 53 a "5" or "6" in its PortDiscriminator. An NPIV FCPort (either Host or Fabric) contains a "7" in its 54

PortDiscriminator. An FCPort that is virtualized on an Ethernet port has a PortDiscriminator of "10". Other

55

Ports are "undistinguished" (Platform FCPorts, non-virtual FCPorts, Gateway FCPorts and Network 56

Ports). For these ports they would need to be distinguished by association. For example, a Network Port

57 would be the Antecedent of a HostedDependency association from an FCPort. A Gateway FCPort would 58 NOT be a Fabric NPIV FCPort that is also associated to a Gateway (N Port Virtualizer) ComputerSystem. 59 Platform FCPorts would have a SystemDevice association to either a Platform ComputerSystem (Host or 60 Storage) or to a Fabric AdminDomain. Non-virtual FCPorts are ports that have a SystemDevice to one of 61 the switch ComputerSystems and do not have a PortDiscriminator property or it has one that does not 62 contain the value "3".

Annex B (Informative) Structure of Fabric Profiles

63	B.3	Property and Association Requirements

64

For each of the multiple types of class instances for CIM_AdminDomain, CIM_ComputerSystem and

65

CIM_FCPort there are differences in property requirements and association patterns.

66

67

B.3.1	CIM_AdminDomain Requirements

68

69 Table B.1 identifies the properties of CIM_AdminDomain to be supported by SAN, Fabric, and Ethernet 70 instances. The values for the distinguishing properties are identified. Otherwise, the property merely 71 identifies whether it is Mandatory, Optional or Conditional. If it is Conditional, the condition is identified in 72 the notes at the bottom of Table B.1.

73

Table B.1 -  AdminDomain Properties

| AdminDomain Property | SAN | Fabric | Ethernet (2) |
| --- | --- | --- | --- |
| CreationClassName | Mandatory | Mandatory | Mandatory |
| Name | Mandatory | Mandatory | Mandatory |
| NameFormat | Mandatory | Mandatory | Mandatory |
| ElementName | Optional | Optional |  |
| OtherIdentifyingInfo | Mandatory - “SAN” | Mandatory - “Fabric” | Mandatory - “Ethernet” |
|  |  | Conditional (1) - Virtual Fabric ID |  |
| IdentifyingDescriptions | Mandatory - “SNIA:DetailedType” | Mandatory - “SNIA:DetailedType” | Mandatory - “SNIA:DetailedType” |
|  |  | Conditional (1) - “SNIA:VF_ID” |  |
| - - If the Virtual Fabrics Profile is implemented, then the OtherIdentifyingInfo will contain a virtual fabric ID and the IdentifyingDescriptions will contain "SNIA:VF_ID" - - The Ethernet AdminDomain is only required inf the FCoE Fabric profile is implemented. |  |  |  |

74

75

76

77

78

79

80

81

82

83

A SAN AdminDomain has an OtherIdentifyingInfo value of "SAN" and a Fabric AdminDomain has an OtherIdentifyingInfo value of "Fabric". Each must have a corresponding value of "SNIA:DetailedType" in the IdentifyingDescriptions array property. In addition, if the Virtual Fabrics Profile is implemented, the Fabric AdminDomain would also have a virtual fabric ID in the OtherIdentifyingInfo array property (with a corresponding "SNIA:VF_ID" in the IdentifyingDescriptions array property).

In addition, the fabric shall have a "Ethernet" AdminDomain if the fabric is an FCoE Fabric (that is, the FCoE Fabric profile is implemented).

84

85

86

87

88

SMI-S 1.8.0 Revision 5	SNIA Technical Position

- Graphically, this is illustrated in Figure B.2. The SAN, Fabric, and Ethernet instances are different (as

- distinguished by OtherIdentifyingInfo and IdentifyingDescriptions). For Virtual Fabrics, a virtual fabric ID also shows up in the OtherIdentifyingInfo array property.

91

Figure B.2 - AdminDomain Properties

The association pattern for CIM_AdminDomain instances is illustrated in Table B.2

Table B.2 -  AdminDomain Associations

94

| Association on AdminDomain | SAN | Fabric | Ethernet |
| --- | --- | --- | --- |
| ContainedDomain to AdminDomain (SAN or Fabric) | Mandatory | Mandatory | N/A |
| ElementConformsToProfile to RegisteredProfile (for Fabric) | N/A | Mandatory | N/A |
| ElementConformsToProfile to RegisteredProfile (for Virtual Fabric) | Conditional (1) | N/A | N/A |
| ElementConformsToProfile to RegisteredProfile (for FCoE Fabric) | N/A | N/A | Conditional (5) |
| Component to ComputerSystem (Partitioning) | Conditional (2) | N/A | N/A |

95

96

Table B.2 -  AdminDomain Associations

| Association on AdminDomain | SAN | Fabric | Ethernet |
| --- | --- | --- | --- |
| Component to ComputerSystem (Switch, Partitioned Switch or IFR Switch) | N/A | Mandatory | N/A |
| Component to ComputerSystem (Ethernet Switch) | N/A | N/A | Mandatory |
| ElementCapabilities to ZoneCapabilities | N/A | Mandatory | N/A |
| HostedCollection to ConnectivityCollection | N/A | Mandatory | Mandatory (to collection of LANEndpoints) |
| HostedCollection to Zone (Active or Inactive) | N/A | Mandatory | N/A |
| HostedCollection to ZoneSet (Active or Inactive) | N/A | Mandatory | N/A |
| Component to ComputerSystem (Storage or Host) | N/A | Optional | N/A |
| HostedAccessPoint to ProtocolEndpoint (Host or Storage) | N/A | Optional | N/A |
| HostedAccessPoint to LANEndpoint | N/A | N/A | Mandatory |
| SystemDevice to FCPort (non-Switch FCPort) | N/A | Optional | N/A |
| SystemDevice to EthernetPort (non-Switch EthernetPort to Ethernet AdminDomain) | N/A | N/A | Mandatory |
| HostedService to a ZoneService | N/A | Optional (3) | N/A |
| HostedCollection to a NamedAddressCollection | N/A | Conditional (4) | N/A |
| - - The ElementConformsToProfile from the SAN AdminDomain to the Virtual Fabrics RegisteredProfile is conditional on the implementation of the Virtual Fabrics Profile - - The Component from the SAN AdminDomain to the ComputerSystem (Partitioning) is conditional on the implementation of the Virtual Fabrics Profile (and Switch Partitioning) - - The HostedService association is optional if the Zone Control Profile is implemented. - - The HostedCollection to a NamedAddressCollection is conditional on the implementation of the Enhanced Zoning and Zoning Control Profile - - This is required if the FCoE Fabric profile is implemented |  |  |  |

97

98

99 Both the SAN and Fabric share the ContainedDomain association (they point to each other). All other 100 associations are unique to either the SAN or Fabric AdminDomain instances. Note that some associations 101 are conditional on implementation of certain component profiles. For example, if the Virtual Fabric 102 component profile is implemented, an ElementConformsToProfile is required between the SAN

103	AdminDomain and the Virtual Fabric RegisteredProfile. And if Switch Partitioning is implemented, then an

ElementConformsToProfile is required between the Partitioning System and the Switch Partitioning 104 RegisteredProfile. 105

- Some of the associations are listed as Optional (#). For example, the HostedService association between

- a Fabric AdminDomain and a ZoneService is optional if the Zone Control Profile is implemented. It is optional, because the ZoneService may be hosted on either a Fabric AdminDomain or a Switch ComputerSystem.

- Graphically, this is illustrated in Figure B.3 (Optional associations are not shown for simplicity).

### Figure B.3 - Associations on AdminDomains

109 For simplicity, the optional associations are not shown in Figure B.3. A SAN AdminDomain only has the ContainedDomain association to the Fabric AdminDomain. But if Virtual Fabrics (and Switch Partitioning) are implemented, then there are two more associations: An ElementConformsToProfile to the Virtual

110

Fabrics RegisteredProfile and a Component association to the Partitioning ComputerSystem.

111

- All associations on the Fabric AdminDomain would also apply to the Virtual Fabric. The

- ElementConformsToProfile goes to the Fabric RegisteredProfile in both cases. All Switches that participate in the Fabric have a Component association to the Fabric AdminDomain. This includes 114 partitioned switches (and IFR Switches, if the Inter Fabric Routing Profile is implemented). In addition, all 115

ConnectivityCollections, Zones and ZoneSets for the Fabric are Hosted on the Fabric AdminDomain. And

116 the ZoneCapabilities are associated to the Fabric AdminDomain.

117

- Other 	associations 	(e.g., 	HostedService 	to 	the 	ZoneService 	and 	HostedCollection 	to

- NamedAddressCollections) are required if certain component profiles are implemented (as indicated in Figure B.3). 120

121	For FCoE Fabric, the LANEndpoints and non-switch Ethernet Ports are mandatory, so the 122	HostedAccessPoint and SystemDevice associations are also mandatory.

- B.3.2	CIM_ComputerSystem Requirements

- The various types of CIM_ComputerSystems instances may or may not be referring to an instance. For 125 example, if both the Fabric and Switch Profiles are implemented, the Switch ComputerSystems defined in 126 both profiles refer to the same instance (and the requirements from Fabric and Switch Profiles both apply 127 to the instance).

128 Similarly, if a Partitioned ComputerSystem also happens to be a Partitioned Switch, then both the 129 requirements of the Partitioned ComputerSystem and the Partitioned Switch apply to the 130 ComputerSystem instance.

131	In order to explain the various types of ComputerSystems, the various types of Switch ComputerSystems 132	are treated first. Then the other types of ComputerSystems will be addressed.

- B.3.2.1	Switch ComputerSystems

- Table B.3 identifies the properties of CIM_ComputerSystem to be supported by Switch ComputerSystem 135 instances. The values for the distinguishing properties are identified. Otherwise, the property merely 136 identifies whether it is Mandatory, Optional or Conditional. If it is Conditional, the condition is identified in 137 the footnotes of Table B.3

Table B.3 -  Switch ComputerSystem Properties

| Computer System Property | Switch | Partitioned Switch | IFR Switch | Backbone Switch |
| --- | --- | --- | --- | --- |
| CreationClassName | Mandatory | Mandatory | Mandatory | Mandatory |
| Name | Mandatory | Mandatory | Mandatory | Mandatory |
| ElementName | Conditional (1) | Conditional (1) | Mandatory | Mandatory |
| NameFormat | Mandatory | Mandatory | Mandatory | Mandatory |
| OperationalStatus | Mandatory | Mandatory | Mandatory | Mandatory |
| OtherIdentifyingInfo | Conditional (1) | Conditional (1) Virtual Fabric ID Conditional (3) "Virtual Switch" | Mandatory "Translate Domain" or "Front Domain" | Mandatory "Backbone" |
| IdentifyingDescriptions | Conditional (1) | Conditional (1) "SNIA:VF_ID" Conditional (3) "SNIA:DetailedType" | Mandatory "SNIA:DetailedType" | Mandatory "SNIA:DetailedType" |
| Dedicated | Mandatory - "5" | Mandatory - "5" | Mandatory - "5" | Mandatory - "5" |
| EnabledState | Conditional (1) | Conditional (1) | Mandatory | Mandatory |
| RequestedState | Conditional (2) | Conditional (2) | Conditional (2) | Conditional (2) |
| EnabledDefault | Optional | Optional | Optional | Optional |
| RequestedStateChange() | Conditional (2) | Conditional (2) | Conditional (2) | Conditional (2) |
| - - ElementName, OtherIdentifyingInfo and IdentifyingDescriptions are Mandatory if the Fabric Profile is implemented. - - RequestedState is Mandatory if the Switch Profile is implemented and FCSwitchCapabilities.RequestedStatesSupported is not NULL. - - OtherIdentifyingInfo and IdentifyingDescriptions are Mandatory if Switch Partitioning is implemented. If Switch Partitioning is implemented the Partitioned system that is a Switch has the value "Virtual Switch" and a corresponding "SNIA:DetailedType" |  |  |  |  |

138

139 The current assumption is that a partitioned switch and an IFR Switch are logically subclasses of Switch 140 and if the Switch Profile is implemented, then the RequestedStateChange method is to be supported as

141 defined for the base switch. However, state change is not currently covered in either the Switch Partitioning or the Inter Fabric Routing Profiles.

The first thing to notice is that a Switch always has Dedicated="5". That is, it is a ComputerSystem that

142

performs some switch related functions. If the Fabric Profile has been implemented, then

143

OtherIdentifyingInfo and IdentifyingDescriptions are also required. If OtherIdentifyingInfo only contains 144 the DomainID of the Switch, then it is not a Partitioned Switch or an IFR Switch. If it also contains "Translate Domain" or "Front Domain", then it is an IFR Switch. If it also contains "Backbone", then it is a backbone switch. If it contains "Virtual Switch", then it is a Partitioned Switch and it will also contain the 145 virtual fabric ID.

If only the Switch Profile has been implemented, then OtherIdentifyingInfo and IdentifyingDescriptions 146 may not be present. If the Switch Profile has been implemented, then the RequestedStateChange method 147 may also be supported. This would be conditional on the FCSwitchCapabilities have a non-NULL value 148 for the RequestedStatesSupported property.

149

150	Graphically, the various switches are represented by Figure B.4.

151

### Figure B.4 - Properties of various Switches

The Fabric and Switch Profile versions of the Switch ComputerSystem are different. However, they represent the same instance. So if both profiles are implemented, then the requirements are merged. The Fabric requirement for OtherIdentifyingInfo applies, as does the extra properties called for by the Switch Profile definition of the Switch ComputerSystem. If the Virtual Fabrics or Switch Partitioning Profiles are implemented, then the Partitioned Switch ComputerSystem(s) are implemented. These augment the definitions of the base Switch profiles. So, in any given implementation one of the above definitions of the Switch or Partitioned Switch ComputerSystem definitions apply (depending on what profiles have been implemented). 161

In contrast, the IFR Switch and the backbone Switch ComputerSystems are constrained versions of the

162

Fabric (or Virtual Fabric) definition of a Switch. Specifically, it defines the set of switches that are

163 constrained to "Translate Domain", "Front Domain" or "Backbone" switches.

164

165	The association pattern for Switch CIM_ComputerSystem instances is illustrated in Table B.4.

166

167	Table B.4 -  Switch ComputerSystem Associations

168

| Association on Switch ComputerSystem | Switch | Partitioned Switch | IFR Switch | Backbone Switch |
| --- | --- | --- | --- | --- |
| SystemDevice to an FCPort (Switch FCPort) | Mandatory | Mandatory | Mandatory | Mandatory |

169

170

171

- Table B.4 -  Switch ComputerSystem Associations

| Association on Switch ComputerSystem | Switch | Partitioned Switch | IFR Switch | Backbone Switch |
| --- | --- | --- | --- | --- |
| Component to an AdminDomain (Fabric) | Conditional (1) | Conditional (1) | Mandatory | Mandatory |
| ElementCapabilities to a ZoneCapabilities | Conditional (1) | Conditional (1) | Mandatory | Mandatory |
| HostedAccessPoint to a ProtocolEndpoint | Conditional (1) | Conditional (1) | Mandatory | Mandatory |
| HostedCollection to a Zone | Conditional (1) | Conditional (1) | Mandatory | Mandatory |
| HostedCollection to a ZoneSet | Conditional (1) | Conditional (1) | Mandatory | Mandatory |
| InstalledSoftwareIdentity to a SoftwareIdentity | Conditional (2) | Conditional (2) | Conditional (2) | Conditional (2) |
| ComputerSystemPackage to a PhysicalPackage | Conditional (2) | Conditional (2) | Conditional (2) | Conditional (2) |
| ElementCapabilities to an FCSwitchCapabilities | Conditional (2) | Conditional (2) | Conditional (2) | Conditional (2) |
| HostedCollection to a StatisticsCollection | Conditional (2) | Conditional (2) | Conditional (2) | Conditional (2) |
| HostedDependency to a ComputerSystem (Partitioning) | Conditional (3) | Conditional (3) | Conditional (3) | Conditional (3) |
| SystemDevice to a LogicalModule | Conditional (4) | Conditional (4) | Conditional (4) | Conditional (4) |
| HostedService to a ZoneService | Optional (5) | Optional(5) | Optional (5) | Optional(5) |
| HostedCollection to a NamedAddressCollection | Conditional (6) | Conditional (6) | Conditional (6) | Conditional (6) |
| - - Conditional on the implementation of the Fabric Profile - - Conditional on the implementation of the Switch Profile - - Conditional on the implementation of the Switch Partitioning Profile (or Virtual Fabrics) - - Conditional on the implementation of the Blades Profile - - Optional presence with the implementation of the Zone Control Profile - - The HostedCollection to a NamedAddressCollection is conditional on the implementation of the Enhanced Zoning and Zoning Control Profile |  |  |  |  |

From Table B.4 the only association that is common to both all definitions of switches is the SystemDevice association to FCPorts. All other associations are conditional on one of three profiles: The Fabric Profile, the Switch Profile, or the Switch Partitioning Profile.

The associations defined in the Fabric Profile are to Fabric-related classes (e.g., Zones or Zonesets). The

- associations defined in the Switch Profile are to Switch-related classes (e.g., FCSwitchCapabilities). The

- one association introduced by the Switch Partitioning Profile is the HostedDependency association to the Partitioning ComputerSystem.

175

NOTE  The IFR Switch has no unique associations. It inherits its associations from the Fabric (and Switch) definitions.

176

177

178

179

180

181

182

192

### Figure B.5 - Associations on Switches

193

194

As illustrated by Figure B.5 there is "shrub" of associations defined by the Fabric Profile for Switch (and 195 Partitioned Switch) ComputerSystems. These associations are to classes defined in the Fabric Profile

- (e.g., the Fabric AdminDomain). Similarly, there is a "shrub" of associations defined by the Switch Profile for Switch (and Partitioned Switch) ComputerSystems. These associations are to classes defined in the Switch Profile (e.g., FCSwitchCapabilities). Both the Fabric and Switch Profiles define the same association to FCPorts (the SystemDevice association). If Fabric Profile implementation does not also implement the Switch Profile, then only the shrub of Fabric associations need be implemented. And if a Switch Profile implementation does not also implement the Fabric Profile, then only the shrub of Switch associations need to be implemented.

Partitioned Switches, in either case, inherit the associations defined for the Switch ComputerSystems. So, for example, a Partitioned Switch needs to have an associated PhysicalPackage.

The Switch Partitioning Profile introduces the requirement to model the HostedDependency from the Partitioned Switch ComputerSystem to a Partitioning ComputerSystem.

- B.3.2.2	Non-Switch ComputerSystems

- Table B.5 identifies the properties of CIM_ComputerSystem to be supported by non-Switch 199 ComputerSystem instances. The values for the distinguishing properties are identified. Otherwise, the 200 property merely identifies whether it is Mandatory, Optional or Conditional. If it is Conditional, the 201 condition is identified in the footnotes of Table B.5.

Table B.5 -  Non-Switch ComputerSystem Properties

| Computer System Property | Host Platform | Storage Platform | Gateway | Partitioning | Partitioned | Ethernet |
| --- | --- | --- | --- | --- | --- | --- |
| CreationClassName | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory |
| Name | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory |
| ElementName | Mandatory | Mandatory | Mandatory |  |  | Mandatory |
| NameFormat | Mandatory | Mandatory | Mandatory |  |  | Mandatory |
| OperationalStatus |  |  | Mandatory |  |  | Mandatory |
| Dedicated | "0" or "1" | "1" or "3" | "20" | "0" Plus | Mandatory | “38” |
| OtherIdentifyingInfo |  |  |  |  | Conditional (1) |  |
| IdentifyingDescriptions |  |  |  |  | Conditional (1) |  |
| EnabledState |  |  |  |  | Mandatory | Mandatory |
| (1) - The OtherIdentifyingInfo and IdentifyingDescriptions are required if Virtual Fabrics is implemented (to identify the VF_ID for the Partitioned System) |  |  |  |  |  |  |

The primary way of distinguishing the non-switch ComputerSystems is via the Dedicated property, 202 although it is not always deterministic from just the Dedicated property. For example, a Host Platform 203 ComputerSystem may have Dedicated="0" (for Host) or "1" ("unknown"). A Storage Platform may have

204 Dedicated="3" (for storage) or "1" ("unknown"). There is no way to determine if a Platform is storage or host, if the value "1" ("unknown") is used. All we know is that it would be an end node (a Platform).

A Gateway ComputerSystem is distinguished by Dedicated="20". A Partitioning system would have 205 multiple values in the Dedicated property and one of those values would be "0" (NOTE: This would 206 distinguish it from a Host Platform, which would only have the single value of "0").

A Partitioned system would have a Dedicated value that corresponds to the function performed by the 207 Partitioned system (e.g., "20" if the partitioned system is a Gateway).

208

209	A Ethernet Switch would have a Dedicated value of "38" (Ethernet Switch).

210

211	Figure B.6 illustrates these systems and their properties graphically.

212

213

214

215

216

217

### Figure B.6 - General ComputerSystem Properties

218 For the most part, each of the instances shown in Figure B.6 represent different instances. The only 219 exception is the Partitioned ComputerSystem which can be any partitioned switch or a Gateway 220 ComputerSystem. A Platform cannot be a Partitioned System (it is virtual with no physical 221 representation). The Ethernet Switch behaves like a partitioning switch in an FCoE environment.

The association pattern for non-Switch CIM_ComputerSystem instances is illustrated in Table B.6.

222

223	Table B.6 -  Non-Switch ComputerSystem Associations

| Association on ComputerSystem | Host Platform | Storage Platform | Gateway | Partitioning | Partitioned | Ethernet |
| --- | --- | --- | --- | --- | --- | --- |
| Component to a Fabric AdminDomain | Optional Conditional (1) | Optional Conditional (2) | Conditional (3) |  | Conditional (4) |  |
| HostedAccessPoint to ProtocolEndpoints | Mandatory Conditional (1) | Mandatory Conditional (2) |  |  |  |  |
| SystemDevice to FCPorts | Optional Conditional (1) | Optional Conditional (2) |  |  |  |  |

224

225

226

Table B.6 -  Non-Switch ComputerSystem Associations

| Association on ComputerSystem | Host Platform | Storage Platform | Gateway | Partitioning | Partitioned | Ethernet |
| --- | --- | --- | --- | --- | --- | --- |
| HostedAccessPoint to an N Port Virtualizer ProtocolEndpoint |  |  | Mandatory |  |  |  |
| SystemDevice to NPIV or Gateway FCPorts |  |  | Mandatory |  |  |  |
| SystemDevice to a NetworkPort |  |  |  | Mandatory |  |  |
| Component to SAN AdminDomain |  |  |  | Mandatory |  |  |
| HostedDependency to a Partitioned ComputerSystem |  |  |  | Mandatory |  |  |
| ElementConformsTo Profile to a Switch Partitioning RegisteredProfile |  |  |  | Mandatory |  |  |
| SystemDevice to a Virtual FCPort |  |  |  |  | Mandatory |  |
| HostedDependency to a Partitioning System |  |  |  |  | Mandatory |  |
| Component to an Ethernet AdminDomain |  |  |  |  |  | Mandatory |
| HostedAccessPoint to LANEndpoints |  |  |  |  |  | Mandatory |
| SystemDevice to EthernetPorts |  |  |  |  |  | Mandatory |
| HostedDependency to an FC Switch |  |  |  |  |  | Optional |
| - - The Component, HostedAccessPoint and SystemDevice Associations are conditional on the implementation of the Host Platform ComputerSystem - - The Component, HostedAccessPoint and SystemDevice Associations are conditional on the implementation of the Storage Platform ComputerSystem - - The Component Association to the Fabric is conditional on the implementation of the N Port Virtualizer Profile - - The Component Association to the Fabric is conditional on implementation of the Fabric Profile (and the Virtual Fabric Profile). |  |  |  |  |  |  |

227

228

From Table B.6 each type of non-switch ComputerSystem instance has it's own "shrub" of associations. If 229 a Host Platform ComputerSystem is instantiated, then it is to have a Component association to its Fabric, 230 and HostedAccessPoint and SystemDevice associations for its ProtocolEndpoints and FCPorts. A similar 231 set of associations are required for Storage Platform ComputerSystem instances.

232

Notice also that all of these non-switch types of ComputerSystem instances have the Component association to some Fabric AdminDomain, with the exception of the Partitioning ComputerSystem. The Partitioning ComputerSystem has its Component association to a SAN AdminDomain. The Partitioning ComputerSystem instances also have an ElementConformsToProfile association to the RegisteredProfile instance for the Switch Partitioning Profile (to which it conforms).

Finally, notice that a Partitioned ComputerSystem instance has a HostedDependency association to a 233 Partitioning ComputerSystem instance (or an Ethernet Switch ComputerSystem).

234

235	This is graphically illustrated in Figure B.7.

### Figure B.7 - General ComputerSystem Associations

In Figure B.7 we can see the non-Switch ComputerSystems and their associations. Each has some sort of port associated to it via SystemDevice. All except the Partitioning ComputerSystem are associated to an FCPort. The Partitioning ComputerSystem is associated to a NetworkPort (which may or may not be an FCPort). For an FCoE environment, the partitioning ComputerSystem is an Ethernet Switch and the ports are EthernetPorts. ComputerSystems defined in the Fabric Profile (or the N Port Virtualizer component profile of Fabric) also have associations to ProtocolEndpoints. The ComputerSystems in the Switch Partitioning  Profile do not have ProtocolEndpoints (unless, of course, the Partitioned ComputerSystem also happens to be a Switch, N Port Virtualizer or Platform).

B.3.3	Port Requirements

B.3.3.1	Switch CIM_FCPort Requirements

Table B.7 identifies the properties of Switch CIM_FCPort to be supported by FCPort instances. The 248 values for the distinguishing properties are identified. Otherwise, the property merely identifies whether it

249

250

251

252

253

254

255 is Mandatory, Optional or Conditional. If it is Conditional, the condition is identified in the footnotes of Table B.7.

256

257	Table B.7 -  Switch FCPort Properties

| Switch FCPort Property | Switch FCPort | Partitioned Switch FCPort | IFR FCPort |
| --- | --- | --- | --- |
| SystemCreationClassName | Mandatory | Mandatory | Mandatory |
| SystemName | Mandatory | Mandatory | Mandatory |
| CreationClassName | Mandatory | Mandatory | Mandatory |
| DeviceID | Mandatory | Mandatory | Mandatory |
| ElementName | Mandatory | Mandatory | Mandatory |
| PermanentAddress | Mandatory | Mandatory | Mandatory |
| OperationalStatus | Mandatory | Mandatory | Mandatory |
| PortType | Mandatory | Mandatory | Mandatory |
| LinkTechnology | Mandatory - “4” | Mandatory - “4” | Mandatory - “4” |
| EnabledState | Conditional (1) | Conditional (1) | Mandatory |
| Speed | Conditional (2) | Conditional (2) | Conditional (2) |
| DetailedPortState | Conditional (2) | Conditional (2) | Conditional (2) |
| RequestedState | Conditional (2) | Conditional (2) | Conditional (2) |
| EnabledDefault | Conditional (2) | Conditional (2) | Conditional (2) |
| MaxSpeed | Conditional (2) | Conditional (2) | Conditional (2) |
| PortNumber | Conditional (2) | Conditional (2) | Conditional (2) |
| RequestedStateChange() | Conditional (3) | Conditional (3) | Conditional (3) |
| PortDiscriminator | Optional | Conditional (4) - "3" | Mandatory - "5" or "6" |
| NetworkAddresses | N/A | N/A | N/A |
| PortAvailability | Optional | Optional |  |
| SupportedFC4Types |  | N/A | N/A |
| SupportedCOS |  | N/A | N/A |
| - - EnabledState is required if the Fabric Profile is implemented, otherwise it is optional. - - Several properties (e.g., Speed) are required if the Switch Profile is implemented, otherwise they are not required. - - Support for the RequestedStateChange method is required if the Switch Profile is implemented and FCPortCapabilities.RequestedStatesSupported is not empty. - - The PortDiscriminator will contain "3" if the Switch Partitioning (including Virtual Fabrics) is implemented. |  |  |  |

258

259

260

From Table B.7 there are several properties (e.g., DeviceID) that are required for any implementation of a switch FCPort instance. In addition, EnabledState is required if the Fabric Profile is implemented. And there are several properties that are required if the Switch Profile is implemented.

The types of Switch FCPorts may be distinguished by the PortDiscriminator. The base definition of a Switch FCPort does not require a PortDiscriminator (in either Fabric or the Switch Profile). However, if switch partitioning (or Virtual Fabrics) are implemented, then the virtual ports will be distinguished by a PortDiscriminator value of "3". And an IFR Port will be distinguished by the value "5" or "6". Note that these ports are the ports that are associated to a Translate Domain or Front Domain Switch. Ports on the 261	Backbone switch would be undistinguished.

262

### Figure B.8 - Switch FCPort Properties

The base definition of a Switch FCPort in both the Fabric and Switch Profiles are undistinguished (by PortDiscriminator). If Virtual Fabrics or Switch Partitioning are implemented, then the PortDiscriminator will contain "3" for Virtual FCPort. And IFR FCPorts (the ones associated to a Translate Domain or Front Domain) are distinguished by "5" (IFR) or "6" (IFR (Virtual IFR)).

Note that Backbone Switch FCPorts would be undistinguished from other Switch FCPorts. A Backbone Switch FCPort may be a virtual FCPort (PortDiscriminator="3").

The association pattern for Switch CIM_FCPort instances is illustrated in Table B.8.

Table B.8 -  Switch FCPort Associations

| Association on Switch FCPort | Switch FCPort | Partitioned Switch FCPort | IFR FCPort |
| --- | --- | --- | --- |
| SystemDevice to a Switch ComputerSystem | Mandatory | Mandatory | Mandatory |
| DeviceSAPImplementation to a ProtocolEndpoint | Conditional (1) | Conditional (1) | Mandatory |
| ElementCapabilities to an FCPortCapabilities | Conditional (2) | Conditional (2) | Conditional (2) |
| ElementStatisticalData to an FCPortStatistics | Conditional (2) | Conditional (2) | Conditional (2) |
| ElementSettingData to an FCPortSettings | Optional (3) | Optional (3) | Optional (3) |
| ElementStatisticalData to an FCPortRateStatistics | Optional (3) | Optional (3) | Optional (3) |

278

279

280	Table B.8 -  Switch FCPort Associations

281

| Association on Switch FCPort | Switch FCPort | Partitioned Switch FCPort | IFR FCPort |
| --- | --- | --- | --- |
| MemberOfCollection to a RedundancySet | Optional (3) | Optional (3) | Optional (3) |
| ModulePort to a LogicalModule | Conditional (4) | Conditional (4) | Conditional (4) |
| SystemDevice to a Partitioned ComputerSystem (or Switch) |  | Conditional (5) | Conditional (5) |
| HostedDependency to a NetworkPort |  | Conditional (5) | Conditional (5) |
| - - The DeviceSAPImplementation to a ProtocolEndpoint is conditional on implementation of the Fabric Profile - - The ElementCapabilities and ElementStatisticalData to an FCPortStatistics are conditional on the implementation of the Switch Profile - - The ElementSettingData, ElementStatisticalData to an FCPortRateStatistics and MemberOfCollection may be present (optional) if the Switch Profile is implemented - - The ModulePort association is conditional on the implementation of the Blades Profile. - - If the Switch Partitioning Profile is implemented, then virtual FCPorts will have their SystemDevice association to a Partitioned Switch and the Virtual FCPorts will also have a HostedDependency to a Network Port (on a Partitioning System). |  |  |  |

282

283

284

From Table B.8 all Switch FCPorts will have a SystemDevice association to some type of Switch ComputerSystem. All other associations are dependent on what profiles are implemented. For example, implementation of the Fabric Profile requires a DeviceSAPImplementation association from the Switch FCPort to a ProtocolEndpoint. And implementation of the Switch Profile requires the implementation of an

285

286

287

288

289

290

291

292

293

294

295

296 ElementCapabilities association to an FCPortCapabilities instance. Notice that a Virtual FCPort will have 297 a HostedDependency to a NetworkPort of a Partitioning System, as illustrated in Figure B.9.

### Figure B.9 - Switch FCPort Associations

303

304

Figure B.9 shows there are a "shrub" of associations that are called for (required) by the Fabric Profile 305 and another "Shrub" of associations that are called for (required) by the Switch Profile. In general, ProtocolEndpoints are required if the Fabric Profile is implemented and FCPortCapabilities are required if 306 the Switch Profile is implemented. In addition, if the Blades  Profile is implemented, the switch FCPort will

307 have a ModulePort association to the LogicalModule it is part of.

308

- If a Switch FCPort has a PortDiscriminator property value of "3" it is a Partitioned Switch FCPort. The

- only association that is added to the picture if a port is a partitioned switch FCPort is the HostedDependency association to a NetworkPort on a Partitioning System.

B.3.3.2	Non-Switch Port Requirements

Table B.9 identifies the properties of non-switch ports to be supported by NetworkPort instances. The values for the distinguishing properties are identified. Otherwise, the property merely identifies whether it is Mandatory, Optional or Conditional. If it is Conditional, the condition is identified in the footnotes of Table B.9.

Table B.9 -  Non-Switch Port Properties

| Port Property | Fabric NPIV FCPort | Gateway FCPort | Network Port | Ethernet Port | Host or Storage FCPort | Host NPIV FCPort |
| --- | --- | --- | --- | --- | --- | --- |
| SystemCreationClassName | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory |
| SystemName | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory |
| CreationClassName | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory |
| DeviceID | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory | Mandatory |
| ElementName | Optional | Optional |  |  | Optional | Optional |
| PermanentAddress | Mandatory | Mandatory |  | Mandatory | Mandatory | Mandatory |
| OperationalStatus | Mandatory | Mandatory |  | Mandatory | Mandatory | Mandatory |
| PortType | Mandatory - “10” | Mandatory - “10” |  |  | Mandatory - “10” | Mandatory - “10” |
| LinkTechnology | Mandatory - “4” | Mandatory - “4” |  | Mandatory - “2” | Mandatory - “4” | Mandatory - “4” |
| NetworkAddresses | Mandatory | Mandatory |  |  | Mandatory | Mandatory |
| PortDiscriminator | Mandatory - "7" | Mandatory |  |  |  | Mandatory - "7" |
| SupportedFC4Types | Optional | Optional |  |  | Optional | Optional |
| SupportedCOS | Optional | Optional |  |  | Optional | Optional |

311

312

313

314

315

From Table B.9 it can be seen that non-switch FCPorts are largely undistinguished. These, in general, must be distinguished by associations to other classes. Even the NPIV ports (PortDiscriminator="7") can be either a component of a N Port Virtualizer or a component of a Host Platform. Also notice that there are few requirements on the NetworkPorts, which are associated with the Partitioning System. In the case of an FCoE environment, Ethernet ports are both host ports and switch ports.

316

317

318

319

320

321

322	Figure B.10 Illustrates these instances graphically.

### Figure B.10 - Non-Switch Port Properties

There are six different types of non-Switch FCPorts. The N Port Virtualizer FCPorts (Gateway and NPIV) are "switch like", but are associated to the N Port Virtualizer ComputerSystem. The non-switch Ports defined in the Fabric Profile are Platform (Host or Storage) FCPorts. The Host FCPorts may be NPIV FCPorts or not. The NetworkPort of the Switch Partitioning Profile may or may not be an FCPort. In the case of an FCoE environment, the NetworkPort is an EthernetPort and requires more properties.

All but the NetworkPort have a PortType="10" (an N port). NPIV ports (whether in the N Port Virtualizer or a Host Platform) have a PortDiscriminator="7". Other ports are not distinguished by properties.

The association pattern for non-switch port instances is illustrated in Table B.10.

Table B.10 -  Non-Switch Port Associations

| Association on Port | Fabric NPIV FCPort | Gateway FCPort | Network Port | Ethernet Port | Host or Storage FCPort | Host NPIV FCPort |
| --- | --- | --- | --- | --- | --- | --- |
| DeviceSAPImplementation to a ProtocolEndpoint | Conditional (1) | Conditional (1) |  |  | Conditional (2) | Conditional (2) |
| DeviceSAPImplementation to a LANEndpoint |  |  |  | Conditional (6 or 7) |  |  |
| HostedDependency to a NetworkPort | Conditional (3) | Conditional (3) |  |  |  |  |
| HostedDependency to a Host FCPort |  |  |  | Conditional (6) |  | Mandatory |
| SystemDevice to a Gateway ComputerSystem | Mandatory | Mandatory |  |  |  |  |

323

324

325

326

327

328

329

Table B.10 -  Non-Switch Port Associations

| Association on Port | Fabric NPIV FCPort | Gateway FCPort | Network Port | Ethernet Port | Host or Storage FCPort | Host NPIV FCPort |
| --- | --- | --- | --- | --- | --- | --- |
| MemberOfCollection to a LogicalPortGroup |  |  |  |  | Mandatory |  |
| SystemDevice to a Platform (Host or Storage) ComputerSystem |  |  |  |  | Optional (4) | Optional (4) |
| SystemDevice to a Fabric AdminDomain |  |  |  |  | Optional (4) |  |
| SystemDevice to an Ethernet AdminDomain |  |  |  | Conditional (6) |  |  |
| SystemDevice to a Partitioned ComputerSystem (or Switch) |  |  |  |  | Conditional (5) |  |
| SystemDevice to a Partitioning ComputerSystem |  |  | Mandatory |  |  |  |
| SystemDevice to a Ethernet Switch ComputerSystem |  |  |  | Conditional (7) |  |  |
| ElementCapabilities to a NetworkPortCapabilities |  |  | Mandatory |  |  |  |
| ElementSettingData to a NetworkPortSettings |  |  | Mandatory |  |  |  |
| - - The DeviceSAPImplementation to a ProtocolEndpoint is conditional on the implementation of the N Port Virtualizer (for Fabric NPIV FCPorts and Gateway FCPorts). - - The DeviceSAPImplementation to a ProtocolEndpoint is conditional in the implementation of the Fabric Profile (for Host or Storage FCPorts, including Host NPIV FCPorts). - - The HostedDependency to a NetworkPort is conditional on the implementation of the Switch Partitioning Profile. - - For Platform (Host or Storage) FCPorts, a SystemDevice association is required to either a Fabric AdminDomain or a Platform ComputerSystem. - - This case may never happen. - - This is conditional on implementation of the FCoE Profile and for host ethernet ports - - This is conditional on implementation of the FCoE Profile and for ethernet switch ports |  |  |  |  |  |  |

From Table B.10 one can see the unique associations for the various non-switch Ports. For Fabric NPIV FCPorts, the FCPort has a PortDiscriminator of "7" and is associated to an N Port Virtualizer. For a Gateway FCPort, it too is associated to an N Port Virtualizer, but its PortDiscriminator does not contain the value "7". For a NetworkPort, and an EthernetPort, it is associated to a Partitioning System and is the antecedent in a HostedDependency association. For a Host or Storage FCPort, it will have a SystemDevice association to a Host or Storage Platform ComputerSystem or to the Fabric AdminDomain. For the Host NPIV Port, it will also have a PortDiscriminator of "7", as illustrated in Figure B.11.

For  simplicity, only the EthernetPort associated to the Ethernet Switch is shown in Figure B.11.

Figure B.11 - Non-Switch Port Associations

EXPERIMENTAL